3GPP TSG-RAN WG4 Meeting # 104-e	R4-2214048

# Electronic Meeting, August 15 - 26, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Input on PC1 FWA back-off for edge allocations

Agenda Item:	10.15.2	UE RF requirements	[LTE_NR_HPUE_FWVM_R18-Core]

Document for:	Approval

# Introduction

In Release 18 WI for PC1 FWA and vehicle mounted use cases [1], one of the objectives is:”UE A-MPR, and impact of other related parameters” with reference to WF from RAN4#103e [2]. Since the WF already highlighted issues related to edge allocations and WOLA filtering effects, in this contribution we provide further background on the potential impact for PC1.

# Discussion

## Edge allocations for PC2 and PC1.5

As previously discussed in WF [2], if PC1 PA are intrinsically more linear than PC2 and PC3 PAs with an ACLR requirement 37dB of versus 31dB and 30dB respectively, FWA and potentially vehicle mounted UEs may still reuse the BB IC and the RF transceiver from a smartphone platform. This means that both TRX carrier, image and CIM3/5 impairments apply but also the BB WOLA filtering will seek the same compromise that is found in smartphone platforms in terms of transient performance and margin to the channel guard-band on the waveform spectrum.

If there are agreed assumptions for carrier, image and CIM3/5 impairments, there is no agreed WOLA filtering. Still, some information is available through the PC2 and PC3 experience.

Edge allocation were first introduced for PC2 MPR for QPSK and higher order modulations. Only later this concept was reused for PC3 in the context of Pi/2 BPSK with power boosting.

If we compare the different MPR tables for PC3, PC2, PC1.5 and PC1 extracted from 38.101-1 and copied at the bottom, and focus for QPSK modulation, it can be found that:

- PC3 does no have edge allocations and outer MPR is 1dB and 3dB for DFT and CP respectively

- PC2 has edge allocation MPR of minimum 3.5dB across all modulations and outer MPR is 1dB and 3dB for DFT and CP respectively. Edge allocations are allocations within the two edge RBs on each side of the channel

- For PC1.5 (using FWA case to neglect issues related to reverse IMD) has edge allocation MPR of minimum 6.5dB across all modulations and outer MPR is 2dB and 4dB for DFT and CP respectively. Edge allocations are allocations within the four edge RBs on each side of the channel

- PC1 MPR does not have edge allocation,s but if the same transceiver and BB would be assumed than for PC2 and PC1.5, then it would be needed.

Observation: while outer MPR is relatively constant across PC3 to PC1.5, edge A-MPR is increased dB/dB with the power class level, it is constant across modulation orders and edge RB region doubles in size every 3dB of PCmax. This means that this is not related to the transmit path linearity, but the consequence of the input spectrum from the BB.

With the above observation and if the same BB WOLA filtering would be assumed, the PC1 MPR should also have an edge allocation type at 8.5dB minimum MPR (2dB above PC1.5) and cover the 7RB at both edge of the channel (4*10^(2/10) is 6.4 and correspond to a 2dB increase of the edge region).

Note that this edge allocation MPR is worst for channel bandwidths with lower guard-bands, but when considering A-MPR, one should further check these allocations as some NS have more stringent requirements in the OOB first 1MHz, either in terms of level or measurement bandwidth.

Proposal on edge allocations:

- If PC1 MPR needs to be representative of implementations using BB and RF transceivers from smartphone platforms, it should consider the addition of an edge allocation type with a minimum MPR of 8.5dB and an edge region of seven RBs.

  - It may be feasible to agree a slightly better value or add signalling for the UE to declare the need for this additional allocation type

- Edge allocations with similar WOLA performance than evaluated for PC2 should be carefully checked for NS where the requirement in the first OOB MHz is more stringent or use smaller measurement bandwidth than NR SEM.; especially for channel bandwidths with small guard-bands

## MPR references for PC3, PC2, PC1.5 (FWA) and PC1

Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 | ≤ 0.52 | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | 02 | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40 % of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Table 6.2.2-2 Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 6.2D.2-3 Maximum power reduction (MPR) for power class 1.5 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 1.5 | ≤ 0 |
|  | QPSK | ≤ 6.5 | ≤ 2 | ≤ 0 |
|  | 16 QAM | ≤ 6.5 | ≤ 3 | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Table 6.2.2-5 Maximum power reduction (MPR) for power class 1

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

# Conclusions

In this contribution, we provide the background information on the characteristics of edge allocations for MPR and A-MPR for HPUEs and make the following proposal.

Proposal on edge allocations:

- If PC1 MPR needs to be representative of implementations using BB and RF transceivers from smartphone platforms, it should consider the addition of an edge allocation type with a minimum MPR of 8.5dB and an edge region of seven RBs.

  - It may be feasible to agree a slightly better value or add signalling for the UE to declare the need for this additional allocation type

- Edge allocations with similar WOLA performance than evaluated for PC2 should be carefully checked for NS where the requirement in the first OOB MHz is more stringent or use smaller measurement bandwidth than NR SEM.; especially for channel bandwidths with small guard-bands

# References

- RP-221841 New WID: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands, Nokia, RAN#96

- R4-2210569	WF on A-MPR for bands n71 and n85,	 Skyworks Solutions Inc., RAN4#103e