3GPP TSG-RAN WG4 #104-e	R4-2214213

August 15 – August 26, 2022

Source: 	ZTE

Title:  	Reply LS on LocationMeasurementIndication contents and measurement gap parameters

Agenda Item:	9.9.1.1

Document for:	Discussion

Introduction

An LS on SSB measurement for L1-RSRP on inter-cell beam management was sent to RAN4 [1]. In this contribution, we discuss the questions in RAN1 LS and provide the draft reply LS.

1. Overall Description: Regarding the above agreements in R4, when the UE autonomously activates/deactivates the pre-configured measurement gap by sending the RRC message LocationMeasurementIndication, RAN2 would like to understand: - Question1: - whether the request for configuration in LocationMeasurementIndication can be un-ambiguously mapped to a certain pre-configured measurement gap? - whether anything needs to be added to the current contents of the LocationMeasurementIndication? - Question2: whether the UE should only autonomously activate/deactivate the pre-configured measurement gap indicated by LocationMeasurementIndication?

Disucssion

Regarding to the UE autonomous activatation/deactivation mechanism, during 103 meeting, RAN4 has some detailed discussion focus on the initiation of LocationMeasurementIndication procedure. Finally RAN4 realized that possible misunderstanding between UE and NW would be caused by such initiation of LocationMeasurementIndication procedure. So RAN4 approved the following agreements.

- Agreements: - Remove LocationMeasurementIndication from the trigger events in clause 9.1.7.3.1. [TS38.133 v17.5.0] “9.1.7.3. ….. The UE can autonomously change the Pre-MG status from activation to deactivation or vice versa based on any of the following triggering conditions: - DCI, timer or RRC based active BWP switching, - Activation/deactivation of SCell(s), - Addition/removal of any measurement object(s) - Addition/release/change of a SCell in carrier aggregation, - Initiation of LocationMeasurementIndication procedure specified in clause 5.5.6 [2]..

Based on such agreements in RAN4, it means LocationMeasurementIndication procedure is no longer a trigger event for the pre-configured MG’s activation/deactivation. So it is not necessary to discuss about Question 1 and Question 2 raised by RAN2 any more.

Proposal: Since the trigger event of initiation of LocationMeasurementIndication procedure has been approved to be removed by RAN4 in 103 meeting, it is not necessary to discuss Question 1 and Question 2 raised by RAN2 any more.

Conclustion

In this contribution, we discuss RAN2 LS on LocationMeasurementIndication contents and measurement gap parameters and provide our views, the draft reply LS are provided in the Annex.

- Proposal: Since the trigger event of initiation of LocationMeasurementIndication procedure has been approved to be removed by RAN4 in 103 meeting, it is not necessary to discuss Question 1 and Question 2 raised by RAN2 any more.

References

- R1-2211520 LS on LocationMeasurementIndication contents and measurement gap parameters

Appendix

3GPP TSG RAN WG4 Meeting #104-e		R4-22XXXXX

Electronic Meeting, 15 – 26 August, 2022

Title:	Reply LS on LocationMeasurementIndication contents and measurement gap parameters

Response to:	R2-2206388

Release:	Rel-17

Study Item:	NR_pos_enh-Core, NR_MG_enh-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	chenchen ZHANG

E-mail Address:	zhangchenchen@zte.com.cn

Attachments

1. Overall Description:

RAN4 discussed the question in RAN2 LS and agree that:

Since the trigger event of initiation of LocationMeasurementIndication procedure has been approved to be removed by RAN4 in 103 meeting, it is not necessary to discuss Question 1 and Question 2 raised by RAN2 any more.

2. Actions:

To RAN2

ACTION: 	RAN4 kindly inform RAN2 to take the above responses into consideration.