3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214299

Electronic Meeting, 15– 26 August 2022

Agenda item:			9.14.4, 9.14.5

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:						Email discussion summary for [104-e][310] NR_exto71GHz_BSRF

Document for:		Information

# Introduction

This email discussion summary covers BS RF requirements for extending NR operation to 71 GHz. Also performance part of the work and BS demod test methodology have been included. Based on the input contributions the discussion is split into three major topics:

- Core requirements

- Conformance testing

- BS OTA demod test methodology.

Within each topic individual requirements and issues are discussed in various sub-topics. Generally, proposals and requirements having most dependency have been grouped together.

The template has been adapted to include comment section separately for each issue to facilitate discussion.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| CATT | Huiping Shan | shanhuiping@catt.cn |
| Qualcomm | Mustafa Emara | memara@qti.qualcomm.com |
| Ericsson | Torbjorn Elfstrom | torbjorn.elfstrom@ericsson.com |
| Nokia | Man Hung Ng | man_hung.ng@nokia.com |
| Rohde & Schwarz | Niels Petrovic | Niels.petrovic@rohde-schwarz.com |
| ZTE | Fei Xue | Xue.fei25@zte.com.cn |
| Keysight | Takao Miyake | takao_miyake@keysight.com |
| NEC | Tetsu Ikeda | tetsu.ikeda@nec.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: Core requirements

This topic covers Tdocs submitted to core requirement agenda items. Two draft CRs as well as two documents for discussion. Only topic needing further agreements outside the CRs is FRCs and therefore only one sub-topic is needed.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211644 | CATT | Proposal 1: Define G-FR2-A1-10 with allocated 62RBs for 480kHz SCS and G-FR2-A1-11 with allocated 31RBs for 960kHz SCS for ICS for 800MHz CBW/480kHz SCS and 800MHz CBW/960kHz SCS respectively in Table 2-1 below. Table 2-1: OTA in-channel selectivity requirement for BS type 2-O Frequency Range; BS channel bandwidth (MHz); Subcarrier spacing (kHz); Reference measurement channel; Wanted signal mean power (dBm) (Note 2); Interfering signal mean power (dBm) (Note 2); Type of interfering signal. FR2-2; 100,400; 120; G-FR2-A1-2; EISREFSENS_50M + 3 + ΔFR2_REFSENS; EISREFSENS_50M + 13 + ΔFR2_REFSENS; DFT-s-OFDM NR signal, 120 kHz SCS, 32 RB. 400; 480; G-FR2-A1-8; EISREFSENS_50M + 9 + ΔFR2_REFSENS; EISREFSENS_50M + 19+ ΔFR2_REFSENS; DFT-s-OFDM NR signal, 480 kHz SCS, 32 RB. 800, 1600; 480; G-FR2-A1-10; EISREFSENS_50M + 12 + ΔFR2_REFSENS; EISREFSENS_50M + 22 + ΔFR2_REFSENS; DFT-s-OFDM NR signal, 480 kHz SCS, [54] RB. 400; 960; G-FR2-A1-9; EISREFSENS_50M + 9 + ΔFR2_REFSENS; EISREFSENS_50M + 19+ ΔFR2_REFSENS; DFT-s-OFDM NR signal, 960 kHz SCS, 16 RB. 800, 1600, 2000; 960; G-FR2-A1-11; EISREFSENS_50M + 12 + ΔFR2_REFSENS; EISREFSENS_50M + 22+ ΔFR2_REFSENS; DFT-s-OFDM NR signal, 960 kHz SCS, [27] RB. NOTE 1: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. NOTE 2: EISREFSENS_50M is defined in clause 10.3.3.. Proposal 2: Adopt the following FRC parameters for G-FR2-A1-10 and G-FR2-A1-11 in Table A.1-2 below. Table A.1-2: FRC parameters for FR2 OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity Reference channel; G-FR2-A1-1; G-FR2-A1-2; G-FR2-A1-3; G-FR2-A1-4; G-FR2-A1-5; G-FR2-A1-6; G-FR2-A1-7; G-FR2-A1-8; G-FR2-A1-9; G-FR2-A1-10; G-FR2-A1-11. Subcarrier spacing (kHz); 60; 120; 120; 60; 120; 480; 960; 480; 960; 480; 960. Allocated resource blocks; 66; 32; 66; 33; 16; 66; 33; 33; 17; 62; 31. CP-OFDM Symbols per slot (Note 1); 12; 12; 12; 12; 12; 12; 12; 12; 12; 12; 12. Modulation; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK. Code rate (Note 2); 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3. Payload size (bits); 5632; 2792; 5632; 2856; 1416; 5632; 2856; 2856; 1480; 5376; 2664. Transport block CRC (bits); 24; 16; 24; 16; 16; 24; 16; 16; 16; 24; 16. Code block CRC size (bits); -; -; -; -; -; -; -; -; -; -; -. Number of code blocks - C; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1; 1. Code block size including CRC (bits) (Note 3); 5656; 2808; 5656; 2872; 1432; 5656; 2872; 2872; 1496; 5400; 2680. Total number of bits per slot; 19008; 9216; 19008; 9504; 4608; 19008; 9504; 9504; 4896; 17856; 8928. Total symbols per slot; 9504; 4608; 9504; 4752; 2304; 9504; 4752; 4752; 2448; 8928; 4464. NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0 = 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15].. |
| R4-2211807 | Nokia, Nokia Shanghai Bell | Proposal 1: No need to further update the new FRC parameters, and the reference sensitivity and FRC in the agreed WF in R4-2210637 can be finalized. Proposal 2: The square brackets around the ICS interfering signal RB for 800MHz, 1600MHz and 200MHz channel bandwidth can be removed to finalize the ICS requirements. |

## Open issues summary

### Sub-topic 1-1: FRCs

This sub-topic covers proposals for FRCs

Open issues and candidate options before e-meeting:

#### Issue 1-1: FRCs

Agreements from previous meeting are shown below, copied from approved WF R4-2210637:

Agreement:

- There is no need to define a new dedicated FRC for 960 kHz SCS and 800 MHz ChBW.

Table A.1-2: FRC parameters for FR2 OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity

| Reference channel | G-FR2-A1-1 | G-FR2-A1-2 | G-FR2-A1-3 | G-FR2-A1-4 | G-FR2-A1-5 | G-FR2-A1-6 | G-FR2-A1-7 | G-FR2-A1-8 | G-FR2-A1-9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 120 | 120 | 60 | 120 | 480 | 960 | 480 | 960 |
| Allocated resource blocks | 66 | 32 | 66 | 33 | 16 | 66 | 33 | 33 | 17 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 5632 | 2792 | 5632 | 2856 | 1416 | 5632 | 2856 | 2856 | 1480 |
| Transport block CRC (bits) | 24 | 16 | 24 | 16 | 16 | 24 | 16 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | - | - | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 3) | 5656 | 2808 | 5656 | 2872 | 1432 | 5656 | 2872 | 2872 | 1496 |
| Total number of bits per slot | 19008 | 9216 | 19008 | 9504 | 4608 | 19008 | 9504 | 9504 | 4896 |
| Total symbols per slot | 9504 | 4608 | 9504 | 4752 | 2304 | 9504 | 4752 | 4752 | 2448 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0 = 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |  |  |  |  |

Agreement:

- Update ICS interfering signal RB for 800MHz, 1600MHz and 200MHz channel bandwidth to ensure both wanted and interfering signal RB are within channel bandwidth.

Table 10.9.3-1: OTA in-channel selectivity requirement for BS type 2-O

| Frequency Range | BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-1 | 50 | 60 | G-FR2-A1-4 | EISREFSENS_50M + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 32 RB |
|  | 100,200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 64 RB |
|  | 50 | 120 | G-FR2-A1-5 | EISREFSENS_50M + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 16 RB |
|  | 100,200,400 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RB |
| FR2-2 | 100,400 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RB |
|  | 400 | 480 | G-FR2-A1-8 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 480 kHz SCS, 32 RB |
|  | 800, 1600 | 480 | G-FR2-A1-6 | EISREFSENS_50M + 12 + ΔFR2_REFSENS | EISREFSENS_50M + 22 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 480 kHz SCS, [54] RB |
|  | 400 | 960 | G-FR2-A1-9 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 960 kHz SCS, 16 RB |
|  | 800, 1600, 2000 | 960 | G-FR2-A1-7 | EISREFSENS_50M + 12 + ΔFR2_REFSENS | EISREFSENS_50M + 22+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 960 kHz SCS, [27] RB |
| NOTE 1: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. NOTE 2: EISREFSENS_50M is defined in clause 10.3.3. |  |  |  |  |  |  |

New proposals in this meeting are provided below

- Proposals

  - Proposal 1: Define G-FR2-A1-10 with allocated 62RBs for 480kHz SCS and G-FR2-A1-11 with allocated 31RBs for 960kHz SCS for ICS for 800MHz CBW/480kHz SCS and 800MHz CBW/960kHz SCS respectively in Table 2-1 below. (moderator: see section 1.1 for table 2-1)

- Proposal 2: Adopt the following FRC parameters for G-FR2-A1-10 and G-FR2-A1-11 in Table A.1-2 below. (moderator: see section 1.1 for table 2-1)

- Proposal 3: No need to further update the new FRC parameters, and the reference sensitivity and FRC in the agreed WF in R4-2210637 can be finalized.

- Proposal 4: The square brackets around the ICS interfering signal RB for 800MHz, 1600MHz and 200MHz (moderator: this should be 2000 MHz) channel bandwidth can be removed to finalize the ICS requirements.

- Recommended WF

  - In previous meeting it was agreed that there is no need to define dedicated FRC for 800 MHz BW, therefore Proposal 3 and Proposal 4 are recommended as WF to respect earlier agreement.

The outcome is to be taken into account in CRs discussed in section 1.2.2

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| CATT | We’re ok with the the recommended WF. We had some misunderstanding. |
| Ericsson | We tend to agree with proposal 3 and 4. |
| Nokia | Propose proposals 3 and 4. On proposals 1 and 2, there is no need for wanted signals to be exactly half of channel bandwidth, LTE 3MHz already have such ICS case. |
| ZTE | Fine with recommended WF |
| NEC | Fine with the recommended WF. |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| R4-2211806 Draft CR to TS 38.104 on correction of TAE requirements Nokia, Nokia Shanghai Bell |  |
| --- | --- |
| Ericsson | The FR2-2 requirements were placed by mistake in the wrong section. This CR fix the issue. Since we change the requirement, it would be worth to consider to also have a table for FR1 TAE to improve specification quality and also make requirement structure between FR1 and FR2 aligned. |
| -Nokia | Reply to Ericsson: The proposal above is for FR1 table, so it should be handled as maintenance CR but not part of this WI for ease of future reference. |
| ZTE | Fine with the CR , current TAE requirement is placed in other wrong sub-clause. |
| NEC | Fine with the CR. |

| R4-2211808 Draft CR to TS 38.104 on finalization of ICS interfering signal RB Nokia, Nokia Shanghai Bell |  |
| --- | --- |
| ZTE | Okay |
| Company B |  |

## Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic #1-1: FRCs | Issue 1-1: FRCs Tentative agreements: All companies agree with - Proposal 3: No need to further update the new FRC parameters, and the reference sensitivity and FRC in the agreed WF in R4-2210637 can be finalized. - Proposal 4: The square brackets around the ICS interfering signal RB for 800MHz, 1600MHz and 200MHz (moderator: this should be 2000 MHz) channel bandwidth can be removed to finalize the ICS requirements. Candidate options: Recommendations for 2nd round: No need for 2nd round discussion or WF, draft CRs aligned with Proposal 3 and Proposal 4 can be agreed already in 1st round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2211806 | Agreeable |
| R4-2211808 | Agreeable |

## Discussion on 2nd round (if applicable)

No discussion on 2nd round needed.

# Topic #2: Conformance testing

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211645 | CATT | Proposal 1: Define independent TC signals for FR2-2 as shown in following Table 4.7.2.1-2a. Table 4.7.2.1-2a: Signal to be used to build NR TCs for BS type 2-O in FR2-2 Operating band characteristics; FDL_high – FDL_low ≤ 14000 MHz. TC signal; BWchannel; 400 MHz (Note 1, Note 2). characteristics; Subcarrier spacing; Smallest supported subcarrier spacing declared per operating band (D.7). NOTE 1: If smallest supported SCS declared per operating band (D.7) is 120 kHz, BS vendor can decide to test with 100 MHz BS channel bandwidth instead of 400 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 100 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used.. Proposal 2: Adopt option 1: only NRTC1 and NRTC2 is applicable for 52.6-71GHz (R4-2209590) Moderator's note: option 1 in the proposal refers to option in issue 3-5-2 in approved WF in R4-2210638 Proposal 3: Support Option 1: agree to reuse the existing test case for FR2-1 NR BS conformance testing for FR2-2. (R4-2209590) Moderator's note: option 1 in the proposal refers to option in issue 3-5-3 in approved WF in R4-2210638 Proposal 4: Adopt 5ms for test model data length for FR2-2. |
| R4-2212465 | Ericsson | Observation: The chamber loss of a CATR can be calculated either by the area formula or by the distance formula. The effective area of the CATR depends on the feeder gain. Observation: The coupling loss in a CATR can be optimized by adapting the CATR size according to the radio size and frequency. By reducing the CATR area a factor 10 from 20 m2 to 2 m2, the coupling loss can be reduced a factor 10 from 71.5 dB to 61.5 dB at 71.0 GHz. This significantly reduces the requirements on the power amplifier. Observation: The improvement in pathloss, by reduction of the effective CATR area, is relevant for both DL and UL. Proposal 1: For FR2-2 consider CATR pathloss values for a CATR suitable for FR2-2 testing. Pathloss values in Table 2.1-2 can be used as baseline for FR2-2. Observation: The test distance and measurement antenna gain can be optimized to make most usage of power and mitigate pathloss. Proposal 2: Add intermediate calibration stage of spectrum analyser absolute power accuracy for DL EIRP measurement, with the intension to break the trend with very large measurement uncertainties for high frequencies. Proposal 3: Introduce external mixer stage test setup and corresponding calibration procedures for receiver requirements and out-of-band requirements to improve measurement uncertainty. Observation: It would be possible to consider a lower frequency limit of 300 to 400 MHz for FR2-2. Proposal 4: Set the upper frequency limit to 130 GHz for FR2-2. |
| R4-2212846 | Nokia, Nokia Shanghai Bell | Observation 1: Current WID does not include any TR to capture the background for base station measurement uncertainty and test tolerance derivation. Proposal 1: Measurement uncertainty and test tolerance analysis needs to be properly captured and therefore update to WID is needed to include TR 37.941. |
| R4-2213702 | ZTE Corporation | Proposal 1: to define the TC signals for 52.6-71GHz as shown in Table Table 4.7.2.1-2a. Table 4.7.2.1-2a: Signal to be used to build NR TCs for FR2-2 BS type 2-O Operating band characteristics; FDL_high – FDL_low ≤ 5000 MHz. TC signal; BWchannel; 400 MHz (Note 1, Note 2). characteristics; Subcarrier spacing; Smallest supported subcarrier spacing declared per operating band (D.7). NOTE 1: BS vendor can decide to test with 100 MHz BS channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 400 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 100 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used.. Proposal 2: only NRTC1 and NRTC2 is applicable for 52.6-71GHz; Proposal 3: agree to reuse the existing test case for FR2-1 NR BS conformance testing for FR2-2. Proposal 4: if there is no operator inputs on the TDD pattern for FR2-2, then agree on the above TDD configuration for FR2-2. Field name; Value. referenceSubcarrierSpacing (kHz); 60; 120; 480; 960. Periodicity (ms) for dl-UL-TransmissionPeriodicity; 1.25; 1.25; 1.25; 1.25. nrofDownlinkSlots; 3; 7; 29; 59. nrofDownlinkSymbols; 10; 6; 10; 6. nrofUplinkSlots; 1; 2; 9; 18. nrofUplinkSymbols; 2; 4; 2; 4. Proposal 5: only NR-FR2-TM1.1, NR-FR2-TM2 and NR-FR2-TM3.1 are applicable for FR2-2. Proposal 6: the existing measurement setup framework in TS 38.141-2 Annex D and Annex E could also been applicable for FR2-2. Proposal 7: to further discuss the feasibility of measurement of the upper limit of spurious emission requirements of FR2-2. |
| R4-2211646 | CATT | Proposal 1: Choose a frequency slightly larger than Fstep,6 (127GHz) as max limit frequency point, e.g., 130GHz. |
| R4-2211809 | Nokia, Nokia Shanghai Bell | Proposal 1: The suitability of each OTA chamber (Far field anechoic chamber, CATR, Near field chamber, PWS, etc.) for each transmitter test in the frequency range between 52.6GHz and 71GHz should be studied and confirmed by TE vendors, or the list of OTA chamber should be updated for each BS type 2-O transmitter test in the frequency range between 52.6GHz and 71GHz. Proposal 2: The measurement capabilities resulting from different TE implementations should be studied and provided by TE vendors to decide the achievable and satisfactory measurement performance for BS type 2-O transmitter testing in the frequency range between 52.6GHz and 71GHz. Proposal 3: The maximum FDL_high – FDL_low of 14000 MHz should replace 5000 MHz in Table 4.7.2.1-2a. (R4-2209590) for BS type 2-O transmitter testing in the frequency range between 52.6GHz and 71GHz. Proposal 4: Focus on using a fixed number of slots for EVM measurement time length and test model data length and select the number of slots to ensure a good trade-off between the test time and MU for BS type 2-O transmitter testing in the frequency range between 52.6GHz and 71GHz. Proposal 5: There is no need to adjust the EVM measurement time length depending on TDD pattern used in the text model for BS type 2-O EVM test in the frequency range between 52.6GHz and 71GHz. |
| R4-2212461 | Ericsson | Proposal: For transmitter requirement test system measurement uncertainty, extend measurement procedures to enable test setups to break the trend where measurement uncertainty grows rapidly as function of frequency. |
| R4-2213925 | Keysight Technologies UK Ltd | Proposal - For FR2-2 Tx off power measurement, as it was agreed for FR2-1, assume On antenna gain used for Off period gain and use EIRP rather TRP for measurement. - Use of LNA should be in MU budget for FR2-2 except Tx off power measurement. - For equipment MU number up to 80 GHz, because these already exist as agreed number, use numbers used in budget for UE. These are; - Power measurement equipment for spurious, 1 sigma number from 40.8 GHz to 80 GHz is 2.00 - Network analyzer, 1 sigma number from 40.8 GHz to 80 GHz is 0.85 - Out of band emission measurement - Consider two configurations for out of band spurious measurement system below 110 GHz and above up to 142 GHz - For out of band spurious measurement system, both use or not to use mixer case to consider and then pick larger MU case to use for calculating total test system MU. For above 110 GHz up to 142 GHz, mixer should be assumed and used for MU budget calculation like FR2-1 spurious emission. |
| R4-2211647 | CATT | Proposal 1: Align the maximum limit frequency point for RX with that for TX. Proposal 2: 120 MHz can be considered as measurement step size for interferer signal step size for 800MHz, 1600MHz, and 2000MHz CBW for OTA in-band blocking and OTA out-of-band blocking. |
| R4-2211810 | Nokia, Nokia Shanghai Bell | Proposal 1: The suitability of each OTA chamber (Far field anechoic chamber, CATR, Near field chamber, PWS, etc.) for each receiver test in the frequency range between 52.6GHz and 71GHz should be studied and confirmed by TE vendors, or the list of OTA chamber should be updated for each BS type 2-O receiver test in the frequency range between 52.6GHz and 71GHz. Proposal 2: The measurement capabilities resulting from different TE implementations should be studied and provided by TE vendors to decide the achievable and satisfactory measurement performance for BS type 2-O receiver testing in the frequency range between 52.6GHz and 71GHz. Proposal 3: Update Table 7.6.4.2.3-1 in TS 38.141-2 to include larger step size 120MHz or 240MHz for minimum supported BS channel bandwidth larger than 400MHz for NR operation in 52.6 – 71 GHz range. |
| R4-2212462 | Ericsson | Observations: 1. The receiver test system uncertainty is larger than transmitter test system uncertainty. 2. The test system uncertainty for the frequency range 43.5 to 48.2 GHz is significantly larger than for lower parts of FR2. Proposal: For receiver test system measurement uncertainty, extend measurement procedures to enable for test setups to break the trend where measurement uncertainty grows rapidly as function of frequency. |
| R4-2213927 | Keysight Technologies UK Ltd | Observation: · Based on rough link budget calculation, there seems some cases need power amplifier to add more power from signal generator. This needs more careful estimation including all other necessary components and loss. · Power sensor/meter can possibly be used for leveling signal generator or up converter output when it meets necessary condition of power sensor use. Otherwise, it’s just add complexity to system. · Regarding with demod link budget estimation, expected power level seem too high then power amplifier required. For use of power amplifier, expected output level should below max power limit of amplifier. · For out of band blocking source, it’s possible to see there is not enough power from generator/amplifier depending on test setup. Proposal · For MU budget table for Rx testing. MU term of up converter (mixer) and additional power amplifier should be added. · Use of power sensor/meter for signal leveling should not be mandated for test system setup because limited condition for use of sensor. · For demod setup, consider using 15dB margin for lowering AWGN level. This reduces risk of not finding appropriate amplifier. For out of band blocking, further consider feasibility. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1: Test configurations and RF channels

Sub-topic description:

Open issues and candidate options before e-meeting:

#### Issue 2-1-1: Signal used to build NR TCs in FR2-2

- Proposals

  - Proposal 1: Define independent TC signals for FR2-2 as shown in following Table 4.7.2.1-2a. (Moderator: see R4-2211645 or section 2.1 of this document for the Table)

  - Proposal 2: The maximum FDL_high – FDL_low of 14000 MHz should replace 5000 MHz in Table 4.7.2.1-2a. (R4-2209590) for BS type 2-O transmitter testing in the frequency range between 52.6GHz and 71GHz.

- Recommended WF

  - Merge proposal 1 and 2, i.e use the table from proposal 1 with FDL_high – FDL_low of 14000 MHz.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| CATT | Proposal 1 already uses FDL_high – FDL_low ≤ 14000 MHz. So proposal 1 should be ok. |
| Qualcomm | Support the recommended WF. |
| Ericsson | We support recommended WF. |
| Nokia | Support recommended WF from moderator. |
| ZTE | Support the recommended WF. |
| NEC | Support the recommended WF. |

#### Issue 2-1-2: Applicable NRTCs

- Proposals

  - Proposal 1: Only NRTC1 and NRTC2 is applicable for 52.6-71GHz

  - Proposal 2: TBA

- Recommended WF

  - Proposal 1

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| CATT | Support the recommended WF. |
| Qualcomm | Support the recommended WF. |
| Ericsson | We support recommended WF. |
| Nokia | Support recommended WF from moderator. |
| ZTE | Support the recommended WF |
| NEC | Support the recommended WF. |

#### Issue 2-1-3: RF channels

- Proposals

  - Proposal 1: agree to reuse the existing test case for FR2-1 NR BS conformance testing for FR2-2. (R4-2209590) (moderator: based on the references in the Tdocs this refers to using existing RF channels)

  - Proposal 2: TBA

- Recommended WF

  - Proposal 1

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| CATT | Support the recommended WF. |
| Qualcomm | Support the recommended WF. |
| Ericsson | We support recommended WF. |
| Nokia | Support recommended WF from moderator. |
| ZTE | Support the recommended WF |
| NEC | Support the recommended WF. |

Sub-topic 2-2: Test models and TDD pattern

#### Issue 2-2-1: Applicable test models

- Proposals

  - Proposal 1: only NR-FR2-TM1.1, NR-FR2-TM2 and NR-FR2-TM3.1 are applicable for FR2-2.71GHz.

  - Proposal 2: TBA

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| CATT | We’re ok with proposal 1. |
| Qualcomm | Support proposal 1. |
| Ericsson | Yes, in principle TM1.1, TM2 and TM3.1 will cover need for FR2-2. But we still have some discussions on EVM testing that may require a new TM depending on RAN4 outcome. |
| Nokia | Ok with proposal 1. |
| ZTE | Support the proposal 1 |
| NEC | Support proposal 1. |
| Moderator | GTW agreement on Aug 17th: Agreement: Proposal 1 agreed |

#### Issue 2-2-2: Test model data length

Agreements in RAN4#103-e captured in R4-2210638:

Issue 3-6-2: Test model data length

Agreement:

- Have the same length on EVM measurement time length and Test model data length

Issue 3-6-3: EVM measurement time length

Agreement reached in issue 3-6-2. Discuss further dependency of TDD pattern, data length and absolute measurement time.

- Proposals

  - Proposal 1: Adopt 5ms for test model data length for FR2-2

  - Proposal 2: Focus on using a fixed number of slots for EVM measurement time length and test model data length and select the number of slots to ensure a good trade-off between the test time and MU for BS type 2-O transmitter testing in the frequency range between 52.6GHz and 71GHz.

  - Proposal 3: There is no need to adjust the EVM measurement time length depending on TDD pattern used in the text model for BS type 2-O EVM test in the frequency range between 52.6GHz and 71GHz.

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| CATT | Proposal 1 is from us. We also support the methodology of proposal 2. |
| Qualcomm | Ok with proposal 2. |
| Nokia | Propose proposals 2 and 3. OK with proposal 1. |
| Rohde & Schwarz | Proposal 2 sound good to us, this is well suited to solve the issue with the high test times for EVM testing. E.g. the test model proposed below in Issue 2-2-3 could be used. |
| ZTE | Okay with proposal 2 and proposal 3, in general, the fixed DL slot for EVM measurement could decided firstly, then further update the following text for FR2-2. The following general parameters are used by all NR test models: - Duration is 2 radio frames for TDD (20 ms) |
| Keysight | Proposal 2 with 80 slots sounds good. Also, we should consider using 1.25ms for all SCS as it was previously proposed. |
| NEC | Fine with proposal 2 and 3. |
| Moderator | GTW agreement on Aug 17: Agreement: Proposal 2 agreed ([80 slot]) |

#### Issue 2-2-3: TDD pattern

- Proposals

  - Proposal 1: if there is no operator inputs on the TDD pattern for FR2-2, then agree on the above TDD configuration for FR2-2.

| Field name | Value |  |  |  |
| --- | --- | --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 60 | 120 | 480 | 960 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 1.25 | 1.25 | 1.25 | 1.25 |
| nrofDownlinkSlots | 3 | 7 | 29 | 59 |
| nrofDownlinkSymbols | 10 | 6 | 10 | 6 |
| nrofUplinkSlots | 1 | 2 | 9 | 18 |
| nrofUplinkSymbols | 2 | 4 | 2 | 4 |

- Proposal 2: TBA

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| CATT | We’re ok with proposal 1. |
| Qualcomm | Ok with proposal 1. |
| Ericsson | Not sure if we have support for 60 kHz SCS at FR2-2. |
| Nokia | OK with proposal 1. |
| Rohde & Schwarz | We are ok with proposal 1. |
| ZTE | Support the proposal 1 |
| Keysight | OK with proposal 1 |

Sub-topic 2-3: General measurement environment/chamber

#### Issue 2-3-1: General framework

- Proposals

  - Proposal 1: The suitability of each OTA chamber (Far field anechoic chamber, CATR, Near field chamber, PWS, etc.) for each test in the frequency range between 52.6GHz and 71GHz should be studied and confirmed by TE vendors, or the list of OTA chamber should be updated for each BS type 2-O transmitter test in the frequency range between 52.6GHz and 71GHz.

  - Proposal 2: the existing measurement setup framework in TS 38.141-2 Annex D and Annex E could also been applicable for FR2-2. (moderator: Already agreed in R4-2210638)

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| CATT | We’re ok with the two proposals. |
| Qualcomm | Ok with proposal 2. The OTA measurement chamber specific calibration and measurements characteristics are discussed in the mentioned annexes. |
| Ericsson | We are not sure what proposal 1 really means. But we agree that we need to work with all test methods relevant for FR2-2. Update the MU to reflect aspects relevant for FR2-2. If needed update test methods and introduced new equipment to maintain reasonable expanded MU. |
| Nokia | Reply to Ericsson: Some of the test methodologies may be applicable for some tests in FR2-1 but not in FR2-2, we need to get confirmation from TE vendors that current list of test methodologies in Annex D and Annex E remain valid in FR2-2. |
| ZTE | we are fine with option 2, however we also agree with option 1 to seek more clarifications from TE vendors. |
| Moderator | GTW agreement Aug 17: Agreement: Proposal 1 agreed; FFS any update needed or not in TS 38.141-2 Annex and/or TR 37.941 |

Sub-topic 2-4: Link budget inside chamber

#### Issue 2-4-1: Path loss

- Proposals

  - Proposal 1: For FR2-2 consider CATR pathloss values for a CATR suitable for FR2-2 testing. Pathloss values in Table 2.1-2 can be used as baseline for FR2-2 (moderator: see R4-2212465 for the values)

  - Proposal 2: TBA

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Qualcomm | Since the proposed values only valid for CATR, do we need some extrapolation to generalize them over other OTA testing environments? |
| Ericsson | We support proposal 1. |
| Nokia | Need further input especially from TE vendors on the proposed values. |
| ZTE | This information would not been captured in TR, this might be useful for MU calculation from our understanding. |
| Keysight | We are not sure about number provided in the table 2.1-2. Calculating back from 61.5dB@71GHz, this is 40cm distance, which can fit 5.7cm Device. (focal length = 3.5 * 2 * D per TR38.810 clause 5.2.5.3) although there could be some system by system difference, the number in the table looks too small, only for smaller size device. |
| Moderator | GTW agreement on Aug 17: Agreement: Further discuss the suitable pathloss values for FR2-2 testing. TE vendors’ feedback is encouraged. |

Sub-topic 2-5: Measurement uncertainty and calibration

#### Issue 2-5-1: General aspects

- Proposals

  - Proposal 1: The measurement capabilities resulting from different TE implementations should be studied and provided by TE vendors to decide the achievable and satisfactory measurement performance for BS type 2-O transmitter testing in the frequency range between 52.6GHz and 71GHz

  - Proposal 2: Measurement uncertainty and test tolerance analysis needs to be properly captured and therefore update to WID is needed to include TR 37.941

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| CATT | We’re ok with the two proposals. |
| Qualcomm | Ok with the two proposals. More input from TE vendors is required. Support proposal 2 to formally include MU and TT aspects in the WID. |
| Ericsson | We support proposal 1, it always better to get input from multiple sources. We also support proposal 2, It was mentioned last meeting that WID needs update to include TR 37.941. But it was not brought up at last RAN plenary. |
| Nokia | Propose proposals 1 and 2. |
| ZTE | Both proposals are fine for us. |
| NEC | Fine with proposal 1 and 2. |

#### Issue 2-5-2: Additional components

- Note observations in R4-2213927:

  - Based on rough link budget calculation, there seems some cases need power amplifier to add more power from signal generator. This needs more careful estimation including all other necessary components and loss.

  - For out of band blocking source, it’s possible to see there is not enough power from generator/amplifier depending on test setup.

- Note content in R4-2212465 on need of components:

| Test | Issue at high frequency | Expected need of change |
| --- | --- | --- |
| Radiated transmit power | Alignment due to more narrow beams | Improved alignment procedure |
| OTA output power dynamics | Limited SNR in test setup | An LNA may be required before measurement receiver |
| OTA transmitted signal quality | Limited SNR for higher modulation formats? |  |
| OTA occupied bandwidth | Limited SNR in test setup |  |
| OTA transmit ON/OFF power | Limited SNR in test setup |  |
| OTA base station output power | Dynamic range | LNA and shorter test distance |
| OTA ACLR | Dynamic range | LNA and shorter test distance |
| OBUE | Dynamic range | LNA and shorter test distance |
| Spurious emissions | Dynamic range | Additional LNA and mixer stage may be required. |
| Receiver spurious emissions | Dynamic range | Additional LNA and mixer stage may be required. |
| OTA reference sensitivity level | Absolute signal power accuracy out from signal generator | The signal generator absolute power can be calibrated using a power meter. An external mixer stage can be added to enhance signal generator frequency capability. |
| OTA ACS | Combination of high frequency signals may cause too large uncertainty. | The signal generator absolute power can be calibrated using a power meter. An external mixer stage can be added to enhance signal generator frequency capability. |
| OTA narrowband blocking | Combination of high frequency signals may cause too large uncertainty. | The signal generator absolute power can be calibrated using a power meter. An external mixer stage can be added to enhance signal generator frequency capability. |
| General OTA blocking | Signal combination and circulator may be difficult at 71 GHz. | Addition of mixer stage is required |
| General OTA out-of-band blocking | See right | For this setup two mixer stages may be required, since wanted signal is within the operating band and interferer is within the range from lower test frequency limit to upper test frequency limit. |

- Proposals

  - Proposal 1: Use of LNA should be in MU budget for FR2-2 except Tx off power measurement.

  - Proposal 2: For MU budget table for Rx testing. MU term of up converter (mixer) and additional power amplifier should be added.

  - Proposal 3: Introduce external mixer stage test setup and corresponding calibration procedures for receiver requirements and out-of-band requirements to improve measurement uncertainty. (moderator: calibration is further discussed in next issue)

  - Proposal 4: Consider two configurations for out of band spurious measurement system below 110 GHz and above up to 142 GHz

    - For out of band spurious measurement system, both use or not to use mixer case to consider and then pick larger MU case to use for calculating total test system MU. For above 110 GHz up to 142 GHz, mixer should be assumed and used for MU budget calculation like FR2-1 spurious emission.

- Recommended WF

  - TBA

As a general principle, if a component if needed, it should be taken into account in MU budget.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| CATT | We support the general principle. |
| Qualcomm | Agree with the recommended WF and the provided principle of accounting additional components within the MU budget. |
| Ericsson | Proposal 1 and Proposal 2: LNA should be included where it’s needed, e.g., many UL tests. We need to work with each test and requirement to figure out if LNA is needed and how it can improve final MU. The introduction of mixer maxes it possible to reduce other MU number since the IF frequency is much lower than RF. The mixer introduction requires a complete overlook of the MU list. We support proposal 3. Regarding proposal 4, we need to study the impact of introducing mixers. Eventually, we can conclude that mixer as used for the complete frequency range, or we need to split it up in multiple sub-regions where mixers is used for some regions. |
| Nokia | Needs further input especially from TE vendors on the MU contributions of the proposed additional components. |
| ZTE | Agree with the recommended WF. In addition, we also think that to use the mixer is one appropriate way for FR2-2. |
| NEC | Fine with the recommended WF in principle. Need further input from TE vendors. Need to know the impact of introducing mixer on MU budget. |
| Moderator | GTW agreement on Aug 17: Use of LNA if applicable should be in MU budget for FR2-2 except Tx off power measurement. FFS whether mixer can be considered for some of test cases if applicable |

#### Issue 2-5-3: Measurement system calibration

- Proposals

  - Proposal 1: Add intermediate calibration stage of spectrum analyser absolute power accuracy for DL EIRP measurement, with the intension to break the trend with very large measurement uncertainties for high frequencies.

  - Proposal 2: For test system measurement uncertainty, extend measurement procedures to enable for test setups to break the trend where measurement uncertainty grows rapidly as function of frequency

  - Proposal 3: Introduce external mixer stage test setup and corresponding calibration procedures for receiver requirements and out-of-band requirements to improve measurement uncertainty. (moderator: focus on calibration, need for components discussed in previous issue)

  - Proposal 4: Use of power sensor/meter for signal leveling should not be mandated for test system setup because limited condition for use of sensor.

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Qualcomm | Ok with the listed proposals. Just to clarify, description of such methodologies would be captured in Annex B in 38.141-2? |
| Ericsson | We support proposal 1 and 2. Regarding proposal 3 we need to study the impact on test procedures, calibrations and test setup. The idea to use mixer stages is to reduce complexity and improving MU. RAN4 needs to study impact of mixers. Regarding proposal 4, we really think MU can be improved by calibration absolute power capabilities of spectrum analyzers and signal generators. This is an approach already used today to improve 3GPP MU. We don’t agree to proposal 4. |
| Nokia | Need further input especially from TE vendors on the calibration proposals, especially on MU impact. |
| ZTE | In general, we are fine with proposals mentioned above, however it’s better to be confirmed with TE vendors. |
| Keysight | For proposal 1,2,3 we are not sure if calibration by itself could reduce uncertainty of system. (we should do calibration carefully all the time) We should continue to use MU budget table to calculate total system MU. For Proposal 4, For SG, while it’s possible to use power sensor as monitoring device to keep monitoring and controlling power level, condition is limited. Actually not sure if it possible for SA. We propose proposal 4. |

#### Issue 2-5-4: Measurement uncertainty contributions

- Proposals

  - Proposal 1: For equipment MU number up to 80 GHz, because these already exist as agreed number, use numbers used in budget for UE. These are;

    - Power measurement equipment for spurious, 1 sigma number from 40.8 GHz to 80 GHz is 2.00

    - Network analyzer, 1 sigma number from 40.8 GHz to 80 GHz is 0.85

- Recommended WF

  - TBA

Companies views’ collection for 1st round

| Company | Comments |
| --- | --- |
| Ericsson | Regarding proposal 1 we believe that power measurements can be done with better measurement uncertainty if power meters are used. This is the reason why we cannot just use spectrum analyzers as input for measuring absolute power. The spectrum analyzer absolute power needs to be calibrated using a power meter. Then MU can be significantly improved. For network analyzer the 0.85 is a reasonable value up to 80 GHz. |
| Nokia | OK with proposal 1 as a starting point, need further input especially from TE vendors. |
| Keysight | With these numbers are already agreed in 3gpp and used, we should use the same agreed number. |
| Ericsson | After gotoweb session I checked TR 37.941. In sub-clause 12.2.3.3 MU value for Network analyzer for spurious emission up to 60 GHz is set to Uncertainty of the network analyzer; 0.30; Gaussian; 1.00; 1; 0.30. The UID is C1-3. It seems that for BS testing 0.3 dB 1 dB sigma is assumed up to 60 GHz. Maybe 0.5 dB is a bit too much to account for frequencies up to 80 GHz. We need to find another value for BS conformance testing. |
| Nokia | Agree we need further input to decide the MU values instead of directly copy from UE test. |
| Moderator | Tentative outcome of GTW on Aug 17: For equipment MU number up to 80 GHz, use numbers used in budget for UE as starting point. These are; - Network analyzer, 1 sigma number from 40.8 GHz to 80 GHz is 0.85 |

Sub-topic 2-6: Upper and lower frequency limits

#### Issue 2-6-1: Upper and lower frequency limits

Note observation in R4-2212465: Observation: It would be possible to consider a lower frequency limit of 300 to 400 MHz for FR2-2.

Please also comment with views on lower limit.

- Proposals

  - Proposal 1: Choose a frequency slightly larger than Fstep,6 (127GHz) as max limit frequency point, e.g., 130GHz.

  - Proposal 2: Align the maximum limit frequency point for RX with that for TX.

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| CATT | Support proposal 1 and proposal 2. |
| Qualcomm | Ok with the listed proposals. |
| Ericsson | We support proposal 1 and 2. Sound like a reasonable compromise. |
| Nokia | On proposal 1, 142 GHz which fully covers 2nd harmonics of the band, but feasibility needs to be confirmed by TE vendors. Ok with proposal 2. |
| ZTE | Support the proposal 1 and proposal 2. |
| Keysight | We wonder where 130G comes from. It seems it’s just picked up. |
| NEC | Support the proposal 1 and 2. |
| Moderator | GTW agreement on Aug 17: Proposal 2 agreed Max limit frequency point: [142] GHz |

Sub-topic 2-7: Measurement frequency step size

#### Issue 2-7-1: Interferer signal step size for OOB blocking

- Proposals

  - Proposal 1: 120 MHz can be considered as measurement step size for interferer signal step size for 800MHz, 1600MHz, and 2000MHz CBW for OTA in-band blocking and OTA out-of-band blocking.

  - Proposal 2: Update Table 7.6.4.2.3-1 in TS 38.141-2 to include larger step size 120MHz or 240MHz for minimum supported BS channel bandwidth larger than 400MHz for NR operation in 52.6 – 71 GHz range.

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| CATT | We proposed 120MHz, we can also accept 240MHz for larger CBW. |
| Ericsson | We support proposal 1. We also support proposal 2, using 120 MHz in tables. |
| Nokia | Propose proposal 2, 240 MHz step size for 1600 and 2000 MHz channel bandwidth to save test time without degrading test coverage. |
| ZTE | Proposal 2 is more preferred considering the testing workload. |
| NEC | Support proposal 2. |
| Moderator | GTW agreement on Aug 17: Update Table 7.6.4.2.3-1 in TS 38.141-2 to include larger step size 240MHz for minimum supported BS channel bandwidth larger than 400MHz for NR operation in 52.6 – 71 GHz rang |

Sub-topic 2-8: Tx OFF measurement

#### Issue 2-8-1: Antenna gain during OFF period

- Proposals

  - Proposal 1: For FR2-2 Tx off power measurement, as it was agreed for FR2-1, assume On antenna gain used for Off period gain and use EIRP rather TRP for measurement.

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| CATT | We’re ok with proposal 1. |
| Qualcomm | Ok with proposal 1. |
| Ericsson | We support to keep the same assumptions as for FR2-1, hence we support proposal 1. |
| Nokia | Ok with proposal 1 |
| ZTE | Okay with proposal 1 |
| NEC | Ok with proposal 1. |

Sub-topic 2-9: OOB blocking feasibility

#### Issue 2-9-1: OOB blocking feasibility

- Proposals

  - Proposal 1: For out of band blocking, further consider feasibility.

  - Proposal 2: TBA

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | The challenge it to generate power at very low frequencies and very high frequencies without changing the test setup too much. E.g., how many test antennas will be needed to cover the out-of-band frequency region. Another aspect to consider it the ability to generate power at high frequencies. Even though, testing may be difficult out-of-band blocking is an important requirement since many strong interferers outside the band exist at high frequencies. |
| Nokia | Need input especially from TE vendors on the feasibility. |
| ZTE | Similar understanding as Nokia |
| Moderator | GTW agreement on Aug 17: Further consideration required for the OOB blocking test feasibility. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| R4-2212463 Draft CR to 38.141-2: Addition of FR2-2 transmitter support in clause 6 Ericsson |  |
| --- | --- |
| Nokia | RAN4 should focus on technical decisions e.g. on MU, before reviewing the draft CR contents which will be updated by the decisions. |
| ZTE | Draft CR could be postponed until we reach the consensus for FR2-2 |

| R4-2212464 Draft CR to 38.141-2: Addition of FR2-2 receiver support in caluse 7 Ericsson |  |
| --- | --- |
| Nokia | RAN4 should focus on technical decisions e.g. on MU, before reviewing the draft CR contents which will be updated by the decisions. |
| ZTE | Draft CR could be postponed until we reach the consensus for FR2-2 |

## Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic #2-1: Test configurations and RF channels | Issue 2-1-1: Signal used to build NR TCs in FR2-2 Background: - Proposal 1: Define independent TC signals for FR2-2 as shown in following Table 4.7.2.1-2a. (Moderator: see R4-2211645 or section 2.1 of this document for the Table) - Proposal 2: The maximum FDL_high – FDL_low of 14000 MHz should replace 5000 MHz in Table 4.7.2.1-2a. (R4-2209590) for BS type 2-O transmitter testing in the frequency range between 52.6GHz and 71GHz. Tentative agreements: - Merge proposal 1 and 2, i.e use the table from proposal 1 with FDL_high – FDL_low of 14000 MHz. Candidate options: Recommendations for 2nd round: Capture agreement in WF. Issue 2-1-2: Applicable NRTCs Tentative agreements: All companies agree: Proposal 1: Only NRTC1 and NRTC2 is applicable for 52.6-71GHz Recommendations for 2nd round: Capture agreement in WF. Issue 2-1-3: RF channels Tentative agreements: All companies agree Proposal 1: agree to reuse the existing test case for FR2-1 NR BS conformance testing for FR2-2. (R4-2209590) (moderator: based on the references in the Tdocs this refers to using existing RF channels) Recommendations for 2nd round: Capture agreement in WF. |
| Sub-topic #2-2: Test models and TDD pattern | Issue 2-2-1: Applicable test models Tentative agreements: GTW agreement on Aug 17th: Agreement: Proposal 1 agreed Proposal 1: only NR-FR2-TM1.1, NR-FR2-TM2 and NR-FR2-TM3.1 are applicable for FR2-2.71GHz. Recommendations for 2nd round: Capture agreement in WF. Issue 2-2-2: Test model data length Tentative agreements: GTW agreement on Aug 17: Agreement: Proposal 2 agreed ([80 slot]) Proposal 2: Focus on using a fixed number of slots for EVM measurement time length and test model data length and select the number of slots to ensure a good trade-off between the test time and MU for BS type 2-O transmitter testing in the frequency range between 52.6GHz and 71GHz. Recommendations for 2nd round: Capture agreement in WF. Issue 2-2-3: TDD pattern All companies agree Proposal 1, but 60 kHz SCS spacing should be removed as it is not supported in FR2-2. Tentative agreements: Proposal 1: if there is no operator inputs on the TDD pattern for FR2-2, then agree on the above TDD configuration for FR2-2. Field name; Value. referenceSubcarrierSpacing (kHz); 60; 120; 480; 960. Periodicity (ms) for dl-UL-TransmissionPeriodicity; 1.25; 1.25; 1.25; 1.25. nrofDownlinkSlots; 3; 7; 29; 59. nrofDownlinkSymbols; 10; 6; 10; 6. nrofUplinkSlots; 1; 2; 9; 18. nrofUplinkSymbols; 2; 4; 2; 4. Recommendations for 2nd round: Capture agreement in WF without 60 kHz SCS. |
| Sub-topic #2-3 General measurement environment/ chamber | Issue 2-3-1: Background: Tentative agreements: GTW agreement Aug 17: Agreement: Proposal 1 agreed; FFS any update needed or not in TS 38.141-2 Annex and/or TR 37.941 Proposal 1: The suitability of each OTA chamber (Far field anechoic chamber, CATR, Near field chamber, PWS, etc.) for each test in the frequency range between 52.6GHz and 71GHz should be studied and confirmed by TE vendors, or the list of OTA chamber should be updated for each BS type 2-O transmitter test in the frequency range between 52.6GHz and 71GHz. Recommendations for 2nd round: Capture agreement in WF, discuss further if necessary |
| Sub-topic #2-4 Link budget inside chamber | Issue 2-4-1: Path loss Tentative agreements: GTW agreement on Aug 17: Agreement: Further discuss the suitable pathloss values for FR2-2 testing. TE vendors’ feedback is encouraged. Recommendations for 2nd round: Follow GTW agreement |
| Sub-topic #2-5 Link budget inside chamber | Issue 2-5-1: General aspects Tentative agreements: All companies agree both proposals: Proposal 1: The measurement capabilities resulting from different TE implementations should be studied and provided by TE vendors to decide the achievable and satisfactory measurement performance for BS type 2-O transmitter testing in the frequency range between 52.6GHz and 71GHz Proposal 2: Measurement uncertainty and test tolerance analysis needs to be properly captured and therefore update to WID is needed to include TR 37.941 Recommendations for 2nd round: Capture agreement in WF. Issue 2-5-2: Additional components Tentative agreements: GTW agreement on Aug 17: Use of LNA if applicable should be in MU budget for FR2-2 except Tx off power measurement. FFS whether mixer can be considered for some of test cases if applicable Also companies agree with the general principle: As a general principle, if a component if needed, it should be taken into account in MU budget. Recommendations for 2nd round: Capture agreement in WF, discuss further if necessary Issue 2-5-3: Measurement system calibration Candidate options: - Proposal 1: Add intermediate calibration stage of spectrum analyser absolute power accuracy for DL EIRP measurement, with the intension to break the trend with very large measurement uncertainties for high frequencies. - Proposal 2: For test system measurement uncertainty, extend measurement procedures to enable for test setups to break the trend where measurement uncertainty grows rapidly as function of frequency - Proposal 3: Introduce external mixer stage test setup and corresponding calibration procedures for receiver requirements and out-of-band requirements to improve measurement uncertainty. (moderator: focus on calibration, need for components discussed in previous issue) - Proposal 4: Use of power sensor/meter for signal leveling should not be mandated for test system setup because limited condition for use of sensor. Recommendations for 2nd round: Discuss further in the WF Issue 2-5-4: Measurement uncertainty contributions Candidate options: For equipment MU number up to 80 GHz, use numbers used in budget for UE as starting point. These are; - Network analyzer, 1 sigma number from 40.8 GHz to 80 GHz is 0.85 Based on TR 37.941 Network analyzer, 1 sigma number up to 60 GHz is 0.3 dB Recommendations for 2nd round: Discuss further in the WF. |
| Sub-topic #2-6 Upper and lower frequency limits | Issue 2-6-1: Upper and lower frequency limits Tentative agreements: GTW agreement on Aug 17: Proposal 2 agreed: Proposal 2: Align the maximum limit frequency point for RX with that for TX. Max limit frequency point: [142] GHz Recommendations for 2nd round: Capture agreements in WF. |
| Sub-topic #2-7 Measurement frequency step size | Issue 2-7-1: Interferer signal step size for OOB blocking Tentative agreements: GTW agreement on Aug 17: Update Table 7.6.4.2.3-1 in TS 38.141-2 to include larger step size 240MHz for minimum supported BS channel bandwidth larger than 400MHz for NR operation in 52.6 – 71 GHz range Recommendations for 2nd round: Capture agreement in WF. |
| Sub-topic #2-8 Tx OFF measurement | Issue 2-8-1: Antenna gain during OFF period Tentative agreements: All companies agree: Proposal 1: For FR2-2 Tx off power measurement, as it was agreed for FR2-1, assume On antenna gain used for Off period gain and use EIRP rather TRP for measurement. Recommendations for 2nd round: Capture agreement in WF. |
| Sub-topic #2-9 OOB blocking feasibility | Issue 2-9-1: OOB blocking feasibility Tentative agreements: GTW agreement on Aug 17: Further consideration required for the OOB blocking test feasibility. Recommendations for 2nd round: Capture agreement in WF, discuss further if possible. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2212463 | Not pursued / postponed to future meeting |
| R4-2212464 | Not pursued / postponed to future meeting |

## Discussion on 2nd round (if applicable)

Second round discussion to take place in WFs, moderator will copy relevant comments to the summary document.

No comments were provided to R4-2214374, WF on FR2-2 BS conformance testing.

Discussion in R4-2214373 WF on FR2-2 BS test environments:

Comments were provided on “Use of power sensor/meter for RF power measurement should not be mandated for  all test requirements and related test setups.

| Keysight | Changed RF power measurement to signal leveling. |
| --- | --- |
| Ericsson | We do not agree to this statement. Power meters are used to improve absolute power accuracy. For some requirements (like EIRP and EIS where absolute power is of interest) the use of power meter is required. Before we exclude power meters we need to show the technical background and feasibility. We suggest to further look into the possibility to use power meters to improve MU as suggested in bullet i and ii. If we agree to iv as Keysight suggest, that would exclude the possibility to use power meters to calibrate the spectrum analyzer. Hence the bullets contradict. The word “leveling” sound for me something related to building construction not RF measurements, but I can be wrong. For requirements related to absolute power level and FR2-2 we suggest to use a power meter to improve measurement uncertainty. For other requirements the power meter is not used (not mandatory). That’s the reason for the work “all” here. |
| Nokia | I changed this as FFS in the bullet heading as we currently have no conclusion whether this should be considered. |
| Keysight | To Ericsson: Please keep original test here. I don’t think this against above (I, ii). Also no need to add “all” Thank you for comment and suggestion, as Man suggests let’s leave this as FFS for this meeting. |
| Huawei | Ok to keep this open and study further. It may be beneficial to set a deadline by the next meeting to take related decision and progress in related MU aspects. |

# Topic #3: BS demod OTA test methodology

This topic covers Tdocs submitted to core requirement agenda items. Two draft CRs as well as two documents for discussion. Only topic needing further agreements outside the CRs is FRCs and therefore only one sub-topic is needed.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2213927 | Keysight Technologies UK Ltd | Observation: - Based on rough link budget calculation, there seems some cases need power amplifier to add more power from signal generator. This needs more careful estimation including all other necessary components and loss. - Power sensor/meter can possibly be used for leveling signal generator or up converter output when it meets necessary condition of power sensor use. Otherwise, it’s just add complexity to system. - Regarding with demod link budget estimation, expected power level seem too high then power amplifier required. For use of power amplifier, expected output level should below max power limit of amplifier. - For out of band blocking source, it’s possible to see there is not enough power from generator/amplifier depending on test setup. Proposal - For MU budget table for Rx testing. MU term of up converter (mixer) and additional power amplifier should be added. - Use of power sensor/meter for signal leveling should not be mandated for test system setup because limited condition for use of sensor. - For demod setup, consider using 15dB margin for lowering AWGN level. This reduces risk of not finding appropriate amplifier. - For out of band blocking, further consider feasibility. |
| R4-2212674 | Nokia, Nokia Shanghai Bell | Proposal 2: RAN4 to define demodulation requirements for FR2-2 including opportunity for AWGN_offset reduction as specified for FR2-1. |

## Open issues summary

### Sub-topic 3-1: AWGN offset

Open issues and candidate options before e-meeting:

#### Issue 3-1: AWGN offset

New proposals in this meeting are provided below

- Proposals

  - Proposal 1: RAN4 to define demodulation requirements for FR2-2 including opportunity for AWGN_offset reduction as specified for FR2-1

  - Proposal 2: For demod setup, consider using 15dB margin for lowering AWGN level. This reduces risk of not finding appropriate amplifier.

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Qualcomm | Ok with proposal 1. For proposal 2, it is not clear on what basis was the 15 dB AWGN level margin was selected. The papers says that it is selected to make test system to meet requirement, is this the 20 dB SNR or something else? It is not clear form Table 1 in R4-2213927. |
| Ericsson | We have a slight preference towards proposal 2. This would give needed margins with respect to selection of power amplifier. |
| Nokia | Propose proposal 1. Ok with proposal 2. Using lower AWGN-offset shall be allowed as already done in FR2-1. |
| Keysight | For answering Qualcomm question and adding some more on proposal 2 (which is from us), reason of 15dB is actually the same reasoning as Proposal 1, which has been agreed for FR2-1 to possibly lower AWGN level up to 15dB. I believe both proposals are actually the same, so OK with both Proposal 1 and 2. |
| Moderator | GTW agreement on Aug 17: RAN4 to define demodulation requirements for FR2-2 including opportunity for AWGN_offset reduction as specified for FR2-1 - AWGN_offset = [0-15] dB Demodulation requirements can be proceeded based on the agreements from RF session. |

CRs/TPs comments collection

No TP or CR submitted.

## Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic #3-1: BS demod OTA test methodology | Issue 3-1: AWGN offset Tentative agreements: GTW agreement on Aug 17: RAN4 to define demodulation requirements for FR2-2 including opportunity for AWGN_offset reduction as specified for FR2-1 - AWGN_offset = [0-15] dB Demodulation requirements can be proceeded based on the agreements from RF session. Recommendations for 2nd round: Capture agreement in WF, discuss further if needed |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

No TP/CR submitted.

## Discussion on 2nd round (if applicable)

Second round discussion to take place in WFs, moderator will copy relevant comments to the summary document.

See section 2.4

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on FR2-2 BS test environments | Ericsson | Cover sub-topic 2-3, 2-4, 2-5 |
|  | WF on FR2-2 BS conformance testing | Nokia, Nokia Shanghai Bell | Cover sub-topic 2-1, 2-2, 2-6, 2-7, 2-8, 2-9, 3-1 |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2211806 |  | Draft CR to TS 38.104 on correction of TAE requirements | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2211644 |  | Discussion on the BS RX RF requirements for 52.6-71GHz | CATT | Noted |  |
| R4-2211807 |  | Proposals on finalizing BS receiver requirements for extending current NR operation to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2211808 |  | Draft CR to TS 38.104 on finalization of ICS interfering signal RB | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2211645 |  | Discussion on general part of conformance requirements for 52.6-71GHz | CATT | Noted |  |
| R4-2212465 |  | On general aspects relevant for conformance testing up to 71 GHz | Ericsson | Noted |  |
| R4-2212846 |  | Measurement uncertainty considerations for NR in 52.6GHz – 71GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2213702 |  | Further discussion on BS conformance testing for 52.6-71GHz | ZTE Corporation | Noted |  |
| R4-2211646 |  | Discussion on the BS transmitter conformance testing for 52.6-71GHz | CATT | Noted |  |
| R4-2211809 |  | Proposals on BS transmitter conformance testing for extending current NR operation to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2212461 |  | On transmitter test requirement level derivation for FR2-2 | Ericsson | Noted |  |
| R4-2212463 |  | Draft CR to 38.141-2: Addition of FR2-2 transmitter support in clause 6 | Ericsson | Not pursued / postponed to future meeting |  |
| R4-2213925 |  | about FR2-2 BS conformance test system, Tx system link budget | Keysight Technologies UK Ltd | Noted |  |
| R4-2211647 |  | Discussion on the BS receiver conformance testing for 52.6-71GHz | CATT | Noted |  |
| R4-2211810 |  | Proposals on BS receiver conformance testing for extending current NR operation to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2212462 |  | On receiver test requirement level derivation for FR2-2 | Ericsson | Noted |  |
| R4-2212464 |  | Draft CR to 38.141-2: Addition of FR2-2 receiver support in caluse 7 | Ericsson | Not pursued / postponed to future meeting |  |
| R4-2213927 |  | about FR2-2 BS conformance test system, Rx test link budget | Keysight Technologies UK Ltd | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2214373 |  | WF on FR2-2 BS test environments | Ericsson | Agreeable |  |
| R4-2214374 |  | WF on FR2-2 BS conformance testing | Nokia, Nokia Shanghai Bell | Agreeable |  |

A new Tdoc for email approval should be still allocated for big CR to include draft CRs agreed in 1st round.

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents