3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214314

Electronic Meeting, 15– 26 August 2022

Agenda item:			9.14.8.2

Source:	Qualcomm

Title:	Email discussion summary for [104-e][325] NR_exto71GHz_Demod_Part2

Document for:	Information

# Introduction

Treated Agenda Items:

9.14.8.2        UE Demodulation and CSI requirements

9.14.8.2.1        PDSCH requirements

9.14.8.2.2        PDCCH/PBCH requirements

9.14.8.2.3        SDR requirements

9.14.8.2.4        CSI reporting requirements

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |  |
| --- | --- | --- | --- |
| Ericsson | Kamel Tourki | Kamel.tourki@ericsson.com |  |
| Qualcomm | Pier Vallese | pvallese@qti.qualcomm.com |  |
| Anritsu | Osamu Yamashita | Osamu.Yamashita@anritsu.com |  |
| Apple | Manasa Raghavan | Manasa.raghavan@apple.com |
| Huawei | Like | like54@hisilicon.com |
| Nokia, Nokia Shanghai Bell | Karsten Petersen | Karsten.petersen@nokia-bell-labs.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: General Scope of the Demodulation Requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211868 | Apple | General Proposal #1: Use Max Doppler frequency as 200Hz for TDL-A channel. Proposal #2: Do not introduce requirements with 960KHz SCS. Observation #1: Only 400MHz CBW is mandatory for 480Khz. Observation #2: Maximum testable SNR is very limited at higher CBW for FR2-2. Proposal #3: Do not define requirements with 1600MHz for 480KHz SCS. PDSCH Demod Proposal #4: HARQ-ACK codebook and HARQ bundling can be left to RAN5 specification and need not be discussed in RAN4. Proposal #5: Do not define requirement at 30% max throughput. Proposal #6: Use CPE only compensation when PN is enabled for FR2-2. Proposal #7: Do not define requirements with rank 2 for FR2-2. Proposal #8: Do not define requirements with 64QAM for FR2-2. Proposal #9: For PDSCH demod requirements in FR2-2 we propose to introduce requirements for QPSK, 16QAM with rank 1 for 120KHz SCS/100MHz; QPSK for 480KHz SCS/400MHz. |
| R4-2211869 | Apple | Proposal #1: Define requirements for PDCCH demod with 480KHz SCS with multi-slot PDCCH monitoring – 1 slot every 4 slots. Proposal #2: Define PDCCH demod requirements for initial set of testcases agreed for FR2-2. |
| R4-2211870 | Apple | Proposal #1: Define CQI reporting requirement with 120KHz SCS/ 100MHz CBW for FR2-2. Proposal #2: Limit CQI requirements in FR2-2 to SNR ranges corresponding to QPSK and 16QAM. |
| R4-2212106 | Nokia, Nokia Shanghai Bell | Observation 1: 32 HARQ processes are required for the agreed TDD pattern (29D3S8U, S1=10D:4G:0U, S2=0D:12G:2U, S3=0D:0G:14U) when using 960kHz SCS Proposal 1: RAN4 to define requirements with 32 DL HARQ processes if 960kHz SCS is agreed. Whether to define requirements with 16 DL HARQ processes Observation 2: 16 HARQ process requirements are already agreed to be defined for 480kHz SCS, hence same should apply to 960kHz SCS. Observation 3: Requirements for 16 HARQ processes should be defined even if agreed to define requirements for 32HARQ processes as 32 HARQ is optional capability. Proposal 2: RAN4 to define requirements with 16 DL HARQ processes for 960kHz SCS if requirements for 960kHz SCS are agreed to be defined. HARQ-ACK Codebook Proposal 3: RAN4 to leave the decision of HARQ-ACK Codebook to RAN5. HARQ Bundling for multi-PDSCH scheduling Observation 4: If RAN5 is to specify the multi-PDSCH scheduling, it is important, RAN4 ensures that all options are covered in the test cases. This is specifically important for the 32 HARQ case as 32 HARQ is a new capability. Proposal 4: RAN5 to specify the multi-PDSCH scheduling and RAN4 to take care that the chosen configuration do not lead to issues in RAN5 test cases with respect to HARQ bundling. Whether to define requirements at 30% of peak throughput Observation 5: The 30% peak throughput test is related to verifying HARQ soft combining. Observation 6: Existing requirements are only for FR2-1 and will not apply for FR2-2 Proposal 5: RAN4 to define requirements at 30% of peak throughput Whether to define Rank1 requirements with FD-OCC disabled Observation 7: As we have not been able to find specific optional capability definition for the feature “FD-OCC-Disabled”, we assume it is mandatory. Proposal 6: Since no information to our knowledge is provided to show the feature “FD-OCC-Disabled” as optional, we support to define requirements with FD-OCC disabled. Whether to define requirements with Rank2 Observation 8: UE declaring support for FR2-2 only will never be tested for Rank 2 if no requirements are defined. Proposal 7: RAN4 to define requirements for rank 2. MCS for 64QAM Observation 9: Observation 9: RAN4 should not limit requirement definition based on current capability of legacy TEs, which are not developed for 71 GHz, hence we do not see a reason to exclude 64QAM for 480kHz and 960kHz. Observation 10: MCS22 seems to be a valid choice for SCS 120kHz, and MCS20 for 480 kHz and 960 kHz SCS. Proposal 8: We propose the following MCS to be used: - MCS 22 for 120 kHz SCS - MCS 20 for 480kHz and 960kHz SCS Phase Noise Compensation Observation 11: Only a baseline set of test cases have been agreed. It is not yet agreed if requirements for larger MCS and CBW will be defined. Proposal 9: CPE or CPE+ICI decision shall depend on the simulation results for the test cases. Proposal 10: Adopt CPE+ICI at least in the following test cases - 120 kHz SCS + 100 MHz + MCS>17 - 120 kHz SCS + 400 MHz + MCS>13 - 480 kHz SCS + 800 MHz + MCS>13 - 480 kHz SCS + 1600 MHz + MCS>13 - Adopt CPE only otherwise Test Cases Observation 12: We see it possible portable device will prefer to support 960kHz SCS instead of 480kHz SCS, to avoid the complexity of implementing ICI. Observation 13: Even if it is currently debatable whether all scenarios of 960kHz are testable with legacy TE equipment, it is useful for RAN4 to have good requirement coverage for 960kHz SCS to anticipate the new Tes developed for FR2-2 and to inform operators about expected minimum performance in deployment. RAN4 to include requirements for 960kHz SCS in the test cases. |
| R4-2212108 | Nokia, Nokia Shanghai Bell | PDCCH Performance Requirements Whether to define requirements for multi-slot PDCCH monitoring Observation 1: Multi-slot PDCCH monitoring is a mandatory feature for 480kHz/960kHz SCS, hence requirements should be defined for the feature We see implicitly testing multi-PDCCH monitoring in the PDSCH requirements as a viable way, however directly defining requirements would be preferred. Proposal 1: Define direct requirements for multi-slot PDCCH monitoring. Test Cases Cover following for 120kHz/100MHz. Align propagation condition with PDSCH. Cover following for 480kHz/400MHz and 960kHz/400MHz. Align propagation condition with PDSCH. PBCH Performance Requirements SSB index assumption SSB index detection is used for RRM measurements, so in our understanding it is important to have requirements with “not known SSB index”. Defining requirements with both cases would be preferred. Define requirement for both known and not known SSB index. |
| R4-2212111 | Nokia, Nokia Shanghai Bell | CBW/SCS Combination CSI Requirements should include at least one of the new SCS optional capabilities. When UE supports higher SCS it is mandatory to support 400MHz CBW, hence it makes sense to include 400MHz CBW/480kHz SCS. RAN4 to define CSI requirements for CBW/SCS combinations 100MHz/120kHz and 400MHz/480kHz SNR value RAN4 should not limit requirement definition based on current capability of TE which are not developed for 71GHz. RAN4 to define SNR values including 64 QAM |
| R4-2212570 | Ericsson | Observation 1: Is it worth that the endorsed Proposal 4 in [2] states that: “For 64QAM scenarios (both Demod and CSI), consider fading backoff margin of 11.08 dB (replacing the current working assumption of 17.71 dB) corresponding to the 1e-3 faded signal clipping probability”. This has led to the SNRs increase in FR2-1. We expect same behavior in FR2-2 by considering a fading backoff margin of 11.08 dB instead of 17.7 dB. Observation 2: We consider the reference SNRs as indicated in Table 7.2.3-4, and we trust that the companies and TE vendors will spend the needed efforts to review the reference SNRs whose may allow for higher MCS for 120 KHz/100 MHz, 480 KHz/400 MHz, and possibly 120 KHz/400 MHz and 480 KHz/800 MHz. Proposal 1: RAN4 defines the PDSCH demodulation requirements with: • FR2-2 TDD: SCS = 120 kHz with min CBW = 100 MHz (66 RBs), and possibly [Max CBW = 400 MHz] FR2-2 TDD: SCS = 480 kHz with min CBW = 400 MHz (66 RBs), and possibly [CBW = 800 MHz] Observation 3: Using SCS 960 KHz, we increase max CBW by 25 % (from 1600 MHz to 2000 MHz), while the CP is halved (looses 50% of its duration). Proposal 2: RAN4 defines the UE demodulation and CSI reporting requirements with: • Number of receive antennas: 2Rx • Modulation order: Up to 64QAM • Both single carrier (FR2-2) and NR-DC FR1 + FR2-2 scenarios Observation 4: In Rel-17, NR operation in FR2-2 will only support Rel-15 patterns for CP-OFDM. Proposal 3: Define PDSCH requirements using both PN models in TR 38.808, where PN is considered at the UE only. Observation 5: When applying TR 38.808 Set 1, we set the design margin as 0 dB at the BS and 5 dB at the UE. Proposal 4: Using Rel-15 PTRS patterns, define PDSCH requirements when PN with CPE compensation only when allocating a low PRB number (66 or below). Proposal 5: Define PDSCH demodulation requirements for UE at 70% of the peak throughput using the following test setup. Observation 6: The proposed TDD UL/DL pattern for test (FR2.120-1) is given by DDDSU, S: 10D:2G:2U. Observation 7: For SCS 480 KHz, we defined a TDD UL-DL pattern in [2], given by 14D2S4U, S1 = 12D:2G:0U, S2 = 0D:6G:8U. Proposal 6: Define PDSCH demodulation requirements for UE at 30% of the peak throughput using the following test setup. Proposal 7: Define PDSCH performance requirements for multi-PDSCH scheduling, where • 120 KHz SCS: Single-TB scheduling • 480 KHz SCS: 4-TB scheduling Observation 8: Do not specify HARQ bundling for multi-PDSCH scheduling and leave it to RAN5. Proposal 8: For HARQ processes in FR2-2: • Do not define requirements with 32 DL HARQ processes for both 120 KHz SCS and 480 KHz SCS. Define requirements with 16 DL HARQ processes for 480 KHz SCS. |
| R4-2212572 | Ericsson | Observation 1: TDD UL-DL pattern should be defined for tests. Observation 2: While RAN1 defined multi-slot PDCCH monitoring for 480 KHz SCS, we do not believe that it is necessary here since it could be implicitly tested with PDSCH. Proposal 1: Define PDCCH demodulation requirements for UE in FR2-2 with the following test setup. Proposal 2: Define PBCH demodulation requirements with the following test setup |
| R4-2212574 | Ericsson | Proposal 1: Define SDR test for UE in FR2-2 considering 2Rx UE considering the following parameters where the following numerologies have been considered (under PN effects): 120 KHz/100 MHz (66 RBs), 120 KHz/400 MHz (264 RBs), 480 KHz/400 MHz (66 RBs), and 480 KHz/800 MHz (124 RBs). Proposal 2: Define SDR requirements for following scenarios: o SA FR2-2 o FR1 (CA or single CC) CA with FR2-2 (CA or single CC) o FR1 (CA or single CC) DC with FR2-2 (CA or single CC) |
| R4-2212576 | Ericsson | Proposal 1: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM), see Appendix A.1, by reusing the existing test setup and metrics for SCS 120 KHz. • Consider 120 KHz/100 MHz (66 RBs) and use PN with CPE compensation only. Proposal 2: For SCS 480 KHz in static conditions, define CSI reporting by considering  Consider 480 KHz/400 MHz (66 RBs) and use PN with CPE compensation only.  Periodicity/offset = 32/1 slot and follow TDD UL-DL pattern FR2.120-2 as: FR2-2 TDD 480 KHz: 11D S 4U, S: 2D:12G:0U  Keep same CQI delay as 120 KHz SCS, i.e., 8.375 ms. |
| R4-2213805 | Huawei,HiSilicon | Observation 1: RF part only defined CA scenarios with CA_n41-n263, CA_n77-n263 and CA_n79-n263 but not defined DC scenarios for FR1+FR2-2 Proposal 1: RAN4 to consider following scenarios for FR2-2 requirements definition:  Single carrier  FR1+FR2-2 CA scenarios Proposal 2: Configure following parameters for FR1 Pcell:  TDD  30kHz SCS with 7D1S2U  All HARQ feedback should be transmitted on UL slot of FR1 carrier Observation 2: Extra simplification work will be needed if TDL-A20 is agreed. Proposal 3: Not consider TDL-A20. Proposal 4: Only consider 200Hz doppler spread. Proposal 5: Not consider 960kHz and 1600MHz CBW for 480kHz SCS. Proposal 6: For PDSCH requirements definition with 64QAM, RAN 4 to select max MCS to satisfy that performance degradation due to phase noise is less than 1dB. I.e. MCS 17 Observation 3, we have following observation for testability.  For 120 kHz SCS, the target SNR related to MCS17 is larger than maximum DL testable SNR.  For 480 kHz SCS, the target SNR related to MCS 13 and MCS 17 are larger than maximum DL testable SNR Proposal 7: Consider following options to make test cases testable:  Option 1 (1st priority): Define requirements with full bandwidth allocation. I.e. (66RBs for both 120kHz/100MHz and 480kHz/400MHz). Define the test setup as following: The number of allocated RB should be the maximum number of RB related to maximum testable SNRBB declared by TE vendor and the tested RB should be put on centre of the tested carrier  Option 2 (2nd priority): Define requirements with partial bandwidth allocation. Take following configurations as an example:  120kHz/100MHz: MCS 4 and 13: Configure 66 RBs  120kHz/100MHz: MCS 17: Configure 33 RBs  480kHz/400MHz: MCS4: Configure 66RBs  480kHz/400MHz: MCS13 rank 1: Configure 16RBs  Option 3: (3rd priority): Define requirements and test setup with full bandwidth allocation, not consider testability like PBCH requirements definition. Proposal 8: Not to define requirements at 30% of peak throughput Proposal 9: Not define rank 1 requirements with FD-OCC disabled Proposal 10: Define the requirements with rank 2+TDLD+MCS13 Observation 4: ICI compensation only bring 0.2dB performance gain under following condition:  Phase noise model set2 only modelled in RX side  MCS 22 Proposal 11: Only consider CPE compensation. Proposal 12: Use type 1 HARQ-ACK codebook and HARQ bundling 4 for 480 kHz SCS Proposal 13: Use cases in Table 2-2 for requirements definition. |
| R4-2213807 | Huawei,HiSilicon | Proposal 1: Implicitly test UE supporting 4-slots monitoring in PDCCH and PDSCH requirements Proposal 2: Use Table 2-1 as final PDCCH test cases Proposal 3: Define FR2-2 PBCH requirements for both known SSB index and unknown index. |
| R4-2213809 | Huawei,HiSilicon | Proposal 1: Define FR2-2 SDR requirements for following scenarios:  SA FR2-2  FR1 (CA or single CC) CA with FR2-2 single CC Proposal 2: For FR1+FR2-2 CA scenario, all ACK/NACK should be transmitted separately on each CC Proposal 3: Define FR2-2 SDR requirements for both rank 1 and rank 2. Observation 1: 480kHz and 960kHz SCS are optional for UE and 120kHz is mandatory for UE Observation 2: 120 kHz SCS is more sensitive to ICI caused by phase noise which is related to minimum requirements. Observation 3: Maximum DL testable SNR is quite low for 480kHz. I.e. <-20dB for 1600MHz and -0.6dB for 400MHz. SDR is untestable under such quite low SNR. Proposal 4: Only consider 120kHz SCS for SDR test Proposal 5: Only consider CPE compensation. Observation 4: Due to the more phase noise impact, performance for cases with 400MHz CBW is worse than 100MHz CBW when MCS is larger than 20. Proposal 6: Define separate MCS to SNR mapping tables for different CBWs Proposal 7: Configure MCS24 for Scaling factor 1 for FR2-2 64QAM Rank 1. Proposal 8: Configure MCS24 for Scaling factor 1 for FR2-2 64QAM Rank 2. Proposal 9: Set MCS0 for minimum MCS for FR2-2 MCS to SNR mapping. |
| R4-2213811 | Huawei,HiSilicon | Observation 1: 120kHz SCS is more susceptible to ICI caused by phase noise. Observation 2: 480kHz/400MHz means quite low testable SNR which means only CQI index related to QPSK can be tested. Proposal 1: Only consider 120kHz/100MHz Proposal 2: Select SNR=6/7 dB to verify CQI reporting related to 16QAM and -2/-1 dB to verify CQI reporting related to QPSK. |
| R4-2213967 | Qualcomm Incorporated | Proposal 1: Until there improvements are agreed to the TE on the achievable SNR for band n263, prioritize FR2-2 UE Demodulation requirements coverage for 120kHz and 100MHz (PDSCH, PDCCH, CQI, PMI, RI); Proposal 2: Further study in the future whether to introduce SDR requirements for FR2-2 with 120kHz, if the achievable SNR for band n263 increases; Proposal 3: RAN4 to consider a limited scope for FR2-2 UE Demodulation requirements with CBW=400MHz for 120 kHz and 480kHz, covering low MCS/SNR tests for PDSCH and PDCCH only; Proposal 4: Further study in the future whether to introduce UE Demodulation requirements for 960kHz or CBW> 400MHz in FR2-2, if the achievable SNR for band n263 increases; Proposal 5: For UE Demod requirements in FR2-2, focus only on Rank=1 and use Channel Model TDLD10-200 to reduce required SNR and increase the margin with maximum TE SNR; Proposal 6: RAN4 to prioritize PDSCH requirements defined at 70% of the peak throughput only, and to keep requirements for 30% of peak throughput FFS; Proposal 7: For UE Demod requirements in FR2-2, use the TDLD10 Delay Profile from Table 1 in this document; Proposal 8: For the purposes of simulation alignment, RAN4 should assume CPE correction only for UE Phase Noise compensation; Proposal 9: Support Option 4: Do not introduce new subsections for FR2-2 requirements. Requirement tables should specify whether they apply to FR2-1 or FR2-2, and the applicability of the requirements for operating bands should list the applicable tables separately; Proposal 10: For the purposes of FR2-2 PDSCH Rank 1 Requirements, RAN4 should consider not consider FD-OCC disabled; Proposal 11: For PBCH Demodulation in FR2-2, RAN4 should introduce only requirements with unknown SSB index; Proposal 12: For FR2-2, only introduce CQI requirements for 120kHz with CBW=100MHz; Proposal 13: Only define CQI requirements in the testable SNR regime not considering CQI for 64QAM reporting; Proposal 14: For FR2-2, introduce CQI requirements for 120kHz/100MHz reusing where applicable the tests setup and metrics from the correspondent FR2-1 requirements; |
| R4-2212105 | Nokia, Nokia Shanghai Bell | Proposal 8: For UE Demodulation requirements update number of PRBs of 960kHz/2000 MHz to follow RF agreements, i.e., 148 PRBs. Proposal 9: Do not introduce new subsections in the specification. Update legacy table headings to include FR1-1. Add new table with table heading FR2-2. Change applicability section (“Applicability of requirements for operating bands”) to only required testing of table with FR2-2 in FR2-2 bands. |
| R4-2213804 | Huawei, HiSilicon | Proposal 1: Use the same sub-clauses and separate tables for FR2-1 and FR2-2 UE requirements. Proposal 2: Use Table 2.1 for TS 38.101-4 spec structure. |
| R4-2212107 | Nokia, Nokia Shanghai Bell | Simulation Results |
| R4-2212109 | Nokia, Nokia Shanghai Bell | Simulation Results |
| R4-2212571 | Ericsson | Simulation Results |
| R4-2212573 | Ericsson | Simulation Results |
| R4-2212575 | Ericsson | Simulation Results |
| R4-2212577 | Ericsson | Simulation Results |
| R4-2213806 | Huawei,HiSilicon | Simulation Results |
| R4-2213808 | Huawei,HiSilicon | Simulation Results |
| R4-2213810 | Huawei,HiSilicon | Simulation Results |

## Open issues summary

### Sub-topic 1-1: General on FR2-2 Requirements

Issue 1-1-2: TDL Channel Model for FR2 UE Demodulation requirements

- Proposals

  - Option 2: TDLD10, TDLA10; (Huawei, Nokia, Qualcomm);

    - Option 2a: TDLA10 for QPSK, TDLD10 for the rest;

  - Option 3: TDLD10, TDLA10, TDLA20; (previous WF, Ericsson, Apple)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-1-2: TDL Channel Model for FR2 UE Demodulation requirements We believe that both TDLA10/20 and TDLD10 channel models are applicable for low/moderate MCS (4, 9, and 13). However, for high MCS (17 onward), we can define requirements using TDLD10. We opt for Option 3. |
| Apple | Issue 1-1-2: TDL Channel Model for FR2 UE Demodulation requirements We are fine with option 3. |
| Huawei | Prefer Option 2 |
| Nokia, Nokia Shanghai Bell | We support option 2 |
| Qualcomm | We can compromise to option 2 but TDLA10 should be used only for QPSK. TDLA 10 for QPSK only, TDLD10 for other MCS: |  |

Issue 1-1-3: Maximum Doppler Frequency for TDLA Channel Model

- Proposals

  - Option 1: 200 Hz (Apple, Huawei, Qualcomm);

  - Option 2: 650 Hz

  - Option 3: 650 Hz for low MCS, 200Hz for the rest (Ericsson);

    - Option 3a: 650Hz for MCS based on simulation results, 200Hz for the rest (Nokia);

    - Option 3b: 650Hz for QPSK only, 200Hz for the rest (Qualcomm);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-1-3: Maximum Doppler Frequency for TDLA Channel Model Based on our simulation results, a max Doppler frequency of 650 Hz is possible (considering the test SNR Table) for low MCS (Up to 13), while only 200 Hz is applicable to reach higher MCS order. We opt for Option 3. |  |
| Apple | Issue 1-1-3: Maximum Doppler Frequency for TDLA Channel Model We support option 1 due to low mobility scenarios for FR2-2. |
| Huawei | Option 1 |
| Nokia, Nokia Shanghai Bell | We prefer to have requirements for 650Hz for cases where possible. We propose Option 3A: The decision on which MCS should be defined with 650Hz shall be based on aligned simulation results. |
| Qualcomm | Similarly to issue 1-1-2, we can compromise to using 650Hz, but limited to QPSK cases only; |

Issue 1-1-4: Whether to define UE Demodulation requirements for 960kHz SCS

- Proposals

  - Option 1: Yes (Nokia);

    - Option 1a: Define the following separate sets of requirements and agree on the capability rule below:

      - Set 1) for legacy TE capabilities (480kHz with not too large CBW, and 960 KHz with non-full FDRA).

      - Set 2) for future 71GHz capable test equipment (480 and 960 kHz with larger CBW).

      - Define capability rules to allow that one of the 2 sets of the requirements are passed depending on what test equipment is available.

  - Option 2: No (Apple, Ericsson, Huawei, Qualcomm, Anritsu);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-1-4: Whether to define UE Demodulation requirements for 960kHz SCS We do not define requirement for SCS 960 KHz. We do believe that 480 KHz SCS is enough as an additional SCS for FR2-2. It is worth reminding that 960 KHz SCS will increase the max CBW by 25% (from 1600 MHz to 2000 MHz) while it will reduce the CP by 50% (with all related RF and RRM challenging issues). |  |
| Anritsu | Support option 2. As also commented to topic summary 324 issue 1-2-2, since there is a note in TS38.101-2 clause 5.3.2 that SCS 480 kHz and 960 kHz are optional in Rel-17, we suppose the demodulation requirement shall also have the same note even if we decide to define the requirements for 480 kHz and 960 kHz. |
| Apple | Issue 1-1-4: Whether to define UE Demodulation requirements for 960kHz SCS Option 2. Both 480 and 960 are optional for FR2-2. We suggest to limit defining demod requirements to 480KHz SCS. The larger CBW will not be testable anyway. There are several open issues for 480KHz SCS and we don’t see the need to add to demod work load given the limed time. |
| Huawei | Option 2 |
| Nokia, Nokia Shanghai Bell | It is our view, that requirements shall be defined for all main capabilities in FR2-2. We see having requirements for both 480kHz and 960kHz as being equally important, as there might be UEs which only support one of them. Without full requirement coverage, UEs supporting only 960kHz SCS would not be tested when corresponding requirements are missing as they do not support 480kHz SCS. Having 960kHz only chipsets make sense, as ICI can be avoided in this SCS without losing too much performance. In addition, it is our understanding, that in the last RF GTW session (on August 17th, 2022) on test capabilities, it was agreed that the work could continue for BS Demod requirement definition, as it is feasible to build TEs even with 2GHZ CBW and 20dB SNR; given enough investment. Hence we do not see any reason to limit the RAN4 UE Demod requirements based on capabilities for TEs that were not developed with 71GHz in mind. Ultimately, we could propose the following as a compromise, with the goal of satisfying the concerns of testability of larger CBW and enabling RAN4 to define a more comprehensive set of requirements: - Define one set of requirements, which match the currently available legacy TE capabilities (480kHz with not too large CBW, and 960 KHz with non-full FDRA). - Define one set of requirements, which match the feasibility of 71GHz capable test equipment (480 and 960 kHz with larger CBW). - Define capability rules to allow that one of the 2 sets of the requirements are passed depending on what test equipment is available. Hence, we support option 1, with the above mentioned possible limitations. |
| Qualcomm | Support option 2 to reduce workload, keep FFS 960kHz SCS requirements; |

Issue 1-1-5: Max CBW for UE Demodulation Requirements definition for SCS 120kHz

- Proposals

  - Option 1: 100 MHz (Ericsson, Apple, Huawei, Qualcomm);

    - Option 1a: Choose only minimum CBW for each SCS; FFS larger CBW/SCS combinations (Huawei);

    - Option 1b: Keep FFS 400 MHz (Ericsson);

  - Option 2: 400 MHz (Nokia, );

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-1-5: Max CBW for UE Demodulation Requirements definition for SCS 120kHz It is worth mentioning that the reference SNRs, in the Maximum DL testable SNR for different allocations Table, have been derived based on the following parameters, gathered in the following Table Parameter; Comment. REFSENS; Using REFSENS agreed for band n263 -72.0 dBm/400 MHz. Multi-band relaxation; TBD. TE amplifier 1dB compression; 23 dBm. Backoff from P1dB; -13 dB (RAN4 assumption) -17.7 dB (RAN5 assumption). Cable loss; 10.3 dB, assuming 1m length. Connector insertion loss; 0. FS path loss; 66.7 dB, assuming 0.725m range length. TE DL absolute power setting uncertainty; +/-6 dB. Probe antenna gain; 12 dBi. Beam peak search procedure error; 0.5 dB. However, the endorsed Proposal 4 in R5-221628 states that: “For 64QAM scenarios (both Demod and CSI), consider fading backoff margin of 11.08 dB (replacing the current working assumption of 17.71 dB) corresponding to the 1e-3 faded signal clipping probability”. This has led to the SNRs increase in FR2-1. We expect same behavior in FR2-2 by considering a fading backoff margin of 11.08 dB instead of 17.7 dB. And we trust that the companies and TE vendors will spend the needed efforts to review the reference SNRs whose may allow for higher MCS for 120 KHz/100 MHz, 480 KHz/400 MHz, and possibly 120 KHz/400 MHz and 480 KHz/800 MHz. Our view is Choose Option 1 FFS Option 2. |
| Apple | Issue 1-1-5: Max CBW for UE Demodulation Requirements definition for SCS 120kHz Option 1 given limited testable SNR at higher CBW. |
| Huawei | Option 1. For CBW, we suggest to choose minimum CBW for each SCS in Rel-17 considering limited testable SNR. We can further discuss higher CBW in the future Release if testable SNR improved. |
| Nokia, Nokia Shanghai Bell | We see testing also higher CBW as important for completeness of the UE Demod requirements coverage. Based on our proposal in Issue 1-1-4, we see it feasible to define requirements with higher CBW. Hence, we support Option 2 |
| Qualcomm | Ok with option 1 to keep only minimum BW, keep 400MHz FFS after testability concerns are addressed; |

Issue 1-1-6: Max CBW for UE Demodulation Requirements definition for SCS 480kHz

- Proposals

  - Option 1: 400 MHz (Qualcomm, Ericsson, Anritsu, Apple, Huawei)

    - Option 1a; FFS 800 MHz (Ericsson);

  - Option 2: 800 MHz (, Nokia);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-1-6: Max CBW for UE Demodulation Requirements definition for SCS 480kHz We keep same arguments as we provided for Issue 1-1-5. Our view is Choose Option 1 FFS Option 2. |  |
| Anritsu | Support Option 1. Need more time to confirm the testability of 800 MHz. |
| Apple | Issue 1-1-6: Max CBW for UE Demodulation Requirements definition for SCS 480kHz Option 1 given limited testable SNR at higher CBW. |
| Huawei | Issue 1-1-6: Max CBW for UE Demodulation Requirements definition for SCS 480kHz Option 1 |
| Nokia, Nokia Shanghai Bell | As Issue 1-1-7 includes discussion on defining requirements for 480kHz/1600MHz we cannot agree to the options in this issue until Issue 1-1-7 is agreed. In case it is agreed not to define requirements for 480kHz/1600MHz in Issue 1-1-7, we have the same comments as in Issue 1-1-7, thus we support option 2. |
| Qualcomm | Option 1; |

Issue 1-1-7: Whether to define UE Demodulation Requirements with CBW = 1600MHz (for SCS 480kHz, 960kHz)

- Proposals

  - Option 1: Yes (Nokia)

  - Option 2: No (Apple, Ericsson, Huawei, Qualcomm, Anritsu);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-1-7: Whether to define UE Demodulation Requirements with CBW = 1600MHz (for SCS 480kHz, 960kHz) We cannot reach any acceptable SNR levels when using 1600 MHz CBW. We choose Option 2 |  |
| Anritsu | Support option 2. Same as issue 1-1-6, we need more time to investigate the feasibility of 1600 MHz/ 2000 MHz CBW by TE. |
| Apple | Option 2 given very low testable SNR at higher CBW. |
| Huawei | Option 2 |
| Nokia, Nokia Shanghai Bell | Based on our proposal in Issue 1-1-4, we see it feasible to define requirements with higher SNR levels, thus we support option 1 |
| Qualcomm | Option 2, as of now we cannot test this CBW; |

Issue 1-1-8: Whether to update the number of PRBs for UE Demodulation requirements for 960kHz/2000MHz to follow RF agreements

- Proposals

  - Option 1: Yes, set to 148PRBs (Nokia);

- Recommended WF

  - Follow agreements in RF and update size to 148 PRBs;

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-1-8: Whether to update the number of PRBs for UE Demodulation requirements for 960kHz/2000MHz to follow RF agreements We do not define performance requirements for 960 KHz SCS. |  |
| Apple | If requirements are defined, we can update, but we don’t think its feasible to define requirements with 960KHz/2000MHz |
| Nokia, Nokia Shanghai Bell | Support suggested WF. |
| Qualcomm | Support WF; |

### Sub Topic 1-2: PDSCH Demodulation Requirements

Issue 1-2-1: Scenarios for FR2-2 PDSCH Requirements definition

- Proposals

  - Option 1:

    - FR2-2 Single Carrier and FR1+FR2-2 CA Scenarios (Huawei, Apple, Ericsson, Nokia, Qualcomm);

  - Option 2: TBA

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |  |
| Apple | Okay with Option 1. |
| Huawei | RF define CA combination for FR1+FR2-2 but not define DC combination for FR1+FR2-2. We support option 1 |
| Ericsson2 | We are fine with Option 1 |
| Nokia, Nokia Shanghai Bell | We support option 1 |
| Qualcomm | We are ok with option 1; |

Issue 1-2-2: How to address low maximum DL Testable SNR

- Reference table from R4-2213805 included for information:

- Table 2-1: Maximum DL testable SNR for different allocations

| SCS, kHz | Allocation size, PRB | Maximum DL testable SNR |
| --- | --- | --- |
| 120 | 264 | [-0.6] |
|  | 132 | [4.1] |
|  | 66 | [7.7] |
|  | 33 | [11] |
|  | 30 | [11.5] |
|  | 16 | [14.3] |
| 480 | 264 | < -20 (NOTE 1) |
|  | 132 | [-14.5] |
|  | 66 | [-0.6] |
|  | 33 | [4.1] |
|  | 16 | [7.9] |
|  | 7 | [11.5] |
| 960 | 165 | < -20 (NOTE 1) |
|  | 132 | < -20 (NOTE 1) |
|  | 66 | [-14.5] |
|  | 33 | [-0.6] |
|  | 16 | [4.2] |

- Proposals:

  - Option 1 (Huawei): Define requirements with full bandwidth allocation. I.e. (66RBs for both 120kHz/100MHz and 480kHz/400MHz). Define the test setup as following: The number of allocated RB should be the maximum number of RB related to maximum testable SNRBB declared by TE vendor and the tested RB should be put on centre of the tested carrier;

  - Option 2 (Huawei (preferred)): Define requirements with partial bandwidth allocation. Take following configurations as an example:

    - 120kHz/100MHz: MCS 4 and 13: Configure 66 RBs

    - 120kHz/100MHz: MCS 17:  Configure 33 RBs

    - 480kHz/400MHz: MCS4: Configure 66RBs

    - 480kHz/400MHz: MCS13 rank 1: Configure 16RBs

  - Option 3 (Huawei): Define requirements and test setup with full bandwidth allocation, not consider testability like PBCH requirements definition.

  - Option 4 (Qualcomm):

    - Prioritize requirements coverage for 120kHz and 100 MHz;

    - Consider a limited scope for CBW=400MHz for 120 and 480kHz, covering low MCS/SNR tests for PDSCH and PCCH;

- Option 6 (Ericsson)

  - 120 KHz/100 MHz (Rank 1): MCS 4, 13, 17, and 20: Configure 66 RBs

  - 480 KHz/400 MHz (Rank 1): MCS 4 and 9: Configure 66 RBs

  - 480 KHz/400 MHz (Rank 1): MCS 13: Configure 33 RBs

  - 480 KHz/400 MHz (Rank 1): MCS 17: Configure 16 RBs

- Option 7 (Apple, Qualcomm)

  - Option 7b: Support list below but align PRB number with PRB bundling size and allocate the center portion of the CBW; (Qualcomm)

- For 120KHz/100MHz

  - MCS 4,13: 66 PRBs

  - MCS17: 30PRBs

- For 480KHz/400MHz

  - MCS 4: 66 PRBs

  - MCS 13: 16 PRBs

No requirements with 64QAM

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-2-2: How to address low maximum DL Testable SNR We choose Option 5 to allow for higher MCS. |  |
| Apple | For 120KHz/100MHz MCS 4,13: 66 PRBs MCS17: 30PRBs For 480KHz/400MHz MCS 4: 66 PRBs MCS 13: 16 PRBs No requirements with 64QAM MCS22 is not feasible as degradation with PN is large. |
| Huawei | We prefer option 2. For option 4, the test coverage is too limited which seems not feasible. We slightly prefer option 2. For 120KHz/100MHz MCS 4,13: 66 PRBs MCS17: 33 PRBs For 480KHz/400MHz MCS 4: 66 PRBs MCS 13: 16 PRBs |
| Ericsson2 | We revise Option 5 as such - 120 KHz/100 MHz (Rank 1): MCS 4, 13, 17, and 20: Configure 66 RBs - 480 KHz/400 MHz (Rank 1): MCS 4 and 9: Configure 66 RBs - 480 KHz/400 MHz (Rank 1): MCS 13: Configure 33 RBs - 480 KHz/400 MHz (Rank 1): MCS 17: Configure 16 RBs |
| Qualcomm | Option 5 seems too optimistic given the current testing constraints; We can be ok with option 2, but for the test cases with partial allocation the number of RB should be aligned with the PRB bundling size and the RB location should be in the center; |

Issue 1-2-3: Whether RAN4 needs to specify HARQ-ACK Codebook and HARQ Bundling for multi-PDSCH scheduling

- Proposals

  - Option 1: No, leave it up to R5 (Apple, Ericsson);

    - Option 1a: Leave specification up to R5, but R4 to ensure no issue in the configuration of the with respect to HARQ bundling (Nokia, Apple, Qualcomm);

  - Option 2: HARQ-ACK codebook type 1, HARQ Bundling 4 for 480kHz (Huawei);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-2-3: Whether RAN4 needs to specify HARQ-ACK Codebook and HARQ Bundling for multi-PDSCH scheduling We leave this issue to RAN5. We choose Option 1. |  |
| Apple | We are fine with option 1a. |
| Huawei | Option2 |
| Nokia, Nokia Shanghai Bell | We support option 1a (corrected the option to 1a from 1b above, as we expected this to be the intention) |
| Qualcomm | Fine with option 1a; |

Issue 1-2-4: Whether RAN4 should introduce requirements for 30% of peak throughput

- Proposals

  - Option 1: No (Apple, Huawei, Qualcomm);

  - Option 2: Yes (Nokia, Ericsson);

  - Option 3: Keep FFS (Qualcomm);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-2-4: Whether RAN4 should introduce requirements for 30% of peak throughput We already performed some simulation considering 30% of peak throughput as a metric for PDSCH. We do believe that it is possible to define requirement for 30% of peak throughput. We choose Option 2. |  |
| Apple | Option 1. We prefer to de-prioritize this. Focus on other test case parameters given the limited time for WI completion. |
| Huawei | Option1 |
| Nokia, Nokia Shanghai Bell | We share similar view as Ericsson. Support Option 2. |
| Qualcomm | We think that 30% should be de-prioritized, and we don’t see these requirements as necessary right now. Support option 1 and 3; |

Issue 1-2-5: UE Phase Noise Model for Simulation Alignment

- Proposals

  - Option 1: Both PN Models in TR 38.808 (Ericsson);

  - Option 2: TBA

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-2-5: UE Phase Noise Model for Simulation Alignment Since RAN4 was not able to agree on a single representative PN model and for the sake of fairness, we believe that it is better to provide results using both PN model in TR38.808. We choose Option 1. |  |
| Apple | We will not enable PN model for simulation results collection. Its only for the initial phase to decide on suitable parameters. We don’t see the need to further discuss this and agree on a model. Could proponents please clarify. |
| Huawei | Companies can bring the simulation results without PN modelled and add the margin including PN impact as impairment results. |
| Ericsson2 | In order to have a better view and for the sake of fairness (results’ alignment), it is better to consider both PN sets. Based on our simulation results, we have been able to achieve MCS 20 for 120 KHz/100 MHz (66 PRBs) under TDLD10-200 channel model while using TR 38.808 Set 1. In our simulations, we considered the performance degradation due to the PN while applying CPE compensation only and it turned out that MCS 20 has been the highest MCS that kept this degradation below 1dB. |
| Qualcomm | For the purposes of simulation alignment, we are ok with considering both PN models, and not selecting only one of them as reference (if this was the proponent’s intention); |

Issue 1-2-6: UE Phase Noise compensation Model for Simulation Alignment

- Proposals :

  - Option 1: CPE compensation only (Apple, Qualcomm, Huawei, Ericsson (Preferred), Nokia);

    - Option 1b: Keep FFS ICI compensation scheme for higher MCS (Nokia, Ericsson);

  - Option 2: CPE+ICI compensation

  - Option 3: CPE or CPE+ICI decision depends on the test case (Nokia, Ericsson (if large CBW applicable))

    - Option 3a (Nokia): Adopt CPE+ICI at least in the following test cases:

      - 120 kHz SCS + 100 MHz + MCS>17

      - 120 kHz SCS + 400 MHz + MCS>13

      - 480 kHz SCS + 800 MHz + MCS>13

      - 480 kHz SCS + 1600 MHz + MCS>13

      - CPE only otherwise

    - Option 3b (Ericsson): CPE only if (Num PRB <= 66), CPE+ICI otherwise

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-2-6: UE Phase Noise compensation Model for Simulation Alignment Based on our (extensive) simulation results, the acceptable SNR level are only reachable for low PRB allocation (Up to 66). Therefore, CPE compensation only is enough. However, and since we kept FFS the intermediate and max CBW, such as 120 KHz/400 MHz and 480 KHz/800 MHz, we can choose Option 1 FFS Option 3b |
| Apple | We support option 1 as that would be the baseline and we will not have large CBW and MCS. We would define requirements based on simulations without PN enabled in my understanding. Please clarify. |
| Huawei | Option 1 |
| Ericsson2 | Please refer to [334], AI 11.5, where the discussion is being held on the max DL testable SNRs for FR2-2. So far, we referred to SNR levels that could increase, thus allowing for higher MCS. For that reason, we would like to keep Option 3b as FFS |
| Nokia, Nokia Shanghai Bell | We can compromise to option 1 as baseline assumption but keep option 3b FFS. |
| Qualcomm | We do not see a technical reason to discriminate the reference UE implementation for requirements based on allocated RB size, we support Option 1 as baseline; |  |

### Sub Topic 1-3: PDSCH Coverage of the requirements

Issue 1-3-1: Modulation Order to be used for the definition of PDSCH requirements

- Proposals:

  - Option 1: QPSK, 16 QAM; (Apple)

  - Option 2: QPSK, 16QAM, 64QAM (Nokia, Ericsson, Huawei);

    - Nokia:

      - MCS 22 (120kHz);

      - MCS 20 (480kHz, 960kHz);

    - Huawei:

      - MCS 17 (120kHz);

      - MCS 13 (480kHz);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-3-1: Modulation Order to be used for the definition of PDSCH requirements Based on our simulation results, we opt for Option 2. |  |
| Apple | This would need to be discussed per SCS/CBW combo and related to Issue 1-2-2. Taking into account testable SNR for different BW, we propose the same as in Issue 1-2-2: For 120KHz/100MHz MCS 4,13: 66 PRBs MCS17: 30PRBs For 480KHz/400MHz MCS 4: 66 PRBs MCS 13: 16 PRBs No requirements with 64QAM |
| Huawei | For 120kHz SCS, MCS 4,13,17; For 480kHz SCS: MCS4 and 13 |
| Nokia, Nokia Shanghai Bell | Support Option 2 |
| Qualcomm | We are ok with Apple’s proposal above; |

Issue 1-3-2: MCS choice for PDSCH requirements for 64QAM (if requirements with 64QAM are agreed)

- Proposals :

  - Option 1: RAN 4 to select max MCS to satisfy that performance degradation due to phase noise is less than 1dB with the chosen PN compensation technique (Huawei, Apple, Qualcomm, Ericsson)

    - Shared tentative MCS for reference:

      - I.e. MCS 17; (Huawei)

      - MCS 20 (Ericsson)

  - Option 2: TBA

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-3-2: MCS choice for PDSCH requirements for 64QAM (if requirements with 64QAM are agreed) Based on our simulation results for 120 KHz/100 MHz, we can achieve MCS 20, using Rank 1 and under TDLD10-200 satisfying the performance degradation due to PN lean than 1 dB. We propose: Option 2: Select highest MCS under agreed propagation environment where the performance degradation due PN is kept below 1dB while using the agreed compensation technique. (Ericsson, MCS 20 for 120 KHz/100 MHz (66 RBs), with CPE compensation only) |  |
| Apple | Option 1, for 120KHz/100MHz. Considering testable SNR and lowest degradation with PN enabled. |
| Huawei | Based on our simulation results, MCS17 can meet option 1 |
| Qualcomm | We are ok with following Option 1. MCS can be decided based on simulation results; |

Issue 1-3-3: Rank to be used for the definition of requirements

- Proposals :

  - Option 1: Only Rank 1 (Apple, Qualcomm, Ericsson, Huawei);

  - Option 2: Both Rank 1 and Rank 2 (Nokia,);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-3-3: Rank to be used for the definition of requirements Based on the achievable SNR levels, we can only talk about Rank 1. We choose Option 1. |  |
| Apple | Option 1. Given testable SNR limit. |
| Huawei | Option 1. |
| Nokia, Nokia Shanghai Bell | Based on our proposal in Issue 1-1-4, we see it feasible to define requirements with higher CBW, hence we see it feasible to define requirements with higher SNR levels. Support option 2. |
| Qualcomm | Support option 1; |

Issue 1-3-4: Whether to define Rank 1 requirements assuming that the feature ‘FD-OCC Disabled’

- Proposals :

  - Option 1: Yes ()

  - Option 2: No (Huawei, Qualcomm, Apple, Nokia);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |  |
| Apple | Option 2. Could proponents clarify the necessity. Is there significant performance delta with FD-OCC disabled? |
| Huawei | Option 2. Based on our simulation results in last meeting, only 960kHz with large delay spread can see performance gain. |
| Nokia, Nokia Shanghai Bell | Based on Huawei’s comments, we can compromise to option 2. |
| Qualcomm | Option 2; |

Issue 1-3-5: PDSCH Requirements Coverage for SCS 120kHz, CBW = 100MHz;

- Proposals :

  - Option 1: Rank1, QPSK and 16 QAM; ()

  - Option 2: Rank 1, Up to 64QAM (Ericsson, Apple, Huawei, Nokia, Qualcomm (if reduced #RBs));

    - Tentative MCS for reference: MCS 4, 13, 17 (Apple, Huawei);

  - Option 3: Include Rank 2 if agreed (Nokia);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-3-5: PDSCH Requirements Coverage for SCS 120kHz, CBW = 100MHz; Based on our simulation results, we believe that 64QAM is achievable (MCS 20). We choose Option 2. |  |
| Apple | Related to Issue 1-2-2, 1-3-1 For 120KHz/100MHz MCS 4,13: 66 PRBs MCS17: 30PRBs |
| Huawei | Option 2. MCS 4,13,17 |
| Nokia, Nokia Shanghai Bell | We support option 2, however requirements coverage for Rank 2 shall be considered in case it is agreed to define requirements with Rank 2. |
| Qualcomm | Assuming the reduced RB allocation proposed by Apple above, we are ok to consider 64QAM if SNR based on simulation results is testable; |

Issue 1-3-6: PDSCH Requirements Coverage for SCS 480kHz, CBW = 400MHz;

- Proposals :

  - Option 1: Rank1, QPSK; ()

  - Option 2: Rank 1, QPSK, 16QAM (Ericsson, Apple, Huawei, Nokia, Qualcomm (if reduced #RBs));

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-3-6: PDSCH Requirements Coverage for SCS 480kHz, CBW = 400MHz; We believe that we can achieve SNR levels for 16QAM related MCS when using partial allocation, such as 33 or 16 RBs. Choose Option 2, Rank 1, QPSK and 16QAM. |  |
| Apple | Related to Issue 1-2-2, 1-3-1 For 480KHz/400MHz MCS 4: 66 PRBs MCS 13: 16 PRBs No requirements with 64QAM |
| Huawei | Option 2 |
| Nokia, Nokia Shanghai Bell | Based on our proposal in Issue 1-1-4, we see it feasible to define requirements with higher CBW, hence higher modulation should be considered. For requirement set matching the currently available legacy TE capabilities (see our comments in Issue 1-1-4) we support option 2. |
| Qualcomm | Assuming the reduced RB allocation proposed by Apple above, we are ok to consider 64QAM if SNR based on simulation results is testable; |

Issue 1-3-7: Number of HARQ processes to be used for SCS=960kHz requirement definition if requirements are agreed;

- Proposals :

  - Option 1: Separate requirements for 16 and 32 HARQ processes (Nokia)

  - Option 2: Only 16 HARQ Processes (Mandatory);

  - Option 3: Only 32 HARQ Processes (Optional Capability);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-3-7: Number of HARQ processes to be used for SCS=960kHz requirement definition if requirements are agreed; We do not define requirements for 960 KHz SCS. |  |
| Apple | Do not define requirements with 960Khz. |
| Nokia, Nokia Shanghai Bell | As we support defining requirements for 960kHz with our proposal in Issue 1-1-4, we support option 1. |

Issue 1-3-8: Scheduling pattern for CA Scenario

- Proposals:

  - Option 1: Configure following parameters for FR1 Pcell (Huawei, as baseline: Apple, Ericsson, Qualcomm);

    - TDD

    - 30kHz SCS with 7D1S2U

    - All HARQ feedback should be transmitted on UL slot of FR1 carrier

  - Option 2: TBA

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |  |
| Apple | Option 1 as baseline assumption. Can revisit if we see any issues. |
| Huawei | Option 1 |
| Ericsson2 | We agree with Apple, and can consider Option 1 as a baseline assumption |
| Qualcomm | Ok with Option 1 as baseline; |

### Sub Topic 1-4: PDSCH Test cases and simulation assumptions

Issue 1-4-1: List of test cases to be used for requirement definition;

- Proposals:

  - Option 1: R4-2213805 (Huawei) Table 2-2

    - Option 1a: Use Table 2-2, add #RBs and de-prioritize Rank 2 tests (Apple);

    - Option 1c: add 480/1600, 960/400 & 960/1600 with non-full FDRA, i.e., only allocating [16] PRB in the center of the CBW for the currently available legacy TE equipment. Add performance requirements for the same SCS/CBW combinations with full bandwidth/FDRA to cover new truly 71GHz capable test equipment.

- Table 2-2: Proposed cases

| CBW(MHz) / SCS (kHz) | MCS/Rank | Channel Model | Antenna Configuration and Correlation Matrix | % of peak thpt |
| --- | --- | --- | --- | --- |
| 100/120 | MCS4/Rank1 | TDLA10-200 | 2x2 ULA Low | 70 |
| 100/120 | MCS13/Rank1 | TDLA10-200 | 2x2 ULA Low | 70 |
| 100/120 | MCS13/Rank2 | TDLD10-200 | 2x2 ULA Low | 70 |
| 100/120 | MCS17/Rank1 | TDLD10-200 | 2x2 ULA Low | 70 |
| 400/480 | MCS4/Rank1 | TDLA10-200 | 2x2 ULA Low | 70 |
| 400/480 | MCS13/Rank1 | TDLA10-200 | 2x2 ULA Low | 70 |
| 400/480 | MCS13/Rank2 | TDLD10-200 | 2x2 ULA Low | 70 |
| 400/480 | MCS17/Rank1 | TDLD10-200 | 2x2 ULA Low | 70 |

Option 1a: (Huawei)

    - Option 1ab: Change MCS13 tests to TDLD10 (Qualcomm);

| CBW(MHz) / SCS (kHz) | MCS/Rank | Channel Model | PRB allocation | Antenna Configuration and Correlation Matrix | % of peak thpt |
| --- | --- | --- | --- | --- | --- |
| 100/120 | MCS4/Rank1 | TDLA10-200 | 66 | 2x2 ULA Low | 70 |
| 100/120 | MCS13/Rank1 | TDLA10-200 | 66 | 2x2 ULA Low | 70 |
| 100/120 | MCS17/Rank1 | TDLD10-200 | 30 or 33 | 2x2 ULA Low | 70 |
| 400/480 | MCS4/Rank1 | TDLA10-200 | 66 | 2x2 ULA Low | 70 |
| 400/480 | MCS13/Rank1 | TDLA10-200 | 16 | 2x2 ULA Low | 70 |

  - Option 2: R4-2212570 (Ericsson)

| Test number | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compen- Sation | % of peak Throughput | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100MHz / 120kHz (66) | 4 Rank 1 | FR2.120-1 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | CPE | 70 | New |
| 1-2 | 100MHz / 120kHz (66) | 13 Rank 1 | FR2.120-1 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | CPE | 70 | New |
| 1-3 | 100MHz / 120kHz (66) | 17 Rank 1 | FR2.120-1 | TDLA10-200 TDLA20-200 TDLD10-200 | 2x2 ULA Low | CPE | 70 | New |
| 1-4 | 100 MHz/ 120 KHz (66) | 20 Rank 1 | FR2.120-1 | TDLD10-200 | 2x2 ULA Low | CPE | 70 | New |
| 2-1 | - / 120kHz (33) | 22 Rank 1 | FR2.120-1 | TDLD10-200 | 2x2 ULA Low | CPE | 70 | New |
| 2-2 | - / 120kHz (16) | 22 Rank 1 | FR2.120-1 | TDLD10-200 | 2x2 ULA Low | CPE | 70 | New |
| 3-1 | 400MHz / 480kHz (66) | 4 Rank 1 | FR2.120-1 | TDLA10-200 TDLA20-200 TDLD10-200 | 2x2 ULA Low | CPE | 70 | New |
| 3-2 | 400MHz / 480kHz (66) | 9 Rank 1 | FR2.120-1 | TDLA10-200 TDLA20-200 TDLD10-200 | 2x2 ULA Low | CPE | 70 | New |
| 4-1 | - / 480kHz (33) | 13 Rank 1 | FR2.120-1 | TDLD10-200 | 2x2 ULA Low | CPE | 70 | New |
| 4-2 | - / 480 KHz (16) | 17 Rank 1 | FR2.120-1 | TDLD10-200 | 2x2 ULA Low | CPE | 70 | New |
| 5-1 | 100MHz / 120kHz (66) | 13 Rank 1 | FR2.120-1 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | CPE | 30 | New |
| 5-2 | 400MHz / 480kHz (66) | 13 Rank 1 | FR2.120-1 | TDLA10-200 TDLA20-200 TDLD10-200 | 2x2 ULA Low | CPE | 30 | New |

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | We choose Option 2, R4-2212570. |  |
| Apple | Support option 1. Option 2 has many test cases, all channel model combinations. It would be impractical to consider all. We prefer Option 1, and would also need to add num PRBs to ensure testable SNR. Rank 2 test cases can be de-prioritized. |
| Huawei | We propose the following cases: CBW(MHz) / SCS (kHz); MCS/Rank; Channel Model; PRB allocation; Antenna Configuration and Correlation Matrix; % of peak thpt. 100/120; MCS4/Rank1; TDLA10-200; 66; 2x2 ULA Low; 70. 100/120; MCS13/Rank1; TDLA10-200; 66; 2x2 ULA Low; 70. 100/120; MCS17/Rank1; TDLD10-200; 33; 2x2 ULA Low; 70. 400/480; MCS4/Rank1; TDLA10-200; 66; 2x2 ULA Low; 70. 400/480; MCS13/Rank1; TDLA10-200; 16; 2x2 ULA Low; 70. |
| Nokia, Nokia Shanghai Bell | Would be helpful for the proponents to the options to explain why their options are superior to the other options. It is difficult for us to discern why one should be chosen over the other. We also note that 960 kHz is not included in any of the options and should be added according to the outcome of the SCS discussion on the general agenda item. We see option 1 as a good starting point. With reference to our proposal in Issue 1-1-4, we should add 480/1600, 960/400 & 960/1600 with non-full FDRA, i.e., only allocating [16] PRB in the center of the CBW for the currently available legacy TE equipment. And additionally, we should add performance requirements for the same SCS/CBW combinations with full bandwidth/FDRA to cover new truly 71GHz capable test equipment. In addition Doppler frequency of 650Hz should be considered pending outcome of Issue 1-1-3. |
| Qualcomm | We can support Huawei’s proposal, but change MCS13 tests to TDLD to increase margin over testable SNR 100/120; MCS13/Rank1; TDLA10-200 TDLD10-200; 66; 2x2 ULA Low; 70. 400/480; MCS13/Rank1; TDLA10-200 TDLD10-200; 16; 2x2 ULA Low; 70. |

Issue 1-4-2: Channel Delay Profile for simulation alignment: TLD10;

- Proposals:

  - Option 1: R4-2213967 (Qualcomm) Table 1

Moderator’s note: This topic of channel delay profile is being discussed in [324][NR_exto71GHz_Demod_Part1], companies are invited not to comment here and refer to that thread for related proposals and comments;

Issue 1-4-3: Test Setup for 70% peak throughput PDSCH Requirements ;

- Proposals:

  - Option 1: Test Setup table (Ericsson, as baseline: Apple, Nokia)

    - Proposed modifications:

      - Velocity 3km/h; (Apple);

      - Add 960kHz SCS if agreed (Nokia);

      - PN Model and compensation? (Apple, Nokia)

      - MCS to follow previous issues outcomes (Qualcomm);

      - Remove TDLA20 (Qualcomm, Nokia);

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (10 ns and 20 ns delay spread) and TDLD (10 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | TR38.808 PN model Set 1 and Set 2 |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| CSI-RS / TRS | CSI-RS/TRS is assumed to be off (for RS overhead) |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 22 (64QAM). Note: It is assumed that NohPRB = 0 for MCS calculations. |

  - Option 2: TBA

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-4-3: Test Setup for 70% peak throughput PDSCH Requirements ; The presented Table summarizes the agreed general parameters that have been discussed so far. Choose Option 1 |  |
| Apple | Fine with Option 1 as starting point. Velocity 3km/h only Do we need to enable PN and compensation for simulation alignment? |
| Nokia, Nokia Shanghai Bell | Ok with option 1 as starting point, however SCS=960kHz should be included if agreed. Suggest to not include TDLA20 In case PN is enabled, consider based on existing simulation results if we can select only one set. Consider adding Rank 2 depending on discussion outcome. |
| Qualcomm | Remove TDLA20; MCS should follow the outcome of the simulation assumptions discussion in the previous issues; |

Issue 1-4-4: Test Setup for 30% peak throughput PDSCH Requirements (if 30% peak throughput requirement is agreed)

- Proposals:

  - Option 1: Test Setup table (Ericsson, Huawei, as baseline: Nokia);

    - Option 1b: Align to Table in previous option, add 32 HARQ processes if 960kHz is included (Nokia)

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (10 ns and 20 ns delay spread) and TDLD (10 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | TR38.808 PN model Set 1 and Set 2 |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| CSI-RS / TRS | CSI-RS/TRS is assumed to be off (for RS overhead) |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 22 (64QAM). Note: It is assumed that NohPRB = 0 for MCS calculations. |

  - Option 2: TBA

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 1-4-4: Test Setup for 30% peak throughput PDSCH Requirements (if 30% peak throughput requirement is agreed) Choose Option 1. |  |
| Apple | We prefer to de-prioritize this. |
| Huawei | Option 1 |
| Nokia, Nokia Bell Labs | Options 1 as starting point with same additional comments as Issue 1-4-3 As this is testing HARQ Soft Combining, 32HARQ configuration should be added if SCS=960kHz is agreed. |
| Qualcomm | Focus on 70% throughput; |

## Summary for 1st round

### Open issues

Outcome of the discussions from the 1st round is summarized below.

|  | Status summary |
| --- | --- |
| Sub-topic 1-1 | Issue 1-1-2: TDL Channel Model for FR2 UE Demodulation requirements - Proposals - Option 2: TDLD10, TDLA10; (Huawei, Nokia, Qualcomm); - Option 2a: TDLA10 for QPSK, TDLD10 for the rest; - Option 3: TDLD10, TDLA10, TDLA20; (previous WF, Ericsson, Apple) - Recommended WF - Split this item in 2 separate issues in the next round: 1) whether to include TDLA20 and 2) highest modulation order for TDLA10; Issue 1-1-3: Maximum Doppler Frequency for TDLA Channel Model - Proposals - Option 1: 200 Hz (Apple, Huawei, Qualcomm); - Option 2: 650 Hz - Option 3: 650 Hz for low MCS, 200Hz for the rest (Ericsson); - Option 3a: 650Hz for MCS based on simulation results, 200Hz for the rest (Nokia); - Option 3b: 650Hz for QPSK only, 200Hz for the rest (Qualcomm); - Recommended WF - No company has expressed an opinion to exclude 200Hz, so it is agreed to be included; - Discuss further whether to consider 650Hz and for which MCS; Issue 1-1-4: Whether to define UE Demodulation requirements for 960kHz SCS - Proposals - Option 1: Yes (Nokia); - Option 1a: Define the following separate sets of requirements and agree on the capability rule below: - Set 1) for legacy TE capabilities (480kHz with not too large CBW, and 960 KHz with non-full FDRA). - Set 2) for future 71GHz capable test equipment (480 and 960 kHz with larger CBW). - Define capability rules to allow that one of the 2 sets of the requirements are passed depending on what test equipment is available. - Option 2: No (Apple, Ericsson, Huawei, Qualcomm, Anritsu); - Recommended WF - Only 1 company has expressed interest to introduce requirements for 960kHz. We recommend to discuss Nokia’s proposal in the second round, if no interest is received can Nokia agree to Option 2? Issue 1-1-5: Max CBW for UE Demodulation Requirements definition for SCS 120kHz - Proposals - Option 1: 100 MHz (Ericsson, Apple, Huawei, Qualcomm); - Option 1a: Choose only minimum CBW for each SCS; FFS larger CBW/SCS combinations (Huawei); - Option 1b: Keep FFS 400 MHz (Ericsson); - Option 2: 400 MHz (Nokia, ); - Recommended WF - Based on first round input, we recommend as WF to use 100MHz for 120kHz and keep 400MHz FFS. Can Nokia agree? Issue 1-1-6: Max CBW for UE Demodulation Requirements definition for SCS 480kHz - Proposals - Option 1: 400 MHz (Qualcomm, Ericsson, Anritsu, Apple, Huawei) - Option 1a; FFS 800 MHz (Ericsson); - Option 2: 800 MHz (, Nokia); - Recommended WF - Based on first round input, we recommend as WF to use 400MHz for 480kHz and keep 800MHz FFS. Can Nokia agree? Issue 1-1-7: Whether to define UE Demodulation Requirements with CBW = 1600MHz (for SCS 480kHz, 960kHz) - Proposals - Option 1: Yes (Nokia) - Option 2: No (Apple, Ericsson, Huawei, Qualcomm, Anritsu); - Recommended WF - Only 1 company has expressed interest to introduce requirements for 1600MHz. Can Nokia agree to Option 2? Issue 1-1-8: Whether to update the number of PRBs for UE Demodulation requirements for 960kHz/2000MHz to follow RF agreements Agreement on WF: In case requirements are introduced, follow agreements in RF and update size to 148 PRBs; |
| Sub Topic 1-2 | Issue 1-2-1: Scenarios for FR2-2 PDSCH Requirements definition Agreement on WF: Introduce requirements for FR2-2 Single Carrier and FR1+FR2-2 CA Scenarios; Issue 1-2-2: How to address low maximum DL Testable SNR - Proposals: - Option 1 (Huawei): Define requirements with full bandwidth allocation. I.e. (66RBs for both 120kHz/100MHz and 480kHz/400MHz). Define the test setup as following: The number of allocated RB should be the maximum number of RB related to maximum testable SNRBB declared by TE vendor and the tested RB should be put on centre of the tested carrier; - Option 2 (Huawei (preferred)): Define requirements with partial bandwidth allocation. Take following configurations as an example: - 120kHz/100MHz: MCS 4 and 13: Configure 66 RBs - 120kHz/100MHz: MCS 17: Configure 33 RBs - 480kHz/400MHz: MCS4: Configure 66RBs - 480kHz/400MHz: MCS13 rank 1: Configure 16RBs - Option 3 (Huawei): Define requirements and test setup with full bandwidth allocation, not consider testability like PBCH requirements definition. - Option 4 (Qualcomm): - Prioritize requirements coverage for 120kHz and 100 MHz; - Consider a limited scope for CBW=400MHz for 120 and 480kHz, covering low MCS/SNR tests for PDSCH and PCCH; - Option 6 (Ericsson) - 120 KHz/100 MHz (Rank 1): MCS 4, 13, 17, and 20: Configure 66 RBs - 480 KHz/400 MHz (Rank 1): MCS 4 and 9: Configure 66 RBs - 480 KHz/400 MHz (Rank 1): MCS 13: Configure 33 RBs - 480 KHz/400 MHz (Rank 1): MCS 17: Configure 16 RBs - Option 7 (Apple, Qualcomm) - Option 7b: Support list below but align PRB number with PRB bundling size and allocate the center portion of the CBW; (Qualcomm) - For 120KHz/100MHz - MCS 4,13: 66 PRBs - MCS17: 30PRBs - For 480KHz/400MHz - MCS 4: 66 PRBs - MCS 13: 16 PRBs - No requirements with 64QAM - Recommended WF Separately discuss the following issues in the second round: - Whether to introduce Demod requirements with partial bandwidth allocation; - RB sizes table for MCS 4, 13, 17 for SCS 120/480kHz; - Partial bandwidth location in the CBW; - Whether to align PRB number with PRB bundling size; Issue 1-2-3: Whether RAN4 needs to specify HARQ-ACK Codebook and HARQ Bundling for multi-PDSCH scheduling - Proposals - Option 1: No, leave it up to R5 (Apple, Ericsson); - Option 1a: Leave specification up to R5, but R4 to ensure no issue in the configuration of the with respect to HARQ bundling (Nokia, Apple, Qualcomm); - Option 2: HARQ-ACK codebook type 1, HARQ Bundling 4 for 480kHz (Huawei); - Recommended WF - Only 1 company wants to specify codebook type and bundling in RAN4. Can Huawei agree to define this in RAN5, while RAN4 checks that no conflict is introduced in the test setup? Issue 1-2-4: Whether RAN4 should introduce requirements for 30% of peak throughput - Proposals - Option 1: No (Apple, Huawei, Qualcomm); - Option 2: Yes (Nokia, Ericsson); - Option 3: Keep FFS (Qualcomm); - Recommended WF - Keep discussing in the second round; Issue 1-2-5: UE Phase Noise Model for Simulation Alignment - Proposals - Option 1: Both PN Models in TR 38.808 (Ericsson); - Recommended WF - Companies clarified that PN modelling can be used to run simulations for the choice of simulation parameters, while simulation results collection for alignment is not expected to use PN models (and expected PN impact is added as a margin to the impairment results). Unless there are diverging views, do not discuss this issue further in the second round; Issue 1-2-6: UE Phase Noise compensation Model for Simulation Alignment - Proposals : - Option 1: CPE compensation only (Apple, Qualcomm, Huawei, Ericsson (Preferred), Nokia); - Option 1b: Keep FFS ICI compensation scheme for higher MCS (Nokia); - Option 2: CPE+ICI compensation - Option 3: CPE or CPE+ICI decision depends on the test case (Nokia, Ericsson (if large CBW applicable)) - Option 3a (Nokia): Adopt CPE+ICI at least in the following test cases: - 120 kHz SCS + 100 MHz + MCS>17 - 120 kHz SCS + 400 MHz + MCS>13 - 480 kHz SCS + 800 MHz + MCS>13 - 480 kHz SCS + 1600 MHz + MCS>13 - CPE only otherwise - Option 3b (Ericsson): CPE only if (Num PRB <= 66), CPE+ICI otherwise - Recommended WF - CPE compensation only is agreed for low MCS. Discuss in the second round whether to keep FFS CPE+ICI compensation for higher MCS/CBW allocations |
| Sub Topic 1-3 | Issue 1-3-1: Modulation Order to be used for the definition of PDSCH requirements - Proposals: - Option 1: QPSK, 16 QAM; (Apple) - Option 2: QPSK, 16QAM, 64QAM (Nokia, Ericsson, Huawei); - Nokia: - MCS 22 (120kHz); - MCS 20 (480kHz, 960kHz); - Huawei: - MCS 17 (120kHz); - MCS 13 (480kHz); - Recommended WF - Discuss this per SCS/CBW combination separately; Issue 1-3-2: MCS choice for PDSCH requirements for 64QAM (if requirements with 64QAM are agreed) - Agreement: RAN 4 to select the max MCS that satisfies the following criteria: performance degradation due to phase noise is less than 1dB with the agreed PN compensation technique; - Recommended WF - In the 2nd round, discuss further the following tentative MCS shared for reference: - MCS 17; - MCS 20; Issue 1-3-3: Rank to be used for the definition of requirements - Proposals : - Option 1: Only Rank 1 (Apple, Qualcomm, Ericsson, Huawei); - Option 2: Both Rank 1 and Rank 2 (Nokia,); - Recommended WF - Only one company wants to introduce Rank 2 requirements. Can Nokia agree to Option 1? Issue 1-3-4: Whether to define Rank 1 requirements assuming that the feature ‘FD-OCC Disabled’ - Agreement: Define Rank 1 requirement assuming that RRC is signaling FD-OCC ON for the DMRS configuration chosen; Issue 1-3-5: PDSCH Requirements Coverage for SCS 120kHz, CBW = 100MHz; - Proposals : - Option 2: Rank 1, Up to 64QAM (Ericsson, Apple, Huawei, Nokia, Qualcomm (if reduced #RBs)); - Tentative MCS for reference: MCS 4, 13, 17 (Apple, Huawei); - Option 3: Include Rank 2 if agreed (Nokia); - Recommended WF - Tentative agreements: Include up to 64QAM for Rank 1. Do not discuss further unless Rank 2 requirements are agreed; Issue 1-3-6: PDSCH Requirements Coverage for SCS 480kHz, CBW = 400MHz; - Agreement: Include up to 16QAM for Rank 1; Discuss FD allocation size in the related issue; Issue 1-3-7: Number of HARQ processes to be used for SCS=960kHz requirement definition if requirements are agreed; - Proposals : - Option 1: Separate requirements for 16 and 32 HARQ processes (Nokia) - Option 2: Only 16 HARQ Processes (Mandatory); - Option 3: Only 32 HARQ Processes (Optional Capability); - Recommended WF - Do not discuss further unless 960kHz SCS requirements are agreed; Issue 1-3-8: Scheduling pattern for CA Scenario - Agreement: Configure following parameters for FR1 Pcell as baseline. Revisit only in case of issues - TDD - 30kHz SCS with 7D1S2U - All HARQ feedback should be transmitted on UL slot of FR1 carrier |
| Sub-topic 1-4 | Issue 1-4-1: List of test cases to be used for requirement definition; Recommended WF: most of the companies’ support was received for the table below. We recommend to keep it as baseline and update it based on the outcome of the detailed issues in the previous sub-topics regarding MCS, Doppler Spread, Channel Model, RB size, etc. Test; CBW(MHz) / SCS (kHz); MCS/Rank; Channel Model; Antenna Configuration and Correlation Matrix; RB Allocation Size; % of peak thpt. 1-1; 100/120; MCS4/Rank1; TDLA10-200; 2x2 ULA Low; [Full]; 70. 1-2; 100/120; MCS13/Rank1; TDLA10-200; 2x2 ULA Low; [x]; 70. 1-3; 100/120; MCS13/Rank2; TDLD10-200; 2x2 ULA Low; 70. 1-4; 100/120; MCS17/Rank1; TDLD10-200; 2x2 ULA Low; 70. 2-1; 400/480; MCS4/Rank1; TDLA10-200; 2x2 ULA Low; 70. 2-2; 400/480; MCS13/Rank1; TDLA10-200; 2x2 ULA Low; 70. 2-3; 400/480; MCS13/Rank2; TDLD10-200; 2x2 ULA Low; 70. 2-4; 400/480; MCS17/Rank1; TDLD10-200; 2x2 ULA Low; 70. Issue 1-4-3: Test Setup for 70% peak throughput PDSCH Requirements ; - Agreements: Use the table below as baseline for test setup. Some fields have been updated with [] to reflect pending discussions Assumptions; Value. Carrier Frequency [GHz]; 70 GHz. Subcarrier Spacing [KHz]; 120 KHz, 480 KHz. Waveform; CP-OFDM. CP Type; Normal CP. Channel Model; [TDLA (10 ns and 20 ns delay spread) and TDLD (10 ns delay spread)]. Antenna configuration; 2x2 ULA Low. Velocity; [3 km/h, 10 km/h]. PA Model; None. gNB PN Model; None. [UE PN Model]; [TR38.808 PN model Set 1 and Set 2]. Pre-loaded Tx EVM; 6%. Additive Rx EVM; 0%. I-Q Imbalance; None. Frequency Offset; 0 ppm. Channel Estimation; Realistic channel estimation. Transmission Rank; Rank 1. DMRS Configuration; 2 DMRS symbols at (2,11) symbol index. PTRS Configuration; For CP-OFDM: (K = 2, L = 1). CSI-RS / TRS; CSI-RS/TRS is assumed to be off (for RS overhead). MCS/TBS; From MCS Table 1 (TS38.214): [Up to MCS 22 (64QAM)] Note: It is assumed that NohPRB = 0 for MCS calculations.. Issue 1-4-4: Test Setup for 30% peak throughput PDSCH Requirements (if 30% peak throughput requirement is agreed) - Proposals: - Option 1: Test Setup table (Ericsson, Huawei, as baseline: Nokia); - Option 1b: Align to Table in previous option, add 32 HARQ processes if 960kHz is included (Nokia) - Recommended WF If 30% throughput requirement is agreed, align the test setup for 70% percent throughput as baseline. Further discuss modifications if necessary. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

### Sub Topic 1-1: General on FR2-2 Requirements

Issue 1-1-2a: Whether to include TDLA20 Channel Model for FR2 UE Demodulation requirements

- Proposals

  - Option 1: No;

  - Option 2: Yes;

- Recommended WF

  - TBA;

| Company | Comments |
| --- | --- |
| Ericsson | Yes. We do believe that TDLA20 is applicable for low/moderate MCS (up to 17) with low PRB allocation (up to 66 for 120 KHz SCS). Choose Option 2. |
| Huawei | This issue have relevant to delay resolution. Based on the GTW agreements from BS side, 5ns for CBW lower than 200MHz and 2ns for CBW larger than 200MHz are agreed. We propose to reuse it for UE requirements. If 5ns resolution is considered, we think 20ns RMS delay can’t be enough to reflect the channel model define in 38.901. We propose the following: TDLA10 for 400MHz TDLA30 for 100MHz |
| Apple | No strong view, need to further check based on tap delay resolution agreed in GTW what RMS delay spread would be feasible. |
| Ericsson2 | We are fine with Huawei comments, and we kindly remind that we have been supporting TDLA30 for 120 KHz SCS in last meeting (#103). We agree to align with [324], and use TDLA30 for 100 MHz TDLA10 and TDLD10 for 400 MHz |
| Qualcomm | According to TE vendors’ feedback, we support Huawei’s proposal to consider TDLA-30 for 100MHz and TDLA/TDLD10 for 400MHz and above. So including TDLA20 would be redundant, support option 2; |
| Nokia, Nokia Shanghai Bell | We support option 1 to not include TDLA20 channel model. We can accept to adopt the agreement from BS side as proposed by Huawei. |

Issue 1-1-2b: Highest Modulation order for TDLA10 Channel Model for FR2 UE Demodulation requirements

- Proposals

  - Option 1: QPSK;

  - Option 2: Decide based on simulation results;

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Ericsson | We choose Option 2. Our simulation results showed that we can reach 64QAM in some configurations. |
| Huawei | We support option 1. Considering we need to allow some margin between testable SNR and required SNR, we propose to only use TDLA for 120kHz/QPSK. |
| Apple | For 120kHz SCS 16QAM could be considered with smaller PRB allocation For 480kHz SCS QPSK |
| Ericsson2 | We would like to provide some simulation results (already presented in our draft R4-2212571) Therein, we have considered both PN models from TR38.808, and we can note that 16QAM is also achievable for 120 KHz SCS. We can even say that 64QAM is achievable under TDLD10-200 channel model for 120 KHz SCS. CBW (MHz) / SCS (KHz) = 100/120 (66 PRBs); PN @ RX (UE) only. Test case; MCS; Propagation condition; NO PN SNR0 (dB) @ 70% Max Throughput; PN compensa-tion; SNR1 (dB) @ 70% Max Throughput; D1 (dB); SNR2 (dB) @ 70% Max Throughput; D2 (dB). Test 1-1; 4; TDLA10-200; -3.3788; No Comp; -3.2522; -; -3.206; -. CPE; -3.1424; 0.2364; -3.1289; 0.2499. TDLA10-650; -3.1813; No Comp; -3.028; -; -2.9786; -. CPE; -2.9126; 0.2687; -2.8743; 0.307. TDLA20-200; -3.3421; No Comp; -3.2319; -; -3.1826; -. CPE; -3.1246; 0.2175; -3.0797; 0.2624. TDLA20-650; -3.0976; No Comp; -2.9745; -; -2.9579; -. CPE; -2.8692; 0.2284; -2.851; 0.2466. Test 1-2; 13; TDLA10-200; 4.4407; No Comp; 4.7784; -; 4.9897; -. CPE; 4.7578; 0.3171; 4.9312; 0.4905. TDLA10-650; 5.0223; No Comp; 5.3213; -; 5.5581; -. CPE; 5.3199; 0.2976; 5.4865; 0.4642. TDLA20-200; 4.4508; No Comp; 4.8033; -; 5.0328; -. CPE; 4.7688; 0.318; 4.9456; 0.4948. TDLA20-650; 4.9883; No Comp; 5.3312; -; 5.5643; -. CPE; 5.286; 0.2977; 5.474; 0.4857. Test 1-3; 17; TDLA10-200; 7.7469; No Comp; 8.4733; -; 8.9909; -. CPE; 8.3185; 0.5716; 8.7127; 0.9658. TDLA10-650; 8.3275; No Comp; 9.0621; -; 9.5952; -. CPE; 8.9399; 0.6124; 9.2739; 0.9464. TDLA20-200; 7.7642; No Comp; 8.5336; -; 9.1027; -. CPE; 8.3566; 0.5924; 8.7411; 0.9769. TDLA20-650; 8.2882; No Comp; 9.0361; -; 9.6357; -. CPE; 8.8699; 0.5817; 9.2879; 0.9997. TDLD10-200; 4.0109; No Comp; 4.6669; -; 5.2091; -. CPE; 4.5089; 0.498; 4.8917; 0.8808. Test 1-4; 20; TDLA10-200; 10.0022; No Comp; 11.3884; -; 12.7607; -. CPE; 10.9281; 0.9259; 11.7925; 1.79. TDLA10-650; 10.7018; No Comp; 12.0465; -; 13.4952; -. CPE; 11.6043; 0.9025; 12.393; 1.6912. TDLA20-200; 10.1098; No Comp; 11.5219; -; 12.8943; -. CPE; 11.0611; 0.9513; 11.8998; 1.79. TDLA20-650; 10.7017; No Comp; 12.0954; -; 13.6599; -. CPE; 11.6132; 0.9115; 12.4859; 1.7842. TDLD10-200; 6.2545; No Comp; 7.521; -; 8.6399; -. CPE; 7.1234; 0.8689; 7.8863; 1.6318. For 480 KHz SCS, we can limit to QPSK @Huawei, we have checked Huawei simulation results, and can note that they have been performed using PN model Set 2 (we made same conclusion for PN Set 2 as well). Thius is why we are advocating the use of both model to draw fair conclusions. |
| Qualcomm | Postpone decision to the next meeting based on aligned simulation results, according to agreements reached on channel model resolution and taps number; |
| Nokia, Nokia Shanghai Bell | We see defining requirement for 64QAM as important; hence we support for option 2 and decide based on aligned simulation results. |

Issue 1-1-3: MCS to use with 650Hz Doppler Frequency for TDLA

- Proposals

  - Option 1: QPSK;

  - Option 2: Decide based on simulation results;

  - Option 3: Do not introduce requirements with 650Hz Doppler Frequency;

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Ericsson | We opt for Option 2. |
| Huawei | We support option 1. We don’t see any performance difference between different Doppler frequency. |
| Apple | We are fine to compromise to option 1. |
| Ericsson2 | Based on the results provided in Issue 1-1-2b, we can note that 16QAM is achievable for 120 KHz SCS under a velocity of 10 Km/h (Max Doppler freq of 650 Hz). We keep choosing Option 2. |
| Qualcomm | Postpone decision to the next meeting based on aligned simulation results, according to agreements reached on channel model resolution and taps number; |
| Nokia, Nokia Shanghai Bell | We would prefer to decide based on aligned simulation results. |

Issue 1-1-4: Whether to define UE Demodulation requirements for 960kHz SCS

- Proposals

  - Option 1: Yes (Nokia);

    - Option 1a: Define the following separate sets of requirements and agree on the capability rule below:

      - Set 1) for legacy TE capabilities (480kHz with not too large CBW, and 960 KHz with non-full FDRA).

      - Set 2) for future 71GHz capable test equipment (480 and 960 kHz with larger CBW).

      - Define capability rules to allow that one of the 2 sets of the requirements are passed depending on what test equipment is available.

  - Option 2: No (Apple, Ericsson, Huawei, Qualcomm, Anritsu);

- Recommended WF

  - Only 1 company has expressed interest to introduce requirements for 960kHz. 
We recommend to discuss Nokia’s proposal in the second round, if no interest is received can Nokia agree to Option 2?

- Agreement (GTW August 22, 2022):

    - RAN4 focuses on others SCSs before test feasibility for supporting 960kHz SCS is confirmed.

| Company | Comments |
| --- | --- |
| Huawei | At current stage we don’t think it is feasible to consider 960kHz SCS. |

Issue 1-1-5: Max CBW for UE Demodulation Requirements definition for SCS 120kHz

- Proposals

  - Option 1: 100 MHz (Ericsson, Apple, Huawei, Qualcomm);

    - Option 1a: Choose only minimum CBW for each SCS; FFS larger CBW/SCS combinations (Huawei);

    - Option 1b: Keep FFS 400 MHz (Ericsson);

  - Option 2: 400 MHz (Nokia, );

- Recommended WF

  - Based on first round input, we recommend as WF to use 100MHz for 120kHz and keep 400MHz FFS. Can Nokia agree?

| Company | Comments |
| --- | --- |
| Ericsson | We agree with the recommended WF |
| Huawei | Option 1a, we have agreed to consider partial bandwidth allocation, it is meaningless to consider large bandwidth. |
| Apple | We support the recommended WF. Same comments as Huawei on why we need to consider large CBW. |
| Ericsson2 | Referring to [334] iterations, we can conclude that the max DL SNR levels would increase, to allow for higher CBW for 120 KHz SCS (with QPSK). |
| Qualcomm | Share Huawei and Apple comments; We don’t see the added coverage from demodulation performances point of view of introducing higher CBW if we cannot test them and we’ll need to reduce the allocation. |
| Nokia, Nokia Shanghai Bell | We still prefer to define requirements for 400MHz, however we can compromise to keeping 400MHz CBW FFS. Our preference is option 2, but we can compromise to Option 1b, hence we support the recommended WF. |

Issue 1-1-6: Max CBW for UE Demodulation Requirements definition for SCS 480kHz

- Proposals

  - Option 1: 400 MHz (Qualcomm, Ericsson, Anritsu, Apple, Huawei)

    - Option 1a; FFS 800 MHz (Ericsson);

  - Option 2: 800 MHz (, Nokia);

- Recommended WF

  - Based on first round input, we recommend as WF to use 400MHz for 480kHz and keep 800MHz FFS. Can Nokia agree?

| Company | Comments |
| --- | --- |
| Ericsson | We agree with the recommended WF. |
| Huawei | Same as 120kHz, we have agreed to consider partial bandwidth allocation, it is meaningless to consider large bandwidth. |
| Apple | We support the recommended WF. Same comments as Huawei on why we need to consider large CBW. |
| Qualcomm | Share Huawei and Apple comments; We don’t see the added coverage from demodulation performances point of view of introducing higher CBW if we cannot test them and we’ll need to reduce the allocation. |
| Nokia, Nokia Shanghai Bell | We still prefer to define requirements for 800MHz, however we can compromise to keeping 800MHz CBW FFS. Our preference is option 2, but we can compromise to Option 1a, hence we support the recommended WF. |

Issue 1-1-7: Whether to define UE Demodulation Requirements with CBW = 1600MHz (for SCS 480kHz, 960kHz)

- Proposals

  - Option 1: Yes (Nokia)

  - Option 2: No (Apple, Ericsson, Huawei, Qualcomm, Anritsu);

- Recommended WF

  - Only 1 company has expressed interest to introduce requirements for 1600MHz. Can Nokia agree to Option 2?

| Company | Comments |
| --- | --- |
| Ericsson | Due to the testability issues, we do not believe that we can define requirements with CBW =1600 MHz. Option 2. |
| Huawei | Option 2. We have agreed to consider partial bandwidth allocation, it is meaningless to consider large bandwidth. |
| Apple | Option 2. |
| Nokia, Nokia Bell Labs | This Issue covers both 480 and 960kHz SCS. Currently the support of 960kHz SCS is pending TE vendor feedback on feasibility. If Issue 1-1-6 agreement is to keep 800MHz CBW FFS, we can compromise to option 2 for 480kHz SCS. We would however need to discuss this issue again, if 960kHz is decided to be feasible by TE vendors. |

### Sub Topic 1-2: PDSCH Demodulation Requirements

Issue 1-2-2: How to address low maximum DL Testable SNR

Issue 1-2-2a: Whether to introduce Demodulation requirements with partial bandwidth allocation

- Proposals

  - Option 1: Yes;

  - Option 2: No;

- Recommended WF

  - TBA;

- Agreement (GTW August 22, 2022): Option 1 agreed.

| Company | Comments |
| --- | --- |

Issue 1-2-2b: Partial bandwidth allocation position in the CBW;

- Proposals

  - Option 1: Center;

  - Option 2: TBA;

- Recommended WF

  - TBA;

| Company | Comments |
| --- | --- |
| Ericsson | We are little bit confused; this appears like a RF issue. Does it matters for a demod perspective? We would like to ask the proponent to further elaborate. |
| Huawei | We support option 1. |
| Apple | Same comment as Ericsson. |
| Qualcomm | Existing PDSCH tests in 38.101-4 that use resource allocation Type 1 specify length and start RB, so we think that this is something that should be agreed and reflected in the test parameters. |

Issue 1-2-2c: Whether to align RB size with PRB bundling size;

- Proposals

  - Option 1: Yes;

  - Option 2: No;

- Recommended WF

  - TBA;

| Company | Comments |
| --- | --- |
| Huawei | Option 1 is OK for us |
| Apple | Does it mean we use the number of PRBs as an integer multiple of the precoding granularity? |
| Qualcomm | @Apple: Yes, that was our proposal’s intention; |
| Nokia Shanghai Bell | We would agree to have mapping it multiple of the bundling size. We propose a new option: Option 1a: RB size should be mapped with multiple of the bundling size. |

Issue 1-2-2d: RB Allocation Table for SCS 120kHz / CBW 100MHz

- Proposals

  - Option 1: MCS 4 and 13: 66 RBs; MCS 17: 33RBs

  - Option 2: MCS 4,13: 66 PRBs; MCS17: 30PRBs

  - Option 3: MCS 4, 13, 17, and 20: Configure 66 RBs;

- Recommended WF

  - TBA;

| Company | Comments |
| --- | --- |
| Ericsson | We opt for Option 3, while we believe that MCS 20 could be tested using whether 66 RBs or at least 33 RBs. |
| Huawei | Our simulation shows: MCS; Target SNR with CPE compensation only; Testable SNR(dB). 4 (Rank1,TDLA); -2.4; 7.7 for 66RBs, 11.0 for 33RBs, 11.5 for 30RBs. 13 (Rank1,TDLA); 5.9. 17 (Rank1,TDLD); 8.4. We can find MCS13, the target SNR is closed to testable SNR(1.8dB margin), We propose to replace TDLA by TDLD for MCS 13 to allow much margin. We can take 66RBs for MCS4 and MCS13 as baseline for simulation and the RB allocation can be updated if there is some problem for testability based on simulation results in next meeting. For 64QAM, we propose to consider MCS17 to avoid impact of phase noise. For RB allocation, we are also OK to use 30RBs to leave much margin. |
| Apple | We are fine with option 1 or 2. Can finalize based on simulation results |
| Ericsson2 | Please refer to our comment for Issue 1-1-2b MCS 17 and 20 are achievable for 120KHz/100 MHz (66 RBs) using TDLD10-200 (SNR1 when using PN Set 1 of TR 38.808) With a higher margin and partial CBW (33 RBs), MCS 20 should be achievable as well. CBW (MHz) / SCS (KHz) = 100/120 (66 PRBs); PN @ RX (UE) only. MCS; Propagation condition; NO PN SNR0 (dB) @ 70% Max Throughput; PN compensa-tion; SNR1 (dB) @ 70% Max Throughput; D1 (dB); SNR2 (dB) @ 70% Max Throughput; D2 (dB). 17; TDLD10-200; 4.0109; No Comp; 4.6669; -; 5.2091; -. CPE; 4.5089; 0.498; 4.8917; 0.8808. 20; TDLD10-200; 6.2545; No Comp; 7.521; -; 8.6399; -. CPE; 7.1234; 0.8689; 7.8863; 1.6318. |
| Qualcomm | Our preliminary results support either option 1 or 2. Postpone to the next meeting based on aligned simulation results and updated testable SNRs; |
| Nokia, Nokia Bell Labs | We support using option 2 for simulations to be delivered. We think the numbers from the TR is very conservative estimates, so we have no issues with going close to these numbers. |

Issue 1-2-2d: RB Allocation Table for SCS 480kHz / CBW 400MHz

- Proposals

  - Option 1: MCS 4: 66 RBs; MCS 13: 16 RBs

  - Option 2: MCS 4, 9: 66 RBs; MCS13: 33RBs; MCS 17: 16 RBs;

- Recommended WF

  - TBA;

| Company | Comments |
| --- | --- |
| Ericsson | We made a correction to Option 2 since we think that it has been a typo. We are for Option 2. |
| Huawei | Our simulation shows: MCS; Target SNR with CPE compensation only; Testable SNR(dB). 4 (Rank1,TDLA); -2.2; -0.6dB for 66RBs, 7.9dB for 16RBs. 13 (Rank1,TDLA); 6.0. 17 (Rank1,TDLD); 9.5. We can find there is only 1.6dB margin for MCS4 and 1.9dB for MCS13. We propose to replace TDLA by TDLD for both MCS4 and MCS13 to allow much margin. We can take this RB allocation for simulation and RB allocation can be updated if there is some problem for testability based on simulation results in next meeting. |
| Apple | Option 1 is a good starting point, we can finalized based on simulation results. Looks like we have to restrict to QPSK for requirements with 480KHz SCS, with partial RB allocation. We really don’t see the feasibility to define requirements with 16QAM and 64QAM for 480/400MHz. Is there really any value in defining requirements for such small RB allocation just for the sake of introducing requirements? |
| Ericsson 2 | We compromise to Option 1 (agree with Huawei and Apple), however, we would like to propose the following: MCS 4: 66 RBs and MCS 9: 33 or 16 RBs; No MCS13. |
| Qualcomm | Postpone to the next meeting based on aligned simulation results and updated testable SNRs; Considering Apple’s point, at the moment in 38.101-4 PDSCH requirements in FR2-1 cover at minimum 6 RBs, we shouldn’t choose an allocation lower than that. @Ericsson: do we need to introduce tests for 2 different coderates for QPSK? Is there a motivation for it? |
| Nokia, Nokia Bell Labs | We support using option 2 for simulations to be delivered. We think the numbers from the TR is very conservative estimates, so we have no issues with going close to these numbers. |
| Ericsson3 | @Qualcomm. Thanks a lot for this relevant remark. Indeed, we do not need, but just tried to propose an alternative, whether to choose the highest MCS for QPSK with partial CBW, or to use MCS4 with 66 RBs. |

Issue 1-2-3: Whether RAN4 needs to specify HARQ-ACK Codebook and HARQ Bundling for multi-PDSCH scheduling

- Proposals

  - Option 1: No, leave it up to R5 (Apple, Ericsson);

    - Option 1a: Leave specification up to R5, but R4 to ensure no issue in the configuration of the with respect to HARQ bundling (Nokia, Apple, Qualcomm);

  - Option 2: HARQ-ACK codebook type 1, HARQ Bundling 4 for 480kHz (Huawei);

- Recommended WF

  - Only 1 company wants to specify codebook type and bundling in RAN4. Can Huawei agree to define this in RAN5, while RAN4 checks that no conflict is introduced in the test setup?

| Company | Comments |
| --- | --- |
| Ericsson | We agree with the recommended WF. |
| Huawei | To move forward, we can compromise to option 1 |
| Apple | We are fine with the recommended WF. |
| Nokia, Nokia Shanghai Bell | We support the recommended WF. |

Issue 1-2-4: Whether RAN4 should introduce requirements for 30% of peak throughput

- Proposals

  - Option 1: No (Apple, Huawei, Qualcomm);

  - Option 2: Yes (Nokia, Ericsson);

  - Option 3: Keep FFS (Qualcomm);

- Recommended WF

  - Keep discussing in the second round;

- Agreement (GTW August 22, 2022): FFS whether introduce requirements for 30% of peak throughput

| Company | Comments |
| --- | --- |
| Ericsson | Option 2 or at least Option 3 |
| Huawei | Option 1, we don’t need to verify every feature defined in FR1. There is no difference between HARQ combination for operating in different frequency range. |
| Apple | Option 1. We should aim to close out the open issues for the test cases we are defining first. |
| Qualcomm | We are okay to keep this FFS and discuss after 70% basic coverage of the requirements; |
| Nokia, Nokia Shanghai Bell | We support option 2, but can compromise to option 3. |

Issue 1-2-6: UE Phase Noise compensation Model for Simulation Alignment

- Agreements;

CPE compensation only is agreed for low MCS.

- Proposals:

  - Option 1: Keep ICI compensation FFS for high MCS/high CBW requirements;

  - Option 2: Do not consider ICI compensation;

- Recommended WF

  - TBA;

| Company | Comments |
| --- | --- |
| Ericsson | We chose Option 1 provided that the max DL testable SNRs increase, thus allow for higher MCS/higher CBW where ICI compensation will be of great significance. |
| Huawei | Our simulation results show ICI compensation only bring performance gain when SNR is closed to 20dB We propose to only consider CPE compensation considering much low testable SNR. |
| Apple | Option 2. |
| Ericsson2 | We can compromise to Option 2, and only consider CPE compensation. |
| Qualcomm | Option 2. Given that the combination of high MCS and large CBW is excluded from the agreements reached on partial allocation, ICI is not expected to bring any meaningful gain in the test and we don’t think it should be further considered; |
| hNokia, Nokia Shanghai Bell | We can compromise to option 2. |

### Sub Topic 1-3: PDSCH Coverage of the requirements

Issue 1-3-2: Highest MCS choice for PDSCH requirements

- Agreement: RAN 4 to select the max MCS that satisfies the following criteria: performance degradation due to phase noise is less than 1dB with the agreed PN compensation technique;

- Proposals:

  - Option 1:

    - 120kHz: MCS 22;

    - 480kHz: MCS 20;

  - Option 2:

    - 120kHz: MCS17;

    - 480kHz: MCS 13;

  - Option 3: Decide based on simulation results separately for 120kHz and 480kHz SCS

    - 120kHz: MCS TBA;

    - 480kHz: MCS TBA;

- Recommended WF

  - TBA;

| Company | Comments |
| --- | --- |
| Ericsson | We choose Option 3, as follow - 120kHz: MCS 20 (66 RBs); - 480kHz: MCS 13 (66 RBs), and 17 (16 RBs); Here we focus on mandatory CBW for each SCS. |
| Huawei | We support MCS17 for 120kHz and not consider 64QAM for 480kHz considering limited testable SNR |
| Apple | Option 2, Option 3. We don’t think MCS13 is feasible for 480/400 |
| Ericsson2 | We choose Option 3, as follow - 120kHz: MCS 20 (66 RBs or 33 RBs); - 480kHz: MCS 9 (33 or 16 RBs); |
| Qualcomm | Postpone to the next meeting based on aligned simulation results and updated testable SNRs; |
| Nokia, Nokia Shanghai Bell | We see 64QAM as important and requirements should be defined if aligned simulation results indicate 64QAM testable. Support option 3. |

Issue 1-3-3: Rank to be used for the definition of requirements

- Proposals :

  - Option 1: Only Rank 1 (Apple, Qualcomm, Ericsson, Huawei);

  - Option 2: Both Rank 1 and Rank 2 (Nokia,);

- Recommended WF

- Only one company wants to introduce Rank 2 requirements. Can Nokia agree to Option 1?

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Huawei | Option 1 |
| Apple | Option 1 |
| Nokia, Nokia Shanghai Bell | We can compromise to option 1. |

Issue 1-3-7: Number of HARQ processes to be used for SCS=960kHz requirement definition if requirements are agreed;

- Proposals :

  - Option 1: Separate requirements for 16 and 32 HARQ processes (Nokia)

  - Option 2: Only 16 HARQ Processes (Mandatory);

  - Option 3: Only 32 HARQ Processes (Optional Capability);

- Recommended WF

  - Do not discuss further unless 960kHz SCS requirements are agreed;

| Company | Comments |
| --- | --- |

### Sub Topic 1-4: PDSCH Test cases and simulation assumptions

Issue 1-4-1: List of test cases to be used for requirement definition;

Recommended WF: most of the companies’ support was received for the table below. We recommend to keep it as baseline and update it based on the outcome of the detailed issues in the previous sub-topics regarding MCS, Doppler Spread, Channel Model, RB size, etc.

| Test | CBW(MHz) / SCS (kHz) | MCS/Rank | Channel Model | Antenna Configuration and Correlation Matrix | RB Allocation Size | % of peak thpt |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100/120 | MCS4/Rank1 | TDLA10-200 | 2x2 ULA Low | [Full] | 70 |
| 1-2 | 100/120 | MCS13/Rank1 | TDLA10-200 | 2x2 ULA Low | [x] | 70 |
| 1-3 | 100/120 | MCS13/Rank2 | TDLD10-200 | 2x2 ULA Low |  | 70 |
| 1-4 | 100/120 | MCS17/Rank1 | TDLD10-200 | 2x2 ULA Low |  | 70 |
| 2-1 | 400/480 | MCS4/Rank1 | TDLA10-200 | 2x2 ULA Low |  | 70 |
| 2-2 | 400/480 | MCS13/Rank1 | TDLA10-200 | 2x2 ULA Low |  | 70 |
| 2-3 | 400/480 | MCS13/Rank2 | TDLD10-200 | 2x2 ULA Low |  | 70 |
| 2-4 | 400/480 | MCS17/Rank1 | TDLD10-200 | 2x2 ULA Low |  | 70 |

| Company | Comments |
| --- | --- |
| Ericsson | We agree with the recommended WF, to update this table based on the progress. We do believe that TDLA20 is possible for MCS 4, 13 and 17, Rank 1. Moreover, the PRB number should be added especially for moderate/high MCS orders. |
| Huawei | We propose the following Test; CBW(MHz) / SCS (kHz); MCS/Rank; Channel Model; Antenna Configuration and Correlation Matrix; RB Allocation Size; % of peak thpt. 1-1; 100/120; MCS4/Rank1; TDLA30-200; 2x2 ULA Low; 66; 70. 1-2; 100/120; MCS13/Rank1; TDLD30-200; 2x2 ULA Low; 66; 70. 1-4; 100/120; MCS17/Rank1; TDLD30-200; 2x2 ULA Low; 30; 70. 2-1; 400/480; MCS4/Rank1; TDLD10-200; 2x2 ULA Low; 66; 70. 2-2; 400/480; MCS13/Rank1; TDLD10-200; 2x2 ULA Low; 16; 70. The RB allocation can be updated based on simulation results in next meeting |
| Apple | We prefer Huawei’s proposal above. |
| Ericsson 2 | We are fine with the following, for 480 KHz SCS, we limit MCS to 9 instead of 13. Test; CBW(MHz) / SCS (kHz); MCS/Rank; Channel Model; Antenna Configuration and Correlation Matrix; RB Allocation Size; % of peak thpt. 1-1; 100/120; MCS4/Rank1; TDLA30-650; 2x2 ULA Low; 66; 70. 1-2; 100/120; MCS13/Rank1; TDLA30-200; 2x2 ULA Low; 66; 70. 1-3; 100/120; MCS17/Rank1; TDLD10-200; 2x2 ULA Low; 66; 70. 1-4; 100/120; MCS20/Rank1; TDLD10-200; 2x2 ULA Low; 33; 70. 2-1; 400/480; MCS4/Rank1; TDLD10-200; 2x2 ULA Low; 66; 70. 2-2; 400/480; MCS9/Rank1; TDLD10-200; 2x2 ULA Low; 16; 70. |
| Qualcomm | We support Huawei’s proposal, further refine the RB allocation size in the next meeting; @ Ericsson: no need to specify requirements for both MCS 4 and MCS 9? |
| Nokia, Nokia Shanghai Bell | Agreement here is pending decision on not have requirements for rank 2 (Issue 1-3-3) and if TDLA/D30 is agreed (Issue 1-1-2a). |
| Ericsson3 | Same response as above. For the Table, may be it is more judicious to keep Test 2-1 and drop 2-2 |

Issue 1-4-3: Test Setup for 70% peak throughput PDSCH Requirements ;

- Agreements: Use the table below as baseline for test setup. Some fields have been updated with [] to reflect pending discussions

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | [TDLA (10 ns and 20 ns delay spread) and TDLD (10 ns delay spread)] |
| Antenna configuration | 2x2 ULA Low |
| Velocity | [3 km/h, 10 km/h] |
| PA Model | None |
| gNB PN Model | None |
| [UE PN Model] | [TR38.808 PN model Set 1 and Set 2] |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| CSI-RS / TRS | CSI-RS/TRS is assumed to be off (for RS overhead) |
| MCS/TBS | From MCS Table 1 (TS38.214): [Up to MCS 22 (64QAM)] Note: It is assumed that NohPRB = 0 for MCS calculations. |

| Company | Comments |
| --- | --- |
| Ericsson | Yes, we opt for this table, while using both proposed velocities (max freq Doppler of 200 Hz and 650 Hz, respectively), and both PN models |
| Huawei | OK with proposals |
| Apple | Ok with proposal. What does this parameter setting mean: CSI-RS/TRS is assumed to be off (for RS overhead) TRS is not enabled for simulations? |
| Ericsson2 | @Apple (TRS is enabled for sure, it has been a mistake when reproducing the table from another one) |
| Qualcomm | We are ok with the proposal and Ericssons’ correction; |
| Nokia, Nokia Shanghai Bell | Table should also reflect the discussion to include TDLA/D30 and rank 2. |

Issue 1-4-4: Test Setup for 30% peak throughput PDSCH Requirements (if 30% peak throughput requirement is agreed)

- Proposals:

  - Option 1: Test Setup table (Ericsson, Huawei, as baseline: Nokia);

    - Option 1b: Align to Table in previous option, add 32 HARQ processes if 960kHz is included (Nokia)

- Recommended WF

If 30% throughput requirement is agreed, align the test setup for 70% percent throughput as baseline. Further discuss modifications if necessary.

| Company | Comments |
| --- | --- |
| Ericsson | Agree with the recommended WF |
| Apple | Prefer to de-prioritize |
| Qualcomm | Share same view as Apple; |
| Nokia, Nokia Shanghai Bell | We support recommended WF. |

# Topic #2: PDCCH and PBCH Demodulation Requirements

## Open issues summary

### Sub Topic 2-1: PDCCH Demodulation Requirements

Issue 2-1-1: Whether to define requirements for multi-slot PDCCH monitoring

- Proposals:

  - Option 1: Yes

    - Option 1b: Define requirements for PDCCH demod with 480kHz SCS with 1 slot every 4 slots (Apple, Nokia, Qualcomm);

    - Option 1c: If 960kHz is agreed, define PDCCH requirements with 1 slot every 8 slots (Nokia);

  - Option 2: No, implicitly test this in PDSCH requirements (Ericsson, Huawei);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 2-1-1: Whether to define requirements for multi-slot PDCCH monitoring We agreed to define requirements for multi-PDSCH scheduling which should implicitly consider the multi-slot PDCCH monitoring. We choose Option 2. |  |
| Apple | We support option 1b. For 480kHz UE will not be monitoring PDCCH every slot, so we cannot define requirements with PDCCH every slot. |
| Huawei | Option 2 |
| Nokia, Nokia Bell Labs | In our view, dedicated tests are preferred. Support option 1. |
| Qualcomm | It is our understanding that for 480kHz multi-slot monitoring with (4,1) is mandatory behaviour and per-slot PDCCH monitoring is not supported, so we should use 1b in the test; |

Issue 2-1-2: Set of PDCCH Test Cases

- Proposals :

  - Option 1: Table from previous WF (Previous WF, Apple, Huawei, Qualcomm);

Moderator’s note: AL on Test #6 was changed to 8 with respect to the previous WF in line with other companies’ proposals. Otherwise, Test 5 and 6 have no difference.

| Test number | CBW/SCS | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | TDLA10-200 | 1_0 | 40 | 1x2 Low |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 | TDLA10-200 | 1_1 | 56 | 2x2 Low |
| 4 | 100/120 | 2 | 60 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low |
| 6 | 400/480 | 1 | 60 | 2 | 6 | 8 | TDLA10-200 | 1_1 | 56 | 1x2 Low |

  - Option 2 (Nokia):

    - Baseline test coverage from Option 1;

    - Change propagation conditions to align with PDSCH;

    - Add the tests below (if 960kHz is agreed);

- 960kHz/400MHz. Align propagation condition with PDSCH.

| Test number | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 60 | 2 | 6 | 4 | TBD | 1_1 | 56 | 1x2 Low |
| 2 | 2 | 60 | 3 | 2 | 16 | TBD | 1_0 | 40 | 2x2 Low |

  - Option 3: (Ericsson)

- FR2-2 TDD, SCS 120 KHz/CBW 100 MHz

| Test case | Interleaver Size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNRBB (Db) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 1x2 ULA Low | 1_0 | 40 | New |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 1x2 ULA Low | 1_1 | 56 | New |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | 1_1 | 56 | New |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | 1_0 | 40 | New |

- FR2-2 TDD, SCS 120 KHz/CBW 400 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNRBB (Db) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 2-1 | 3 | 2 | 240 | 1 | 2 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 1x2 ULA Low | 1_0 | 40 | New |
| Test 2-2 | 2 | 6 | 240 | 1 | 4 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 1x2 ULA Low | 1_1 | 56 | New |
| Test 2-3 | 3 | 2 | 240 | 1 | 8 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | 1_1 | 56 | New |
| Test 2-4 | 3 | 2 | 240 | 1 | 16 | TDLA10-200 TDLA10-650 TDLA20-200 TDLA20-650 | 2x2 ULA Low | 1_1 | 56 | New |

- FR2-2 TDD, SCS 480 KHz/CBW 400 MHz

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNRBB (Db) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 3-1 | 3 | 2 | 60 | 1 | 2 | TDLA10-200 TDLA20-200 TDLD10-200 | 1x2 ULA Low | 1_0 | 40 | New |
| Test 3-2 | 2 | 6 | 60 | 1 | 4 | TDLA10-200 TDLA20-200 TDLD10-200 | 1x2 ULA Low | 1_1 | 56 | New |
| Test 3-3 | 3 | 2 | 60 | 1 | 8 | TDLA10-200 TDLA20-200 TDLD10-200 | 2x2 ULA Low | 1_1 | 56 | New |
| Test 3-4 | 3 | 2 | 60 | 2 | 16 | TDLA10-200 TDLA20-200 TDLD10-200 | 2x2 ULA Low | 1_0 | 40 | New |

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 2-1-2: Set of PDCCH Test Cases In the proposed Ericsson Table, we consider different AL, especially for low MCS such as 8 and 16. We can make FFS the Test cases 2-1 and 3-1. Choose Option 7 (that should be Option 3 – Typo) |  |
| Apple | Option 1. We don’t see the need to define so many test cases for PDCCH as suggested in Option 3. |
| Huawei | Option 1 |
| Nokia, Nokia Shanghai Bell | Agree with Apple, that option 3 is too extensive. Support Option 2. |
| Qualcomm | We are ok with the coverage from option 1, considering the testing workload; |

### Sub Topic 2-2: PBCH Demodulation Requirements

Issue 2-2-1: SSB index assumption

- Proposals:

  - Option 1: Only with not known SSB index (Qualcomm, Apple);

  - Option 2: Both known and not known SSB index (Nokia, Ericsson, Huawei);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 2-2-1: SSB index assumption Choose Option 2. |  |
| Apple | Option 1 to reduce number of test cases and simulation effort. Don’t really see why we need requirements with known SSB index. |
| Huawei | We prefer option 2 to keep aligned with legacy test |
| Nokia, Nokia Shanghai Bell | Support option 2. |
| Qualcomm | These requirements, like the legacy ones, cannot be tested and are for information only, to reduce the workload we support introducing requirement for not known SSB index only for reference; |

Issue 2-2-2: Test setup for PBCH requirements

- Proposals :

  - Option 1: R4-2212572 ()

- FR2-2 TDD, SCS 120 KHz/CBW 100 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120 KHz / 100 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLA20-200 | New |  |
|  |  |  | TDLD10-200 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLA20-200 | New |  |
|  |  |  | TDLD10-200 | New |  |

- FR2-2 TDD, SCS 480 KHz/CBW 400 MHz

| Test number | SSB SCS | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 480 KHz / 400 MHz | 1x2 Low | TDLA10-200 | New | unknown |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLA20-200 | New |  |
|  |  |  | TDLD10-200 | New |  |
| 1-2 |  |  | TDLA10-200 | New | known |
|  |  |  | TDLA10-650 | New |  |
|  |  |  | TDLA20-200 | New |  |
|  |  |  | TDLD10-200 | New |  |

  - Option 2: (Huawei, Ericsson, Nokia)

| Test number | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- |
| 1-1 | 1x2 Low | TDLA10-200 | New | unknown |
| 1-2 |  | TDLA10-200 | New | known |
| 1-3 |  | TDLD10-200 | New | unknown |
| 1-4 |  | TDLD10-200 | New | known |

  - Option 3 (Apple):

    - 120KHz/100MHz with TDLA20-200 with unknown SSB index

    - 480KHz/400MHz with TDLD10-200 with unknown SSB index

  - Option 4 (Qualcomm)

| Test number | SCS/CBW | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120kHz/100MHz | 1x2 Low | TDLA10-200 | New | unknown |
| 1-2 | 480kHz/400MHz |  | TDLA10-200 | New | unknown |

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 2-2-2: Test setup for PBCH requirements We choose Option 1. |  |
| Apple | Proposed TCs: 120KHz/100MHz with TDLA20-200 with unknown SSB index 480KHz/400MHz with TDLD10-200 with unknown SSB index |
| Huawei | We don’t need to define so many cases. We prefer the following: Test number; Antenna configuration; Propagation condition; PBCH SNR (dB) @ 1% Pm-bch; SS/PBCH block index. 1-1; 1x2 Low; TDLA10-200; New; unknown. 1-2; TDLA10-200; New; known. 1-3; TDLD10-200; New; unknown. 1-4; TDLD10-200; New; known. |
| Ericsson2 | We agree to down select some channel models and opt for what Huawei proposed. TDLA10-200 and TDLD10-200 are enough. |
| Nokia, Nokia Shanghai Bell | Agree with Huawei’s proposal. |
| Qualcomm | Define the following tests: Test number; SCS/CBW; Antenna configuration; Propagation condition; PBCH SNR (dB) @ 1% Pm-bch; SS/PBCH block index. 1-1; 120kHz/100MHz; 1x2 Low; TDLA10-200; New; unknown. 1-2; 480kHz/400MHz; TDLA10-200; New; unknown. |

## Summary for 1st round

### Open issues

| Sub-topic 1 | Issue 2-1-1: Whether to define requirements for multi-slot PDCCH monitoring - Proposals: - Option 1: Yes - Option 1b: Define requirements for PDCCH demod with 480kHz SCS with 1 slot every 4 slots (Apple, Nokia, Qualcomm); - Option 1c: If 960kHz is agreed, define PDCCH requirements with 1 slot every 8 slots (Nokia); - Option 2: No, implicitly test this in PDSCH requirements (Ericsson, Huawei); - Recommended WF - According to multiple companies’ feedback, for SCS higher than 120kHz the UE will not monitor PDCCH in every slot. So we recommend that RAN4 follow the mandatory behaviour for multi-slot PDCCH scheduling and schedule PDCCH with (4,1) for 480kHz and (8,1) for 960kHz if agreed. Do not discuss this further in the second round unless there are diverging views. Issue 2-1-2: Set of PDCCH Test Cases - Recommended WF - Multiple companies regarded Option 3 as too expensive in terms of workload, so we recommend considering the Table below as baseline in the second round and discuss whether: - 1) To align propagation conditions with PDSCH; - 2) To add 960kHz tests; - 3) Other options not precluded; Test number; CBW/SCS; CORESET duration; CORESET RB; Interleaver size; REG bundle size; Aggregation level; Propagation Condition; DCI format; Information Bit; Antenna configuration and correlation Matrix. 1; 100/120; 1; 60; 3; 2; 2; TDLA10-200; 1_0; 40; 1x2 Low. 2; 100/120; 1; 60; 2; 6; 4; TDLA10-200; 1_1; 56; 1x2 Low. 3; 100/120; 1; 60; 3; 2; 8; TDLA10-200; 1_1; 56; 2x2 Low. 4; 100/120; 2; 60; 3; 2; 16; TDLA10-200; 1_0; 40; 2x2 Low. 5; 400/480; 1; 60; 2; 6; 4; TDLA10-200; 1_1; 56; 1x2 Low. 6; 400/480; 1; 60; 2; 6; 8; TDLA10-200; 1_1; 56; 1x2 Low. 7; 400/480; 2; 60; 3; 2; 16; TDLA10-200; 1_0; 40; 2x2 Low. |
| --- | --- |
| Sub-topic #2 | Issue 2-2-1: SSB index assumption - Proposals: - Option 1: Only with not known SSB index (Qualcomm, Apple); - Option 2: Both known and not known SSB index (Nokia, Ericsson, Huawei); - Recommended WF - Continue discussing in the second round |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

### Sub Topic 2-1: PDCCH Demodulation Requirements

Issue 2-1-1: Whether to define requirements for multi-slot PDCCH monitoring

- Proposals:

  - Option 1: Yes

    - Option 1b: Define requirements for PDCCH demod with 480kHz SCS with 1 slot every 4 slots (Apple, Nokia, Qualcomm);

    - Option 1c: If 960kHz is agreed, define PDCCH requirements with 1 slot every 8 slots (Nokia);

  - Option 2: No, implicitly test this in PDSCH requirements (Ericsson, Huawei);

- Recommended WF

  - According to multiple companies’ feedback, for SCS higher than 120kHz the UE will not monitor PDCCH in every slot. So we recommend that RAN4 follow the mandatory behaviour for multi-slot PDCCH scheduling and schedule PDCCH with (4,1) for 480kHz and (8,1) for 960kHz if agreed. Do not discuss this further in the second round unless there are diverging views.

Issue 2-1-2: Set of PDCCH Test Cases

- Baseline table for reference, other options are not precluded;

| Test number | CBW/SCS | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | TDLA10-200 | 1_0 | 40 | 1x2 Low |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 | TDLA10-200 | 1_1 | 56 | 2x2 Low |
| 4 | 100/120 | 2 | 60 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low |
| 6 | 400/480 | 1 | 60 | 2 | 6 | 8 | TDLA10-200 | 1_1 | 56 | 1x2 Low |
| 7 | 400/480 | 2 | 60 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low |

| Company | Comments |
| --- | --- |
| Ericsson | Yes, we are fine with the proposed Set of PDCCH Test Cases |
| Huawei | Issue 2-1-2: Set of PDCCH Test Cases Generally we are fine with proposed cases. But noted that for 100MHz the time resolution is only 5ns according to feedback from TE vendor. Hence we propose to use TDL30 instead of TDL10 for 100MHz CBW to make simplified model more accurately. |
| Apple | We are fine to finalize this and not add any more tests. |
| Qualcomm | Ok with proposed tables. Align channel models to DS=30ns based on TE vendors recommendations for 100MHz; |
| Nokia, Nokia Shanghai Bell | We are OK with the proposed baseline table. |

Issue 2-1-1: Whether to align propagation con conditions with PDSCH

- Proposals:

  - Option 1: Yes

  - Option 2: No;

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Ericsson | Yes, Option 1. |
| Apple | Option 1. Yes – |
| Qualcomm | Yes |
| Nokia, Nokia Shanghai Bell | Support option 1: Yes. |

### Sub Topic 2-2: PBCH Demodulation Requirements

Issue 2-2-1: SSB index assumption

- Proposals:

  - Option 1: Only with not known SSB index (Qualcomm, Apple);

  - Option 2: Both known and not known SSB index (Nokia, Ericsson, Huawei);

- Recommended WF

  - Continue discussing in the second round

- Agreement (GTW August 22, 2022): Option 1 agreed.

| Company | Comments |
| --- | --- |

# Topic #2: SDR and CSI Demodulation Requirements

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub Topic 3-1: SDR Demodulation Requirements

Issue 3-1-1: Scope of the FR2-2 UE SDR Requirements:

- Proposals:

  - Option 1: Do not introduce SDR requirements for FR2-2 (Qualcomm, Apple, Huawei);

  - Option 1: Define SDR tests according to the table below (Ericsson);

    - following numerologies have been considered (under PN effects): 120 KHz/100 MHz (66 RBs), 120 KHz/400 MHz (264 RBs), 480 KHz/400 MHz (66 RBs), and 480 KHz/800 MHz (124 RBs).

| Received antenna | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | PN compensation |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PRB (66) | PRB (124 or 264) |
| 2Rx UE | 1 | 6 | 1 | 27 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.8 | 23 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.75 | 22 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.4 | 14 | CPE only | CPE+ICI (u=2) |
|  |  | 4 | 1 | 16 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.4 | 10 | CPE only | CPE only |
|  |  | 2 | 1 | 9 | CPE only | CPE only |
|  |  |  | 0.4 | 4 | CPE only | CPE only |

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 3-1-1: Scope of the FR2-2 UE SDR Requirements: We choose Option 3 (Ericsson proposal). |  |
| Apple | We are wondering how the SDR requirements will be tested with such low testable SNR in FR2-2. Could someone please clarify? SDR tests are meant to be tested at max supported UE CBW for the corresponding SCS. We don’t really see the necessity and purpose of introducing these requirements if they are not testable. |
| Huawei | Sorry for the typo, we propose to only consider 120kHz SCS since it is more sensitive to phase noise. We have the same concern with Apple, maybe we can de-prioritize SDR test in Rel-17 and further discuss it in the future release if necessary. |
| Qualcomm | Support option 1, SDR tests are not meaningful with the low testable SNR assumed. Keep SDR FFS |

Issue 3-1-2: Maximum MCS (Scaling Factor = 1) for FR2-2 in the SDR tables

- Proposals:

  - Option 1: MCS 24 for Rank1 and rank2 (Huawei);

  - Option 2: MCS 27 for Rank 1 (Ericsson, see table in Issue 3-1-1);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 3-1-2: Maximum MCS (Scaling Factor = 1) for FR2-2 in the SDR tables We choose Option 2 (MCS 27, Rank 1) |  |
| Apple | We need to choose highest MCS that is feasible to test. |
| Huawei | Based on our simulation results, maximum MCS that SNR@85% of maximum throughput can be achieved is MCS24. We support option 1 |

Issue 3-1-3: Minimum MCS for FR2-2 in the SDR tables

- Proposals:

  - Option 1: MCS 0 for Rank1 and rank2 (Huawei)

  - Option 1: Depending on maximum Modulation Format (Ericsson, from table):

| Maximum Modulation Format | Minimum MCS |
| --- | --- |
| 6 | 14 |
| 4 | 10 |
| 2 | 4 |

  - ;

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 3-1-3: Minimum MCS for FR2-2 in the SDR tables We choose Option 2 (Ericsson proposal) |  |
| Huawei | Option 1 considering low testable SNR in FR2-2 |

Issue 3-1-4: Rank to use for FR2-2 SDR Requirements

- Proposals:

  - Option 1: Rank 1 and Rank 2 (Huawei, Apple);

  - Option 1: Rank 1 only (Ericsson, from table);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 3-1-4: Rank to use for FR2-2 SDR Requirements To follow our proposals for PDSCH, we define requirements for Rank 1 only. We choose Option 2. |  |
| Apple | Rank2 should be introduced for SDR reqt if feasible to test. |
| Huawei | We think rank 2 should be tested since it affect the maximum throughput |

Issue 3-1-5: Whether to define separate MCS to SNR mapping table for different CBW

- Proposals:

  - Option 1: Yes (Huawei, Apple)

  - Option 2: No;

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |  |
| Apple | Yes, depending on testable SNR. |

Issue 3-1-6: Scenarios to consider for the FR2-2 SDR Requirements:

- Proposals:

  - Option 1: ()

    - SA FR2-2

    - FR1 (CA or single CC) CA with FR2-2 (CA or single CC)

    - FR1 (CA or single CC) DC with FR2-2 (CA or single CC);

  - Option 2: (Huawei, Apple, Ericsson, Qualcomm);

    - SA FR2-2

    - FR1 (CA or single CC) CA with FR2-2 single CC

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 3-1-6: Scenarios to consider for the FR2-2 SDR Requirements: We choose Option 1. |  |
| Apple | We support option 2. |
| Huawei | Option 2. @Ericsson, RF didn’t define any DC combination for FR1+FR2-2 scenario, could you please clarify why consider DC scenario? |
| Ericsson2 | When we define band combinations with FR1+FR2-1, the addition of FR2-1 does not affect the RF requirements, and FR1+FR2-1 DC has been agreed. We except the same for FR2-2. Anyway, we can compromise at this moment, and opt for Option 2. |
| Qualcomm | We are ok with option 2; |

Issue 3-1-7: For FR1+FR2-2 CA scenarios, whether to transmit ACK/NACK separately on each CC

- Proposals:

  - Option 1: Yes (Huawei);

  - Option 2: No;

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |  |
| Apple | Not very clear what this means. We assume in FR1+FR2-1 CA ACK/NACK will be sent on FR1 UL, and on PCC for all other CCs. |
| Huawei | @Apple, due the limitation of HARQ processes, if ACK/NACK are transmitted on FR1 CC (7D1S2U), there will be many empty DL slot in FR2-2 CC which will make SDR test meaningless. |

### Sub Topic 3-2: CQI Demodulation Requirements

Issue 3-2-1: Scope of the FR2-2 CQI Requirements:

- Proposals:

  - Option 1: Only for SCS 120kHz, CBW=100MHz (Qualcomm, Apple, Huawei);

  - Option 2: SCS=120kHz, CBW=100MHz and SCS=480kHz,CBW=400MHz (Nokia, Ericsson);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 3-2-1: Scope of the FR2-2 CQI Requirements: We choose Option 2. |  |
| Apple | Option 1 considering testable SNR. |
| Huawei | Option 1 |
| Qualcomm | Option 1; |

Issue 3-2-2: Target Modulation Order for FR2-2 CQI Requirements:

- Proposals:

  - Option 1: Up to 16 QAM (Qualcomm, Apple, Huawei);

  - Option 2: Include 64QAM (Nokia, Ericsson);

    - Option 3b: Do not limit requirements based on current TE capability (Nokia);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 3-2-2: Target Modulation Order for FR2-2 CQI Requirements: For the same arguments that we have provided above, we believe that 64QAM should be considered. Choose Option 2. |  |
| Apple | Option 1 considering testable SNR. |
| Huawei | Option 1 |
| Qualcomm | Option 1; |

Issue 3-2-3: Whether to reuse CQI requirements test setup and metrics from FR2-1 for 120kHz/100MHz:

- Proposals:

  - Option 1: Yes,  for 120kHz/100MHz (Qualcomm, Ericsson, Apple);

  - Option 2: Test points: SNR = 6/7 for 16QAM, -2/-1Db for QPSK (Huawei);

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 3-2-3: Whether to reuse CQI requirements test setup and metrics from FR2-1 for 120kHz/100MHz: We believe that CQI reporting requirements that have been defined for FR2-1 120 KHz/100 MHz can be reused in FR2-2 120 KHz/100 MHz when applicable. Choose Option 1. |  |
| Apple | Yes, for 120KHz/100MHz |
| Huawei | Option 2 |
| Qualcomm | Option 1 for 120kH/100MHz; |

Issue 3-2-4: CQI requirements test setup and metrics from FR2-1 for 480kHz/400MHz:

- Proposals:

  - Option 1 (Ericsson):

    - Periodicity/offset = 32/1 slot and follow TDD UL-DL pattern FR2.120-2 as:

    - FR2-2 TDD 480 KHz: 11D S 4U, S: 2D:12G:0U

    - Keep same CQI delay as 120 KHz SCS, i.e., 8.375 ms.

  - Option 2: Do not introduce CQI Requirement test for 480kHz/400MHz; (Apple)

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| Ericsson | Issue 3-2-4: CQI requirements test setup and metrics from FR2-1 for 480kHz/400MHz: We choose Option 1. |  |
| Apple | Do not introduce CQI reporting requirements with 480/400MHz |

## Summary for 1st round

### Open issues

| Sub-topic #1 | Issue 3-1-1: Scope of the FR2-2 UE SDR Requirements: - Proposals: - Option 1: Do not introduce SDR requirements for FR2-2 (Qualcomm, Apple, Huawei); - Option 1: Define SDR tests according to the table below (Ericsson); - following numerologies have been considered (under PN effects): 120 KHz/100 MHz (66 RBs), 120 KHz/400 MHz (264 RBs), 480 KHz/400 MHz (66 RBs), and 480 KHz/800 MHz (124 RBs). Received antenna; Maximum number of PDSCH MIMO layers; Maximum modulation format; Scaling factor; MCS; PN compensation. PRB (66); PRB (124 or 264). 2Rx UE; 1; 6; 1; 27; CPE only; CPE+ICI (u=2). 0.8; 23; CPE only; CPE+ICI (u=2). 0.75; 22; CPE only; CPE+ICI (u=2). 0.4; 14; CPE only; CPE+ICI (u=2). 4; 1; 16; CPE only; CPE+ICI (u=2). 0.4; 10; CPE only; CPE only. 2; 1; 9; CPE only; CPE only. 0.4; 4; CPE only; CPE only. - Recommended WF - Only 1 company supports introducing SDR requirements. Can Ericsson agree to option 1? Issue 3-1-2: Maximum MCS (Scaling Factor = 1) for FR2-2 in the SDR tables - Proposals: - Option 1: MCS 24 for Rank1 and rank2 (Huawei); - Option 2: MCS 27 for Rank 1 (Ericsson, see table in Issue 3-1-1); - Recommended WF - Continue discussing and decide based on simulation results; Issue 3-1-3: Minimum MCS for FR2-2 in the SDR tables - Proposals: - Option 1: MCS 0 for Rank1 and rank2 (Huawei) - Option 1: Depending on maximum Modulation Format (Ericsson, from table): Maximum Modulation Format; Minimum MCS. 6; 14. 4; 10. 2; 4. - Recommended WF - Continue discussing; Issue 3-1-4: Rank to use for FR2-2 SDR Requirements - Proposals: - Option 1: Rank 1 and Rank 2 (Huawei, Apple); - Option 2: Rank 1 only (Ericsson, from table); - Recommended WF - Continue discussing; Issue 3-1-5: Whether to define separate MCS to SNR mapping table for different CBW - Proposals: - Option 1: Yes (Huawei, Apple) - Option 2: No; - Recommended WF - No company has expressed interest in Option 2. Considering the testable SNR, we recommend to define different MCS to SNR mapping table for different CBW; Issue 3-1-6: Scenarios to consider for the FR2-2 SDR Requirements: - Agreement: Companies agreed to consider the following scenarios for the definition of FR2-2 SDR requirements - SA FR2-2 - FR1 (CA or single CC) CA with FR2-2 single CC Issue 3-1-7: For FR1+FR2-2 CA scenarios, whether to transmit ACK/NACK separately on each CC - Proposals: - Option 1: Yes (Huawei); - Option 2: No; - Recommended WF - Continue discussing it in the second round |
| --- | --- |
| Sub-topic #2 | Issue 3-2-1: Scope of the FR2-2 CQI Requirements: - Proposals: - Option 1: Only for SCS 120kHz, CBW=100MHz (Qualcomm, Apple, Huawei); - Option 2: SCS=120kHz, CBW=100MHz and SCS=480kHz,CBW=400MHz (Nokia, Ericsson); - Recommended WF - Continue discussing it in the second round Issue 3-2-2: Target Modulation Order for FR2-2 CQI Requirements: - Proposals: - Option 1: Up to 16 QAM (Qualcomm, Apple, Huawei); - Option 2: Include 64QAM (Nokia, Ericsson); - Option 3b: Do not limit requirements based on current TE capability (Nokia); - Recommended WF - Continue discussing it in the second round Issue 3-2-3: Whether to reuse CQI requirements test setup and metrics from FR2-1 for 120kHz/100MHz: - Proposals: - Option 1: Yes, for 120kHz/100MHz (Qualcomm, Ericsson, Apple); - Option 2: Test points: SNR = 6/7 for 16QAM, -2/-1Db for QPSK (Huawei); - Recommended WF - Continue discussing it in the second round Issue 3-2-4: CQI requirements test setup and metrics from FR2-1 for 480kHz/400MHz: - Proposals: - Option 1 (Ericsson): - Periodicity/offset = 32/1 slot and follow TDD UL-DL pattern FR2.120-2 as: - FR2-2 TDD 480 KHz: 11D S 4U, S: 2D:12G:0U - Keep same CQI delay as 120 KHz SCS, i.e., 8.375 ms. - Option 2: Do not introduce CQI Requirement test for 480kHz/400MHz; (Apple) - Recommended WF - Continue discussing it in the second round |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

### Sub Topic 3-1: SDR Demodulation Requirements

Issue 3-1-1: Scope of the FR2-2 UE SDR Requirements:

- Proposals:

  - Option 1: Do not introduce SDR requirements for FR2-2 (Qualcomm, Apple, Huawei);

  - Option 1: Define SDR tests according to the table below (Ericsson);

    - following numerologies have been considered (under PN effects): 120 KHz/100 MHz (66 RBs), 120 KHz/400 MHz (264 RBs), 480 KHz/400 MHz (66 RBs), and 480 KHz/800 MHz (124 RBs).

| Received antenna | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | PN compensation |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PRB (66) | PRB (124 or 264) |
| 2Rx UE | 1 | 6 | 1 | 27 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.8 | 23 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.75 | 22 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.4 | 14 | CPE only | CPE+ICI (u=2) |
|  |  | 4 | 1 | 16 | CPE only | CPE+ICI (u=2) |
|  |  |  | 0.4 | 10 | CPE only | CPE only |
|  |  | 2 | 1 | 9 | CPE only | CPE only |
|  |  |  | 0.4 | 4 | CPE only | CPE only |

- Recommended WF

  - Only 1 company supports introducing SDR requirements. Can Ericsson agree to option 1?

  - Agreement (GTW August 22, 2022): FFS whether SDR test cases will be specified for FR2-2 pending further checking on the test feasibility on supporting SNR range

| Company | Comments |
| --- | --- |
| Huawei | We have the concern that for 400MHz, maximum testable SNR is -0.6dB, only QPSK can be tested. We doubt the feasibility of introduce SDR test in FR2-2 band. We can deprioritize SDR test in Rel-17 and can further discuss in future release if advanced test method are introduced. |
| Apple | Based on agreement in GTW we shouldn’t be discussing this further and focus on reaching consensus on other test cases. |

Issue 3-1-2: Maximum MCS (Scaling Factor = 1) for FR2-2 in the SDR tables

- Proposals:

  - Option 1: MCS 24 for Rank1 and rank2 (Huawei);

  - Option 2: MCS 27 for Rank 1 (Ericsson, see table in Issue 3-1-1);

- Recommended WF

  - Continue discussing and decide based on simulation results;

| Company | Comments |
| --- | --- |
| Ericsson | We are fine with the recommended WF. |
| Huawei | We support option 1 based on our simulation results if SDR test is agreed |

Issue 3-1-3: Minimum MCS for FR2-2 in the SDR tables

- Proposals:

  - Option 1: MCS 0 for Rank1 and rank2 (Huawei)

  - Option 2: Depending on maximum Modulation Format (Ericsson, from table):

| Maximum Modulation Format | Minimum MCS |
| --- | --- |
| 6 | 14 |
| 4 | 10 |
| 2 | 4 |

- Recommended WF

  - Continue discussing;

| Company | Comments |
| --- | --- |
| Ericsson | Option 2. |
| Huawei | We support option 1 if SDR test is agreed. @Ericsson, based on our understanding, minimum MCS depends on testable SNR rather than maximum modulation format. |
| Ericsson2 | @ Huawei, This is the moderator’s summary, please refer to our papers (SDR) R4-2212574 and R4-2212575 |

Issue 3-1-4: Rank to use for FR2-2 SDR Requirements

- Proposals:

  - Option 1: Rank 1 and Rank 2 (Huawei, Apple);

  - Option 2: Rank 1 only (Ericsson, from table);

- Recommended WF

  - Continue discussing;

| Company | Comments |
| --- | --- |
| Ericsson | Option 2. |
| Huawei | We support option 1 if SDR test is agreed |

Issue 3-1-7: For FR1+FR2-2 CA scenarios, whether to transmit ACK/NACK separately on each CC

- Proposals:

  - Option 1: Yes (Huawei);

  - Option 2: No;

- Recommended WF

  - Continue discussing it in the second round

| Company | Comments |
| --- | --- |
| Huawei | We support option 1. Our concern is feedback on Pcell will cause much empty slots due to the TDD pattern (7D1S2U for FR1-1 band) in FR2-2 cell which will reduce the data rate. |

### Sub Topic 3-2: CQI Demodulation Requirements

Issue 3-2-1: Scope of the FR2-2 CQI Requirements:

- Proposals:

  - Option 1: Only for SCS 120kHz, CBW=100MHz (Qualcomm, Apple, Huawei);

  - Option 2: SCS=120kHz, CBW=100MHz and SCS=480kHz,CBW=400MHz (Nokia, Ericsson);

- Recommended WF

  - Continue discussing it in the second round

- Agreement (GTW August 22, 2022): Option 1 agreed, further discuss test metric and test SNR points.

| Company | Comments |
| --- | --- |

Issue 3-2-2: Target Modulation Order for FR2-2 CQI Requirements:

- Proposals:

  - Option 1: Up to 16 QAM (Qualcomm, Apple, Huawei);

  - Option 2: Include 64QAM (Nokia, Ericsson);

    - Option 3b: Do not limit requirements based on current TE capability (Nokia);

- Recommended WF

  - Continue discussing it in the second round

| Company | Comments |
| --- | --- |
| Ericsson | Option 2 |
| Huawei | Option 1 |
| Apple | Option 1 |
| Qualcomm | Option 1: Current requirements for CQI reporting in static channel targeting 64QAM are set for 14 and 15 dB. We don’t see how we can get to this SNR given current restrictions; |
| Nokia, Nokia Shanghai Bell | Option 2 |

Issue 3-2-3: Whether to reuse CQI requirements test setup and metrics from FR2-1 for 120kHz/100MHz:

- Proposals:

  - Option 1: Yes, for 120kHz/100MHz (Qualcomm, Ericsson, Apple);

  - Option 2: Test points: SNR = 6/7 for 16QAM, -2/-1Db for QPSK (Huawei);

- Recommended WF

  - Continue discussing it in the second round

| Company | Comments |
| --- | --- |
| Ericsson | Yes, Option 1. |
| Huawei | Based on our simulation results 6/7dB related to 16QAM and -2/-1dB related to QPSK is feasible. We can wait for other companies’ results. |
| Apple | Yes for test metrics and setup. Use SNR points in option 2 |
| Qualcomm | Agree regarding test setup and metrics. Postpone to the next meeting the discussion on the proposed SNR points in option 2 based on simulation results; |

Issue 3-2-4: CQI requirements test setup and metrics from FR2-1 for 480kHz/400MHz:

- Proposals:

  - Option 1 (Ericsson):

    - Periodicity/offset = 32/1 slot and follow TDD UL-DL pattern FR2.120-2 as:

    - FR2-2 TDD 480 KHz: 11D S 4U, S: 2D:12G:0U

    - Keep same CQI delay as 120 KHz SCS, i.e., 8.375 ms.

  - Option 2: Do not introduce CQI Requirement test for 480kHz/400MHz; (Apple)

- Recommended WF

  - Continue discussing it in the second round

| Company | Comments |
| --- | --- |
| Ericsson | 480 KHz will not be considered anymore. |
| Apple | Not in scope based on agreement in GTW. |

# Topic #4: Specification Structure and comment collections on Draft CRs

## Open issues summary

### Sub-topic 4-1: Specification Structure

Issue 4-1-1: Overall Specification Structure

- Proposals :

  - Option 1: PDSCH: 7.2.2.2.4, PDCCH/PBCH: Update in existing subclauses, SDR:?

  - Option 2: PDSCH: 7.2.2.2.4, PDCCH: 7.3.2.2.4, PBCH: 7.4.2.3, SDR: 7.5A.2

  - Option 3: Do not introduce new subsections. Update legacy table headings to include FR1-1. Add new table with table heading FR2-2. Change applicability section (“Applicability of requirements for operating bands”) to only required testing of table with FR2-2 in FR2-2 bands.(Nokia, Huawei, Qualcomm);

    - Option 3a: Requirement tables should specify whether they apply to FR2-1 or FR2-2, and the applicability of the requirements for operating bands should list the applicable tables separately; (Qualcomm);

    - Option 3b: Huawei, Table 2.1

- Table 2.1: Proposed positions in TS 38.101-4 for FR2-2 requirements

| Requirements | Sub-clause |
| --- | --- |
| PDSCH requirements for FR2-2 single operation: | 7.2.2.2.1 |
| PDSCH requirements for FR1+FR2-2 CA: | 9.2A.1 |
| PDSCH requirements for FR1+FR2-2 NR-DC: | 9.2B.2 |
| PDCCH requirements for FR2-2 single operation: | 7.3.2.2.2 |
| PDCCH requirements for FR1+FR2-2 CA | 9.3A.1 |
| PDCCH requirements for FR1+FR2-2 NR-DC: | 9.3.B.2 |
| PBCH requirements | 7.4.2.2 |
| SDR requirements for FR2-2 single operation: | 7.5.1 |
| SDR requirements for FR1+FR2-2 CA | 9.4A.1 |
| SDR requirements for FR1+FR2-2 NR-DC | 9.4B.2 |
| CQI requirements for FR2-2 single operation | 8.2.2.2.1 |
| CQI requirements for FR1+FR2-2 CA | 10.2A |
| CQI requirements for FR1+FR2-2 NR-DC | 10.2B.2 |

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |  |
| Apple | Option 3 since we are not introducing requirements for new features but for additional band. Also, we prefer not to change the existing tables to say FR2-1, but only introduce table headings with requirements for FR2-2 bands |
| Nokia, Nokia Shanghai Bell | Support option 3. @Apple: The general overall agreement is that FR2 refers to FR2-1 and FR2-2 combined (see our comments to DraftCR R4-2212673).. The existing tables as they would refer to FR2 in our view and not only FR2-1. In addition since RAN4 agreed to not reuse requirements from FR2-1 in FR2-2 and provide separate tables for FR2-2, we see the need to differentiate between the two. Agreement from RAN4#102e Issue 1-5-1: Implementation of FR2-2 requirements into specification Capture FR2-2 demodulation requirement into same section as FR2-1 but with different tables if possible. In our view, this can only be done by appending FR2-1 to existing tables and including FR2-2 in new tables related to FR2-2. This is also the case for the common test parameters as there might be difference between the common parameters of FR2-1 and FR2-2. |
| Qualcomm | Support option 3; We share Nokia’s view, current requirement tables should be modified to FR2-1 to avoid miscomprehension on the applicability of the requirements (we see related issues to this topic also coming up in other threads) |

Issue 4-1-2: Specification Structure for PDCCH:

- Proposals:

  - Option 1: Table below (Ericsson);

- Table 1	Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.3 | PDCCH demodulation requirements | Updates in Table 7.3-1 |
| 7.3.1 | 1RX requirements | Void |
| 7.3.2 | 2RX requirements |  |
| 7.3.2.2 | TDD |  |
| 7.3.2.2.1 | 1 Tx Antenna performances | Updates on FR2-2 |
| 7.3.2.2.2 | 2 Tx Antenna performances | Updates on FR2-2 |
| 7.4 | PBCH demodulation requirements |  |
| 7.4.1 | 1RX requirements |  |
| 7.4.2 | 2RX requirements |  |
| 7.4.2.2 | TDD | Updates on FR2-2 UEs |

  - Option 2: Nokia (according to draft CR)

| Section number | Section name | Note |
| --- | --- | --- |
| 7.3 | PDCCH demodulation requirements | Updates on FR2-2 |
| 7.3.1 | 1RX requirements | Void |
| 7.3.2 | 2RX requirements | Updates on FR2-2 |
| 7.3.2.2 | TDD |  |
| 7.3.2.2.1 | 1 Tx Antenna performances | Updates on FR2-2 |
| 7.3.2.2.2 | 2 Tx Antenna performances | Updates on FR2-2 |
| 7.4 | PBCH demodulation requirements |  |
| 7.4.1 | 1RX requirements |  |
| 7.4.2 | 2RX requirements |  |
| 7.4.2.2 | TDD | Updates on FR2-2 UEs |

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Apple | Okay with option 1. |
| Nokia, Nokia Bell Labs | Based on our comments in Issue 4-1-1 and our draftCR for PDCCH, we have the following additional option: Option 2: Section number; Section name; Note. 7.3; PDCCH demodulation requirements; Updates on FR2-2. 7.3.1; 1RX requirements; Void. 7.3.2; 2RX requirements; Updates on FR2-2. 7.3.2.2; TDD. 7.3.2.2.1; 1 Tx Antenna performances; Updates on FR2-2. 7.3.2.2.2; 2 Tx Antenna performances; Updates on FR2-2. 7.4; PBCH demodulation requirements. 7.4.1; 1RX requirements. 7.4.2; 2RX requirements. 7.4.2.2; TDD; Updates on FR2-2 UEs. |
| Qualcomm | Option 1 and 2 are aligned in structure, updates can be done where necessary to distinguish FR2-1 and FR2-2; |

Issue 4-1-3: Specification Structure for SDR:

- Proposals:

  - Option 1: Table below (Ericsson);

Table 1	Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.5 | Sustained downlink data rate provided by lower layers |  |
| 7.5.1 | FR2 single carrier requirements | Updates on FR2-2 |

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |  |
| Apple | Okay with option 1. |
| Qualcomm | Option 1; |

Issue 4-1-4: Specification Structure for CQI:

- Proposals:

  - Option 1: Table below (Ericsson);

- Table 1	Specification structure for UE CSI reporting requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 8.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 8.2.2 | 2RX requirements |  |
| 8.2.2.2 | TDD |  |
| 8.2.2.2.1 | CQI reporting definition under AWGN | Updates on FR2-2 |
| 8.2.2.2.2 | CQI reporting under fading conditions | Updates on FR2-2 |

- Recommended WF

  - TBA

| Company | Comments |  |
| --- | --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |  |
| Apple | We only agreed on introducing reqt with AWGN fr FR2-2. Fine with option 1. |
| Nokia, Nokia Shanghai Bell | Option 1 |
| Qualcomm | Option 1 – AWGN only; |

## Companies views’ collection for 1st round

### CRs/TPs comments collection

| CR/TP number | Comments collection |  |
| --- | --- | --- |
| R4- 2211871 Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GHz; Apple. | Nokia: Eventhough the current version specifically indicates the operating bands, we propose to use the term FR2-2 to have consistency across the specification (see also our comments to DraftCR: R4-2212673. In earlier agreement the FDL_high should be updated for FR2-2 to 71GHz.: From: R4-2207223: Issue 3-2-3: Other Increase the FLD_high max frequency to cover FR2-2 frequency bands, i.e. “FDL_high may not exceed 71000 MHz” and reconsider existing. |  |
| R4-2212110 Nokia_DraftCR_38101-4_PDCCH; Nokia, Nokia Shanghai Bell. | Apple: We don’t see the need to replicate the common parameters table for FR2-2. We can add the relevant updates to parameters for 480KHz SCS. Also the test parameters for 120 and 480KHz SCS shouldn’t be in separate tables. |  |
|  | Nokia: Thanks Apple for review and comments. We split the common parameters as we agreed to keep separate tables for FR2-1 and FR2-2 where possible. Also in case dedicated changes are needed for either tables, the suggested structure will be able support that. The test parameters are kept in separate tables due to having all 3 (120, 480, 960) in one table would in our view make the table too big for the page width. We understand, that 960kHz SCS is not agreed yet, however even if not agreed now, requirements for 960kHz might be added at a later point in time. |
| R4-2212578 draft CR on PDSCH requirements for 52.6 - 71 GHz band; Ericsson. | Apple: No updates? |  |
| R4-2212579 draft CR on SDR requirements for 52.6 - 71 GHz band; Ericsson. | Apple: The cover sheet shouldn’t have revision marks. These sections exist in spec, why are they with revision marks? Cannot tell where the newly added sections or requirements are. Why are CQI sections deleted? |  |
| R4-2213817 Draft CR Introduction of FR2-2 PDSCH performance requirements in TS 38.101-4; Huawei,HiSilicon. | Nokia: 960kHz SCS is not decided yet, so 960kHz should be added as [960kHz]. Also, please see our comments for our draftCR about splitting the tables. |  |
| R4-2212673 Draft CR - definition of FR2-2; Nokia, Nokia Shanghai Bell. | Apple: We prefer not to rename everything to FR2-1 in spec. We can introduce FR2-2 where necessary. |  |
|  | Nokia: Thanks Apple for the review and comments. Our understanding based on the wording from 38.101-2 v17.6.0 is that FR2 relates to both FR2-1 and FR2-2, hence any existing FR2-1 only tables, etc, would have to be specifically identified as FR2-1. Due to this, we see the new abbreviations needed: Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specification are identified as described in Table 5.1-1. Whenever the FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered, unless otherwise stated. Table 5.1-1: Definition of frequency ranges Frequency range designation; Corresponding frequency range. FR1; 410 MHz – 7125 MHz. FR2; FR2-1; 24250 MHz – 52600 MHz. FR2-2; 52600 MHz – 71000 MHz. |

## Summary for 1st round

### Open issues

| Subtopic#1 | Issue 4-1-1: Overall Specification Structure - Recommended WF: - Companies agreed on not introducing new subsections and changing applicability section (“Applicability of requirements for operating bands”) to only required testing of table with FR2-2 in FR2-2 bands. - In the second round, companies are invited to continue discussing the following details of Option 3: - 1) Whether to update existing tables by specifying FR2-1 to differentiate from FR2-2 - 2) Proposed positions according to table below Requirements; Sub-clause. PDSCH requirements for FR2-2 single operation:; 7.2.2.2.1. PDSCH requirements for FR1+FR2-2 CA:; 9.2A.1. PDSCH requirements for FR1+FR2-2 NR-DC:; 9.2B.2. PDCCH requirements for FR2-2 single operation:; 7.3.2.2.2. PDCCH requirements for FR1+FR2-2 CA; 9.3A.1. PDCCH requirements for FR1+FR2-2 NR-DC:; 9.3.B.2. PBCH requirements; 7.4.2.2. SDR requirements for FR2-2 single operation:; 7.5.1. SDR requirements for FR1+FR2-2 CA; 9.4A.1. SDR requirements for FR1+FR2-2 NR-DC; 9.4B.2. CQI requirements for FR2-2 single operation; 8.2.2.2.1. CQI requirements for FR1+FR2-2 CA; 10.2A. CQI requirements for FR1+FR2-2 NR-DC; 10.2B.2. Issue 4-1-2: Specification Structure for PDCCH: - Proposals: - Option 1: Table below (Ericsson); - Table 1 Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements). Section number; Section name; Note. 7.3; PDCCH demodulation requirements; Updates in Table 7.3-1. 7.3.1; 1RX requirements; Void. 7.3.2; 2RX requirements. 7.3.2.2; TDD. 7.3.2.2.1; 1 Tx Antenna performances; Updates on FR2-2. 7.3.2.2.2; 2 Tx Antenna performances; Updates on FR2-2. 7.4; PBCH demodulation requirements. 7.4.1; 1RX requirements. 7.4.2; 2RX requirements. 7.4.2.2; TDD; Updates on FR2-2 UEs. - Option 2: Nokia (according to draft CR) Section number; Section name; Note. 7.3; PDCCH demodulation requirements; Updates on FR2-2. 7.3.1; 1RX requirements; Void. 7.3.2; 2RX requirements; Updates on FR2-2. 7.3.2.2; TDD. 7.3.2.2.1; 1 Tx Antenna performances; Updates on FR2-2. 7.3.2.2.2; 2 Tx Antenna performances; Updates on FR2-2. 7.4; PBCH demodulation requirements. 7.4.1; 1RX requirements. 7.4.2; 2RX requirements. 7.4.2.2; TDD; Updates on FR2-2 UEs. - Recommended WF - Continue discussing in the second round Issue 4-1-3: Specification Structure for SDR: - Agreement: Table 1 Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements). Section number; Section name; Note. 7.5; Sustained downlink data rate provided by lower layers. 7.5.1; FR2 single carrier requirements; Updates on FR2-2. Issue 4-1-4: Specification Structure for CQI: - Recommended WF: - According to companies’ comments, use the structure from Table 1 below, with the exception of CQI reporting under fading conditions, for which no FR2-2 requiments are agreed to be introduced. - Table 1 Specification structure for UE CSI reporting requirements in FR2-2 (Radiated requirements). Section number; Section name; Note. 8.2; Reporting of Channel Quality Indicator (CQI). 8.2.2; 2RX requirements. 8.2.2.2; TDD. 8.2.2.2.1; CQI reporting definition under AWGN; Updates on FR2-2. 8.2.2.2.2; CQI reporting under fading conditions; Updates on FR2-2. |
| --- | --- |

### CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |
| R4- 2211871 Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GHz; Apple. | Nokia: Eventhough the current version specifically indicates the operating bands, we propose to use the term FR2-2 to have consistency across the specification (see also our comments to DraftCR: R4-2212673. In earlier agreement the FDL_high should be updated for FR2-2 to 71GHz.: From: R4-2207223: Issue 3-2-3: Other Increase the FLD_high max frequency to cover FR2-2 frequency bands, i.e. “FDL_high may not exceed 71000 MHz” and reconsider existing. |
| R4-2212110 Nokia_DraftCR_38101-4_PDCCH; Nokia, Nokia Shanghai Bell. | Apple: We don’t see the need to replicate the common parameters table for FR2-2. We can add the relevant updates to parameters for 480KHz SCS. Also the test parameters for 120 and 480KHz SCS shouldn’t be in separate tables. |
|  | Nokia: Thanks Apple for review and comments. We split the common parameters as we agreed to keep separate tables for FR2-1 and FR2-2 where possible. Also in case dedicated changes are needed for either tables, the suggested structure will be able support that. The test parameters are kept in separate tables due to having all 3 (120, 480, 960) in one table would in our view make the table too big for the page width. We understand, that 960kHz SCS is not agreed yet, however even if not agreed now, requirements for 960kHz might be added at a later point in time. |
| R4-2212578 draft CR on PDSCH requirements for 52.6 - 71 GHz band; Ericsson. | Apple: No updates? |
| R4-2212579 draft CR on SDR requirements for 52.6 - 71 GHz band; Ericsson. | Apple: The cover sheet shouldn’t have revision marks. These sections exist in spec, why are they with revision marks? Cannot tell where the newly added sections or requirements are. Why are CQI sections deleted? |
| R4-2213817 Draft CR Introduction of FR2-2 PDSCH performance requirements in TS 38.101-4; Huawei,HiSilicon. | Nokia: 960kHz SCS is not decided yet, so 960kHz should be added as [960kHz]. Also, please see our comments for our draftCR about splitting the tables. |
| R4-2212673 Draft CR - definition of FR2-2; Nokia, Nokia Shanghai Bell. | Apple: We prefer not to rename everything to FR2-1 in spec. We can introduce FR2-2 where necessary. |
|  | Nokia: Thanks Apple for the review and comments. Our understanding based on the wording from 38.101-2 v17.6.0 is that FR2 relates to both FR2-1 and FR2-2, hence any existing FR2-1 only tables, etc, would have to be specifically identified as FR2-1. Due to this, we see the new abbreviations needed: Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specification are identified as described in Table 5.1-1. Whenever the FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered, unless otherwise stated. Table 5.1-1: Definition of frequency ranges Frequency range designation; Corresponding frequency range. FR1; 410 MHz – 7125 MHz. FR2; FR2-1; 24250 MHz – 52600 MHz. FR2-2; 52600 MHz – 71000 MHz. |

## Discussion on 2nd round (if applicable)

### Sub-topic 4-1: Specification Structure

Issue 4-1-1: Overall Specification Structure

- Agreements:

  - Companies agreed on not introducing new subsections and changing applicability section (“Applicability of requirements for operating bands”) to only required testing of table with FR2-2 in FR2-2 bands.

  - In the second round, companies are invited to continue discussing the proposed locations according to table below

| Requirements | Sub-clause |
| --- | --- |
| PDSCH requirements for FR2-2 single operation: | 7.2.2.2.1 |
| PDSCH requirements for FR1+FR2-2 CA: | 9.2A.1 |
| PDSCH requirements for FR1+FR2-2 NR-DC: | 9.2B.2 |
| PDCCH requirements for FR2-2 single operation: | 7.3.2.2.2 |
| PDCCH requirements for FR1+FR2-2 CA | 9.3A.1 |
| PDCCH requirements for FR1+FR2-2 NR-DC: | 9.3.B.2 |
| PBCH requirements | 7.4.2.2 |
| SDR requirements for FR2-2 single operation: | 7.5.1 |
| SDR requirements for FR1+FR2-2 CA | 9.4A.1 |
| SDR requirements for FR1+FR2-2 NR-DC | 9.4B.2 |
| CQI requirements for FR2-2 single operation | 8.2.2.2.1 |
| CQI requirements for FR1+FR2-2 CA | 10.2A |
| CQI requirements for FR1+FR2-2 NR-DC | 10.2B.2 |

- Recommended WF

  - Continue discussing in the second round

| Company | Comments |
| --- | --- |
| Huawei | We are OK with this table. SDR test should depends on outcome of corresponding open issues. We propose to remove all requirements with NR-DC since RF didn’t define any scenario for DC combination. |
| Apple | Same comments as Huawei. Also need to avoid replication of common parameter tables. 120KHz, 480KHz SCS requirements can be introduced in the same table. |
| Nokia, Nokia Shanghai Bell | We will keep the common table as is with no corrections. |
| Ericsson3 | For PDSCH we can the Table as follow (as proposed in R4-2212570) Section number; Section name; Note. 7.2; PDSCH demodulation requirements; Update Table 7.2-1. 7.2.1; 1RX requirements. 7.2.2; 2RX requirements. 7.2.2.2; TDD. 7.2.2.2.[4]; Minimum requirements for UEs in FR2-2; New section. 7.2A; PDSCH demodulation requirements for CA. 7.2A.1; 1RX requirements. 7.2A.2; 2RX requirements; Updates on FR2-2. …….; ……. A.3.2; Reference measurement channels for PDSCH performance requirements. A.3.2.2; TDD; Updates on FR2-2. We made 7.2.2.2.[4] as optional, because further enhancement on PDSCH would be discussed in the future. Furthermore, we may update the appendix with new Ref channels for PDSCH. |

Issue 4-1-2:Whether to update existing tables by specifying FR2-1 to differentiate from FR2-2

- Proposals:

  - Option 1: Yes;

  - Option 2: No;

- Recommended WF

  - TBA;

| Company | Comments |
| --- | --- |
| Huawei | We prefer option 1. |
| Apple | If we have clear applicability rules, I don’t see the need to update existing tables. |
| Qualcomm | In general we support Option 1 but we think it should be clarified what we expect to be differentiated. Unless they contain mentions to FR2, table names and descriptions should not specify their applicability, use only properly written applicability rules as proposed by Apple; If the tables reference FR2 in their rows, then it should be updated to FR2-1 to avoid potential confusion. For example, TDD DL/UL patterns such as FR2.120-1 should be modified to FR2-1.120-1. |
| Nokia, Nokia Shanghai Bell | We support option 1. If tables are not updated, all tables without specific indication would apply to both FR2-1 and FR2-2. Forcing an update will ensure that defined requirements only apply to the feature they are intended for. Any tables without indication can be updated to indicate FR2 if they cover both FR2-1 and FR2-2. Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specification are identified as described in Table 5.1-1. Whenever the FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered, unless otherwise stated. Table 5.1-1: Definition of frequency ranges Frequency range designation; Corresponding frequency range. FR1; 410 MHz – 7125 MHz. FR2; FR2-1; 24250 MHz – 52600 MHz. FR2-2; 52600 MHz – 71000 MHz. |

Issue 4-1-2: Specification Structure for PDCCH:

- Proposals:

  - Option 1: Table below (Ericsson);

- Table 1	Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.3 | PDCCH demodulation requirements | Updates in Table 7.3-1 |
| 7.3.1 | 1RX requirements | Void |
| 7.3.2 | 2RX requirements |  |
| 7.3.2.2 | TDD |  |
| 7.3.2.2.1 | 1 Tx Antenna performances | Updates on FR2-2 |
| 7.3.2.2.2 | 2 Tx Antenna performances | Updates on FR2-2 |
| 7.4 | PBCH demodulation requirements |  |
| 7.4.1 | 1RX requirements |  |
| 7.4.2 | 2RX requirements |  |
| 7.4.2.2 | TDD | Updates on FR2-2 UEs |

  - Option 2: Nokia (according to draft CR)

| Section number | Section name | Note |
| --- | --- | --- |
| 7.3 | PDCCH demodulation requirements | Updates on FR2-2 |
| 7.3.1 | 1RX requirements | Void |
| 7.3.2 | 2RX requirements | Updates on FR2-2 |
| 7.3.2.2 | TDD |  |
| 7.3.2.2.1 | 1 Tx Antenna performances | Updates on FR2-2 |
| 7.3.2.2.2 | 2 Tx Antenna performances | Updates on FR2-2 |
| 7.4 | PBCH demodulation requirements |  |
| 7.4.1 | 1RX requirements |  |
| 7.4.2 | 2RX requirements |  |
| 7.4.2.2 | TDD | Updates on FR2-2 UEs |

- Recommended WF

  - Continue discussing in the second round

| Company | Comments |
| --- | --- |
| Huawei | We prefer option 1. We don’t see much difference for common test parameters and we can updates to existing table. |
| Apple |  |
| Qualcomm | Support option 1, we would prefer the common test table to be unique and common to all tests. FR2-2 specific changes can be written in their own sections. |
| Nokia, Nokia Shanghai Bell | We can agree with option 1 and will change our draftCR accordingly so the common section is not updated. |

### Comments on Revised CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |
| R4- 2211871 Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GHz; Apple. |  |
| R4-2212110 Nokia_DraftCR_38101-4_PDCCH; Nokia, Nokia Shanghai Bell. | Qualcomm: Do not split common test parameters. |
| R4-2212578 draft CR on PDSCH requirements for 52.6 - 71 GHz band; Ericsson. | Moderator: Original CR received no comments. Postponed to the next meeting; |
| R4-2212579 draft CR on SDR requirements for 52.6 - 71 GHz band; Ericsson. | Moderator: SDR requirements are postponed FFS pending checking on the test feasibility on supporting SNR range; |
| R4-2213817 Draft CR Introduction of FR2-2 PDSCH performance requirements in TS 38.101-4; Huawei,HiSilicon. |  |
| R4-2212673 Draft CR - definition of FR2-2; Nokia, Nokia Shanghai Bell. |  |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on FR2-2 UE demodulation requirements | Qualcomm |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4- 2211871 | R4-2214751 | Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GHz | Apple | Revised |  |
| R4-2212110 | R4-2214756 | Nokia_DraftCR_38101-4_PDCCH | Nokia | Revised |  |
| R4-2212578 | R4-2214783 | draft CR on PDSCH requirements for 52.6 - 71 GHz band | Ericsson | Revised |  |
| R4-2212579 | R4-2214784 | draft CR on SDR requirements for 52.6 - 71 GHz band | Ericsson | Revised |  |
| R4-2213817 | R4-2214848 | Draft CR Introduction of FR2-2 PDSCH performance requirements in TS 38.101-4 | Huawei,HiSilicon | Revised |  |
| R4-2212673 | R4-2215138 | Draft CR - definition of FR2-2 | Nokia, Nokia Shanghai Bell | Revised |  |

## 2nd round

| Tdoc | Revised to | Title | Source | Recommendation | Comment |
| --- | --- | --- | --- | --- | --- |
| R4-2214390 | - | WF on FR2-2 UE demodulation requirements | Qualcomm | Agreeable |  |
| R4-2214751 | - | Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GHz | Apple | Agreeable |  |
| R4-2214756 | - | Nokia_DraftCR_38101-4_PDCCH | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2214783 | - | draft CR on PDSCH requirements for 52.6 - 71 GHz band | Ericsson | Agreeable |  |
| R4-2214784 | - | draft CR on SDR requirements for 52.6 - 71 GHz band | Ericsson | Agreeable |  |
| R4-2214848 | - | Draft CR Introduction of FR2-2 PDSCH performance requirements in TS 38.101-4 | Huawei,HiSilicon | Agreeable |  |
| R4-2215138 | - | Draft CR - definition of FR2-2 | Nokia, Nokia Shanghai Bell | Agreeable |  |