3GPP TSG-RAN4 Meeting #104-e		R4-2214716

# Electronic Meeting, Aug 15 - Aug 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | 1 | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.133: Correction to conditional handover requirements (Rel-16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the conditional measurement event of CHO is triggered, the conditional handover would be directly initiated. UE is not required to report SSB based RRM measurement result related to MO for CHO. SSB index detection is not necessary from measurement report perspective as in section 9 for intra-frequency and inter-frequency measurement. Therefore, for intra-frequency in FDD FR1, it is unreasonable to split the measurement delay of CHO into two scenarios, i.e., with acquiring the index of the SSB or without acquiring the index of the SSB. However, for FDD FR1, SSB index needs to be known for target PCell handover. Therefore, for FDD FR1, the time for CHO measurement needs to include SSB index detection time. Tmeasure in CHO should be Tidentify_intra_with_index. For TDD FR1 and FR2, the time for CHO measurement does not need to include SSB index detection time. Tmeasure in CHO should be Tidentify_intra_without_index. And for inter-frequency, it is also unreasonable to split the measurement delay of CHO into two scenarios, i.e., with acquiring the index of the SSB or without acquiring the index of the SSB. However, SSB index needs to be known for target PCell handover. Therefore, the time for CHO measurement needs to include SSB index detection time. Tmeasure in CHO should be Tidentify_inter_with_index. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For intra-frequency, add corresponding applicable scenarios to the cases in which UE does not/does need to acquire the index of the SSB in measurement time in conditional handover requirement - For intra-frequency measurement in TDD FR1 , add the note of applicable scenarios to Tidentify_intra_without_index in measurment time in conditional handover requirement - For intra-frequency measurement in FDD FR1, add the note of applicable scenario to Tidentify_intra_with_index in measurment time in conditional handover requirement - For intra-frequency measurement in FR2, delete Tidentify_inter_witht_index in measurment time in conditional handover requirement - For inter-frequency, delete the case in which UE needs not to acquire the index of the SSB in measurement time in conditional handover requirement - For inter-frequency measurement, delete Tidentify_inter_without_index in measurment time in conditional handover requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel-16 condtional handover requirements are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2213041 |  |  |  |  |  |  |  |  |

# <Start of change 1>

6.1.4	NR Conditional Handover

6.1.4.1	Introduction

The requirements in this clause are applicable to conditional handover to change the NR PCell to another NR cell.

6.1.4.2	NR FR1 – NR FR1 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR1 cell to NR FR1 cell.

##### 6.1.4.2.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover.

Tmeasure is the measurements time stated in clause 6.1.4.2.2.

TCHO_execution is the conditional execution preparation time in clause 6.1.4.2.3.

Tinterrupt is the interruption time stated in clause 6.1.4.2.4.

##### 6.1.4.2.2	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index (for FDD FR1) or Tidentify_intra_without_index (for TDD FR1)defined in clause 9.2.5.1 or clause 9.2.6.2.

For inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index defined in clause 9.3.4.

When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_with_index for inter-frequency handover. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_with_index for inter-frequency handover becomes undetectable for a period and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200/2µ Tc while the measurement gap has not been available and the L3 filter has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

##### 6.1.4.2.3	Preparation time

TCHO_execution is the UE execution preparation time for conditional handover, and starts after UE realizes the condition of CHO is met and identity of the target cell is determined. TCHO_execution can be up to 10ms.

##### 6.1.4.2.4	Interruption time

The interruption time is the time between when the UE starts to execute the conditional handover to the target cell and the time the UE starts transmission of the new PRACH.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms

Where:

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3]

T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

6.1.4.3	NR FR2 – NR FR1 conditional handover

The requirements in this clause are applicable to inter-frequency conditional handover from NR FR2 cell to NR FR1 cell.

The requirements defined in clause 6.1.4.2 applies assuming inter-frequency handover and:

Tprocessing is time for UE processing. Tprocessing can be up to 40ms.

6.1.4.4	NR FR2 – NR FR2 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR2 cell to NR FR2 cell.

6.1.4.4.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover.

Tmeasure is the measurements time stated in clause 6.1.4.4.2.

TCHO_execution is the conditional execution preparation time in clause 6.1.4.4.3.	Tinterrupt is the interruption time stated in clause 6.1.4.4.4.

6.1.4.4.2	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

For inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index defined in clause 9.3.4.

When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_with_index for inter-frequency handover. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_with_index for inter-frequency handover becomes undetectable for a period and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200/2µ Tc while the measurement gap has not been available and the L3 filter has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

6.1.4.4.3	Preparation time

TCHO_execution is the UE execution preparation time for conditional handover, and starts after UE realizes the condition of CHO is met and identity of the target cell is determined. TCHO_execution can be up 10ms.

6.1.4.4.4	Interruption time

The interruption time is the time between when the UE starts to execute the conditional handover to the target cell and the time the UE starts transmission of the new PRACH.

For intra-frequency or inter-frequency conditional conditional handover, the measurment time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms

Where:

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3]

T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

6.1.4.5	NR FR1 – NR FR2 conditional handover

The requirements in this clause are applicable to inter-frequency conditional handover from NR FR1 cell to NR FR2 cell.

The requirements defined in clause 6.1.4.4 applies assuming inter-frequency handover and:

Tprocessing is time for UE processing. Tprocessing can be up to 40ms.

# <End of change 1>