3GPP TSG-RAN WG4 Meeting #104-e	R4-2215010

Electronic Meeting, 15 August – 26 August 2022

Source:	Ericsson, Telstra

Title:	TP for TR 37.718-21-11: Including band combinations DC_28-(n)7

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.718-21-11 to add DC_28A-(n)7AA.

# 2. Text Proposal

---Start of changes---

## 5.x	DC_28-(n)7

### 5.x.1	Operating bands for EN-DC

Table 5.x.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 28-(n)7 | CA_28-7 | n7 | No |

### 5.x.2	Configuration for DC

Table 5.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_28A-(n)7AA | DC_28A_n7A | CA_28A-7A | n7A |

### 5.x.3		∆TIB and ∆RIB values

For DC_28-(n)7, the TIB,c and RIB,c values are reused from DC_28_n7 and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28-(n)7 | 28 | 0.3 |
|  | 7 | 0.3 |
|  | n7 | 0.3 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28-(n)7 | 28 | 0 |
|  | 7 | 0 |
|  | n7 | 0 |

### 5.x.4	REFSENS requirements

There are no IMD impact from UL 28_7 affecting DL band 28 or band n7.

---End of changes---

# Reference

[1]		RP-221832, New WID: Rel-18 Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei, HiSilicon