3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215326

# Electronic Meeting, October 10 – October 19, 2022

Source:	China Telecom

Title:	On BS PUSCH demodulation requirements for NR coverage enhancements

Agenda Item:	4.4.2.1

Document for:	Discussion

- Introduction

In the RAN4 #104-e meeting, the BS PUSCH demodulation requirements for NR coverage enhancements were discussed. As a outcome, agreements on most of the test parameters were reached and captured in the chairman notes.

In this paper, we provide our views on the remaining issues.

- Discussion

SNR requirement value deriving rule for both PUSCH and PUCCH BE demod requirements

Considering this meeting is scheduled to be the last meeting and we have already collected simulation results from at least 4 companies for each test case, we will need to make decision on the SNR requirement deriving rule, to assist the implementation of the draft CRs.

We propose to reuse the Rel-15 BS demodulation SNR requirement derivation procedure as agreed in [1], which was also used in Rel-17 PUSCH 256QAM demodulation requirements.

Proposal 1: Reuse the SNR requirement value deriving rule for Rel-15 BS demodulation SNR requirement derivation procedure as agreed in [1]:

Procedure to derive the performance requirements: – Only inputs that consist of a pair of ideal and impaired results can be taken into account. – If the ideal span <= [2]dB: • The AVERAGE impairment results can be used for the performance requirement with [] in the draftCRs/CRs; – Else if the ideal span is larger than [2]dB: • The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available: – The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the draftCRs/CRs. • Otherwise put TBD for the related performance requirements. – If the span of the impairment results after removal the outliers (if any) are larger than 4dB, then the procedure cannot be applied, related performance requirement remain TBD.

Number of HARQ process for BS PUSCH demod requirements with JCE

Status in the Chairman note:

  - Agreement: Assuming no such limitation needed as baseline and further check in next meeting based on the simulation results

In the last meeting, there was concern from company that the configuration of different number of HARQ process in the test, will impact the PUSCH JCE performance due to the time diversity gain. In this paper, we provide simulation results of PUSCH JCE under different HARQ process number configuration under TDD 30kHz SCS with 7D1S2U pattern as shown in Table 1.

Table 1. Simulation results for PUSCH JCE under different HARQ process number under TDD 30kHz SCS with 7D1S2U pattern

| Duplex Mode | Antenna Configuration | DMRS | HARQ process number |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 2 | 4 | 8 |
| TDD | 1T2R | DMRS 1+0 | -4.2 | -4.4 | -4.5 |
|  |  | DMRS 1+1 | -4.9 | -5.0 | -5.0 |
|  | 1T4R | DMRS 1+0 | -8.0 | -8.1 | -8.3 |
|  |  | DMRS 1+1 | -8.7 | -8.9 | -8.9 |
|  | 1T8R | DMRS 1+0 | -11.0 | -11.1 | -11.2 |
|  |  | DMRS 1+1 | -11.9 | -11.9 | -12.0 |

Based on the above simulation results, no big performance difference is observed under practical HARQ process number of 2/4/8, due to the negligible time diversity gain under the low speed channel model (TDLA30-10) we have agreed.

Considering the above, we propose not to specify the number of HARQ process which is aligned with all other existing BS demodulation requirements in the spec.

Observation 1: No big performance difference is observed under practical HARQ process number of 2/4/8, due to the negligible time diversity gain under the low speed channel model (TDLA30-10) we have agreed.

Proposal 2: Not to specify the number of HARQ process which is aligned with all other existing BS demodulation requirements in the spec.

- Conclusion

In this paper, China Telecom’s views on the remain issues for PUSCH coverage enhancements are given.

Proposal 1: Reuse the SNR requirement value deriving rule for Rel-15 BS demodulation SNR requirement derivation procedure as agreed in [1]:

Procedure to derive the performance requirements: – Only inputs that consist of a pair of ideal and impaired results can be taken into account. – If the ideal span <= [2]dB: • The AVERAGE impairment results can be used for the performance requirement with [] in the draftCRs/CRs; – Else if the ideal span is larger than [2]dB: • The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available: – The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the draftCRs/CRs. • Otherwise put TBD for the related performance requirements. – If the span of the impairment results after removal the outliers (if any) are larger than 4dB, then the procedure cannot be applied, related performance requirement remain TBD.

Observation 1: No big performance difference is observed under practical HARQ process number of 2/4/8, due to the negligible time diversity gain under the low speed channel model (TDLA30-10) we have agreed.

Proposal 2: Not to specify the number of HARQ process which is aligned with all other existing BS demodulation requirements in the spec.

- References

R4-1904713, WF for requirement SNR derivation procedure, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, RAN4 #90bis, April 2019.