3GPP TSG-RAN WG4 Meeting # 104b-e	R4-2215330

# Electronic Meeting, October 10 - 19, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	A-MPR for PC1.5 TDD bands

Agenda Item:	5.21.2 UE RF requirements [HPUE_NR_FR1_TDD_R18]

Document for:	Approval

# Introduction

In RP#97e, a Release 18 new work item [1] was agreed to introduce PC1.5 UEs in more TDD bands; this includes bands 34, 39 and 40 that are in the same frequency range than n41 already specified. In this contribution we discuss the related A-MPR aspects for PC1.5 in these bands.

# Discussion

## Applicable NS for requested PC1.5 TDD bands

For new PC1.5 TDD bands on [1], since PC1.5 MPR already exists for both smartphone and FWA UEs, the only aspect to be further specified is the need for A-MPR related to a given NS. Table 1 lists the applicable NS beyond NS_01 for the requested PC1.5 TDD bands.

Table 1: applicable NS for PC1.5 TDD bands

| NR band | n34 | n39 | n40 |
| --- | --- | --- | --- |
| Applicable NS |  | NS_50 |  |

The only applicable NS is NS_50 for band n39. Thus, A-MPR is needed for this, but we assume that only the smartphone case should be defined using the same RIMD assumptions (10dB antenna coupling) that has been used for MPR.

Proposal:

- No additional A-MPR work for PC1.5 n34 and n40, PC1.5 can be added directly to those bands.

- PC1.5 n39 NS_50 A-MPR is needed for smartphone UE, 10dB antenna isolation is used like for the related PC1.5 MPR.

## NS_50 A-MPR Requirements for n39

Table 2 is a copy of the NS_50 additional emissions requirement from 38.101-1 which needs to be fulfilled for PC1.5.

Table 2: Additional requirements for NS_50

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 1805 | - | 1855 | -40 | 1 | 1 |
| Frequency range | 1855 | - | 1880 | -15.5 | 5 | 1, 2, 3 |
| NOTE 1: This requirement is applicable for carriers with aggregated channel bandwidths confined in 1885-1920 MHz for ≤ 30MHz channel BWs and confined in 1880-1920 MHz for 40MHz channel BW. NOTE 2: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

## Current PC3 and PC2 A-MPR for n39

The following tables provide the NS_50 A-MPR backoff and regions for PC3 and PC2:

- Table 3 is for PC3 A-MPR regions

- Table 4 is for PC3 A-MPR values

- Table 5 is for PC2 A-MPR regions

- Table 6 is for PC3 A-MPR values

Table 3: A-MPR regions for NS_50 (Power Class 3)

| CBW (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 25 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 20 | ≤ 1.44 | A8 |
| 30 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 25 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32, ≤ 10.44 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8 | A2 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A6 |
|  | > 31.68 | > 0 | A5 |

Table 4: A-MPR for PC3 NS_50 (DFT-s-OFDM/CP-OFDM)

| Modulation | A1 (dB) | A2 (dB) | A3 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| Pi/2 BPSK | ≤ 11/NA | ≤ 7/NA | ≤ 3/NA | ≤ 5/NA | ≤ 2/NA | ≤ 4/MA | ≤ 2 |
| QPSK | ≤ 11/12 | ≤ 7/8 | ≤ 3/4.5 | ≤ 5/5 | ≤ 2/3.5 | ≤ 5/6.5 | ≤ 2 |
| 16 QAM | ≤ 11/12 | ≤ 7/8 | ≤ 3/4.5 | ≤ 5/5 | ≤ 2/3.5 | ≤ 5/6.5 | ≤ 2.5 |
| 64 QAM | ≤ 11/12 | ≤ 7/8 | ≤ 3/4.5 | ≤ 5/5 |  | ≤ 5/6.5 |  |
| 256 QAM | ≤ 11/12 | ≤ 7/8 |  | ≤ 5/0 |  | ≤ 5/6.5 |  |

Table 5: A-MPR regions for NS_50 (Power Class 2)

| CBW (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10 MHz | ≤ 1.44 | < 1.44 | A5 |
|  | ≤ 1.8 | ≥ 2.7+2* RBstart*12*SCS | A4 |
|  | >1.8 | ≥ 8.1- RBstart*12*SCS | A4 |
| 15 MHz | ≤ 2.88 | < 2.7 | A5 |
|  | ≤ 3.24 | ≥ 2.7+2* RBstart*12*SCS | A3 |
|  | >3.24 | ≥ 12.42- RBstart*12*SCS | A4 |
| 20 MHz | ≤ 4.32 | < 3.6 | A5 |
|  | ≤ 4.5 | ≥ 3.6+2* RBstart*12*SCS | A3 |
|  | >4.5 | ≥ 17.1- RBstart*12*SCS | A4 |
| 25 MHz | ≤ LCRB*12*SCS – 5 | > 5 | A2 |
|  | ≤ 6.3 | ≤ 1.44 | A5 |
|  | > 8.28 | > max (21.6 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | >1.8, ≤6.12 | > 1.44, ≤ 3.6 | A6 |
|  | > LCRB *12*SCS – 5, ≤ 5.04 | > 1.44 | A4 |
| 30 MHz | ≤ LCRB*12*SCS – 5 | >5 | A2 |
|  | ≤ 7.56 | ≤ 1.44 | A5 |
|  | >1.8, ≤7.56 | > 1.44, ≤ 3.6 | A6 |
|  | ≤ 1.8 | >1.44, <RBstart*12*SCS+5 | A4 |
|  | > 10.8 | > max (26.64 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32 | > RBstart*12*SCS + 11.88 | A1 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8, <= RBstart*12*SCS + 11.88 | A7 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A4 |
|  | > 31.68 | > 0 | A1 |

Table 6: A-MPR for PC2 NS_50 (DFT-s-OFDM/CP-OFDM)

| Modulation | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| Pi/2 BPSK | ≤ 11.5/NA | ≤4.5/NA | ≤ 4/NA | ≤ 2.5/NA | ≤ 4/NA | ≤ 1/NA | ≤ 3.5/NA |
| QPSK | ≤ 11.5/12.5 | ≤ 5.5/7 | ≤ 4/5.5 | ≤ 2.5/4 | ≤ 4/4 | ≤ 1/2 | ≤ 3.5/5 |
| 16 QAM | ≤ 11.5/12.5 | ≤ 5.5/7 | ≤ 4/5.5 | ≤ 2.5/4 | ≤ 4/4 | ≤ 1.5/2 | ≤ 3.5/5 |
| 64 QAM | ≤ 11.5/12.5 | ≤ 5.5/7 | ≤ 4/5.5 |  | ≤ 4/4 |  | ≤ 3.5/5 |
| 256 QAM | ≤ 11.5/12.5 | ≤ 5.5/7 |  |  |  |  |  |

When comparing PC3 and PC2 A-MPR requirements we can observe the following:

- For PC2, 10, 15 and 20MHz channel bandwidths require A-MPR where MPR is sufficient for PC3.

- For 25, 30 and 40MHz channel bandwidths that require A-MPR for both PC3 and PC2, the A-MPR regions are different with wider and more regions needed for PC2.

- The worst-case A-MPR region is the same for PC3 and PC2 (40MHz CBW region A1) with 0.5dB higher back-off for PC2. These are highlighted in yellow.

- Also, the worst case region for 20 and 30MHz are the same for PC2 (region A2) and PC3 (region A7) again with 0.5dB higher back-off for PC2. These are highlighted in cyan.

This can be explained by the following facts:

- The NS_50 spectrum emission requirement is absolute

- PC2 power in 3dB higher than PC3 thus emissions are higher

- PC2 ACLR is 31dB versus PC3 at 30dB, thus slightly more linear

If the same PA linearity was set for PC2 and PC3 (same ACLR), the emissions would be 3dB higher for PC2 and would thus require 1dB higher back-off for 3rd order related non-linearities. But since PC2 ACLR is 31dB, it is slightly more linear than PC3 which explains why 0.5dB higher back-off is sufficient.

However, for PC1.5, the ACLR is 31dB like for PC2 and the emissions are basically 2x the PC2 emissions plus RIMD contribution:

- For the worst-case region, without accounting for reverse IMD, the emission level is 3dB higher requiring 1dB additional back-off.

- To further account for RIMD in the same way that for the PC1.5 MPR for smartphone, the back-off need a further increase of 0.5dB.

- Given the higher emissions, it is possible the A-MPR is needed for 5MHz and new/different A-MPR regions should be defined.

Proposal for PC1.5 NS_50 A-MPR:

- For PC1.5, the worst case PC2 regions A1 and A2 need 1.5dB higher back-off to account for 3dB higher emissions per antenna and the additional contribution from RIMD.

- A-MPR for 5MHz should be investigated.

- New or different A-MPR regions may need to be defined.

# Conclusions

In this contribution, we discuss the need for A-MPR for PC1.5 requested for n34, n39 and n40 and make following proposals.

Proposal:

- No additional A-MPR work for PC1.5 n34 and n40, PC1.5 can be added directly to those bands.

- PC1.5 n39 NS_50 A-MPR is needed for smartphone UE, 10dB antenna isolation is used like for the related PC1.5 MPR.

Based on only needing NS_50 for n39 and the above study of the existing PC3 and PC2 A-MPR, we formulate the following proposal.

Proposal for PC1.5 NS_50 A-MPR:

- For PC1.5, the worst case PC2 regions A1 and A2 need 1.5dB higher back-off to account for 3dB higher emissions per antenna and the additional contribution from RIMD.

- A-MPR for 5MHz should be investigated.

- New or different A-MPR regions may need to be defined.

# References

- RP-222351 New WID: High power UE (power class 1.5) for NR TDD bands, CMCC, RP#97e