3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215378

10th Oct – 19th Oct, 2022

Source:	Qualcomm Incorporated

Title:	Signalling for low MSD

Agenda item:	6.6.4.2

Document for:	Approval

# 1.	Introduction

In RAN4#104-e there was much discussion on introducing lower MSD for certain band combinations and how this should be signalled. However, there was little agreement on what should be signalled and what it should apply to. This paper discusses various signalling schemes that can be adopted for lowering MSD.

# 2. 	Discussion

In RAN4#104-e the agreed WF [1] proposed continuing discussion of how to signal lower MSD as no agreement was made in this meeting.

In the last meeting it was discussed that certain band combinations may have several different types of impairments. The issue is whether to report only the lowest MSD for all impairments for a given band combination or to report MSDs for all of them. This is an issue between reduced complexity in having only one MSD entry for a given band combination and greater accuracy in cancellation where the values for each impairment type is specified individually. Realizing that these impairments do not usually occur at the same frequency we think that it would be beneficial to have only the largest MSDs for a given band combination above a predetermined threshold capable of being reduced using the low MSD feature. This would mean that some band combinations may have several MSD impairments that can be cancelled while others have only one. A UE should be able to indicate which impairments it can support and declare the lower MSD value it can support for each impairment in each band combination. We think that it would be difficult for companies to agree on specific low MSD values for each band combination where lower MSD is possible. So, we think the ability for the UE to declare the achievable lower MSD as an easier method for implementing this feature.

Observation 1: It would be difficult for companies to agree on specific low MSD values for each band combination where lower MSD is possible. We see the ability for the UE to declare the achievable lower MSD for each band combination as an easier method for implementing this feature

Proposal 1: For each band combination that can support low MSD allow the UE to declare which impairment improvement it supports (IMD2, IMD4, HD2 etc.) and the associated lower MSD value for each impairment using capability signalling.

Proposal 2: The resolution of the UE declared low MSD value is [1.0] dB

We think that the threshold for lower MSD reporting should be further discussed in RAN4.

Proposal 3: Further discuss the lower MSD threshold which should be agreed for a given band combination to qualify for MSD reduction.

For the high MSD band combinations if a network knows that a UE is capable of lower MSD then it will alter its scheduling behaviour to account for this. For example, in a ULCA scenario if the network knows a UE is capable of lower MSD it can schedule both bands simultaneously in the direct hit frequencies which it may not have been able to do before. This is one of the main advantages we see in the network knowing that certain UEs can achieve lower MSDs.

Observation 2: Knowing a UE can achieve lower MSDs enables a network to schedule carriers more efficiently.

# Conclusion

As per the guidelines stated in the latest WF [1] we propose the following for low MSD signalling:

Observation 1: It would be difficult for companies to agree on specific low MSD values for each band combination where lower MSD is possible. We see the ability for the UE to declare the achievable lower MSD for each band combination as an easier method for implementing this feature

Proposal 1: For each band combination that can support low MSD allow the UE to declare which impairment improvement it supports (IMD2, IMD4, HD2 etc.) and the associated lower MSD value for each impairment using capability signalling.

Proposal 2: The resolution of the UE declared low MSD value is [1.0] dB

Proposal 3: Further discuss the lower MSD threshold which should be agreed for a given band combination to qualify for MSD reduction.

Observation 2: Knowing a UE can achieve lower MSDs enables a network to schedule carriers more efficiently.

# References

[1] R4-2214452, WF on study for lower MSD, RAN4#104-bis-e, Aug. 15-26, 2022