3GPP TSG-RAN WG4 Meeting # 104bis-e                                                       R4-2215386

# Electronic Meeting, October 10 – October 19, 2022

Title: 	Further discussion on ATG co-existence simulation assumption

Source: 	CATT

Agenda item:	6.13.2

Document for:	Discussion

- Background

Some preliminary co-existence simulation assumptions were agreed in last meeting [1]. This contribution provides our further consideration of the remaining open items.

- Discussion

  - General issues

The following table captures our views on the open issues in last meeting.

Table 1: General issues assumption

| Issues | Views from our company | Comments |
| --- | --- | --- |
| Whether ATG BS can be co-located with TN BS | Consider the co-located if it’s a valid deployment scenario. |  |
| Assumption on the density of aircraft within the simulation area | Option 1: single UE per ATG BS as a starting point |  |
| ATG ISD | 200km is achievable | This is related to BS, UE power assumptions. |
| UE altitude | High altitude: 10km Low altitude: 3km | The worst case for each scenario is chosen. |
| Network layout of ATG | Option 2: ATG network can be deployed in larger area | Two reasons: Large area may be more stringent than the airline deployment. The layout of option 2 can refer some existing layouts. |

  - Co-existence system parameters

BS parameters are shown in Table 2.

Table 2: BS system parameters

| UE distribution | Randomly |
| --- | --- |
| Indoor UE percentage | 0% |
| Number of DL active UEs per cell (NOTE 2) | 1 |
| Number of UL active UEs per cell (NOTE 2) | 1 |
| DL scheduled bandwidth per UE | Full BW |
| UL scheduled bandwidth per UE | [1/2 full BW] |
| Traffic model | Full buffer |
| ATG BS maximum output power | [50 dBm] |
| ATG BS noise figure | 5dB |
| Handover margin | 3dB |
| NOTE 3: ATG BS max TX power is defined per polarization |  |

UE parameters are shown in the following table.

Table 3: UE system parameters

| ATG UE max TX power in dBm | [40 dBm] |
| --- | --- |
| ATG UE min TX power in dBm | FFS |
| ATG UE noise figure | 9 dB |

TN BS and TN UE system parameters can refer TR 38.863.

  - Antenna and beamforming pattern modelling

The following table is our proposal for ATG BS antenna parameters.

Table 4: ATG BS AAS antenna parameters

| Antenna pattern | TR 38.921 |
| --- | --- |
| Element gain (dBi) | 7.1 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | [8 × 8] elements |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 25 |
| Base station maximum coverage angle in the horizontal plane (degrees) | 120 |
| Base station vertical coverage range (degrees) | 80-90 |
| Mechanical up-tilt (degrees) | 3 |

For ATG UE antenna, we support option 3: antenna mounted on the aircraft are directional without beam steering capability.

For TN BS and UE antenna parameters, they can refer TR 38.863.

- Summary

This contribution provides our views on the remaining co-existence simulation assumptions.

Reference

[1] R4-2214459, “WF on co-existence evaluation for ATG”, Huawei, RAN4#104e