3GPP TSG-RAN WG4 Meeting # 104bis-e   									R4-2215486

Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.17.2.1

Source: 	CMCC

Title: 	Study on the simulation assumption for adjacent channel co-existence

Document for:	Discussion

# 1. Introduction

In last meeting, a WF on Simulation assumption for adjacent co-existence study has been approved [1]. There are still some open issues. In this meeting, we focus on the remaining issues discussion for adjacent channel co-existence.

# 2. Discussion

From our understanding, it’s better to align SBFD sub-band configuration with RAN1 since final ACLR will be used in the same simulation. For RAN1, there is still no final agreement now. SBFD UL sub-band is about [20%] of the channel bandwidth with 20% agreement in the square bracket.

Proposal 1: for SBFD configuration, it’s suggested to be aligned with RAN1 agreements and 20% UL sub-band could be taken as the starting point. i.e. {D,U,D}={40M,20M,40M} and {D,U}={80M,20M}

For the propagation loss, there is no final agreements. We need to compare propagation loss model for following three cases:

| gNB to UE | 38.803 or 38.901 |
| --- | --- |
| gNB to gNB | 38.803 or 38.901 |
| UE to UE | 36.828 or 38.901 or 38.802 |

In the following we focus on the analysis of differences for PL models between 38.803 and 38.901.

- For gNB-UE model,

For UMa, the main difference comes from LOS PL1 model. Comparison of PL1 models between 38.803 and 38.901 is listed as below in fig1. It seems at least for UMa, such two models are very near to each other, e.g. 2.2dB difference if assume 2km distance at 4GHz. For LOS PL2 of UMa scenario, 38.803 has 0.6dB larger PL compared with 38.901 which could be negligible. For NLOS, 38.901 and 38.803 have the same PL model.

fig1. Comparison of PL1 models between 38.803 and 38.901 is listed

For indoor scenario, the PL model is almost the same except for d2D applicable range.

Observation 1: for UMa gNB-UE model, PL difference between 38.803 and 38.901 are very small, e.g. 2.2dB with 2km distance. For indoor gNB-UE model, PL model is the same except for d2D applicable range.

- For gNB-gNB model

For gNB-gNB model, the common assumption is to replace hUT with 25m the same height as hBS. Main difference between 38.803 and 38.901 comes from UMa LOS model, the details are listed as below for information. The hE value used to calculate h'BS and h'UT are not constant anymore. Following fig use different hE to evaluate the difference.

Observation 2: for UMa gNB-gNB model, the PL difference between 38.803 and 38.901 is very small, e.g. max 2.2dB difference at 2km distance with 4GHz. For indoor gNB-gNB model, the PL are the same except for d2D applicable range.

- For UE to UE model

The model in 38.802 actually reuse the same large-scale model as in TR36.843 which is used for D2D scenario from R12. 36.828 path loss is the max value between free space and Winner + B1. 36.828 use the xia model which comes from 2G area. For 38.901, if we use the gNB-UE model in 38.901 to stimulate UE-UE model, we need to adjust legacy hBS height to range 1.5~22.5 considering actual floor height. To be honest we don’t know whether 38.901 is still applicable or not with such adjustment since legacy hBS height is constant and not equal to UE height. Based on above analysis, 36.828 is reasonable.

| model | 38.802 | 36.828 | 38.901 |
| --- | --- | --- | --- |
| characteristics | max value between free space and Winner + B1 | xia model | don’t know whether 38.901 is still applicable or not because we need to adjust hBS height to UE height which is not allowed in legacy spec. |

Observation 3: for UE-UE model, 38.802 use max value between free space and Winner +B1 model, 36.828 use xia model. 38.901 has to adjust hBS to the range 1.5-22.5 considering actual floor height but actually such height range is not applicable for 38.901 model.

Proposal 2: it’s suggested to reuse the same pathloss model as used in 38.828.

About the grid shift, as analyzed in 38.828. 0% grid shift will introduce severe blocking issue which should be avoided. So during the simulation 0% grid shift should be avoided. This doesn’t mean we will exclude co-location blocking analysis. On the contrary, co-location blocking issue should be carefully studied to give more guidance for actual network deployment.

100% grid shift is one choice to reduce interference issues. So 100% grid shift is suggested as the baseline

Proposal 3: during the adjacent-channel simulation, 100% grid shift is suggested as the baseline and 0% grid shift should be avoided. Co-location blocking issue should be carefully analyzed to give more guidance for actual network deployment.

For indoor and outdoor UE radio, normally more UE will be indoor rather than outdoor. It’s more reasonable to assume 80% indoor and 20% outdoor to reflect actual UE distribution for FR1. For FR2, penetration is much severe, normally no operator will use FR2 for O2I coverage. we could only consider 100% outdoor scenario for FR2.

Proposal 4: for FR1 macro and micro, 80% indoor and 20% outdoor UE is assumed. For FR2, 100% outdoor is assumed for macro and micro.

In last RAN1 meeting, UE distribution mechanism is approved as below.

- Baseline (UE clustering):

  - 10 users per macro TRP

    - Step 1: Randomly drop X UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster

    - Step 2: Y% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, (1-Y%) users randomly and uniformly dropped throughout the macro geographical area

    - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h

    - FFS the values of X, Dmacro-to-cluster, Dinter-cluster, R, Y%

- Optional:

  - 10 users per macro TRP, and all users are randomly and uniformly dropped within the macro cell

  - At least for FR1: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h

    - FFS: FR2 details

This method simulates UE behaviour that UE cluster is like indoor environment and operator use outdoor gNB to coverage indoor UE. The main reason for the circular cluster is that RAN1 assume 500/3m ISD which means 120*50 indoor area can’t be dropped into macro area. So they try to simplify the model with circular area. Since RAN4 only consider 500m ISD with 167m cell radius assumption. We could assume the cluster area the same as indoor environment.

As for the distance between macro TRP and cluster, maybe we could reuse the same distribution as in TR 38.828 which assume 35m inter-site distance for macro-indoor interference scenarios. Considering we can’t use inter-site concept we could use distance between TRP and cluster centre with 100m which is equivalent to 35m inter-site distance in TR 38.828.

Proposal 5: UE distribution mechanism is suggested as below to stimulate outdoor gNB cover indoor UE scenario.

Step 1: Randomly drop a cluster within a macro cell geographical area considering the minimum 100m distance between macro TRP to cluster centre, where the size of each cluster is 120 x 50 (m);

Step 2: 80% UEs are randomly and uniformly dropped within the cluster, and 20% UEs are randomly and uniformly dropped outside the cluster.

For BS antenna configuration, we usually use sub-array AAS antenna to provide more flexibility for steering range in commercial network. Such sub-array AAS antenna configuration will also introduce better special/antenna domain isolation between Tx and Rx part of SBFD. So sub-array AAS antenna configuration is suggested. In 38.803, one example of AAS antenna configuration is configured. The example in table 5.2.3.2.4-3 assumes 4*8 sub-arrays with 3 elements per vertical sub-array, which will require M,N,P=12, 8, 2 elements. Such assumption is OK for us for FR1. For FR2, the assumption proposed as option 2 in [1] is also OK for us.

Proposal 6: for SBFD gNB antenna configuration, sub-array AAS antenna is preferred to reflect actual antenna performance. Original option 2 as in last meeting WF is OK for us[1].

For BS Tx power, it’s suggested to keep the same as legacy TDD, i.e. 49dBm for FR1 SBFD macro and 43dBm for FR2 macro. About higher power to reflect actual max power in actual network. We are open to such higher power. It’s true that 55dBm is used in our commercial network, but it only apply for 160MHz which is equivalent to 53dBm for 100MHz. to be honest, if the interference issue is severe, output power may be allowed to reduce to some extent to try to reduce interference. From this point of view, 49dBm is suggested as the baseline and if some companies prefer to use 53dBm it’s also OK as optional to give some guidance for actual network.

Proposal 7: for output power, 49dBm for FR1 and 43dBm for FR2 macro SBFD are suggested. 53dBm could be taken as second priority and companies could contribute simulation results with 53dBm assumption to give more guidance for actual network.

During commercial network deployment, usually, down-tilt including mechanical and electronical down-tilts are designed to make sure main beam point toward cell edge. the larger mechanical down-tilt, the more severe sidelobe leakage. According to our experience, usually mechanical down-tilt is fixed and we adjust electrical down-tilt to make sure main beam point toward cell edge. mechanical down-tilt is independent on electronical down-tilt.

Proposal 8: 6-degree mechanical down-tilt is suggested considering it usually is fixed and independent on electronical down-tilt.

# 3. Conclusions

In this contribution, adjacent channel co-existence simulation assumption is assumed with following observations and proposals:

Proposal 1: for SBFD configuration, it’s suggested to be aligned with RAN1 agreements and 20% UL sub-band could be taken as the starting point. i.e. {D,U,D}={40M,20M,40M} and {D,U}={80M,20M}.

Observation 1: for UMa gNB-UE model, PL difference between 38.803 and 38.901 are very small, e.g. max 2.2dB at 2km distance with 4GHz. For indoor gNB-UE model, PL model is the same except for d2D applicable range.

Observation 2: for UMa gNB-gNB model, PL difference between 38.803 and 38.901 is very small, e.g. max 2.2dB at 2km distance with 4GHz. For indoor gNB-gNB model, PL model are the same except for d2D applicable range.

Observation 3: for UE-UE model, 38.802 use max value between free space and Winner +B1 model, 36.828 use xia model. 38.901 has to adjust hBS to the range 1.5-22.5 considering actual floor height but to be honest such height range is not applicable for 38.901 model.

Proposal 2: it’s suggested to reuse the same pathloss model as used in 38.828.

Proposal 3: during the adjacent-channel simulation, 100% grid shift is suggested as the baseline and 0% grid shift should be avoided. Co-location blocking issue should be carefully analyzed rather than in co-existence simulation to give more guidance for actual network deployment.

Proposal 4: for FR1 macro and micro, 80% indoor and 20% outdoor UE is assumed. For FR2, 100% outdoor is assumed for macro and micro.

Proposal 5: UE distribution mechanism is suggested as below to stimulate outdoor gNB cover indoor UE scenario.

- Step 1: Randomly drop a cluster within a macro cell geographical area considering the minimum 100m distance between macro TRP to cluster centre, where the size of each cluster is 120 x 50 (m);

- Step 2: 80% UEs are randomly and uniformly dropped within the cluster, and 20% UEs are randomly and uniformly dropped outside the cluster.

Proposal 6: for SBFD gNB antenna configuration, sub-array AAS antenna is preferred to reflect actual antenna performance. Original option 2 as in last meeting WF[1] is OK for us.

Proposal 7: for output power, 49dBm for FR1 and 43dBm for FR2 macro SBFD are suggested. 53dBm could be taken as optional and companies could contribute simulation results with 53dBm assumption to give more guidance for actual network.

Proposal 8: 6-degree mechanical down-tilt is suggested considering it usually is fixed and independent on electronical down-tilt.

# 4. Reference

[1] R4-2214379, WF on Simulation assumption for adjacent co-existence study, CMCC