3GPP TSG-RAN WG4 Meeting # 104-bis-e							       R4-2215508

Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.13.4

Source: 	CMCC

Title: 	ATG BS classes, types and requirements

Document for:	Discussion

# 1. Introduction

After the discussion in RAN4 #104 e-meeting, there are still some ATG BS requirements that need to be discussed further.

This contribution provides our consideration on ATG RF requirements for BS classes, types and requirements.

# 2. Discussion

## 2.1. ATG BS class

ATG BS class had been reached an agreement at RAN4 #104 e-meeting, which will follow the approach of HAPS. HAPS BS class is defined by a typical distance from BS and ground UE. Similar with this way, ATG BS class is appropriate to define a typical vertical distance between ATG UE and gNB. The flight altitude range of aircraft is from 7~12km without considering the takeoff and landing of the aircraft. Because the time of the takeoff and landing is short, and this scenario is not typical compared with the whole flight.

Proposal 1: ATG BS class is defined with a BS to ATG UE typical vertical distance around 7~12km.

## 2.2. ATG BS type

1-H and 1-O had been agreed as the base type for ATG BS at RAN4 #104 e-meeting. Considering the incompatibility of BS type 1-C with beamforming, its beam tracking capability does not apply to ATG, hence inferiority to 1-H and 1-O in research and deployment.

Observation 1: Considering the incompatibility of BS type 1-C with beamforming, its beam tracking capability does not apply to ATG.

BR RF requirements of type 1-H can be divided into conducted requirements and OTA requirements. When defining the RF requirements of 1-H, the conducted requirements are defined which are also applicable to the BS type 1-C. The definition of type 1-C RF requirements will not bring additional work, so 1-C can be retained as the possibility of ATG BS type.

Observation 2: While defining the BS type 1-H RF requirements, the BS type 1-C RF requirements are also defined.

Proposal 2: The BS types 1-H and 1-O are the first choice of ATG BS types, however, 1-C is not excluded.

## 2.3. ATG BS requirements

Several RF requirements have been approved at RAN4 #104 e-meeting, and most of them reuse the same requirements defined in TS38.104. According to the Table 1 from CCSA, ATG users need traffic with high throughput. So 256 QAM is needed for ATG. The EVM requirement in TS 38.104 could be reused

Observation 3: ATG users need traffic with high throughput, the details are listed in the following table.

Table 1. throughput calculation per airplane

| Use case | Simultaneous active UE number | DL target throughput Mbps | UL target throughput Mbps |
| --- | --- | --- | --- |
| social media | 7 | 0.25 per user | 0.05 per user |
| Video chat | 5 | 0.5 per user | 0.5 per user |
| standard-definition video | 15 | 1 per user | 0.024 per user |
| high-definition video | 5 | 2 per user | 0.048 per user |
| Web browsing | 25 | 0.25 per user | 0.0375 per user |
| others | 15 | 0.25 per user | 0.016 per user |
| total | 72 | 39.25 | 4.6 |
| Note: above assuming 180 seats per airplane, 80% passengers, 50% of whom use ATG service. |  |  |  |

The following Table 2 shows the link budget to verify whether 256 QAM could be supported by the current BS antenna configurations and output power provided by vendor. Table 2 refer to R4-2213686 from ZTE [1].

Table 2. link budget for 256 QAM

| ATG UE Tx Antenna gain | 20.1dBi |
| --- | --- |
| ATG UE Tx transmit power | 29dBm |
| Frequency band | 4.9GHz |
| Channel bandwidth | 100MHz |
| ATG gNB Rx Antenna gain | 25.1dBi |
| ATG gNB Thermal noise | -114dBm/MHz |
| ATG gNB Noise figure | 5dB |
| SNR | 25dB |
| Pass Lose | 138.2dB |
| Distance | 39.8km |

Observation 4: The link budget shows 256 QAM cloud be supported by current configuration provided by vendor.

Proposal 3: 256 QAM should be supported at ATG gNB, and the same EVM requirements in TS 38.104 could be reused.

# 3. Conclusions

In this contribution, ATG gNB classes and types are discussed with following proposals:

Proposal 1: ATG BS class is defined with a BS to ATG UE typical vertical distance around 7~12km.

Observation 1: Considering the incompatibility of BS type 1-C with beamforming, its beam tracking capability does not apply to ATG.

Observation 2: While defining the BS type 1-H RF requirements, the BS type 1-C RF requirements are also defined.

Proposal 2: The BS types 1-H and 1-O are the first choice of ATG BS types, however, 1-C is not excluded.

Observation 3: ATG users need traffic with high throughput, the details are listed in the following table.

Proposal 3: 256 QAM should be supported at ATG gNB, and the same EVM requirements in TS 38.104 could be reused.

# 4. References

[1] R4-2213686, Discussion on coexistence evaluation for ATG network, ZTE