3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215528

Electronic Meeting, 10 October – 19 October 2022

Source:	Skyworks Solutions, Inc.

Title:	Triple beat MSD for DC_3C_n28A

Agenda item:			5.1.1 UE RF Requirements

Document for:	Approval

# 1. Introduction

This contribution presents measurements that evaluate the Low Noise Amplifier (LNA) contribution to band n28 MSD due to triple beat intermodulation interference in DC_3C_n28A. RF parameters are based upon the last meeting and the agreed upon Way Forward (WF) defined in [1].

# 2. Discussion

## 2.1	Triple-Beat Coupling Paths

For this measurement campaign, we evaluate the agreed upon WF [1] DC_3C_n28A architecture shown in Figure 1. Due to time constraints, this contribution focuses solely on the LNAs contribution to triple beat intermodulation distortion (IMD) products. Triple beat IMDs may result from the coupling of the B3C dual component carriers (CCs) and the band n28 single uplink carrier into the n28 primary and diversity LNAs, through several RF coupling paths. In Figure 1, we consider the following paths, labelled 1 to 12 as listed below, for each IMD contributor.

Figure 1: Triple Beat Coupling Paths.

- B3 PA:

  - path [1] from n28 PA output to B3 PA input via limited PCB isolation;

  - path [2] from n28 PA output to B3 PA output via limited PCB isolation;

  - path [3] from n28 PA output to B3 PA output via diplexer and B3 duplexer Ant/Tx rejection;

- n28 PA:

  - path [4] from B3 PA output to n28 PA input via limited PCB isolation;

  - path [5] from B3 PA output to n28 PA output via limited PCB isolation;

  - Reverse path [3]: from B3 PA output to n28 PA output via diplexer and n28 duplexer Ant/Tx rejection;

- n28 primary LNA:

  - path [6] from B3 PA output to n28 LNA input via diplexer isolation and n28 duplexer Ant/Rx rejection;

  - path [7] from n28 PA output to n28 LNA input via n28 duplexer Tx / Rx isolation;

  - path [9] from n28 PA output to n28 LNA input via limited PCB isolation;

  - path [11] from B3 PA output to n28 LNA input via limited PCB isolation;

- n28 diversity LNA:

  - path [8] from B3 PA and n28 PA output to n28 LNA input via antenna-to-antenna isolation, diplexer and B28 SAW BPF;

  - path [10] from n28 PA output to n28 LNA input via limited PCB isolation;

  - path [12] from B3 PA output to n28 LNA input via limited PCB isolation;

The evaluation of MSD due to the LNA contributions takes into consideration paths 6 to 12.

## 2.2	RF Test Parameters and RF-Front-End Assumptions

The triple beat IMD level is measured using a band n28 LNA. Our RFFE assumptions differ from the parameters proposed in WF [1]. For the diversity branch, we assume that the band n28 BPF may not achieve the same level of performance than the n28 duplexer Rx filter performance. Our assumptions are captured in Table 1 in a separate column from the WF parameters.

Table 1: RF-FE parameters for DC_3C_n28A MSD evaluation

| Attenuation and Isolation Parameter | WF [1] (dB) | This contribution (dB) | Comment |
| --- | --- | --- | --- |
| Antenna ISO | 10 | 10 |  |
| Diplexer ISO | 10 | 10 |  |
| PA gain | 25 | n-a |  |
| PCB isolation | 60 | 60 | PAout to LNAin |
| Band n28 Duplexer Tx rejection at B3 Tx | 30 | n-a | n-a for LNA contribution |
| Band n28 Duplexer Tx rejection at n28 Rx | 43 | n-a | n-a for LNA contribution |
| Band B3 Duplexer Tx rejection at n28 Tx | 32 | n-a | n-a for LNA contribution |
| Band B3 Duplexer Tx rejection at n28 Rx | 32 | n-a | n-a for LNA contribution |
| Band n28 filter/Duplexer Rx rejection at n28 Tx | 50 | n28 Duplexer: 50 n28 Div. BPF: 42 | Diversity n28 BPF may exhibit lower rejection than duplexer |
| Band n28 filter/Duplexer Rx rejection at B3 Tx | 33 | n28 Duplexer: 33 n28 Div. BPF: 25 |  |
| LNA IIP3 assumption | -6 | n-a | The linearity of the device under test may differ. |

- Power class 3 operation: 20dBm+20dBm;

- Test waveforms:

  - Local Oscillator (LO) leakage: -28dBc.

  - IQ Image rejection: -28dB.

  - UL B3C: DFT-s-OFDM, 20MHz + 20MHz channel bandwidth (CBW), and LCRB= 1 (RBstart= 0) + 1 (RBstart= 99) for CC1 and CC2 respectively.

  - UL n28: DFT-s-OFDM, 25MHz CBW, SCS15kHz, LCRB= 25 (RBstart= 108)

- UL carrier frequencies:

  - UL B3C, Fc=1730MHz. n28 UL Fc=715.5MHz.

- Power levels referred to LNA input:

  - n28 UL power level is swept from -21 to -36dBm;

  - For each n28 input level injected to the LNA, the B3C UL power level is swept from -31 to -56dBm.

- Both UL carriers are generated with high-end RF arbitrary waveform generators whose output noise in the DL band n28 is rejected using bandpass filters.

The triple beat IMD product is expected at 765.5MHz, i.e. the product falls within the band n28 Receiver (Rx) 25MHz CBW for DL carrier frequency of 770.5MHz. The IMD power level is measured using band power markers using a 23.94MHz measurement bandwidth. The spectrum analyser noise floor is approximately -81.2dBm/23.94MHz.

## 2.3	LNA IMD Results & MSD Analysis

In Figure 2, the LNA triple beat IMD level is plotted vs. the band B3 power level, in orange, green, red and blue for -21,-26,-31 and -36dBm n28 input levels. Measurement accuracy is limited by the spectrum analyzer noise floor which is plotted in a dashed black line. A linear interpolation from the high SNR region is used to estimate the IMD power levels that are not possible to measure due to test equipment noise floor.

Figure 2: Measured LNA triple beat IMD levels vs n28 and CA_3C UL power levels.

The antenna referred triple beat IMD power levels for the primary and diversity antenna ports vs PCB isolation are shown in Figure 3. The n28 25MHz CBW MSD due to LNA contribution only is estimated at 12.7dB assuming correlated MRC combining.

Figure 3: Measured LNA triple beat IMD interference levels referred to the primary and diversity antenna ports and corresponding Band n28 25MHz CBW MSD.

For comparison purposes, in [2] the MSD for 30MHz CBW was estimated at 4.6dB, meaning that for 25MHz CBW, the MSD may be up to 9.9dB if one assumes that the total interference power levels of [2] remain unchanged.

We offer the following proposal, which ignores PA forward and reverse triple beat IMD contributions.

Proposal: For DC_3C_n28A, adopt the band n28 25MHz CBW triple beat MSD captured in Table 1.

Table 1: Band n28 MSD for DC_3C_n28A due to dual uplink transmission triple beat interference

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENDC band combination | NR/LTE band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
|  | n28 | 715.5 | 25 | 25(RBSTART=108) | 770.5 | 12.7 | FDD | IMD3 |
| DC_3C_n28A | 3 | 1720 | 20 | 1 (RBSTART=0) | 1815 | N/A | FDD | N/A |
|  |  | 1739.8 | 20 | 1 (RBSTART=99) | 1834.8 |  |  |  |

# 3. Conclusion

This contribution evaluates band n28 MSD for DC_3C_n28A using LNA measurements. Due to time constraints, a contribution that measures power amplifier IMD interference could not be evaluated. We therefore make the following proposal based solely on the LNAs contribution.

Proposal: For DC_3C_n28A, adopt the band n28 25MHz CBW triple beat MSD captured in Table 1.

Table 1: Band n28 MSD for DC_3C_n28A due to dual uplink transmission triple beat interference

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENDC band combination | NR/LTE band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
|  | n28 | 715.5 | 25 | 25(RBSTART=108) | 770.5 | 12.7 | FDD | IMD3 |
| DC_3C_n28A | 3 | 1720 | 20 | 1 (RBSTART=0) | 1815 | N/A | FDD | N/A |
|  |  | 1739.8 | 20 | 1 (RBSTART=99) | 1834.8 |  |  |  |

# Reference

[1]	R4-2214426, WF on triple beat MSD of UL DC_3C_n28A, 3GPP TSG-RAN WG4 Meeting # 104-e, Huawei, HiSilicon.

[2] 	R4-2213132, Discussion on triple beat MSD of UL DC_3C_n28A, 3GPP TSG-RAN WG4 Meeting #104-e, Huawei, HiSilicon.