3GPP TSG-RAN WG4 Meeting # 104-bis-e                              R4-2215531

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.9.2.3

Source:	China Telecom

Title:	Discussion on SCell activation without SSB in inter-band scenario

Document for:	Discussion

- Introduction

The WID of even further RRM enhancement for NR and MR-DC on Rel-18 was approved in RP-220977 [1] and revised in RP-221696 [2]. One of the objectives is to study the enhancement of FR2 SCell activation delay. In RAN4 #104-e meeting, the FR2 SCell activation delay reduction solutions were discussed for the first time with the WF [3] approved, and the solutions will be further discussed in detail.

In this paper, we discuss other potential enhancement for FR2 SCell activation and present our views about these issues.

- Discussion

According to the requirement in TS 38.133 section 8.3.2, the unknown FR2 SCell activation delay is very long as illustrated in the following Figure 1. The process of AGC tuning, cell search, L1-RSRP measurement and report, fine timing tracking are required for unknown FR2 SCell activation.

Figure 1. Unknown FR2 SCell activation delay

In order to reduce the unknown FR2 SCell activation delay, the following options are presented in this paper.

Extension of FR2 SCell activation without SSB into inter-band scenario

The requirement of FR2 SCell activation without SSB is defined in TS38.133, as seen in Table 1.

Table 1. Requirement of FR2 SCell activation without SSB in TS 38.133

| FR2 SCell activation | Scenario | Delay (Tactivation_time)) |
| --- | --- | --- |
| At least one active serving cell on the FR2 band | UE is provided with SMTC | TFirstSSB+ 5ms |
|  | UE is not provided with any SMTC | 3 ms |

In Table 1, there is at least one active serving cell on the FR2 band of to-be-activated SCell, if UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms provided that the RSs of SCell being activated are QCL-TypeD with RSs of one active serving cell on that FR2 band. It means that the legacy requirement applies to the FR2 intra-band case without any SMTC.

Status in the WF [3]:

Issue 2-4-1: Other possible enhancement of FR2 unknown SCell activation FFS: - Proposal 4 (Huawei, Apple, CTC, ZTE): - Under certain condition when the timing/beam information of active serving cell on FR2 inter-band is able to be applied for to-be-activated SCell, the SCell activation delay can be further reduced (e.g., Tactivation_time=3ms) - Proposal 5 (ZTE, CTC): - To realize energy saving, similar as scellWithoutSSB defined for intra-band CA in Rel-16, the SCell without SSB or SSB-less can be considered for inter-band CA case. The corresponding SCell activation procedure needs some update for such SCell withou SSB or SSB-less.

In the WF, it was proposed to consider the candidate option of SCell without SSB in inter-band scenario. In our views, the definition is able to extend into inter-band provided that the RSs of SCell being activated are QCL-TypeD with RSs of any other active serving cell in another band. Meanwhile, it needs to be fulfilled that the RTD between target SCell and other inter-band active serving cell is within a certain threshold, and the difference of reception power between these cells is within a certain threshold. These thresholds can be discussed further in details. In practical networks, network can decide whether to use the SCell activation without SSB, based on the real deployment scenario and the frequency gap between the different bands.

Proposal 1: The FR2 SCell being activated without SSB is able to extend into inter-band scenario, and the delay reduction can be significant under certain conditions.

Extension of FR1 SCell activation without SSB into inter-band scenario

Status in the WF [3]:

Issue 2-1-2: Prioritization for initial phase Agreement: - RAN4 to prioritize at least FR2 unknown Scell delay reduction in the 1st phase of the WI. - FR2 unknown Scell without intra-band serving cell is considered for 1st phase. - The extension of the enhancement solutions to FR1 can also be discussed.

In the WF, it was agreed that the extension of the enhancement solutions to FR1 can also be discussed to avoid the duplicated discussion. In legacy RRM requirement, the requirement of FR1 SCell activation without SSB is defined in TS38.133, as seen in Table 2.

Table 2. Requirement of FR1 SCell activation without SSB in TS 38.133

| FR1 SCell activation | Scenario | Delay (Tactivation_time)) |
| --- | --- | --- |
| At least one active serving cell contiguous to the SCell on the FR1 band | UE is not provided with SSB configuration nor SMTC configuration for the target SCell | 3 ms |

In FR1 SCell activation without SSB scenario, the specification also defines intra-band case but doesn’t cover inter-band case. In our views, when some conditions are fulfilled, the extension of FR1 SCell activation without SSB into inter-band can also be introduced. The conditions will be further discussed in details.

Proposal 2: The extension of FR1 SCell activation without SSB into inter-band scenario can also be introduced.

- Conclusion

The contribution presents our views on the solutions of SCell activation delay reduction, with the following proposals:

Proposal 1: The FR2 SCell being activated without SSB is able to extend into inter-band scenario, and the delay reduction can be significant under certain conditions.

Proposal 2: The extension of FR1 SCell activation without SSB into inter-band scenario can also be introduced.

- References

- RP-220977, New WID: Even Further RRM enhancement for NR and MR-DC, Apple, OPPO, RAN Meeting #95e, March 2022.

- RP-221696, Revised WID: Even Further RRM enhancement for NR and MR-DC, Apple, OPPO, RAN Meeting #96, June 2022.

- R4-2214345, WF on R18 eFeRRM, Apple, RAN4 #104-e, August 2022.