3GPP TSG-RAN WG4 Meeting #104-bis-e 	R4-2215532

Electronic Meeting, 10-19 Oct., 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On PDSCH Requirements for ext71GHz

Agenda item:	4.3.7.2.1

Document for:	Discussion

Introduction

During the last RAN4#104-e meeting, good progress was made on the topic of defining PDSCH requirements for 711GHz.

In this contribution we will express our views on the open issues and open new discussions, if necessary.

Discussion

Maximum Testable SNR

The latest version of [3] indicates the following maximum DL testable SNR for band n263. We note that the values are increased from previous version which were used for reference in the previous RAN4 meetings.

Table 7.2.3-2: Maximum DL testable SNR preliminary extension for band n263 CBW (MHz); Test method. IFF. Single band UE; 100; [9.8]. 400; [2.6]. 800; [-2.3]. 1600; < -20 (NOTE 1). 2000; < -20 (NOTE 1). Multi band UE; 100; TBD. 400; TBD. 800; TBD. 1600; TBD. 2000; TBD. NOTE 1: Result does not converge.

The new updated values should be considered when deciding feasibility and definition of PDSCH Demodulation Requirements.

In “TR 38.884 - Study on enhanced test methods for FR2 NR UEs v18.2.0” the Maximum DL testable SNR values have been increased from the previous version and this change should be taken into consideration for deciding which requirements are feasible to be defined.

  - General Issues

    - Highest Modulation order for TDLA10 Channel Model for FR2 UE Demodulation requirements

In RAN4#104e it was discussed what the highest modulation order should be for TDLA10 channel model [1]

Issue 1-1-2b: Highest Modulation order for TDLA10 Channel Model for FR2 UE Demodulation requirements - Option 1: QPSK; - Option 2: Decide based on simulation results;

In previous RAN4#104e it was decided to use TDLA30 instead of TDLA10 for 120kHz SCS, hence this issue should cover both TDLA10 and TDLA30.
For 120kHz SCS cases using TDLA30 our simulations show feasibility to define requirements up to MCS17 with reduced RB.
For 480kHz SCS cases using TDLA10, our simulations show feasibility to define requirements up to MCS13 with reduced RB.

This issue should be extended to also include TLDA30.

We see it feasible to define requirements up to MCS13 for 480kHz SCS using TDLA10 with reduced RB.

We see it feasible to define requirements up to MCS17 for 120kHz SCS using TDLA30 with reduced RB.

For TDLA10, define requirements including 16QAM for 400MHz/480kHz CBW/SCS.

For TDLA30, define requirements including 64QAM for 100MHz/120kHz CBW/SCS.

    - MCS to use with 650Hz Doppler Frequency for TDLA

In RAN4#104e it was discussed if requirements should be introduced with 650Hz Doppler Frequency for TDLA [1]:

Issue 1-1-3: MCS to use with 650Hz Doppler Frequency for TDLA - Option 1: QPSK; - Option 2: Decide based on simulation results; - Option 3: Do not introduce requirements with 650Hz Doppler Frequency;

For a selected number of test cases for lower MCS when 650Hz SNR is feasible to be tested, it would be desirable to select 650Hz to have test cases with more mobility. For other test cases 200Hz can be used.

If for 650Hz doppler frequency at lower MCS the SNR is feasible to be tested, 650Hz doppler can be selected.

Introduce requirements with 650Hz Doppler frequency for lower MCS settings based on aligned simulation results.

    - Whether to define UE Demodulation requirements for 960kHz SCS

In RAN4#104e it was discussed If requirements should be introduced for 960kHz SCS [1]:

Issue 1-1-4: Whether to define UE Demodulation requirements for 960kHz SCS - Agreement (GTW August 22, 2022): RAN4 focuses on others SCSs before test feasibility for supporting 960kHz SCS is confirmed.

960kHz SCS has been defined by RAN1, hence demodulation requirements should be defined by RAN4 if possible. The decision has been kept open for TE vendors to provide updated information about TE capabilities which might show feasibility for 960kHz SCS. In the last meetings, information from TE vendors and simulation results has indicated that it seems not feasible to define requirements for 960kHz SCS as it will not be testable, however new updated information indicates that higher SNR can be achieved than what was initially indicated.
Our simulations provided in [2] shows testable SNR values when based on the latest Maximum Testable SNR information from [3].

New updated Maximum Testable SNR values have increased significant from the previous values.

RAN4 can now discuss further, if the updated information provided shows defining requirements for 960kHz is feasible.

Based on RAN4’s updated/increased Maximum Testable SNR, RAN4 to define requirements for 400MHz/960kHz CBW/SNR with lower MCS and if needed reduced RB allocation.

    - Max CBW for UE Demodulation Requirements definition for SCS 120kHz

In RAN4#104e it was agreed to introduce 100MHz CBW for 120kHz SCS, however it is still FFS if requirements are to be introduced for 400MHz CBW / 120kHz SCS [1]:

Issue 1-1-5: Max CBW for UE Demodulation Requirements definition for SCS 120kHz - Agreement: 100MHz for 120kHz and keep 400MHz FFS

It is important for RAN4 to define requirements for higher CBW as well. Based on the updated Maximum Testable SNR values, it seems feasible to define at least one requirement for 400MHz.

New updated Maximum Testable SNR values have increased from the previous values making 400MHz SBW /120kHz SCS feasible.

Based on RAN4’s updated/increased Maximum Testable SNR, RAN4 to define at least one requirement for 400MHz/120kHz CBW/SNR with lower MCS and if needed reduced RB allocation.

    - Max CBW for UE Demodulation Requirements definition for SCS 480kHz

In RAN4#104e it was agreed to introduce 400MHz CBW / 480kHz SCS, however it is still FFS if requirements are to be introduced for 800MHz CBW / 480kHz SCS [1]:

Issue 1-1-6: Max CBW for UE Demodulation Requirements definition for SCS 480kHz - Agreement: 400MHz for 480kHz and keep 800MHz FFS;

It is important for RAN4 to define requirements for higher CBW as well. Based on the updated Maximum Testable SNR values, it is feasible to define requirements for 800MHz with possible reduced number of RBs.

New updated Maximum Testable SNR values have increased from the previous values making 800MHz CBW / 480kHz SCS feasible.

Based on RAN4’s updated/increased Maximum Testable SNR, RAN4 to define at least one requirement for 800MHz/480kHz CBW/SNR with lower MCS and if needed reduced RB allocation.

    - Whether to define UE Demodulation Requirements with CBW = 1600MHz

In RAN4#104e it agreed not to define requirements for 1600MHz CBW / 480kHz SCS, however it is still FFS if requirements are to be introduced for 1600MHz CBW / 960kHz SCS [1]:

Issue 1-1-7: Whether to define UE Demodulation Requirements with CBW = 1600MHz - Agreement: - Don’t define for SCS 480kHz; - FFS the discussion on 960kHz if requirements with 960kHz are agreed;

With the updated Maximum Testable SNR values it does not seem feasible to support CBW=1600MHz, however in case later feasibility is shown, we see it valid to keep the existing agreement.

Keep current agreement “FFS the discussion on 960kHz if requirements with 960kHz are agreed”

PDSCH Demodulation Requirements

Partial bandwidth allocation position in the CBW

In RAN4#104e it was discussed where in the CBW a partial bandwidth allocation should be positioned [1]:

Issue 1-2-2b: Partial bandwidth allocation position in the CBW; - Option 1: Center; - Option 2: TBA;

We do not see this as a demodulation issue, however since resource allocation requires length and start TB is specified, option 1 to place the partial bandwidth in center of CBW would be fine.

We do not see it as a demodulation task to define the position of the partial bandwidth allocation, however using resource allocation requires length and start TB specified.

Place the partial bandwidth allocation at the centre of the CBW (Option 1)

RB Allocation Table for SCS 120kHz / CBW 100MHz

In RAN4#104e it was discussed which TB allocation to use for 120kHz/100MHz CBW/SCS [1]:

Issue 1-2-2d: RB Allocation Table for SCS 120kHz / CBW 100MHz - Option 1: MCS 4 and 13: 66 RBs; MCS 17: 33RBs - Option 2: MCS 4,13: 66 PRBs; MCS17: 30PRBs - Option 3: MCS 4, 13, 17, and 20: Configure 66 RBs;

Based on the lates official version of [3] (see section 2.1) and to match the indicated capabilities of the TE with our simulated results, the number of RBs should be reduced to a level where then TE will be able to sustain the power needed for the agreed SNR.

Based on our simulations results in [4] we see the following possible for 120kHz/100MHz SCS/CBW:

MCS4, 13: 66 RBs; MCS17: 33 RBs; MCS20: 16 RBs

Based on the updated maximum DL testable SNR in “TR 38.884 - Study on enhanced test methods for FR2 NR UEs v18.2.0” it is in our understanding possible to achieve testability up to MCS20 with specific RB allocation.

Use the following RB allocation for 120kHz/100MHz SCS/CBW
MCS4, 13: 66 RBs; MCS17: 33 RBs; MCS20: 16 RBs

RB Allocation Table for SCS 480kHz / CBW 400MHz

In RAN4#104e it was discussed which MCS and PRB allocations to use for 400MHzCBW / 480kHz SCS requirements definition [1]:

Issue 1-2-2e: RB Allocation Table for SCS 480kHz / CBW 400MHz - Option 1: MCS 4: 66 RBs; MCS 13: 16 RBs - Option 2: MCS 4, 9: 66 RBs; MCS13: 33RBs; MCS 17: 16 RBs;

Based on the lates official version of [3] (see section 2.1) and to match the indicated capabilities of the TE with our simulated results, the number of RBs should be reduced to a level where then TE will be able to sustain the power needed for the agreed SNR.

Based on our simulations results in [4] we see the following possible for 480kHz/400MHz SCS/CBW:

MCS4: 66 RBs; MCS13: 33 RBs; MCS17: 8 RBs

Based on the updated maximum DL testable SNR in “TR 38.884 - Study on enhanced test methods for FR2 NR UEs v18.2.0” it is in our understanding possible to achieve testability up to MCS17 with specific RB allocation.

Use the following RB allocation for 480kHz/400MHz SCS/CBW
MCS4: 66 RBs; MCS13: 33 RBs; MCS17: 8 RBs

PDSCH Coverage of the requirements

MCS choice for PDSCH requirements for 64QAM (if requirements with 64QAM are agreed)

In RAN4#104e it was discussed which MCS settings to use for 64QAM requirements definition [1]:

Issue 1-3-2: MCS choice for PDSCH requirements for 64QAM (if requirements with 64QAM are agreed) - Agreement: RAN 4 to select the max MCS that satisfies the following criteria: performance degradation due to phase noise is less than 1dB with the agreed PN compensation technique; - Option 1: - 120kHz: MCS 22; - 480kHz: MCS 20; - Option 2: - 120kHz: MCS17; - 480kHz: MCS 13; - Option 3: - 120kHz: MCS 20 (66 RBs); - 480kHz: MCS 13 (66 RBs), and 17 (16 RBs); - Option 4: - 120kHz: MCS 20 (66 RBs or 33 RBs); - 480kHz: MCS 9 (33 or 16 RBs);

As shown in sections 2.2.2 and 2.2.3 it is in our view possible to achieve MCS17(33Rbs) and MCS 20 (16RBs) for 120kHz SCS/100MHz CBW and MCS17 (8RBs) for 480kHz SCS/400MHz CBW.

It is in our understanding possible to define requirements for 64QAM with 120kHz:MCS17/MCS20 and 480kHz:MCS17.

Define PDSCH requirements for 64QAM for the following: MCS17(33RBs) & MCS20(16RBs) for 120kHz SCS/100MHz CBW and MCS17 (8RBs) for 480kHz SCS/400MHz CBW

Number of HARQ processes to be used for SCS=960kHz requirement definition if requirements are agreed

In RAN4#104e it was discussed what number of HARQ processes to use when defining requirements for 960kHz SCS [1]:

Issue 1-3-7: Number of HARQ processes to be used for SCS=960kHz requirement definition if requirements are agreed; - Do not discuss further unless 960kHz SCS requirements are agreed;

We see 32 HARQ processes to be needed, if requirements are to be defined for 960kHz SCS.

At this point, it is still not decided if requirements for 960kHz SCS is to be introduced.

Introduce requirements for 32 HARQ processes, if defining requirements for 960kHz is agreed.

PDSCH Test cases and simulation assumptions

List of test cases to be used for requirement definition

In RAN4#104e the PDSCH test cases and simulation assumptions were discussed [1]:

Issue 1-4-1: List of test cases to be used for requirement definition; Option 1: Table below as baseline and update it based on the outcome of the detailed issues regarding MCS, Doppler Spread, Channel Model, RB size, etc. Test; CBW(MHz) / SCS (kHz); MCS/Rank; Channel Model; Antenna Configuration and Correlation Matrix; RB Allocation Size; % of peak thpt. 1-1; 100/120; MCS4/Rank1; TDLA30-200; 2x2 ULA Low; [66]; 70. 1-2; 100/120; MCS13/Rank1; TDLA30-200; 2x2 ULA Low; [x]; 70. 1-4; 100/120; MCS17/Rank1; TDLD30-200; 2x2 ULA Low; [x]; 70. 2-1; 400/480; MCS4/Rank1; TDLA10-200; 2x2 ULA Low; [x]; 70. 2-2; 400/480; MCS13/Rank1; TDLA10-200; 2x2 ULA Low; [x]; 70. 2-4; 400/480; MCS17/Rank1; TDLD10-200; 2x2 ULA Low; [x]; 70. Option 2: Test; CBW(MHz) / SCS (kHz); MCS/Rank; Channel Model; Antenna Configuration and Correlation Matrix; RB Allocation Size; % of peak thpt. 1-1; 100/120; MCS4/Rank1; TDLA30-200; 2x2 ULA Low; 66; 70. 1-2; 100/120; MCS13/Rank1; TDLD30-200; 2x2 ULA Low; 66; 70. 1-4; 100/120; MCS17/Rank1; TDLD30-200; 2x2 ULA Low; 30; 70. 2-1; 400/480; MCS4/Rank1; TDLD10-200; 2x2 ULA Low; 66; 70. 2-2; 400/480; MCS13/Rank1; TDLD10-200; 2x2 ULA Low; 16; 70. Option 3 Test; CBW(MHz) / SCS (kHz); MCS/Rank; Channel Model; Antenna Configuration and Correlation Matrix; RB Allocation Size; % of peak thpt. 1-1; 100/120; MCS4/Rank1; TDLA30-650; 2x2 ULA Low; 66; 70. 1-2; 100/120; MCS13/Rank1; TDLA30-200; 2x2 ULA Low; 66; 70. 1-3; 100/120; MCS17/Rank1; TDLD10-200; 2x2 ULA Low; 66; 70. 1-4; 100/120; MCS20/Rank1; TDLD10-200; 2x2 ULA Low; 33; 70. 2-1; 400/480; MCS4/Rank1; TDLD10-200; 2x2 ULA Low; 66; 70.

To have best coverage of defined requirements, at least one requirement should be defined per modulation type per SCS/CBW combination.

At least one demodulation requirement should be defined for each feasible modulation types per agreed SCS/CBW combinations.

Define requirements for the following:

| Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS | Rank | Channel Model | Antenna Configuration and Correlation Matrix | RB Allocation Size | Fraction of maximum throughput (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 100/120 | MCS4 | 1 | TDLA30-650 | 2x2 ULA Low | 66 | 70 |
| 100/120 | MCS13 | 1 | TDLA30-200 | 2x2 ULA Low | 66 | 70 |
| 100/120 | MCS17 | 1 | TDLA30-200 | 2x2 ULA Low | 33 | 70 |
| 100/120 | MCS20 | 1 | TDLD10-200 | 2x2 ULA Low | 16 | 70 |
| 400/480 | MCS4 | 1 | TDLA10-200 | 2x2 ULA Low | 66 | 70 |
| 400/480 | MCS13 | 1 | TDLA10-200 | 2x2 ULA Low | 33 | 70 |
| 400/480 | MCS17 | 1 | TDLD10-200 | 2x2 ULA Low | 8 | 70 |
| 400/480 | MCS17 | 1 | TDLD10-200 | 2x2 ULA Low | 66 | 30 |
| Requirements for higher CBW and 960kHz SCS if agreed: |  |  |  |  |  |  |
| 400/120 | MCS4 | 1 | TDLA30-200 | 2x2 ULA Low | 264 | 70 |
| 800/480 | MCS4 | 1 | TDLD10-200 | 2x2 ULA Low | 66 | 70 |
| 400/960 | MCS4 | 1 | TDLD10-200 | 2x2 ULA Low | 32 | 70 |

Test Setup for 30% peak throughput PDSCH Requirements (if 30% peak throughput requirement is agreed)

In RAN4#104e it was discussed if requirements should be defined for 30% throughput [1]:

Issue 1-4-4: Test Setup for 30% peak throughput PDSCH Requirements (if 30% peak throughput requirement is agreed) If 30% throughput requirement is agreed, align the test setup for 70% percent throughput as baseline. Further discuss modifications if necessary. From GTW on August 22nd 2022: - Agreement: FFS whether introduce requirements for 30% of peak throughput - Option 1: not introduced - Option 2: introduce only one new dedicated test case - Option 3: Replace one of existing test case

The 30% peak throughput requirements have been defined in FR2-1 for verifying HARQ combining. Since it was already agreed to not reuse requirements from FR2-1 for FR2-2, we see a need to introduce requirements for 30% peak throughput.

With the updated information of Maximum testable SNR (see 2.1), we see it possible to have requirements for 70% for 400MHz/480kHz CBW/SCS, hence there is no immediate need to replace this testcase with 30% peak throughput.

To support test of HARQ combining, requirements for 30% throughput has validity.

In our view, the updated information of Maximum Testable SNR shows it is possible to define requirements for 400MHz/480kHz CBW/SCS with MCS17 with 70% throughput. Based on this, we do not see a need to replace one of the 70% throughput test cases with a 30% throughput test.

Introduce separate test case for 30% throughput (option 2) using 400MHz/480kHz CBW/SCS with MCS17.

Conclusion

In this contribution we have provided our views on various open issues with relation to PDSCH requirements for the extension to 71GHz.

We have made the following observations and proposals:

Maximum Testable SNR

- In “TR 38.884 - Study on enhanced test methods for FR2 NR UEs v18.2.0” the Maximum DL testable SNR values have been increased from the previous version and this change should be taken into consideration for deciding which requirements are feasible to be defined.

General Issues

Highest Modulation order for TDLA10 Channel Model for FR2 UE Demodulation requirements

This issue should be extended to also include TLDA30.

We see it feasible to define requirements up to MCS13 for 480kHz SCS using TDLA10 with reduced RB.

We see it feasible to define requirements up to MCS17 for 120kHz SCS using TDLA30 with reduced RB.

- For TDLA10, define requirements including 16QAM for 400MHz/480kHz CBW/SCS.

For TDLA30, define requirements including 64QAM for 100MHz/120kHz CBW/SCS.

MCS to use with 650Hz Doppler Frequency for TDLA

If for 650Hz doppler frequency at lower MCS the SNR is feasible to be tested, 650Hz doppler can be selected.

Introduce requirements with 650Hz Doppler frequency for lower MCS settings based on aligned simulation results.

Whether to define UE Demodulation requirements for 960kHz SCS

New updated Maximum Testable SNR values have increased significant from the previous values.

RAN4 can now discuss further, if the updated information provided shows defining requirements for 960kHz is feasible.

Based on RAN4’s updated/increased Maximum Testable SNR, RAN4 to define requirements for 400MHz/960kHz CBW/SNR with lower MCS and if needed reduced RB allocation.

Max CBW for UE Demodulation Requirements definition for SCS 120kHz

New updated Maximum Testable SNR values have increased from the previous values making 400MHz SBW /120kHz SCS feasible.

Based on RAN4’s updated/increased Maximum Testable SNR, RAN4 to define at least one requirement for 400MHz/120kHz CBW/SNR with lower MCS and if needed reduced RB allocation.

Max CBW for UE Demodulation Requirements definition for SCS 480kHz

New updated Maximum Testable SNR values have increased from the previous values making 800MHz CBW / 480kHz SCS feasible.

Based on RAN4’s updated/increased Maximum Testable SNR, RAN4 to define at least one requirement for 800MHz/480kHz CBW/SNR with lower MCS and if needed reduced RB allocation.

Whether to define UE Demodulation Requirements with CBW = 1600MHz

Keep current agreement “FFS the discussion on 960kHz if requirements with 960kHz are agreed”

PDSCH Demodulation Requirements

Partial bandwidth allocation position in the CBW

We do not see it as a demodulation task to define the position of the partial bandwidth allocation, however using resource allocation requires length and start TB specified.

Place the partial bandwidth allocation at the centre of the CBW (Option 1)

RB Allocation Table for SCS 120kHz / CBW 100MHz

Based on the updated maximum DL testable SNR in “TR 38.884 - Study on enhanced test methods for FR2 NR UEs v18.2.0” it is in our understanding possible to achieve testability up to MCS20 with specific RB allocation.

Use the following RB allocation for 120kHz/100MHz SCS/CBW
MCS4, 13: 66 RBs; MCS17: 33 RBs; MCS20: 16 RBs

RB Allocation Table for SCS 480kHz / CBW 400MHz

Based on the updated maximum DL testable SNR in “TR 38.884 - Study on enhanced test methods for FR2 NR UEs v18.2.0” it is in our understanding possible to achieve testability up to MCS17 with specific RB allocation.

Use the following RB allocation for 480kHz/400MHz SCS/CBW
MCS4: 66 RBs; MCS13: 33 RBs; MCS17: 8 RBs

PDSCH Coverage of the requirements

MCS choice for PDSCH requirements for 64QAM (if requirements with 64QAM are agreed)

It is in our understanding possible to define requirements for 64QAM with 120kHz:MCS17/MCS20 and 480kHz:MCS17.

Define PDSCH requirements for 64QAM for the following: MCS17(33RBs) & MCS20(16RBs) for 120kHz SCS/100MHz CBW and MCS17 (8RBs) for 480kHz SCS/400MHz CBW

Number of HARQ processes to be used for SCS=960kHz requirement definition if requirements are agreed

At this point, it is still not decided if requirements for 960kHz SCS is to be introduced.

Introduce requirements for 32 HARQ processes, if defining requirements for 960kHz is agreed.

PDSCH Test cases and simulation assumptions

List of test cases to be used for requirement definition

At least one demodulation requirement should be defined for each feasible modulation types per agreed SCS/CBW combinations.

Define requirements for the following:

| Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS | Rank | Channel Model | Antenna Configuration and Correlation Matrix | RB Allocation Size | Fraction of maximum throughput (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 100/120 | MCS4 | 1 | TDLA30-650 | 2x2 ULA Low | 66 | 70 |
| 100/120 | MCS13 | 1 | TDLA30-200 | 2x2 ULA Low | 66 | 70 |
| 100/120 | MCS17 | 1 | TDLA30-200 | 2x2 ULA Low | 33 | 70 |
| 100/120 | MCS20 | 1 | TDLD10-200 | 2x2 ULA Low | 16 | 70 |
| 400/480 | MCS4 | 1 | TDLA10-200 | 2x2 ULA Low | 66 | 70 |
| 400/480 | MCS13 | 1 | TDLA10-200 | 2x2 ULA Low | 33 | 70 |
| 400/480 | MCS17 | 1 | TDLD10-200 | 2x2 ULA Low | 8 | 70 |
| 400/480 | MCS17 | 1 | TDLD10-200 | 2x2 ULA Low | 66 | 30 |
| Define requirements for higher CBW and 960kHz SCS if agreed: |  |  |  |  |  |  |
| 400/120 | MCS4 | 1 | TDLA30-200 | 2x2 ULA Low | 264 | 70 |
| 800/480 | MCS4 | 1 | TDLD10-200 | 2x2 ULA Low | 66 | 70 |
| 400/960 | MCS4 | 1 | TDLD10-200 | 2x2 ULA Low | 32 | 70 |

Test Setup for 30% peak throughput PDSCH Requirements (if 30% peak throughput requirement is agreed)

To support test of HARQ combining, requirements for 30% throughput has validity.

In our view, the updated information of Maximum Testable SNR shows it is possible to define requirements for 400MHz/480kHz CBW/SCS with MCS17 with 70% throughput. Based on this, we do not see a need to replace one of the existing test cases.

Introduce separate test case for 30% throughput (option 2) using 400MHz/480kHz CBW/SCS with MCS17.

- References

- R4-2214390 - WF on FR2-2 UE demodulation requirements

- TS38.101-4 v17.6.0 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

- TR 38.884 - Study on enhanced test methods for FR2 NR UEs v18.2.0

- R4-2215533 - PDSCH simulation results for ext71GHz