3GPP TSG-RAN WG4 Meeting # 104-bis-e                	R4-2215533

Electronic Meeting, 10-19 Oct, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PDSCH simulation results for ext71 GHz

Agenda item: 	4.3.7.2.1

Document for: 	Discussion

Introduction

This paper presents PDSCH simulation results for the definition of demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Simulation assumptions

Simulation assumptions are shown below and are derived considering the agreements on the WF [1].

The simulations are done using the proposed channel models defined in [1].

Table 1 PDSCH Simulation Assumptions agreed [1]

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | [TDLA (10 ns and 30 ns delay spread) and TDLD (10 and 30 ns delay spread)] Doppler Spread: [200, 650] Hz |
| Antenna configuration | 2x2 ULA Low |
| Velocity | [3 km/h, 10 km/h] |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | TR38.808 PN model Set 1 and Set 2 |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| CSI-RS / TRS | Periodicity: [20] ms; Offset: [10] ms; |
| MCS/TBS | From MCS Table 1 (TS38.214): [Up to MCS 22 (64QAM)] Note: It is assumed that NohPRB = 0 for TBS calculations. |

PDSCH Simulation Results

Table 2 Results for PDSCH

| Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS | Rank | Channel Model | Antenna Configuration and Correlation Matrix | Fraction of maximum throughput (%) | No PN | PN set 1 CPE | PN set2 CPE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SNRBB (dB) | SNRBB (dB) | SNRBB (dB) |
| 100/120 | MCS4 | 1 | TDLA30-200 | 2x2 ULA Low | 70 | -0.8 | 0.27 | 0.18 |
| 100/120 | MCS4 | 1 | TDLA30-650 | 2x2 ULA Low | 70 | -0.97 | -0.07 | -0.03 |
| 100/120 | MCS13 | 1 | TDLA30-200 | 2x2 ULA Low | 70 | 7.28 | 7.77 | 7.58 |
| 100/120 | MCS13 | 1 | TDLD30-200 | 2x2 ULA Low | 70 | 7.39 | 8.32 | 8.27 |
| 100/120 | MCS17 | 1 | TDLA30-200 | 2x2 ULA Low | 70 | 10.92 | 11.59 | 11.14 |
| 100/120 | MCS17 | 1 | TDLD10-200 | 2x2 ULA Low | 70 | 11.02 | 11.6 | 11.21 |
| 100/120 | MCS20 | 1 | TDLD10-200 | 2x2 ULA Low | 70 | 13.53 | 14.34 | 13.65 |
| 100/120 | MCS20 | 1 | TDLD10-200 | 2x2 ULA Low | 30 | 4.96 | 6.25 | 6.1 |
| 400/120 | MCS4 | 1 | TDLA30-200 | 2x2 ULA Low | 70 | -0.91 | 0.2 | 0.09 |
| 400/120 | MCS13 | 1 | TDLD30-200 | 2x2 ULA Low | 70 | 7.21 | 7.76 | 7.51 |
| 400/480 | MCS4 | 1 | TDLA10-200 | 2x2 ULA Low | 70 | -0.94 | 0.11 | 0.09 |
| 400/480 | MCS4 | 1 | TDLD10-200 | 2x2 ULA Low | 70 | 0.06 | 0.98 | 0.95 |
| 400/480 | MCS13 | 1 | TDLA10-200 | 2x2 ULA Low | 70 | 7.2 | 7.72 | 7.53 |
| 400/480 | MCS13 | 1 | TDLD10-200 | 2x2 ULA Low | 70 | 7.74 | 8.16 | 8.05 |
| 400/480 | MCS17 | 1 | TDLD10-200 | 2x2 ULA Low | 70 | 11.15 | 11.68 | 11.36 |
| 400/480 | MCS17 | 1 | TDLD10-200 | 2x2 ULA Low | 30 | 3.98 | 4.67 | 4.53 |
| 800/480 | MCS4 | 1 | TDLD10-200 | 2x2 ULA Low | 70 | 0.1 | 1.06 | 1.03 |
| 800/480 | MCS13 | 1 | TDLD10-200 | 2x2 ULA Low | 70 | 7.71 | 8.19 | 8.04 |
| 400/960 | MCS4 | 1 | TDLD10-200 | 2x2 ULA Low | 70 | 0.2 | 1.24 | 1.17 |
| 400/960 | MCS13 | 1 | TDLD10-200 | 2x2 ULA Low | 70 | 7.72 | 8.23 | 8.08 |

References

- R4-2214390 - WF on FR2-2 UE demodulation requirements