3GPP TSG-RAN WG4 Meeting #104-bis-e 	R4-2215537

Electronic Meeting, 10-19 Oct., 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On CSI Reporting Requirements for ext71GHz

Agenda item:	4.3.7.2.4

Document for:	Discussion

Introduction

During the last RAN4#104-e meeting, good progress was made on the topic of CSI reporting requirements for 71GHz.

In this contribution we will express our views on the open issues and open new discussions, if necessary.

Discussion

CQI Demodulation Requirements

Target Modulation Order for FR2-2 CQI Requirements

In RAN4#104e it was discussed what the maximum modulation order should be for CQI demodulation requirements definition [1]:

Issue 3-2-2: Target Modulation Order for FR2-2 CQI Requirements: - Option 1: Up to 16 QAM; - Option 2: Include 64QAM;

The maximum modulation order for CQI demodulation requirements can be considered based on the updated Testable SNR values in [2] and our provided simulation results from [3].

The Maximum testable SNR values have been increased in latest version of “TR 38.884 - Study on enhanced test methods for FR2 NR UEs v18.2.0” to [9.8dB] for 100MHz CBW.

Assuming testable SNR of up to 9.8dB will based on our simulation results mean a CQI of 7/8 should be feasible, which represents 16QAM.

RAN4 to define CQI requirements up to and including 16QAM.

Whether to reuse CQI requirements test setup and metrics from FR2-1 for 120kHz/100MHz

In RAN4#104e it was agreed to reuse the test setup and test metrics from FR2-1. It was left open whether the FR2-1 SNR test points can be re-used for FR2-2 [1]:

Issue 3-2-3: Whether to reuse CQI requirements test setup and metrics from FR2-1 for 120kHz/100MHz: - Agreement: For FR2-2 CQI requirements for 120kHz/100MHz, reuse test setup and test metrics from FR2-1; - Further discuss whether FR2-1 SNR test points can be reused - Option 1: Yes; - Option 2: Test points: SNR = 6/7 for 16QAM, -2/-1dB for QPSK;

Based on the updated Maximum testable SNR values of [9.8dB] for 100MHz CBW [2], SNR points of 8/9dB should be feasible.

Maximum testable SNR is currently set to [9.8dB], making SNR points of 8/9dB feasible.

Use test points: SNR=8/9 for 16QAM, -2/-1dB for QPSK.

Conclusion

In this contribution we have provided our views on various open issues with relation to CSI reporting requirements for the extension to 71GHz.

We have made the following observations and proposals:

Target Modulation Order for FR2-2 CQI Requirements

- The Maximum testable SNR values have been increased in latest version of “TR 38.884 - Study on enhanced test methods for FR2 NR UEs v18.2.0” to [9.8dB] for 100MHz CBW.

Assuming testable SNR of up to 9.8dB will based on our simulation results mean a CQI of 7/8 should be feasible, which represents 16QAM.

RAN4 to define CQI requirements up to and including 16QAM.

Whether to reuse CQI requirements test setup and metrics from FR2-1 for 120kHz/100MHz

Maximum testable SNR is currently set to [9.8dB], making SNR points of 8/9dB feasible.

Use test points: SNR=8/9 for 16QAM, -2/-1dB for QPSK.

- References

- R4-2214390 - WF on FR2-2 UE demodulation requirements.

- TR 38.884 - Study on enhanced test methods for FR2 NR UEs v18.2.0

- R4-2216707 - CSI Simulation Results for ext71GHz