3GPP TSG-RAN4 Meeting #104-bis-e	R4-2215541
Electronic Meeting, 10th October – 19th October 2022

Agenda item:	6.8.2.2

Source:	Keysight Technologies

Title:	Multi-Rx Chain DL Spherical Coverage Test Proposal

Document for:	Approval

# Introduction

This contribution discusses various UE RF spherical coverage test options for multi-RX DL operation.

# Discussion

In [1], a new Study Item (SI) was endorsed to define test methodologies for UE RF/demodulation/RRM testing to support reception of DL signals simultaneously from multiple Angles of Arrival (AoAs). This SI directly supports the Work Item (WI) in [2] with testability aspects to eventually enable testing of 4-layer MIMO reception which states [2]:

“The existing Rel-15 NR FR2 minimum UE requirements are defined with an assumption that UE is equipped with a single antenna panel and capable to perform DL reception using a single RX beam/chain reception. Furthermore, the UE performance requirements are limited for DL MIMO rank 1 and 2. In FR2, 4-layer MIMO reception requires beam reception from at least two directions. Although this is supported by the MIMO features since Rel-15, no performance requirements have yet been specified.” and “This work item aims to introduce the requirements for UEs capable of multi-beam/chain simultaneous DL reception on a single component carrier to achieve improved RF, RRM and UE demodulation performance.”

The main focus in the Rel-18 WI and SI is to enable new spherical coverage requirements/testing for devices with simultaneous reception from 2 AoAs.

The remainder of this contribution outlines various testing approaches together with proposals of a new spherical coverage tests for 4-layer MIMO reception.

##### Full Degree of Rotation Freedom for each AoA

In RAN4#104-e, the first meeting of the SI, some discussions were held whether to support testing with “full degree of rotation freedom,” i.e., any permutation of possible AoAs shall be supported. An example test system which could support two simultaneous AoAs with (almost) any combination of DL directions was originally introduced in the original FR2 OTA Study Item [4], i.e., the Direct Far Field (DFF) test method with the measurement setup illustrated in Figure 5.2.1.1-1 of [4].

Figure 5.2.1.1-1: DFF measurement setup of UE RF characteristics

Such sample measurement setup when adapted to two-simultaneous AoA testing is illustrated in Figure 1. It should be noted that no positioning architecture/details are included in this Figure 1 as they are immaterial to the following discussions. However, it should be highlighted that such systems yield a very large footprint/height, complexity and cost increase due to the need for independent positioning of the two probes [6].

Observation 1: The independent positioning needs of the two probes yield larger system footprint/height, complexity and cost.

The full degree of rotation freedom is highlighted with the yellow arrows behind each probe. Additionally, the type of probe implementation, e.g., DFF vs IFF, is not further discussed here in this discussion as the focus is on impact of this test approach utilizing (almost) full degree of rotation freedom on test effort/test time. The finite size of the probes will limit the minimum angular separation between probes and thus prevent a “full” degree of rotation freedom.

Figure 1: Example measurement setup for two-simultaneous AoA testing supporting (almost) all AoA combinations.

Without any prior knowledge of the performance of any panel/chain, e.g., beam peak directions of each array, the spherical coverage test for this multi-chain UE system configuration would require a spherical coverage test with N*(N - 1)≈N2 AoA combinations. Here, N is the minimum number of spherical coverage grid points for the legacy spherical coverage test case, defined in Clause M.3 of [5], and it is assumed that the same measurement grids/grid density are required for multi-chain operations. The effort in terms of minimum number of test points is quantified in Table 1.

Table 1: Spherical Coverage effort for 4-DL multi-chain operation (full degrees of freedom for both AoAs)

| Power Class | Grid Type | Minimum Number of Spherical Coverage Test Points N (single chain) as defined in M.3 of [5] | Number of Spherical Coverage Test Points N (multi chain) N*(N-1) |
| --- | --- | --- | --- |
| PC1 | constant density | 200 | 39,800 |
| PC3 | constant density | 180 | 32,220 |
| PC1/PC3 | constant-step size | 266 | 70,490 |

Clearly, this test approach is test time prohibitive given the very large number of test points. Additional testability related concerns for test systems supporting full degrees of freedom are summarized in [6].

Observation 2: The test effort for the 4-DL spherical coverage test based on Full Degree of Rotation Freedom for each AoA is tremendous.

Proposal 1: A Spherical coverage test with full degree of rotation freedom for each AoA should not be considered due to test time limitations.

##### Full Degree of Rotation Freedom for 1st DL direction and discrete, fixed AoAs for 2nd DL direction

An alternate example measurement setup of a more efficient and cost-effective 4-DL MIMO multi-chain UE RF spherical coverage test approach is illustrated in Figure 2. In this example setup, the (fixed) probe locations, the number of probes, and probe type (DFF, IFF) are selected arbitrarily; specific implementation details could be discussed at a later time. In this specific measurement setup, all probe locations are fixed while the DUT is rotated in 3D using a 2-axis positioner (not illustrated). The 1st DL direction is introduced over the probe labelled AoA1 (typically oriented towards the z axis of the system) while the 2nd DL direction is introduced over any of the M (in this example, M=4) fixed probes labelled AoA2.m (with m = 1, …, M).

The proposed spherical coverage measurement approach is as follows:

- AoA1: Probe AoA1 (together with one Probe AoA2) performs a spherical coverage test similar to the legacy spherical coverage test, outlined in Clauses 7.3.4 and K.1.6 of [5], with the metric of EISAoA1 (Link=Spherical coverage grid, Meas=AoA1 angle, Optimized AoA2 angle) where EIS is sampled over the 3D sphere while for each grid point the TP (or EIS) for AoA2 is maximized beforehand. However, instead of testing two orthogonal polarizations sequentially, the MIMO signals are introduced concurrently on each polarization of both probes.

- AoA2: Prior to performing the EISAoA1 measurements for each grid point, the TPAoA2 for a fixed DL power level (or alternatively EISAoA2) from each probe AoA2.m (with m = 1, …, M) is determined and the maximum TPAoA2 (or minimum EISAoA2) is recorded for each grid point. Subsequently, the 2nd DL (AoA2) is introduced from the discrete probe that yielded the maximum TPAoA2 (or EISAoA2) before EISAoA1 is determined.

Not further discussed here are the power levels set on each of the signal paths (AoA1, AoA2) during the TPAoA2, EISAoA1, EISAoA2 measurements.

The detailed test sequences are further illustrated in the flow diagram of Figure 3.

Figure 2: Example measurement setup for two-simultaneous AoA testing supporting 3D AoAs for the 1st DL direction and discrete, fixed AoAs for the 2nd DL direction.

Figure 3: Flow diagram of test sequences.

The final metrics for this approach would be

- AoA1: the CDF of all EISAoA1 measurements determined and collected over the 3D sphere with N grid points

- AoA2: the CDF of the maximum TPAoA2 (or minimum EISAoA2) collected for each of the N grid points.

This test effort (primarily in terms of overall test time without taking positioning and switching times into account) is summarized in Table 2 separately for the case when TP and EIS metric is considered for AOA2 with an assumption the number of discrete probes for AoA2 is limited to M=4. The test times considered here are merely approximate (actual test time depends on many factors and can be better than stated below) and estimated as follows:

- tEIS: average time for an EIS measurement (~45s)

- tdwell: dwell time (per 38.521-2: “BEAM_SELECT_WAIT_TIME for the UE TX beam selection to complete”) (~3s)

- tEIS’: sum of tEIS and tdwell (~48s)

- tTP: average time for a TP measurement (~1s)

Clearly, the effort of this test approach is minimized if only the TPAoA2 (instead of EISAoA2) is collected as the TP for a fixed DL power level is significantly faster than performing an EIS search (tTP << tEIS). The 4-DL MIMO approach does not require polarizations to be tested sequentially but concurrently instead which is why the overall spherical coverage test time for the 4-DL multi-chain operation is ~33% less when compared to the legacy case if TP is considered as metric for the 2nd AoA.

Table 2: Spherical Coverage effort/test time for 4-DL multi-chain operation (full rotational freedom for AoA1 with fixed, discrete AoA2)

| Power Class | Grid Type | Minimum Number of Spherical Coverage Test Points N (single chain) as defined in M.3 of [5] | Effort/Test Time for single-chain spherical coverage test | Effort/Test Time for 4-DL multi-chain spherical coverage test |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | AOA2 Test: TPAoA2 (M=4) | AOA2 Test: EISAoA2 (M=4) |
| PC1 | constant-step size | 200 | 200*2*tEIS’≈320min | 200*[tEIS’ + M*(tdwell+tTP)] ≈213min | 200*(M+1)*tEIS’ ≈800min |
| PC3 | constant-step size | 180 | 200*2*tEIS’≈288min | 180*[tEIS’ + M*(tdwell+tTP)] ≈192min | 180*(M+1)*tEIS’ ≈720min |
| PC1/PC3 | constant density | 266 | 200*2*tEIS’≈426min | 266*[tEIS’ + M*(tdwell+tTP)] ≈284min | 266*(M+1)*tEIS’ ≈1064min |

Observation 3: The test effort for the 4-DL multi-chain spherical coverage test based on full rotational freedom for AoA1 with fixed and discrete AoA2 is manageable.

Proposal 2: Adopt A spherical coverage test based on full rotational freedom for AoA1 with fixed and discrete AoA2 with two performance metrics: CDF of EISAoA1 and CDF of the maximum TPAoA2.

In order to enable the activation of multiple antenna modules with good 4-layer DL performance, it is proposed to support a wide range of AoA angular differences between AoA1 and AoA2, e.g., 60°, 90°, 120°, and 150° (similar to the 2 AoA RRM requirements). This is further illustrated with an example UE with three integrated antenna arrays as shown in Figure 4. The different antenna arrays are shown with different colours, and it is assumed that the good coverage can be achieved by steering the respective beams. When two AoAs are selected that fall predominantly into the coverage sector of a single antenna array, as shown in Figure 5 on the left, very poor 4-DL MIMO performance can be expected while much better performance can be expected when the AoAs are spread out and thus fall into coverage areas of different antenna arrays as shown in Figure 5 on the right.

Figure 4: Spherical Coverage Map (2D View) of example UE with 3 antenna arrays

Figure 5: Examples of poor 4-DL (left) and good 4-DL performance (right)

Proposal 3: Consider a wide range of AoA angular differences between AoA1 (full degree of freedom) and AoA2 (limited, fixed degree of freedom)

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: The independent positioning needs of the two probes yield larger system footprint/height, complexity and cost.

Observation 2: The test effort for the 4-DL spherical coverage test based on Full Degree of Rotation Freedom for each AoA is tremendous.

Observation 3: The test effort for the 4-DL multi-chain spherical coverage test based on full rotational freedom for AoA1 with fixed and discrete AoA2 is manageable.

Proposal 1: A Spherical coverage test with full degree of rotation freedom for each AoA should not be considered due to test time limitations.

Proposal 2: Adopt A spherical coverage test based on full rotational freedom for AoA1 with fixed and discrete AoA2 with two performance metrics: CDF of EISAoA1 and CDF of the maximum TPAoA2.

Proposal 3: Consider a wide range of AoA angular differences between AoA1 (full degree of freedom) and AoA2 (limited, fixed degree of freedom)

# References

- RP-220988, New SI: Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements, Qualcomm Incorporated, 3GPP TSG RAN Meeting #95e, March 2022

- RP-221753, “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm Incorporated, 3GPP TSG RAN Meeting #96, June 2022

- R4-2214357, WF on NR FR2 OTA testing enhancements, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting # 104-e	, August 2022

- TR38.810, Study on test methods, V16.6.1 (2020-09)

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone, V16.12.0 (2022-06)

- R4-2215540, Testing Considerations for Multi AoA UE RF Spherical Coverage Test Case and FR2 Demodulation, Keysight Technologies, 3GPP TSG-RAN4 Meeting #104-bis-e, October 2022