3GPP TSG-RAN4 Meeting #104Bis-e	R4-2215567

# Online, , Oct 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR |  | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38.101-1 DMRS for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2022 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarifying CA requirements for DMRS bundling for DL CA and UL CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding coverge for DL CA with sentence: For inter-band carrier aggregation with uplink assigned to one NR band, DMRS bundling requirements in clause 6.4.2.6 apply for the uplink carrier. For UL CA with sentence: For inter-band carrier aggregation with uplink assigned to two NR bands within FR2, DMRS bundling requirements for phase continuity in clause 6.4.2.6 apply when UE is not scheduled for transmissions on an any other uplink carrier overlapping with a scheduled bundle on a uplink carrier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DMRS bundlign requirements for CA remain unclear and more than one interpretation of the applicable requirements exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4A.2.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<start of change>

### 6.4A.2 Transmit modulation quality

#### 6.4A.2.0 General

For intra-band contiguous and non-contiguous carrier aggregation, the requirements in clauses 6.4A.2.1, 6.4A.2.2, and 6.4A.2.3.

All the parameters defined in clause 6.4A.2 are defined using the measurement methodology specified in Annex F.

All the requirements in 6.4A.2 are defined as directional requirement.

The requirements are verified in beam locked mode on beam peak direction.

For inter-band carrier aggregation with uplink assigned to one NR band, DMRS bundling [maxDurationDMRS-Bundling-r17] requirements in clause 6.4.2.6 apply for the uplink carrier.

For inter-band carrier aggregation with uplink assigned to two NR bands, and each UL band is configured with a single CC, the transmit modulation quality requirements are specified in clause 6.4.2 and are applicable for each CC with all CCs active with non-zero UL RB allocation.

For inter-band carrier aggregation with uplink assigned to two NR bands within FR2, DMRS bundling [maxDurationDMRS-Bundling-r17] requirements for phase continuity in clause 6.4.2.6 apply when UE is not scheduled for transmissions on an any other uplink carrier overlapping with a scheduled bundle on an uplink carrier.

##### 6.4A.2.1 Error Vector magnitude

<end of change>