3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215576

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Feasibility study on FR2 multi-band RIB and multi-band BS

Agenda item:	6.4.2

Document for:	Approval

Introduction

TSG RAN#97-e has approved the revised SID on NR BS RF requirement evolution [1]. Following the agreed WF from TSG RAN4#104-e meeting, one of the aspects needs to be considered for FR2 multi-band BS is

- The following technical challenges need to be studied for FR2 multi-band BS - RF front-end - Antenna array - Phase shifters - Beamforming architectures - Others are not excluded

This contribution provides investigation on feasibility of FR2 multi-band BS, focusing on the feasibility of multi-band RIB w.r.t. different band combinations.

Discussion

Feasibility study on FR2 multi-band RIB

FR1 multi-band RIB was defined in [1] as following

multi-band RIB: operating band specific RIB associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

As BS type 1-O and BS type 2-O share the same definition for RIB [1, Clause 4.4.3], the above definition should be also applicable for FR2 multi-band RIB which means FR2 multi-band RIB should use the same physical antenna array and/or RF frontend to process signals of multiple operating bandwidths simultaneously.

Observation 1:  FR2 multi-band RIB should have same definition as FR1 multi-band RIB

The following feasibility analysis for FR2 multi-band BS based on the assumption that same physical antenna array and RF frontend for multi-band processing. Table 2.1-1 lists all possibilities of band combinations, including the list from SI [1] and the remaining possibilities. This table is the extension of [2, Table 2.2.1-1] for a more comprehensive study.

Table 2.1-1: Band combinations for FR2 multi-band BS

| Band combination | Frequency range (fB,high-fA,low) (MHz) | Maximum supported radio bandwidth (MHz) | Fractional Bandwidth (fB,high-fA,low)/fvc (%) |
| --- | --- | --- | --- |
| n258+n261 | 24250 – 28350 | 4100 | 15.58 |
| n258+n2571 | 24250 – 29500 | 5250 | 19.53 |
| n260+n2591 | 37000 – 43500 | 6500 | 16.15 |
| n259+n2621 | 39500 – 48200 | 8700 | 19.84 |
| n260+n2621 | 37000 – 48200 | 11200 | 26.29 |
| n261+n260 | 27500 – 40000 | 12500 | 37.03 |
| n257+n260 | 26500 – 40000 | 13500 | 40.6 |
| n258+n2601 | 24250 – 40000 | 15750 | 49.02 |
| n261+n259 | 27500 – 43500 | 16000 | 45.07 |
| n257+n259 | 26500 – 43500 | 17000 | 48.57 |
| n261+n262 | 27500 – 48200 | 20700 | 54.69 |
| n257+n262 | 26500 – 48200 | 21700 | 58.1 |
| n258+n262 | 24250 – 48200 | 23950 | 66.11 |
| NOTE 1: band combination NOT listed in SID [1] |  |  |  |

Note that combination n261+n257 is not listed here and it should be considered as a single band since any BS bandwidth in n261 is a sub-band of n257, as such this combination is not covered by current FR1 multi-band RIB definitions. Moreover, a single-band BS that is capable of operating on band n261 is also able to handle transmission/reception of carriers in band n257.

Observation 2: Combination n261+n257 should be considered as single-band RIB following the current FR1 multi-band RIB definition as any BS bandwidth in n261 is a sub-band of n257.

Wideband antenna array in FR2 multi-band RIB

As discussed in our previous paper [2], antenna array for wideband transmission imposes a trade-off between directivity and maximum steering angle that not causing grating lobe, i.e., the lowest frequency band suffers directivity degradation while the highest frequency band has limited steering angle. Table 2.1.1-1 provides an example of directivity variation and maximum steerable angles w.r.t. different band combinations for 32x32-element antenna array with inter-element spacing 0.5λ at the center frequency of composite bandwidth.

Table 2.1.1-1: Wideband antenna array performance with different band combinations for FR2 multi-band BS, inter-element spacing is 0.5λ at (fB,high+fA,low)/2

| Band combination | Inter-element spacing @fA,low-@fB,high | Directivity variation at boresight2,3 (dB) | Maximum azimuth steerable angle at highest frequency 4 (degree) |
| --- | --- | --- | --- |
| n258+n261 | 0.46λ – 0.54λ | -1.35 | ±58.80 |
| n258+n2571 | 0.45λ – 0.55λ | -1.66 | ±55.29 |
| n260+n2591 | 0.46λ – 0.54λ | -1.40 | ±58.27 |
| n259+n2621 | 0.45λ – 0.55λ | -1.68 | ±55.03 |
| n260+n2621 | 0.43λ – 0.56λ | -2.27 | ±50.14 |
| n261+n260 | 0.41λ – 0.59λ | -3.19 | ±43.43 |
| n257+n260 | 0.40λ – 0.60λ | -3.53 | ±41.49 |
| n258+n2601 | 0.38λ – 0.62λ | -4.25 | ±37.31 |
| n261+n259 | 0.39λ – 0.61λ | -3.89 | ±39.21 |
| n257+n259 | 0.38λ – 0.62λ | -4.20 | ±37.53 |
| n261+n262 | 0.36λ – 0.63λ | -4.79 | ±34.79 |
| n257+n262 | 0.35λ – 0.64λ | -5.07 | ±33.35 |
| n258+n262 | 0.33λ – 0.66λ | -5.87 | ±30.21 |
| NOTE 1: band combination NOT listed in SID [1] NOTE 2: result is achieved by simulation given frequencies and inter-element spacing and assuming an URA of elements with 32x32 elements NOTE 3: Directivity variation = directivity at the lowest frequency - directivity at the lowest frequency NOTE 4: = sin-1(1Inter-element spacing@fB,high-1) |  |  |  |

As can be seen, those band combinations with the supported radio bandwidth < 9GHz or FBW < 20% result in directivity variation being less than (absolute) 1.7dB on the boresight direction. Other band combinations with the supported radio bandwidth > 11.2 GHz or FBW > 26% have large directivity variations being higher than (absolute) 2.2dB. Thus, very wideband antenna array poses a huge challenge to meet the radiated transmit power requirements and EIPR accuracy, given that there are also limitations on other RF components operating at wideband condition, e.g., power amplifier.

Observation 3: Wideband antenna array for band combinations with the supported radio bandwidth < 9GHz or FBW < 20% has directivity variation being less than (absolute) 1.7dB on the boresight direction. Whereas that of band combinations with the supported radio bandwidth > 11.2 GHz or FBW > 26% have directivity variation being higher than (absolute) 2.2dB on the boresight direction.

From azimuth steerable angle perspective, antenna array of the combinations with supported radio bandwidth < 9GHz or FBW < 20% can cover > ±55 degrees in horizontal direction which is nearly a sector of ±60 degrees. If only sub-band of operating band A and/or B are considered, the maximum steering angle can be further optimized. The relation between inter-spacing and the maximum steering angle is plotted in Figure 2.1.1-1 which shows that antenna array with inter-element spacing larger than 0.535λ at highest frequency fB,high will have steerable angle (not to creating grating lobes) less than ±60 degrees.

Figure 2.1.1-1: Maximum steering angle versus inter-element spacing

To see the array performance when inter-element spacing is designed to ensure ±60 degrees scanning range in azimuth angle for the wideband array, we can set inter-element spacing 0.535λ at the highest frequency. Table 2.1.1-2 provides the corresponding result for 32x32-element antenna array.

Table 2.1.1-2: Wideband antenna array performance with different band combinations for FR2 multi-band BS, inter-element spacing is 0.535λ at fB,high

| Band combination | Inter-element spacing @fA,low-@fB,high | Directivity variation at boresight2,3 (dB) | Maximum azimuth steerable angle at highest frequency 4 (degree) |
| --- | --- | --- | --- |
| n258+n261 | 0.46λ – 0.535λ | -1.36 | ±60 |
| n258+n2571 | 0.44λ – 0.535λ | -1.67 | ±60 |
| n260+n2591 | 0.45λ – 0.535λ | -1.41 | ±60 |
| n259+n2621 | 0.44λ – 0.535λ | -1.70 | ±60 |
| n260+n2621 | 0.41λ – 0.535λ | -2.26 | ±60 |
| n261+n260 | 0.37λ – 0.535λ | -3.23 | ±60 |
| n257+n260 | 0.35λ – 0.535λ | -3.54 | ±60 |
| n258+n2601 | 0.32λ – 0.535λ | -4.31 | ±60 |
| n261+n259 | 0.34λ – 0.535λ | -3.95 | ±60 |
| n257+n259 | 0.32λ – 0.535λ | -4.27 | ±60 |
| n261+n262 | 0.30λ – 0.535λ | -4.84 | ±60 |
| n257+n262 | 0.29λ – 0.535λ | -5.15 | ±60 |
| n258+n262 | 0.27λ – 0.535λ | -5.94 | ±60 |
| NOTE 1: band combination NOT listed in SID [1] NOTE 2: result is achieved by simulation given frequencies and inter-element spacing and assuming an URA of elements with 32x32 elements NOTE 3: Directivity variation = directivity at the lowest frequency - directivity at the lowest frequency NOTE 4: = sin-1(1Inter-element spacing@fB,high-1) |  |  |  |

The directivity variation is similar to the results in Table 2.1.1-2. However, we can see that inter-element spacing is really low at the lowest frequency fA,low for some band combinations which implies that directivity of antenna array at lower band is very poor as can be seen in Figure 2.1.1-2

Figure 2.1.1-2: Directivity versus inter-element spacing of URA 32x32 isotropic elements

From the results in Tables 2.1.1-1, 2.1.1-2 and Figures 2.1.1-1, 2.1.1-2, we can see that wideband antenna array for band combinations with the supported radio bandwidth < 9GHz or FBW < 20% seems to have better trade-off between directivity (of the lowest frequency) and maximum azimuth steering angle (of the highest frequency) compared to the other combinations with the supported radio bandwidth > 11.2 GHz or FBW > 26%.

Observation 4: Wideband antenna array for band combinations with the supported radio bandwidth < 9GHz or FBW < 20% achieves better trade-off between directivity (of the lowest frequency) and maximum azimuth steering angle (of the highest frequency) compared to the other combinations with the supported radio bandwidth > 11.2 GHz or FBW > 26%.

Wideband power amplifier and DAC in FR2 multi-band RIB

Wideband devices operating at FR2 bands often have lower power efficiency and output power than FR1 devices due to the parasitic loss. Thus, there is a demand for the wideband PA working in FR2 bands to provide sufficient output power to maintain reasonable coverage for FR2 BS while achieving good power efficiency. To that purpose, PA may be required to have high efficiency and linearity over the wide bandwidth. However, linearity and performance stability of PA over the wideband is in part limited by video bandwidth of the PA, and increasing video bandwidth, e.g., up to 4GHz, is already challenging for PA implementation. Thus, for band combinations listed in Table 2.1-1, high linearity and high efficiency of the PA is difficult, or somewhat infeasible to obtain. This means PA for such wide bandwidth seems to be highly nonlinearity.

Observation 5: High linearity and high efficiency wideband-PA is challenging to obtain due to the requirement on the high video bandwidth, e.g., up to 4GHz video bandwidth is already challenging for PA implementation.

It should be noted that while it is unclear whether commercial PA for wide bandwidth, e.g.,  > 6GHz may be yet available and what would be the power performance, there are some commercial PAs which can operating up to 6GHz. However for band combinations of which the supported radio bandwidth < 6GHz, multi-band transmission on the same PA may generate the IM3 and IM5 frequencies which falls into the operating frequency ranges, as such increase intermodulation distortion level.

Observation 6: Multi-band signals going through same PA for combinations with the supported radio bandwidth < 6GHz may generate the IM3 and IM5 frequencies which falls into the operating frequency ranges as such increase intermodulation distortion level.

For the highly nonlinearity PA, operating point at high efficiency region close to the saturation point is expected since operating at high power backoff to meet the EVM floor may not be sufficient to meet the radiated power and coverage requirement. In addition, since PA would tend to be integrated to the antenna array to minimize the size of FR2 devices, the load impedance of the PA would vary with the scanning angle of the antenna array causing the variation on the output power and efficiency of the PA. This certainly raises the concerns on high-level nonlinearity distortions which increase in-band and out-of-band emission causing degradation of EVM and ACLR and EIRP accuracy. This fact was also mentioned in [3]. A DPD algorithm is needed to minimize the risk of nonlinear distortion generated by PA. However, such algorithm requires high sampling rate from DACs and baseband signal processors. Typically sampling rate of DAC needs to be 5x signal bandwidth to be able to cope with 5th order distortion. Advanced signal processing techniques have been studied of which results show expectation of lower required sampling rate, i.e., 1.2x-2x signal’s instantaneous bandwidth. However, even with theoretical sampling rate reduction to 1.2x bandwidth, band combinations with the supported radio bandwidth > 11.2 GHz or FBW > 26% already need >13GSPS sampling rate which may not be widely commercially available.

Observation 7: DPD is needed to minimize nonlinear distortions caused by PA, but such algorithms require high DAC’s sampling rate which is expensive or may be currently unavailable for band combinations with the supported radio bandwidth > 11.2 GHz (or FBW > 26%).

Note that ADC and DAC capability for normal data transmission/reception purposes needs to support very wide bandwidth. Currently commercial RFICs can support sampling rate to handle less than 10GHz RF bandwidth.

Phase shifter in FR2 multi-band RIB

Phase shifters (PS) with frequency-invariable phase and gain control is typically used to in FR2 analog and hybrid beamforming implementation. Such PS can only control one set of phase and amplitude at a time which raises a question on capability of concurrent beamforming in two different operating bands simultaneously if beam for each band points to different direction to the other. There are approaches which may be able to do concurrent beamforming for multi-band transmission, and they impact to the feasibility of multi-band RIB.

The first option is to use single frequency-invariable PS per RF chain, but the beamforming weights of different operating bands are combined. For the ease of description, let us assume xA and xB to be transmitted signals, and wA and wB are the corresponding beamforming weight for transmission at band A and B, respectively. The combined beamformer, i.e., w=wA+wB, is applied to PS to enable concurrent beamforming at the two bands. The outgoing signal from RIB at time instance t is

xt=xA+xBw=xA+xBwA+wB=xAwA+xBwB+xAwB+xBwAinterference

Obviously this approach will cause intra-band interference as part of the power from band A’s signal is steered to other direction due to the band B’s beamformer and vice versa. This may have impact to the OTA receiver out-of-band blocking requirement. In addition, it is not energy efficient either. Optimization could be applied to minimize the interference level, but would require complicated signal processing with knowledge of channels.

Observation 8: Using single frequency-invariable PS per RF chain to do concurrent beamforming transmission for multi-band signals increases intra-band interference which impacts OTA receiver out-of-band blocking requirement while being energy inefficient.

The second option is to split the multi-band signals to multiple single-band ones, each is fed to a single band PS. This approach not only requires extra PSs but also passive components to be added, and the number of additional components is scaling w.r.t. the number of the antennas and supporting operating bands. As such the size and cost, as well as energy consumption of the RF front end are significantly increased.

Typically phase shifter, PA and LNA are integrated into RFIC beamformers. Thus the second option would increase challenges to RFIC implementation, and may not be of practical interest. Such RFIC beamformers with splitting architecture may not be yet commercially available.

Observation 9: Using multiple frequency-invariable PSs per RF chain to do concurrent beamforming transmission for multi-band signals will increase size, cost and energy consumption for the FR2 radio which might not be of practical interest.

The third option, which is straight forward one, is to use multi-band PS, i.e., frequency-selective PSs. However, the technologies for such multi-band PS have not been mature yet, and may not be soon commercially available.

Observation 10: Multi-band PS, i.e., frequency-selective PSs might not be soon commercially available yet.

Observation 11: In general the use of phase shifter poses a challenge for practical implementations of FR2 multi-band RIB.

It should be noted that PS can be either an active or passive component depending on technologies and implementation. As ability of FR2 multi-band RIB is defined as ‘to process two or more carriers in common active RF components simultaneously’, therefore the implementation of PS in beamforming architecture may impact the declaration of FR2 multi-band RIB. For example, if active PSs are used in the splitting architecture (the second option discussed above), the RIB may not be declared as multi-band because the active components are not commonly processed multi-band signals simultaneously.

Observation 12: The implementation of PS in beamforming architecture may impact the declaration of FR2 multi-band RIB.

Based on the discussion above, we propose:

Proposal 1: The maximum supported bandwidth for FR2 multi-band RIB needs to be limited from the perspective of capabilities and performance of RF frontend to make the practical implementation feasible.

Proposal 2: To clarify if the use of active phase shifters in analog/hybrid beamforming in FR2 multi-band RIB can impact to the declaration of multi-band RIB, e.g., FR2 RIB would be considered as FR2 multi-band RIB if active phase shifters are used to process two or more bands simultaneously.

Proposal 3: To investigate further if the requirements of EVM, ACLR and EIRP accuracy, transmitter intermodulation requirement for FR2 multi-band RIB needs to be changed or relaxed compared to those for single band RIB.

Feasibility study on FR2 multi-band BS

As specified in clause 4.8 of TS 38.104 [4]:

BS type 1-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary: - All RIBs are single-band RIBs. - All RIBs are multi-band RIBs. - A combination of single-band RIBs and multi-band RIBs provides support of the BS type 1-O capability of operation in multiple operating bands.

The above definition should be also applicable for BS type 2-O as we have seen no relevant justification to introduce a new definition for FR2.

We discuss in the following some issues from implementation perspective for FR2 multi-band BS, focusing on the first two bullets. The implementation in the third bullet is a combination of the first two ones and should inherit issues from them.

FR2 multi-band BS using multi-band RIBs

As discussed in Section 2.1, RF frontend components and RF performance of FR2 multi-band RIB are dependent on the supported radio bandwidth. There is a need to clarify the maximum supported radio bandwidth would be supported by FR2 multi-band RIB. The beamforming architecture with the use of PS in FR2 multi-band may need to be further clarified from definition and practical implementation perspective.

Nevertheless, when the supported radio bandwidth would be clarified, the following requirements may need to be revisited to see if changes or relaxation would be required:

- OTA Adjacent Channel Leakage Power Ratio (ACLR)

- OTA operating band unwanted emissions

- OTA transmitter spurious emissions

- OTA adjacent channel selectivity

- OTA receiver intermodulation

- OTA EVM

- EIRP accuracy

This is due to the impact of wideband antenna array performance and nonlinearity properties of wideband FR2 antenna array, PAs and other components in the RF frontend.

Proposal 4: There should be maximum radio bandwidth defined for FR2 multi-band BS with multi-band RIBs. Some requirements would need to be revisited.

FR2 multi-band BS using multiple single-band RIBs

Section 2.1 also pointed out issues from practical implementation perspective of FR2 multi-band RIB. In some band combinations, e.g., those with the supported radio bandwidth > 9GHz, the use of multiple single-band RIBs would be a more feasible solution from antenna array performance and capability and performance of RF frontend components.

Observation 13: FR2 multi-band BS with multiple single-band RIBs is a more suitable solution for some band combinations, e.g., those with the supported radio bandwidth > 9GHz.

Conclusion

This contribution has provided the investigation on feasibility of FR2 multi-band RIBs w.r.t. different band combinations, as well as the discussion on feasibility of FR2 multi-band BS solutions. The observations and proposals are summarized as follows:

Observation 1:  FR2 multi-band RIB should have same definition as FR1 multi-band RIB.

Observation 2: Combination n261+n257 should be considered as single-band RIB following the current FR1 multi-band RIB definition as any BS bandwidth in n261 is a sub-band of n257.

Observation 3: Wideband antenna array for band combinations with the supported radio bandwidth < 9GHz or FBW < 20% has directivity variation being less than (absolute) 1.7dB on the boresight direction. Whereas that of band combinations with the supported radio bandwidth > 11.2 GHz or FBW > 26% have directivity variation being higher than (absolute) 2.2dB on the boresight direction.

Observation 4: Wideband antenna array for band combinations with the supported radio bandwidth < 9GHz or FBW < 20% achieves better trade-off between directivity (of the lowest frequency) and maximum azimuth steering angle (of the highest frequency) compared to the other combinations with the supported radio bandwidth > 11.2 GHz or FBW > 26%.

Observation 5: High linearity and high efficiency wideband-PA is challenging to obtain due to the requirement on the high video bandwidth, e.g., up to 4GHz video bandwidth is already challenging for PA implementation.

Observation 6: Multi-band signals going through same PA for combinations with the supported radio bandwidth < 6GHz may generate the IM3 and IM5 frequencies which falls into the operating frequency ranges which increase intermodulation distortion level.

Observation 7: DPD is needed to minimize nonlinear distortions caused by PA, but such algorithms require high DAC’s sampling rate which is expensive or may be currently unavailable for band combinations with the supported radio bandwidth > 11.2 GHz (or FBW > 26%).

Observation 8: Using single frequency-invariable PS per RF chain to do concurrent beamforming transmission for multi-band signals increases intra-band interference which impacts OTA receiver out-of-band blocking requirement while being energy inefficient.

Observation 9: Using multiple frequency-invariable PSs per RF chain to do concurrent beamforming transmission for multi-band signals will increase size, cost and energy consumption for the FR2 radio which might not be of practical interest.

Observation 10: Multi-band PS, i.e., frequency-selective PSs might not be soon commercially available yet.

Observation 11: In general the use of phase shifter poses a challenge for practical implementations of FR2 multi-band RIB.

Observation 12: The implementation of PS in beamforming architecture may impact the declaration of FR2 multi-band RIB.

Proposal 1: The maximum supported bandwidth for FR2 multi-band RIB needs to be limited from the perspective of capabilities and performance of RF frontend to make the practical implementation feasible.

Proposal 2: To clarify if the use of active phase shifters in analog/hybrid beamforming in FR2 multi-band RIB can impact to the declaration of multi-band RIB, e.g., FR2 RIB would be considered as FR2 multi-band RIB if active phase shifters are used to process two or more bands simultaneously.

Proposal 3: To investigate further if the requirements of EVM, ACLR and EIRP accuracy, transmitter intermodulation requirement for FR2 multi-band RIB needs to be changed or relaxed compared to those for single band RIB.

Proposal 4: There should be maximum radio bandwidth defined for FR2 multi-band BS with multi-band RIBs. Some requirements would need to be revisited.

Observation 13: FR2 multi-band BS with multiple single-band RIBs is a more suitable solution for band combinations, e.g., those with the supported radio bandwidth > 9GHz.

- References

[1]	RP-221001, “New SID: NR BS RF requirement evolution”, Huawei, HiSilicon.

[2]	R4-2211812, “Discussion on possible issues on performance of wideband RF and antenna”, Nokia, Nokia Shanghai Bell

[3]	R4-2212622, “Multi-band BS in mm wave”, Ericsson

[4]	3GPP TS 38.104 v17.6.0: “NR; Base Station (BS) radio transmission and reception”.