3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215587

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	4.3.7.2.1

Source:	Apple

Title:	On PDSCH demod requirements for 52.6 - 71 GHz

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#104-e demodulation requirements for 52.6 - 71 GHz was discussed and way forward [1] was agreed. In this contribution we present our views on the open issues on general aspects and PDSCH demodulation requirements and initial simulation results.

# 2. Discussion

## Testable SNR

In RAN4#104-e, there was an update to the parameter for demodulation SNR calculation and the testable SNR levels for band n263, and in [2]:

Table 1: Maximum DL testable SNR preliminary extension for band n263

|  | CBW (MHz) | Test method |
| --- | --- | --- |
|  |  | IFF |
| Single band UE | 100 | [9.8] |
|  | 400 | [2.6] |
|  | 800 | [-2.3] |
|  | 1600 | < -20 (NOTE 1) |
|  | 2000 | < -20 (NOTE 1) |
| Multi band UE | 100 | TBD |
|  | 400 | TBD |
|  | 800 | TBD |
|  | 1600 | TBD |
|  | 2000 | TBD |
| NOTE 1: Result does not converge |  |  |

Based on the limited testable SNR for band n263, it was also agreed to introduce PDSCH demod requirements with partial bandwidth allocation to cover more scenarios.

Based on the updated test parameters, we calculated the testable SNR levels for partial allocation for different SCS.

Table 2: PRB allocation and testable SNR for FR2-2

| SCS (KHz) | Num PRB | Testable SNR (dB) |
| --- | --- | --- |
| 120 | 66 | 9.8 |
|  | 33 | 13 |
| 480 | 66 | 2.6 |
|  | 33 | 6.4 |
|  | 20 | 8.9 |
|  | 16 | 10 |

We propose to use the updated testable SNR as a guideline to define demod requirements for FR2-2.

Proposal #1: Use the updated testable SNR as guide to define PDSCH requirements for FR2-2.

| SCS (KHz) | Num PRB | Testable SNR |
| --- | --- | --- |
| 120 | 66 | 9.8 |
|  | 33 | 13 |
| 480 | 66 | 2.6 |
|  | 33 | 6.4 |
|  | 20 | 8.9 |
|  | 16 | 10 |

## PDSCH Demod Requirements

In [1] we had an agreement on the channel models to be used for 120KHz SCS and 480KHz SCS:

Based on the agreed channel models, we provide simulation results for 120KHz with 100MHz and 480KHz SCS with 400MHz CBW.

Table 3: Simulation results for PDSCH

| SCS (KHz)/ CBW (MHz) | MCS | Propagation Channel | SNR @ 70% Max TP |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | PN Disabled | PN+CPE Enabled | Degradation with PN |
| 120/100 | MCS4 | TDLA30-200 | -1.5 | -0.8 | 0.6 |
|  | MCS4 | TDLA30-650 | -1.4 | -0.8 | 0.6 |
|  | MCS13 | TDLA30-200 | 6.3 | 6.8 | 0.5 |
|  | MCS17 | TDLA30-200 | 9.5 | 10.3 | 0.8 |
|  | MCS20 | TDLA30-200 | 12.2 | 13.5 | 1.3 |
|  | MCS17 | TDLD30-200 | 8.5 | 9.7 | 1.2 |
|  | MCS20 | TDLD30-200 | 11.1 | 12.7 | 1.6 |
| 480/400 | MCS4 | TDLA10-200 | -1.4 | -0.8 | 0.6 |
|  | MCS4 | TDLA10-650 | -1.3 | -0.7 | 0.6 |
|  | MCS13 | TDLA10-200 | 6.3 | 6.7 | 0.4 |
|  | MCS17 | TDLA10-200 | 9.5 | 10.2 | 0.7 |
|  | MCS20 | TDLA10-200 | 12.2 | 13.4 | 1.2 |
|  | MCS17 | TDLD10-200 | 8.6 | 9.8 | 1.1 |
|  | MCS20 | TDLD10-200 | 11.2 | 12.7 | 1.5 |

Based on the performance results above, we observe > 1dB degradation with PN enabled for MCS20 with TDL-A and MCS17,20 with TDL-D channel.

Observation #1: > 1dB degradation observed for TDL-A with MCS20; TDL-D with MCS17, 20.

Based on the degradation due to phase noise, MCS 20 is not suitable for either 120KHz or 480KHz SCS. We propose not to define requirements with MCS20 for FR2-2. MCS17 would be suitable for defining requirements based on degradation due to phase noise, but other aspects of testable SNR need to be considered.

Proposal #2: Do not define requirements with MCS20 for FR2-2.

Based on the testable SNR in table 1, and simulation results, the feasible PDSCH allocation is determined as follows taking implementation margin into consideration:

Table 4: Possible allocation based on testable SNR

| SCS (KHz)/ CBW (MHz) | MCS | Propagation Channel | SNR @ 70% Max TP | Num PRB |
| --- | --- | --- | --- | --- |
| 120/100 | MCS4 | TDLA30-200 | -1.5 | 66 |
|  | MCS4 | TDLA30-650 | -1.4 | 66 |
|  | MCS13 | TDLA30-200 | 6.3 | 66 |
|  | MCS17 | TDLA30-200 | 9.5 | 33 |
|  | MCS20 | TDLA30-200 | 12.2 | N/A |
|  | MCS17 | TDLD30-200 | 8.5 | 33 |
|  | MCS20 | TDLD30-200 | 11.1 | N/A |
| 480/400 | MCS4 | TDLA10-200 | -1.4 | 66 |
|  | MCS4 | TDLA10-650 | -1.3 | 66 |
|  | MCS13 | TDLA10-200 | 6.3 | 20 |
|  | MCS17 | TDLA10-200 | 9.5 | N/A |
|  | MCS20 | TDLA10-200 | 12.2 | N/A |
|  | MCS17 | TDLD10-200 | 8.6 | N/A |
|  | MCS20 | TDLD10-200 | 11.2 | N/A |

Considering the span in companies results, implementation margin, additional margin to derive requirements, based on the results and testable SNR, we think for 120KHz SCS, only MCS 17 is feasible for 64QAM modulation, even with partial BW allocation.

Observation #2: For 120KHz SCS only MCS17 is feasible for FR2-2 for 64QAM with partial BW allocation.

Based on the results and testable SNR for 480KHz SCS, it is not feasible to introduce requirements with 64QAM modulation.

Observation #3: It is not feasible to define requirements with 64QAM with 480kHz SCS due to testable SNR limit.

The following open issues from [1] were suggested to be addressed based on simulation results.

Issue 1-1-2b: Highest Modulation order for TDLA10 Channel Model for FR2 UE Demodulation requirements

- Option 1: QPSK;

- Option 2: Decide based on simulation results;

Issue 1-1-3: MCS to use with 650Hz Doppler Frequency for TDLA

- Option 1: QPSK;

- Option 2: Decide based on simulation results;

- Option 3: Do not introduce requirements with 650Hz Doppler Frequency;

Based on the results for 480KHz/400MHz using TDLA10 channel model and updated testable SNR, it is feasible to introduce requirements with MCS13/ 16QAM modulation with partial allocation .

Observation #4: It is feasible to introduce requirements with 16QAM (MCS13) with TDLA10 channel for 480KHz/400MHz with partial allocation.

For FR2-2, the use case is low mobility scenarios, hence 200Hz Doppler is more suitable to define requirements which corresponds to 3km/hr @ 70GHz. 650 Hz would be for 10km/hr, which seems high mobility for FR2-2 use case. We evaluated performance at 650Hz for QPSK and the performance delta is not much. Adding an additional test case with higher Doppler of 650 Hz is okay for QPSK.

Observation #5: It is feasible to add testcase with higher Doppler of 650Hz  for TDLA channel for QPSK.

Based on the above observations, we propose to define requirements for FR2-2 with the following simulation assumptions.

Table 4: Possible allocation based on testable SNR

| SCS (KHz)/ CBW (MHz) | MCS | Propagation Channel | Num PRB |
| --- | --- | --- | --- |
| 120/100 | MCS4 | TDLA30-200 | 66 |
|  | MCS4 | TDLA30-650 | 66 |
|  | MCS13 | TDLA30-200 | 66 |
|  | MCS17 | TDLD30-200 | 32 |
| 480/400 | MCS4 | TDLA10-200 | 66 |
|  | MCS13 | TDLA10-200 | 20 |

Proposal #3: Define PDSCH demod requirements for FR2-2 with the following simulation assumptions:

| SCS (KHz)/ CBW (MHz) | MCS | Propagation Channel | Num PRB |
| --- | --- | --- | --- |
| 120/100 | MCS4 | TDLA30-200 | 66 |
|  | MCS4 | TDLA30-650 | 66 |
|  | MCS13 | TDLA30-200 | 66 |
|  | MCS17 | TDLD30-200 | 32 |
| 480/400 | MCS4 | TDLA10-200 | 66 |
|  | MCS13 | TDLA10-200 | 20 |

# 3. Conclusion

In this paper we present our views on the open issues for UE demodulation requirements definition for 52.6-71GHz. Our observations and proposals are captured below:

Proposal #1: Use the updated testable SNR as guide to define PDSCH requirements for FR2-2.

| SCS (KHz) | Num PRB | Testable SNR |
| --- | --- | --- |
| 120 | 66 | 9.8 |
|  | 33 | 13 |
| 480 | 66 | 2.6 |
|  | 33 | 6.4 |
|  | 20 | 8.9 |
|  | 16 | 10 |

Observation #1: > 1dB degradation observed for TDL-A with MCS20; TDL-D with MCS17, 20.

Proposal #2: Do not define requirements with MCS20 for FR2-2.

Observation #2: For 120KHz SCS only MCS17 is feasible for FR2-2 for 64QAM with partial BW allocation.

Observation #3: It is not feasible to define requirements with 64QAM with 480kHz SCS due to testable SNR limit.

Observation #4: It is feasible to introduce requirements with 16QAM (MCS13) with TDLA10 channel for 480KHz/400MHz with partial allocation.

Observation #5: It is feasible to add testcase with higher Doppler of 650Hz  for TDLA channel for QPSK.

Proposal #3: Define PDSCH demod requirements for FR2-2 with the following simulation assumptions:

| SCS (KHz)/ CBW (MHz) | MCS | Propagation Channel | Num PRB |
| --- | --- | --- | --- |
| 120/100 | MCS4 | TDLA30-200 | 66 |
|  | MCS4 | TDLA30-650 | 66 |
|  | MCS13 | TDLA30-200 | 66 |
|  | MCS17 | TDLD30-200 | 32 |
| 480/400 | MCS4 | TDLA10-200 | 66 |
|  | MCS13 | TDLA10-200 | 20 |

# Reference

- R4-2214390, “WF on FR2-2 UE demodulation requirements”, Qualcomm Incorporated.

- TR 38.884, v18.2.0.