3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215589

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	4.3.7.2.4

Source:	Apple

Title:	On CQI reporting requirements for 52.6 - 71 GHz

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#104-e demodulation requirements for 52.6 - 71 GHz was discussed and way forward [1] was agreed. In this contribution we present our views on CQI reporting requirements.

# 2. Discussion

For CQI reporting requirements it was agreed to only define requirements in AWGN channel for FR2-2. It was also agreed that requirements will only be defined for 120KHz/100MHz.

Issue 3-2-1: Scope of the FR2-2 CQI Requirements: - Agreement (GTW August 22, 2022): Only for SCS 120kHz, CBW=100MHz, further discuss test metric and test SNR points.

The test setup and test metrics were also agreed to be reused from existing FR2-2 test cases from Rel-15.

The SNR test points are the only remaining open issue for QI reporting requirements for FR2-2.

Issue 3-2-2: Target Modulation Order for FR2-2 CQI Requirements:

  - Option 1: Up to 16 QAM;

  - Option 2: Include 64QAM;

Issue 3-2-3: Whether to reuse CQI requirements test setup and metrics from FR2-1 for 120kHz/100MHz:

- Agreement: For FR2-2 CQI requirements for 120kHz/100MHz, reuse test setup and test metrics from FR2-1;

- Further discuss whether FR2-1 SNR test points can be reused

  - Option 1: Yes;

  - Option 2: Test points: SNR = 6/7 for 16QAM, -2/-1dB for QPSK;

The testable SNR for band n263 with 120KHz SCS 100MHz is < 10dB.

Table 1: Testable SNR for band n263

| SCS (KHz) | Num PRB | Testable SNR (dB) |
| --- | --- | --- |
| 120 | 66 | 9.8 |
|  | 33 | 13 |

The median CQI for AWGN in FR2-2 is captured in Table 2.

Table 2: Median CQI in AWGN

| SNR | Med CQI | Modulation |
| --- | --- | --- |
| -2 | 3 | QPSK |
| -1 | 3 |  |
| 0 | 5 |  |
| 1 | 4 |  |
| 2 | 5 |  |
| 3 | 5 |  |
| 4 | 5 |  |
| 5 | 6 |  |
| 6 | 6 |  |
| 7 | 7 | 16QAM |
| 8 | 7 |  |
| 9 | 8 |  |
| 10 | 8 |  |
| 11 | 9 |  |
| 12 | 9 |  |
| 13 | 10 | 64QAM |
| 14 | 10 |  |

From the above results, we see that the SNR for reporting CQI in 64QAM range is > 12 dB.

Observation #1: The testable SNR for 120KHz/100MHz is < 10dB, while the operating SNR for reporting CQI in 64QAM range is > 12dB.

Given the testable SNR range, it is not practical to introduce requirements with higher SNR for CQI reporting in FR2-2. Hence, we propose to choose SNR points corresponding to QPSK and 16QAM range for defining requirements.

Proposal #1: Define CQI requirements in FR2-2 for SNR ranges corresponding to QPSK and 16QAM.

The existing CQI reporting requirements in AWGN for FR2-1 are for SNRs [8,9] and [14,15] dB. We propose to use SNR values of 8/9 dB for 16QAM, 0/1dB for QPSK.

Proposal #2: Use SNR values of 8/9 dB for 16QAM, 0/1dB for QPSK for CQI reporting in FR2-2.

# 3. Conclusion

In this paper we present our views on the open issues for UE demodulation requirements definition for 52.6-71GHz. Our observations and proposals are captured below:

Observation #1: The testable SNR for 120KHz/100MHz is < 10dB, while the operating SNR for reporting CQI in 64QAM range is > 12dB.

Proposal #1: Define CQI requirements in FR2-2 for SNR ranges corresponding to QPSK and 16QAM.

Proposal #2: Use SNR values of 8/9 dB for 16QAM, 0/1dB for QPSK for CQI reporting in FR2-2.

# Reference

- R4-2214390, “WF on FR2-2 UE demodulation requirements”, Qualcomm Incorporated.