3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215606

# Electronic Meeting, Oct 10 – Oct 19, 2022

Agenda item:	4.6.3.1.5

Source:	Apple

Title:	On remaining issues of RRM requirement for RedCap UE

Document for:	Discussion

# Introduction

In RAN4 #104e meeting, the WF[1] for RedCap RRM requirements was agreed, but there are still some open issues as followings,

SDT for RedCap with eDRX FFS: TA validation requirements for RedCap CG-SDT is defined for UE configured with eDRX in RRC_INACTIVE state by reusing the principles from DRX requirements introduced in R17 SDT WI. Note: eDRX in RRC_INACTIVE state does not have PTW. Whether to define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17 No consensus reached. Serving cell threshold associated SSB FFS: The serving cell thresholds of s-MeasureConfig for Connected mode should be checked based on reference SSB measurement.

In this contribution, we discuss the above remaining issues and corresponding CR [2] for serving cell threshold associated SSB is proposed.

# Discussion on RAN4 remaining issues

SDT for RedCap with eDRX

The existing TA validation for CG-SDT in Inactive mode has been specified as,

RSRP1 and RSRP2 are considered valid provided that the conditions in Table 5.5.3-1 and Table 5.5.3-2 are met for FR1 and FR2-1. Table 5.5.3-1 Valid measurement for FR1 Measurement; FR1. RSRP1; (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)). RSRP2; (T2 – min(640ms, M1*TDRX)) ≤ T2’ ≤ T2. Table 5.5.3-2 Valid measurement for FR2-1 Measurement; FR2-1. RSRP1; (T1 – [max{480ms, 8*SMTC periodicity}]) ≤ T1’ ≤ (T1 + [max{480ms, 8*SMTC periodicity}]). RSRP2; (T2 – [max{480ms, 8*SMTC periodicity}]) ≤ T2’ ≤ T2. If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT. Additionally, the UE shall not transmit in an CG-SDT occasion that occurs more than [640 ms] after T2.

The TA validation for CG-SDT in FR2-1 is not associated with DRX cycle, and therefore the existing TA validation requirement in FR2-1 can be reused for RedCap UE. Regarding FR1, since in RedCap inactive mode the serving cell measurement period is specified based on T in table Table 5.1B.2.2-1, the TA validation requirement shall also be based on T instead of TDRX.

Table 5.1B.2.2-1: Nserv_RedCap for inactive Redcap UE configured with eDRX_IDLE cycle, (Frequency range FR1) eDRX_IDLE cycle length [s]; DRX or eDRX INACTIVE cycle length[s]; T [s]; Nserv _RedCap [number of T ]. 2.56 ≤eDRX_IDLE cycle length ≤10485.76; 0.32 ≤DRX_Inactive cycle length ≤2.56; or 2.56 ≤eDRX_Inactive cycle length ≤10.24 if inactive eDRX is configured; 0.32; 4*M1. 0.64; 4*M1. 1.28; 2. 2.56; 2. 5.12; 2. 10.24; 2. Note1: T is dertermined according to clause 7.1 in [1]. Note2: M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second..

Thus, the TA validation window length for RedCap UE shall be min(640ms, M1*T) in Inactive mode.

Proposal 1: For UE configured with eDRX in RRC_INACTIVE state, the TA validation requirements for RedCap CG-SDT is defined as,

- For FR1,

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1*T)) ≤ T1’ ≤ (T1 + min(640ms, M1*T)) |
| RSRP2 | (T2 – min(640ms, M1*T)) ≤ T2’ ≤ T2 |
| Note: T is as defined in table 5.1B.2.2-1 |  |

- For FR2-1, same as the requirement with DRX in R17 SDT WI

Whether to define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17

In latest WID RP-220966, it was clearly stated the scope of R17 RedCap WI that,

  - No RRM measurement relaxations are specified for the serving cell.

In RAN4 LS (R4-2206977), it was agreed that,

RAN4 would like to thank RAN2 for the incoming LS in R2-2109218. RAN4 has discussed the RAN2 agreements on UE capabilities from a RRM perspective and further discussed the Rel-16 UE features and their applicability for RedCap in Rel-17. Based on the discussions, following conclusion is reached: Rel-15/Rel-16 features in TS 38.133; RedCap RRM requirements applicability in R17. Dual connectivity and carrier aggregation; Not applicable. 2-step RA; Applicable. NR measurements with autonomous gaps; Applicable. Rel-17 features in TS 38.133. SDT; Applicable. Note: RAN4 will not define any RRM requirements for RedCap UE for other release 16/release 17 features which are not listed above in release 17..

Thus, we think RAN4 shall not define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17. It is also like the previous discussion about RedCap UE to support unlicensed band: as in RP-212634, RedCap UE can support such feature but no requirement or spec modification shall happen in R17 timeline.

Proposal 2: RAN4 will not define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17.

Serving cell threshold associated SSB

In serving cell measurement, there is a threshold configured by network to determine if neighbor cell measurement is needed, i.e., s-MeasureConfig for Connected mode. We also think those thresholds are also checked based on reference SSB measurement, and also the CR[2] of this proposal is submitted in this meeting.

Proposal 3: The serving cell thresholds of s-MeasureConfig for Connected mode should be checked based on reference SSB measurement.

# Conclusion

In this contribution, we discuss the above remaining issues and corresponding CR [2] for serving cell threshold associated SSB is proposed.

Proposal 1: For UE configured with eDRX in RRC_INACTIVE state, the TA validation requirements for RedCap CG-SDT is defined as,

- For FR1,

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1*T)) ≤ T1’ ≤ (T1 + min(640ms, M1*T)) |
| RSRP2 | (T2 – min(640ms, M1*T)) ≤ T2’ ≤ T2 |
| Note: T is as defined in table 5.1B.2.2-1 |  |

- For FR2-1, same as the requirement with DRX in R17 SDT WI

Proposal 2: RAN4 will not define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17.

Proposal 3: The serving cell thresholds of s-MeasureConfig for Connected mode should be checked based on reference SSB measurement.

# References

[1] R4-2214485, WF on RedCap RRM requirements, Ericsson, RAN4 #104e.

[2] R4-2215607, CR for serving cell thresholds of s-MeasureConfig for RedCap, Apple, RAN4 #104-bis-e.