3GPP TSG-RAN WG4 Meeting # 104bis-e												   R4-2215608

Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.20.3

Source:	Apple

Title:	On R18 mobility enhancement - L1/L2 inter-cell mobility RRM

Document for:	Discussion

# Introduction

RAN4 had initial discussion on L1/L2 inter-cell mobility in RAN4#104e, with agreement captured in [1]:

# Topic #3: L1/L2 based inter-cell mobility ## Sub-topic 3-1: RRM requirements to specify < Agreement>: Issue 3-1-1: L1/L2 inter-cell mobility delay requirements Specify L1/L2 inter-cell mobility delay and analyse each component of L1/L2 inter-cell mobility delay. <Way forward >: Issue 3-1-2: inter-cell mobility interruption requirements Merged in issue 3-1-1. <Way forward >: Issue 3-1-3: L1-RSRP measurement delay requirements Come back to this issue after the procedure and supported scenario is clear. < Agreement>: Issue 3-1-4: Timing management requirements Wait for RAN1/2 input and then discuss whether and how to define timing management requirements. <Way forward >: Issue 3-1-5: Timing requirements - Revised Option 1 (Ericsson): Specify MRTD of serving cell and neighbour cell if covering the scenario that time offset of serving cell and neighbour cell is more than CP. - Option 2 (Apple, QC, HW, Nokia) : FFS, need further clarification ## Sub-topic 3-2: Scenarios <Way forward >: Issue 3-2-1: Whether to consider simultaneous Rx/Tx with both source cell and target cell - Option 1 (MTK, Apple, vivo, HW, Ericsson): Not consider simultaneous Rx/Tx with both source cell and target cell. - Option 2 (QC, Nokia): FFS, need further clarification <Way forward >: Issue 3-2-2: Whether to consider simultaneous multi-panel in FR2 - Option 1 (MTK, Apple, vivo, QC, HW, LGE): Not consider simultaneous multi-panel in FR2 - Option 2 (Ericsson, Nokia): Consider simultaneous multi-panel in FR2 <Way forward >: Issue 3-2-3: Definition of intra-frequency/inter-frequency in inter-cell operation - Option 1 (Apple, Ericsson, vivo, HW, Nokia): RAN4 to discuss the definition of intra-frequency/inter-frequency in inter-cell operation - Option 1a (MTK): For SSB L1-RSRP measurement, follow the definition of L3 measurement. - Option 1b: For SSB L1-RSRP measurement, intra-frequency is defined as long as the SSB-based L1 measurement is performed within active BWP(s) of the UE. - Option 2 (QC): discuss later <Way forward >: Issue 3-2-4: Whether to cover inter-frequency - Option 1 (MTK, vivo, QC, apple): Further discuss the necessity, feasibility, and pros/cons of specifying inter-frequency L1-RSRP measurement. - Option 2 (CMCC, HW, Ericsson, Nokia): Yes <Way forward >: Issue 3-2-5: Whether to cover non-synchronous scenarios - Option 1: Not consider non-synchronous scenarios. - Option 2 (Ericsson, Nokia): Consider non-synchronous scenarios. - Option 3 (Ericsson, HW): FFS - Option 3a (Apple, MTK, vivo): discuss the definition of synchronous and non-synchronous - Option 3b (Apple, QC, vivo, Nokia): wait for RAN2’s progress <Way forward >: Issue 3-2-6: Whether to support L1 measurement on multiple cells with PCI different from serving cell - Option 1(Apple, HW, vivo, Ericsson, MTK, Nokia): to discuss the number of supported cells with PCI different from serving cell. - Option 1a: To discuss the number of supported cells with PCI different from serving cell on a FR2 band - Option 1b: To discuss the total number of cells to be monitored on all bands - Option 1c: To discuss the total number of cells to be monitored per frequency layer - Option 2 (QC): discuss this issue later ## Sub-topic 3-3: Measurement accuracy <Way forward >: Issue 3-3-1: Intra-frequency L1-RSRP measurement accuracy requirements - Option 1 (Nokia, vivo): Discuss whether intra-frequency L1-RSRP measurement accuracy can be improved for L1/L2 mobility - Option 2 (MTK, HW): reuse the legacy intra-frequency L1-RSRP measurement accuracy for L1/L2 mobility. - Option 3 (Apple, Ericsson, QC): Discuss this issue later, e.g. in performance part <Way forward >: Issue 3-3-2: Inter-frequency L1-RSRP measurement accuracy requirements - Option 1 (Nokia): Define inter-frequency L1-RSRP measurement accuracy requirements on non-serving cell - Option 2 (Apple, Ericsson, vivo, MTK, HW): Further discuss whether and how to define inter-frequency L1-RSRP measurement accuracy requirements on non-serving cell later, e.g. in performance part.

Besides, RAN2 approved an LS [2] to RAN4 in August meeting on this topic. In this contribution, we will further discuss RRM aspects to support L1/L2 inter-cell mobility.

# Discussion

The first issue is about L1/L2 inter-cell mobility delay and interruption requirements:

< Agreement>: Issue 3-1-1: L1/L2 inter-cell mobility delay requirements

Specify L1/L2 inter-cell mobility delay and analyse each component of L1/L2 inter-cell mobility delay.

<Way forward >: Issue 3-1-2: inter-cell mobility interruption requirements

Merged in issue 3-1-1.

In legacy handover procedure, RRM delay requirement typically is defined from the time when handover command is received to the time when UE is ready to perform RACH toward target cell. In RACH-less handover (in LTE), the ending point is the time when UE is ready to start the transmission of the new uplink PUSCH channel to target cell.

We believe similar framework can be reused in L1/L2 inter-cell mobility. Actually, RAN2 had similar discussion in August meeting and reached the following assumption as can be found in their LS:

Assumption: HO interruption time for L1/L2-based inter-cell mobility is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. FFS if TRS tracking after HO and CSI RS measurement should also be included, i.e. the time to use a high-performance beam (can be clarified further).

Note that RAN2 called it ‘HO interruption time’. However, from RAN4 RRM requirements point of view, this is more like the handover delay. The difference is that UE may still be able to perform data transmission/reception with source cell during the cell switch command processing time. Thus, in RAN4 we probably won’t consider this period as interruption.

Another thing we would like to highlight is that it is unclear whether RACH has been considered or not. RACH is one of the procedures to acquire UL timing. It is unclear now whether RACH is allowed during this procedure. Nevertheless, from RAN4 requirements perspective, if UE doesn’t have valid UL timing for the target cell, UE may need to perform RACH before it can start transmission toward target cell.

Besides, we may also need to consider another execution time, similar with CHO. It is possible that network pre-configures multiple candidate cells for L1 measurement, together with cell parameters of the candidate cells. But eventually UE only needs to switch to one target cell. Therefore, it is unnecessary for UE to perform parameters validation and apply them for all the candidate cells. Our expectation is that UE keeps measuring all the candidate cells. Once it receives switch command to certain target cell, it can make the corresponding cell parameters valid and apply them (similar to CHO).

Based on above discussion, the L1/L2 inter-cell mobility delay and interruption can be defined as:

L1/L2 inter-cell mobility delay (DL1/L2_mobility) is defined as

DL1/L2_mobility = Tprocess + Tinterrupt + Texecution

Where:

DL1/L2_mobility is the L1/L2 inter-cell mobility delay.

- For RACH-less case (if supported), it is defined as the time UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

- For RACH-based case (if supported), it is defined as the time UE receives the cell switch command to UE starts transmission of the new uplink PRACH channel to the target cell.

Tprocess is the processing time of cell switch command, which depends on command design, and it is up to other working groups.

Tinterrupt is the data interruption time during the mobility procedure. Length of Tinterrupt includes several aspects, such as to acquire fine T/F tracking, suitable beam and so on. Under certain conditions some of the components can be zero.

Texecution is execution preparation time, which can be FFS. TCHO_execution can be a good starting point.

Proposal 1: L1/L2 inter-cell mobility delay (DL1/L2_mobility) depends on whether RACH is needed/allowed:

- For RACH-less case (if supported), it is defined as the time UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

- For RACH-based case (if supported), it is defined as the time UE receives the cell switch command to UE starts transmission of the new uplink PRACH channel to the target cell.

Proposal 2: cell switch command processing time (Tprocess) needs to be considered in L1/L2 inter-cell mobility delay requirements.

Proposal 3: interruption time during the mobility procedure (Tinterrupt) includes several aspects, such as to acquire fine T/F tracking, suitable beam and so on. Under certain conditions some of the components can be zero.

Proposal 4: L1/L2 inter-cell mobility execution time needs to be considered.

The second issue we would like to discuss is about simultaneous Rx/Tx and multi-panels assumption.

<Way forward >: Issue 3-2-1: Whether to consider simultaneous Rx/Tx with both source cell and target cell

  - Option 1 (MTK, Apple, vivo, HW, Ericsson): Not consider simultaneous Rx/Tx with both source cell and target cell.

  - Option 2 (QC, Nokia): FFS, need further clarification

<Way forward >: Issue 3-2-2: Whether to consider simultaneous multi-panel in FR2

  - Option 1 (MTK, Apple, vivo, QC, HW, LGE): Not consider simultaneous multi-panel in FR2

  - Option 2 (Ericsson, Nokia): Consider simultaneous multi-panel in FR2

Regarding simultaneous Rx/Tx during inter-cell mobility procedure, it is not mentioned in the WID. However, this has significant impact on procedure design, especially from UE complexity perspective. Correspondingly, RAN4 requirements would be different if simultaneous Rx/Tx needs to be supported. On necessity of simultaneous Rx/Tx in this work item, we don’t think this is necessary. At least it should not be the baseline assumption for UE to support L1/L2 inter-cell mobility. The key reason is we already have similar procedure, i.e. DAPS handover which was introduced in R16. According to DAPS handover design the interruption is very limited. In legacy handover, long handover latency means long interruption on business. However, in DAPS handover, data transmission and reception are still allowed during handover delay. Therefore, reducing latency in the handover procedure wherein UE can still perform data transmission/reception it not that attractive. In other word, no need to consider L1/L2 based DAPS handover.

Observation 1: complexity would be significantly increased if UE needs to support simultaneous data Rx/Tx with both source and target cells during L1/L2 inter-cell mobility. However, the gain is not that attractive since the functionality has already been supported in R16 DAPS handover.

Proposal 5: as baseline assumption, UE is not required to perform simultaneous data Rx/Tx with both source and target cells during L1/L2 inter-cell mobility.

Proposal 6: as baseline, do not consider simultaneous multi-Rx in FR2.

The next issue is about definition of intra/inter-frequency L1 measurement:

<Way forward >: Issue 3-2-3: Definition of intra-frequency/inter-frequency in inter-cell operation

  - Option 1 (Apple, Ericsson, vivo, HW, Nokia): RAN4 to discuss the definition of intra-frequency/inter-frequency in inter-cell operation

    - Option 1a (MTK): For SSB L1-RSRP measurement, follow the definition of L3 measurement.

    - Option 1b: For SSB L1-RSRP measurement, intra-frequency is defined as long as the SSB-based L1 measurement is performed within active BWP(s) of the UE.

  - Option 2 (QC): discuss later

When defining requirement framework for L3 measurement in early R15 design, RAN4 had extensive discussion on whether and how to define intra-frequency and inter-frequency RRM measurement. Eventually RAN4 agreed to introduce the concepts of intra-frequency and inter-frequency RRM measurement, even though they both can be gap-based or gap-less measurement. To avoid redundant discussion, RAN4 shall reuse the methodology as much as possible. Thus, RAN4 shall introduce the definition of intra-frequency/inter-frequency in inter-cell mobility.

Proposal 7: RAN4 to discuss the definition of intra-frequency/inter-frequency in L1/L2 inter-cell mobility.

Existing L1 measurement requirements in TS38.133 only apply for measurements on serving cells, including PCell, PSCell, or SCell, on the resources configured for L1 measurements within the active BWP. However, in this work item, UE is expected to perform L1 measurement on neighbour cell for candidate handover, which is also reflected in RAN2 agreement:

Assume that we rely on L1 measurements to trigger L1L2 mobility (still measurement for preparation could be L3, FFS)

Observation 2: UE is expected to perform L1 measurement on neighbour cell for L1/L2 inter-cell mobility.

Proposal 8: for definition of intra-frequency/inter-frequency SSB based L1-RSRP measurement, follow the definition of L3 measurement, i.e. an SSB based L1 measurement is defined as an intra-frequency SSB based L1 measurement provided the centre frequency of the SSB of the serving cell indicated for measurement and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. Otherwise, it is an inter-frequency SSB based L1 measurement.

Regarding the following issues:

<Way forward >: Issue 3-2-5: Whether to cover non-synchronous scenarios

  - Option 1: Not consider non-synchronous scenarios.

  - Option 2 (Ericsson, Nokia): Consider non-synchronous scenarios.

  - Option 3 (Ericsson, HW): FFS

    - Option 3a (Apple, MTK, vivo): discuss the definition of synchronous and non-synchronous

    - Option 3b (Apple, QC, vivo, Nokia): wait for RAN2’s progress

<Way forward >: Issue 3-2-6: Whether to support L1 measurement on multiple cells with PCI different from serving cell

  - Option 1(Apple, HW, vivo, Ericsson, MTK, Nokia): to discuss the number of supported cells with PCI different from serving cell.

    - Option 1a: To discuss the number of supported cells with PCI different from serving cell on a FR2 band

    - Option 1b: To discuss the total number of cells to be monitored on all bands

    - Option 1c: To discuss the total number of cells to be monitored per frequency layer

  - Option 2 (QC): discuss this issue later

<Way forward >: Issue 3-3-1: Intra-frequency L1-RSRP measurement accuracy requirements

  - Option 1 (Nokia, vivo): Discuss whether intra-frequency L1-RSRP measurement accuracy can be improved for L1/L2 mobility

  - Option 2 (MTK, HW): reuse the legacy intra-frequency L1-RSRP measurement accuracy for L1/L2 mobility.

  - Option 3 (Apple, Ericsson, QC): Discuss this issue later, e.g. in performance part

<Way forward >: Issue 3-3-2: Inter-frequency L1-RSRP measurement accuracy requirements

  - Option 1 (Nokia): Define inter-frequency L1-RSRP measurement accuracy requirements on non-serving cell

  - Option 2 (Apple, Ericsson, vivo, MTK, HW): Further discuss whether and how to define inter-frequency L1-RSRP measurement accuracy requirements on non-serving cell later, e.g. in performance part.

Whether to consider non-synchronous scenario, we think it can be discussed when UE behaviour is clear and stable during the mobility procedure, which is being discussed in other working groups. Also, for number of supported cells with PCI different from serving cell, it can be addressed as UE measurement capability. RAN4 typically discusses it after UE measurement behaviour is clear. As for accuracy, it belongs to performance part and can be discussed later.

Besides, as RAN2 agreed, ICBM is not a prerequisite for using L1L2 mobility, RAN4 needs to consider the case wherein ICBM is not supported/configured.

ICBM is one scenario considered for L1L2 mobility, but is not the only one, and is not a prerequisite for using L1L2 mobility.

Proposal 9: as agreed in RAN2, ICBM is not a prerequisite for using L1L2 mobility, RAN4 requirements need to cover the scenario that ICBM is not supported/enabled.

To align with RAN2, RAN4 shall focus on PCell mobility first:

R2 will initially focus on PCell mobility. R2 assumption: Rel-18 L1/L2 mobility includes both non-CA (PCell only) and CA scenarios (PCell and SCell). This includes the following cases - a) the target PCell/target SCell(s) is not a current serving cell (CA CA scenario with PCell change) - b) FFS the target PCell is a current SCell - c) FFS the target SCell is the current PCell. DC scenarios are FFS (e.g. PSCell mobility may be a low hanging fruit FFS).

Proposal 10: RAN4 shall initially focus on PCell mobility. Other scenarios like CA can be discussed later.

# Conclusion

In this contribution, we continue discuss L1/L2 inter-cell mobility from RRM perspective. After discussion the following conclusions are provided:

Proposal 1: L1/L2 inter-cell mobility delay (DL1/L2_mobility) depends on whether RACH is needed/allowed:

- For RACH-less case (if supported), it is defined as the time UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

- For RACH-based case (if supported), it is defined as the time UE receives the cell switch command to UE starts transmission of the new uplink PRACH channel to the target cell.

Proposal 2: cell switch command processing time (Tprocess) needs to be considered in L1/L2 inter-cell mobility delay requirements.

Proposal 3: interruption time during the mobility procedure (Tinterrupt) includes several aspects, such as to acquire fine T/F tracking, suitable beam and so on. Under certain conditions some of the components can be zero.

Observation 1: complexity would be significantly increased if UE needs to support simultaneous data Rx/Tx with both source and target cells during L1/L2 inter-cell mobility. However, the gain is not that attractive since the functionality has already been supported in R16 DAPS handover.

Proposal 4: L1/L2 inter-cell mobility execution time needs to be considered.

Proposal 5: as baseline assumption, UE is not required to perform simultaneous data Rx/Tx with both source and target cells during L1/L2 inter-cell mobility.

Proposal 6: as baseline, do not consider simultaneous multi-Rx in FR2.

Proposal 7: RAN4 to discuss the definition of intra-frequency/inter-frequency in L1/L2 inter-cell mobility.

Observation 2: UE is expected to perform L1 measurement on neighbour cell for L1/L2 inter-cell mobility.

Proposal 8: for definition of intra-frequency/inter-frequency SSB based L1-RSRP measurement, follow the definition of L3 measurement, i.e. an SSB based L1 measurement is defined as an intra-frequency SSB based L1 measurement provided the centre frequency of the SSB of the serving cell indicated for measurement and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. Otherwise, it is an inter-frequency SSB based L1 measurement.

Proposal 9: as agreed in RAN2, ICBM is not a prerequisite for using L1L2 mobility, RAN4 requirements need to cover the scenario that ICBM is not supported/enabled.

Proposal 10: RAN4 shall initially focus on PCell mobility. Other scenarios like CA can be discussed later.

# References

- R4-2214348, WF on Further NR Mobility Enhancements, MediaTek Inc.

- R2-2209257, LS on L1/L2-based inter-cell mobility, MediaTek Inc.