3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215626

# Electronic Meeting, October 10 – October 22, 2022

Agenda item:	5.6.5.2.1

Source:	Apple Inc.

Title:	On CQI Reporting Requirements for Reduced Capability Devices in NR

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#104-e RedCap UE demodulation and CSI reporting requirements for RedCap UEs were discussed and a way forward [1] was agreed. We present our views on the remaining open issues.

# 2. Discussion

During RAN4#104-e the following agreements were made

Agreement: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD

The specifics of the agreement consisted in configuring the following parameters for CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD:

CSI-RS periodicity and offset: 10/1

CSI-Report periodicity and offset: 10/9

CQI/RI/PMI delay: [14ms]

It has been suggested that interested companies should consider evaluating the performance difference between CQI delay 14ms and 10ms during RAN4#104-bis-e. The rationale is that if significant performance degradation is observed compared with CQI delay 10ms, RAN4 may revisit the CQI/RI/PMI delay.

As stated in our previous contributions, we support the tentative agreement of CQI delay of 14ms.

Proposal #1: Introduce CQI/RI/PMI delay of 14ms, along with proposed Resource/Report configuration of [10/1] [10/9] respectively.

Way forward: Static channel matrix used for 1Rx UE and SNR test point offset for CQI reporting tests

Here, according to some interested companies, the WF encourages to evaluate the options for the static channel matrix and SNR test point offset in contrast with the WF of RAN4#103-e. Here

Option 1: Set the static channel matrix in the frequency domain as H1RX=1j. Set SNR test point X=[0]dB lower than 2Rx test case.

Option 2: Keep the previous agreement on the static channel matrix in the frequency domain, that is, H1RX=11. Set SNR test point X=3dB lower than 2Rx test case.

During RAN#103-e, the initial agreement was that for CQI reporting test in static condition for 1Rx, the SNR test points should be 3 dB lower than 2Rx test case. Now, testing both channel options, H=[1 1] and H=[1 j], we can observe that the 3dB offset may not be needed. However, we see no strong technical reason that precludes the use of H=[1 1]. Hence, we support keeping the original agreement.

# 3. Conclusion

In this contribution we provide our position on the remaining open issues and we provide the requested simulation results for CQI reporting requirements for RedCap UE. Our observations and proposals are captured below:

Proposal #1: Introduce CQI/RI/PMI delay of 14ms, along with proposed Resource/Report configuration of [10/1] [10/9] respectively.

Proposal #2: Introduce proposed Aperiodic Report Slot Offset 3 slots and CQI/RI/PMI delay of 6ms.

# Reference

- R4-2214394, “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.

- R4-2214317, “Email discussion summary for [104-e][328] NR_RedCap_Demod”, Ericsson