3GPP TSG-RAN WG4 Meeting # 104bis-e                               R4-2215632

Electronic Meeting, 10th – 19th August 2022

Source: 	Apple

Title: 	Discussion on Beam correspondence test issues

Agenda Item:	6.7.3.3

Document for:	Discussion

# 1. Introduction

Beam correspondence requirement was extensively discussed in RAN4#104-e meeting [1]. A WF was agreed for BC in RRC_INACTIVE and initial access [2], where the following agreement and open issues were identified for further study.

Agreement

- BC test for RRC_INACTIVE (if applicable) and RRC_Idle is at UE maximum output power

- BEAM_LOCK is not available in RRC_INACTIVE and IA mode

Open issue for FFS

- Discuss if and how UE Tx beam should/would change during IA and what are the impact on the test method.

- Feasibility of testing UEs in Inactive mode

- Feasibility of triggering SDT in test mode for different UE implementations

- Whether and how UE Tx beam should/would change without beam lock function during IA when test equipment is holding RAR to achieving UE max output power.

- How to achieve and keep max power in testing

- Number of tests, test time implications and proposed test time reductions

- Polarization aspects without beam lock function in testing

This contribution will present our views for the identified open issues.

# 2. Discussion

According to the current specification, beam correspondence requirement for a UE is fulfilled if the UE meets the minimum peak EIRP requirement and the spherical coverage requirement for the corresponding power class. All these requirement tests should be done at UE maximum output power. Currently the beam correspondence test is designed for connected state with a closed loop power control. The test results will be recorded after the UE reaches its maximum output power and the maximum output power should be maintained during the test. A new mechanism should be designed in initial access to achieve and maintain the maximum output power during test.

Our understanding is also that the beam correspondence test should be based on power related test. The absolute power tolerance test can be considered as the starting point for BC test development in initial access. The following points are key aspects for the test feasibility and should be discussed further.

- How to make sure the UE reach maximum output power

- How to maintain the maximum output power during the test

- Whether it is possible to lock the beam or allow beam change during the test.

In the initial access, the UE measures the RSRP and estimates the PL based on the broadcasted DL RS power at first. Then the required power for PRACH transmission will be calculated based on the estimated PL and the target Rx power at gNB and compares with the Pcmax. Hence the PRACH power is determined by the following formula in 38.213. With this, we can set the UE to transmit at Pcmax for PRACH by well design the signalling parameter.

[dBm],

In the following measurement process, the UE maximum transmission power can be maintained by holding RAR(msg2) and the target received power, preamble power step and number of preamble transmissions as descripted in [4]

Another issue is the beam change issue. We think the performance verification for beam correspondence in initial access should be based on similar consideration on beam change as for RRC_CONNECTED. Currently, RAN5 only enabled beam lock function for RRC_CONNECTED state in 38.509. We propose to send LS to RAN5 to check whether the beam lock function can be extended to initial access.

Proposal 1: It is proposed to check with RAN5 whether beam lock function can be defined for beam correspondence testing in initial access.

Regarding the test for RA-SDT and CG-SDT, we think it is not necessary to double the test effort as the UE behaviour and requirement are same as analysed in [5]. The beam correspondence in initial access is the most fundamental one. To save the test effort, it is proposed to focus IA for beam correspondence test.

# 3. Summary

This contribution presented our consideration on beam correspondence test in initial access. The following proposals are concluded.

Proposal 1: It is proposed to check with RAN5 whether beam lock function can be defined for beam correspondence testing in initial access.

# 4. References

[1] RP-2214244, Email discussion summary for [104-e][133] FR2_enh_req_Ph3, Nokia

[2] R4-2214454, WF on for BC in RRC_INACTIVE and initial access, Nokia

[3] R4-2211915, Beam correspondence for RRC_INACTIVE and initial access, Apple

[4]	R4-1914121, Test configuration for beam correspondence during initial access, Ericsson

[5] R4-211xxxx, Further consideration on BC requirement applicability for IA/RA-SDT/CD-SDT, Apple