3GPP TSG-RAN WG4 Meeting # 104bis-e                               R4-2215633

Electronic Meeting, 10th – 19th August 2022

Source: 	Apple

Title: 	Discussion on ATG general aspects

Agenda Item:	6.13.1

Document for:	Discussion

# Introduction

There were extensive discussions on ATG in the last RAN4 meeting [1]. A WF on ATG UE co-existence was agreed [1]. This contribution will further discuss the general aspects for ATG UE.

# Discussion

## Differential of ATG network and TN network

- “The ATG network should be distinguished from TN network to enable/disable specific type of UE access”

It was agreed to differentiate the ATG network from the TN network in the last meeting.

The agreement comes from the fact that ATG systems will be deployed in the IMT frequency and TN UE may also support the same frequency. So, it is necessary to have some network access control to forbidden TN UE access. to this end, the network can provide a flag to indicate that it is a ATG network/ATG cell. Only ATG UE decoded this flag can access the ATG cell.

The ATG network access control needs to involve RAN2 work. It is proposed to either send RAN2 to ask for such consideration or update the WID scope and then involve RAN2 work.

Proposal 1: it is proposed to send LS to RAN2 to trigger the network access control discussion.

It is understood that ATG UE is mainly used to provide service to aircraft passengers in flight mode. However, it seems better not to preclude that it can also provide services when on the ground. E.g. when in foreigner country, it can provide service to some passenger boarded but not have cellular roaming service.

## Density of aircraft

<Way forward 3>: Clarify an assumption on the density of aircraft within the simulation area.

- Option 1: It’s assumed single UE per ATG BS as a starting point.

- Option 2: FFS

The following was agreed in the last RAN4 meeting.

According to the WID description, the time needed to fly across the ATG cell with 300km radius is about 30 minutes. Our understanding is that there might be multiple aircraft along the same trajectory but with some vertical (e.g. 300m ~1200m) or horizontal distance (>10minutes in time). we believe it is not reasonable to only assume single UE per ATG BS in the simulation. Maybe 3-4 UEs per ATG cell could be considered as the starting point for 200~300 km cell radius.

Proposal 2: More than one UE per ATG BS should be considered for co-existence study.

## Network layout

<Way forward 10>: Potential assumption for network layout of ATG.

- Option 1: ATG network can be deployed on the airline routes.

- Option 2: ATG network can be deployed in larger area

<Way forward 11>: The assumption for network layout of NR terrestrial network

- 19-sites 57 sectors with wrap-around can be assumed.

- FFS how to drop the TN cluster.

  - Option 1: drop TN clusters close to ATG BS

  - Option 2: Dropping clusters of TN cells randomly

The following was agreed in the last RAN4 meeting.

Regarding ATG network, we think it is typic case to be deployed along the airline routes.

If there is no regulation on geographic distance between AGT network and TN network, we should assume that the TN cells are surrounding the ATG BS with random distribution.

Proposal 3: ATG network is deployed along the airline route.

Proposal 4: TN network clusters is dropped randomly surrounding ATG gNB.

## UE altitude

<Way forward 5>: For commercial aircraft flight, what is the range of UE altitude?

- The following upper and lower boundary can be considered.

  - UE altitude (upper boundary): range: 10km

  - UE altitude (lower boundary): range: 3~7km

- Choose one fixed value for one simulation.

The UE altitude for co-existence simulation was agreed as following,

From co-existence point of view, there is no value to consider 7km in addition to 10km, there will be no obvious difference between 7km and 10km. 3km is preferred.

Another factor to be considered is that 3km is the typical altitude for private aircraft and helicopter. We think it’s necessary to consider altitude down to 3km since it is likely that ATG UE is equipped in such aircraft.

Proposal 5: The lower band of ATG UE altitude is 3km.

# Summary

This contribution presented our view on several general aspects for ATG. The following observations and proposals are concluded.

Proposal 1: it is proposed to send LS to RAN2 to trigger the network access control discussion.

Proposal 2: More than one UE per ATG BS should be considered for co-existence study.

Proposal 3: ATG network is deployed along the airline route.

Proposal 4: TN network clusters is dropped randomly surrounding ATG gNB.

Proposal 5: The lower band of ATG UE altitude is 3km.

# References

[1] RP-2214244, Email discussion summary for [104-e][133] FR2_enh_req_Ph3, Nokia

[2] RP-2214459, WF on co-existence evaluation for ATG, Apple