3GPP RAN4 Meeting #104bis-e		R4-2215656

Online meeting, October 10 – 19, 2022

Agenda item:	6.14.1

Source:	Apple

Title:	On TRP TRS requirement development prioritization

Release:	Rel-18

Document for:	Discussion

# 1	Introduction

Following the conclusion of the Rel-17 RAN4-led work item on the Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) [1], [2], 3GPP approved a new Rel-18 work item on the Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies [3] with the following work objectives:

### 4.1 Objective of Core part WI The following objectives are considered in this WI: - Enhancements of TRP TRS test methodology - Specify necessary enhancement of the anechoic-chamber based test methodology (i.e. reference test methodology) to support (test methodology defined in TR 38.834 is the baseline): - UE with NR 2Tx configuration - Case 1: TxD (i.e., TxD capability supported) - Case 2: single layer UL-MIMO (i.e., codebook-based capability supported) - Study proper configuration from UE implementation and test system feasibility perspective - Define test case applicability for case 1 and case 2 - RedCap devices (focused on wearable device only) - Wrist-worn device as 1st priority - Considering UEs with antenna configurations of 1 Tx, 1 Rx, 2 Rx - Forearm phantom should be specified - Consider the coordination with CTIA on this aspect - Other phantoms are not precluded - UE with DL/UL carrier aggregation configuration (2nd priority) - Independent measurements of each CC can be the baseline approach - Limited to Inter-band CA with up to 2DL/2UL cases - Study and, if feasible, specify reverb-chamber based test methodology to support SA and EN-DC TRP TRS testing: - Define test methodology for UE type with 1Tx and 2Tx configuration - Test scenarios support: at least browsing mode, talk mode, and wrist-worn mode - Perform harmonization activity with reference test method - Define harmonization framework and pass/fail criteria - Perform harmonization test campaign according to the framework - Note: the harmonization between anechoic chamber and Reverb-chamber based method can be applied independently for the conditions between 1Tx and 2Tx, among browsing mode, talk mode, and wrist-worn mode. - If reverb-chamber based test methodology is found harmonized with the reference test method, then further discuss applicability of reverb-chamber based test methodology for requirements conformance testing - Develop the Measurement Uncertainty (MU) assessment of the above newly defined test methodologies and test configurations [RAN5]: - Measurement Uncertainty (MU) aspects will be handled by RAN5 and the conclusions can be captured in a separate section of the TR - Browsing mode, talk mode, and wrist-worn mode should be considered - Study and specify the testing time reduction methodology for TRP and TRS testing During the course of this work item, ongoing communication with 3GPP RAN WG5, CTIA OTA Working Group, CCSA TC9 WG1, GCF, GSMA TSG-AP, ETSI MSG TFES, and PTCRB shall be maintained to ensure industry coordination on this topic

### 4.2 Objective of Performance part WI Specify the performance part framework (general for both 1Tx and 2Tx requirements work): - Lab alignment outcome and performance campaign framework in Rel-17 should be baseline with the following considerations: - Decide whether lab alignment is needed for new test bands, new test scenarios and EN-DC configuration - Handheld UE type is the first priority - Consider UE size 1 and size 2 - UE size 1 is the first priority - Consider both browsing mode and talk mode - Further prioritization not precluded - Consider Power class 2 and 1.5 for 2Tx requirements, Power class 3 and 2 for 1Tx requirements - Requirement applicability of TxD and single-layer UL MIMO of the same UE shall be specified - Consider further prioritization of test cases - Taking following as a starting point to further discuss which information needed including for information disclosure: - [Number of models tested by the labs] - Number of vendors that produced the models - percentage of tested devices per vendor - Percentage of models per production year - [Power Class of the devices] - Note 1: Size 1 (wide, width >72mm and ≤92mm), Size 2 (narrow, width ≥56mm and ≤72mm); - Note 2: Browsing mode (hand phantom only), Talk mode (head and hand phantom); - Specify TRP TRS requirements and recommended tolerance for UE with NR 2Tx for handheld UE based on enhanced reference test method and defined performance part framework - Specify the requirements and test tolerance for UE with SA mode - Band n41, n77 and n78, as the first priority - FDD bands are not precluded - SA with 1 CC is the first priority - Specify TRP TRS requirements and recommended tolerance for UE with NR 1Tx (SA and EN-DC) based on reference test method and defined performance part framework - Specify the requirements and recommended tolerance for UE - SA: n3, n5, n7, n8, and n28 with PC3, n1, n77 and n79 with PC2 and PC3 - For talk mode requirements, n41 and n78 with PC2 should also be additionally considered - Further prioritization on band list not precluded - EN-DC: NR SA bands related EN-DC band combinations are the first priority, only NR carrier requirement will be specified - Rel-17 band combination conclusions in TS 38.161 should be the basis - For each newly added NR band to the scope, at least one example band combination shall be defined

In the contribution we share our initial views on the prioritization of the performance requirement scope for Rel-18 TRP/TRS.

# 2	Discussion

Considering the very large scope of the performance part of the TRP/TRS work item, it is useful to begin a discussion about the prioritization of the performance tasks.  We can structure this into three main categories:  what lab alignment needs should be addressed in the work item, prioritization of test cases, and prioritization of bands/configurations.

Considering lab alignment procedures, we anticipate the Rel-18 work item will define requirements for the talk mode (beside head and hand phantom) test case.  Since the Rel-17 lab alignment campaign only measured the browsing mode (hand phantom) test case, the talk mode alignment results are necessary.

Observation 1:	Talk mode (beside head and hand phantom) test case should be included in Rel-18 TRP/TRS lab alignment procedures.

With band n28 in scope of performance requirement work, the frequency range over which labs need to be aligned increases from 2496 – 3800 MHz (bands n41 and n78) down to 703 MHz.  Additionally, the band is the first FDD band to be considered for NR OTA work, and lab alignment verification for this band is necessary.

Observation 2:	Lab alignment for band n28 should be included in Rel-18 TRP/TRS lab alignment procedures.

Considering the remaining two high bands, n77 and n79, we observe that n77 overlaps with n78, for which lab alignment procedures were already concluded in Rel-17.  Band n79, however, increases the upper limit of the overall frequency span so far considered for OTA work to 5000 MHz.

Observation 3:	Lab alignment for band n77 is not necessary, as alignment with band n78 should be applicable, while alignment for band n79 should be included in Rel-18 TRP/TRS lab alignment procedures.

Finally, the newly developed test methodology for devices capable of UL MIMO certainly merits lab alignment procedures in the Rel-18 work plan.

Observation 4:	Lab alignment for 2Tx test cases, such as UL MIMO, should be included in Rel-18 TRP/TRS lab alignment procedures pending completion of definition of NR 2Tx test methodology.

In terms of test cases, the Rel-18 TRP/TRS work item aims to define requirements for 1 Tx UEs, according to procedures already defined in Rel-17 (we can refer to this work as performance requirement continuation from Rel-17) and for 2 Tx UEs (e.g. UL MIMO capable UEs), according to procedures which will be developed as part of the Rel-18 TRP/TRS core work item objectives.  We consider both of these objectives the highest priority from the performance perspective.

Observation 5:	It is recommended to prioritize browsing and talk mode test cases for 1 Tx and browsing mode test cases for 2 Tx with UL MIMO in the Rel-18 TRP/TRS performance work item (talk mode test cases for 2 Tx could be 2nd priority).

Arriving at the list of bands mentioned in the Rel-18 WID, we generally note that this list is too large for the work item to accommodate in a single release.  Recalling that RAN4 was able to conclude only two bands in Rel-17, realistic scope in terms of band coverage needs to be selected.

Observation 6:	For browsing mode 1 Tx test cases, it is recommended to consider n28 and n77 or n79, depending on operator request.

Observation 7:	For talk mode 1 Tx test cases, it is recommended to consider n41 and n78 as the first priority (to match the Rel-17 outcomes with browsing mode) and then to aim for n28 and n77 or n79 as the second priority.

Observation 8:	For 2 Tx with UL MIMO, it is recommended to consider bands n41 and n78, with bands n28 and n77 or n79 as the second priority.

Observation 9:	Following the Rel-17 arrangement, it is recommended to focus the performance requirement development on devices operating in stand-alone mode, and all test cases which feature other bands not mentioned in Observations 6 – 8, as well as requirements for devices in NSA mode, to be treated as the third priority.

Observation 10:	The lab alignment and subsequent performance campaign for each of the items in Observation 2, Observation 3, Observation 1 and Observation 4 (in that order) be planned sequentially and staggered by up to one meeting cycle gap if possible.

Proposal 1:	It is proposed to take Observations 1 through 10 into account when defining the work scope for the performance part of the Rel-18 TRP/TRS work item.

# Conclusions

In the contribution we share our initial views on the prioritization of the performance requirement scope for Rel-18 TRP/TRS.  The following observations and proposals are presented:

Observation 1:	Talk mode (beside head and hand phantom) test case should be included in Rel-18 TRP/TRS lab alignment procedures.

Observation 2:	Lab alignment for band n28 should be included in Rel-18 TRP/TRS lab alignment procedures.

Observation 3:	Lab alignment for band n77 is not necessary, as alignment with band n78 should be applicable, while alignment for band n79 should be included in Rel-18 TRP/TRS lab alignment procedures.

Observation 4:	Lab alignment for 2Tx test cases, such as UL MIMO, should be included in Rel-18 TRP/TRS lab alignment procedures pending completion of definition of NR 2Tx test methodology.

Observation 5:	It is recommended to prioritize browsing and talk mode test cases for 1 Tx and browsing mode test cases for 2 Tx with UL MIMO in the Rel-18 TRP/TRS performance work item (talk mode test cases for 2 Tx could be 2nd priority).

Observation 6:	For browsing mode 1 Tx test cases, it is recommended to consider n28 and n77 or n79, depending on operator request.

Observation 7:	For talk mode 1 Tx test cases, it is recommended to consider n41 and n78 as the first priority (to match the Rel-17 outcomes with browsing mode) and then to aim for n28 and n77 or n79 as the second priority.

Observation 8:	For 2 Tx with UL MIMO, it is recommended to consider bands n41 and n78, with bands n28 and n77 or n79 as the second priority.

Observation 9:	Following the Rel-17 arrangement, it is recommended to focus the performance requirement development on devices operating in stand-alone mode, and all test cases which feature other bands not mentioned in Observations 6 – 8, as well as requirements for devices in NSA mode, to be treated as the third priority.

Observation 10:	The lab alignment and subsequent performance campaign for each of the items in Observation 2, Observation 3, Observation 1 and Observation 4 (in that order) be planned sequentially and staggered by up to one meeting cycle gap if possible.

Proposal 1:	It is proposed to take Observations 1 through 10 into account when defining the work scope for the performance part of the Rel-18 TRP/TRS work item.

# 4	References

- RP-222269, "Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)," vivo, 3GPP RAN #97, September 2022

- RP-222152, "Status report for WI: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC); rapporteur: vivo," 3GPP RAN #97, September 2022

- RP-222669, “Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies,” vivo, 3GPP RAN #97, September 2022