3GPP RAN WG4 Meeting #104bis-e		R4-2215666

Online, October 10th – 19th, 2022

Agenda item:	6.6.4.1

Source:	Apple

Title:		Further analyses and views on MSD improvement for inter-band CA/DC

WI/SI:	NR_ENDC_ RF_FR1_enh2-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

The desire on seeking for potential MSD improvement for inter-band CA/EN-DC/DC combinations and the feasibility for UE to report lower MSD capability has been formulated as part of the objectives in Rel-18 WID on further RF requirements enhancement for NR and EN-DC in (FR1) [1]. In last RAN4 meeting, the initial discussions on this topic had led to an approved WF [2] which concludes the example band combinations for MSD analysis and the aspects of MSD improvement and lower MSD signaling studies.

In this contribution, in addition to continuing our analyses from last RAN4 meeting [3] on MSD improvement for CA_n3-n78 which is a fallback combination of the example band combination CA_n1-n3-n78, we also share our analysis on MSD improvement for CA_n28-n40 where the n28 REFSENS degradation is caused by 3rd order Rx harmonic mixing.

# Discussion

In last RAN4 meeting, we have provided our MSD analyses on CA_n3-n78 for both UL 2nd harmonic and 2UL IMD2 induced REFSENS degradation and illustrated the improvement of MSD over improved antenna and PCB isolations [2]. The results of our analyses are recaptured as below in Figure 2-1 and Figure 2-2 for UL 2nd harmonic MSD and 2UL IMD2 MSD respectively.

Figure 2-1 CA_n3-n78 UL 2nd harmonic MSD improvement over improved PCB and antenna isolations

Figure 2-2 CA_n3-n78 2UL IMD2 MSD improvement over improved PCB and antenna isolations

In this contribution, we further analyze CA_n3-n78 UL 2nd harmonic and 2UL IMD2 MSD improvement over other RF parameters in section 2.1 and 2.2 respectively. We also provide the MSD improvement analysis for CA_n28-n40 in section 2.3 where the n28 REFSENS degradation is caused by 3rd order Rx harmonic mixing.

## 2.1		CA_n3-n78 UL 2nd harmonic MSD analysis

Figure 2.1-1 shows the MSD improvement over the harmonic filter rejection ratio. It can be seen that when harmonic filter rejection is above 20 dB, the MSD would be dominated by other impairments.

Figure 2.1-1 CA_n3-n78 UL 2nd harmonic MSD improvement over improved harmonic filter rejection

Observation 1: When harmonic filter rejection is above 20 dB, the MSD would be dominated by other impairments.

Figure 2.1-2 shows the MSD improvement over the combined diplexer/n78 filter rejection ratio to n3 Tx. It can be seen that when the combined rejection ratio is above 60 dB, the MSD would be dominated by other impairments.

Figure 2.1-2 UL 2nd harmonic MSD improvement over improved diplexer+n78 filter rejection to n3 Tx

Observation 2: When the combined diplexer+n78 filter rejection ratio to n3 Tx is above 60 dB, the MSD would be dominated by other impairments.

Figure 2.1-3 shows the MSD improvement over the PA 2nd harmonic rejection ratio. It can be seen that when the PA 2nd harmonic rejection ratio is above 56 dB, the MSD would be dominated by other impairments.

Figure 2.1-3 UL 2nd harmonic MSD improvement over improved PA 2nd harmonic rejection ratio

Observation 3: When the PA 2nd harmonic rejection ratio is above 56 dB, the MSD would be dominated by other impairments.

Figure 2.1-4 shows the MSD improvement over LNA IP2 performance. It can be seen that the MSD though improved with increasing LNA IP2, the improvement is quite limited even with LNA IP2 up to 30 dBm.

Observation 4: MSD though improved with increasing LNA IP2, the improvement is quite limited even with LNA IP2 up to 30 dBm.

Figure 2.1-4 UL 2nd harmonic MSD improvement over improved LNA IP2

Based on the above observations, we can conclude that without PCB and antenna isolations improvement, the MSD from other improved RF parameters alone cannot be reduced to below 18 dB.

Observation 5: Without PCB and antenna isolations improvement, the UL 2nd harmonic MSD from other improved RF parameters alone cannot be reduced to below 18 dB.

## 2.2		CA_n3-n78 2UL IMD2 MSD analysis

Figure 2.2-1 shows the MSD improvement over the duplexer rejection ratio for IMD2 at n3 DL. It can be seen that when the duplexer rejection ratio is above 70 dB, the MSD would be dominated by other impairments.

Figure 2.2-1 CA_n3-n78 2UL IMD2 MSD improvement over improved duplexer rejection ratio at n3 DL

Observation 6: When the duplexer rejection ratio for IMD2 at n3 DL is above 70 dB, the MSD would be dominated by other impairments.

Figure 2.2-2 shows the MSD improvement over PA forward mixing IP2. It can be seen that when the PA forward mixing IP2 is above 45 dB, the MSD would be dominated by other impairments.

Figure 2.2-2 CA_n3-n78 2UL IMD2 MSD improvement over improved PA forward mixing IP2

Observation 7: When the PA forward mixing IP2 is above 45 dB, the MSD would be dominated by other impairments.

Figure 2.2-3 shows the MSD improvement over LNA IP2 performance. It can be seen that the MSD though improved with increasing LNA IP2, the improvement is quite limited even with LNA IP2 up to 20 dBm.

Figure 2.2-3 CA_n3-n78 2UL IMD2 MSD improvement over improved LNA IP2

Observation 8: MSD though improved with increasing LNA IP2, the improvement is quite limited even with LNA IP2 up to 20 dBm.

Figure 2.2-4 shows the MSD improvement over diversity n3 filter rejection ratio to n3 and n78 UL. It can be seen that when the n3 diversity Rx filter rejection ratio to n3 and n78 UL is above 46 dB, the MSD would be dominated by other impairments.

Observation 9: when the n3 diversity Rx filter rejection ratio to n3 and n78 UL is above 46 dB, the MSD would be dominated by other impairments.

Figure 2.2-4 UL 2nd harmonic MSD improvement over improved LNA IP2

Based on the above observations, we can conclude that without PCB and antenna isolations improvement, the MSD from other improved RF parameters alone cannot be reduced to below 18 dB.

Observation 10: Without PCB and antenna isolations improvement, the 2UL IMD2 MSD from other improved RF parameters alone cannot be reduced to below 18 dB.

## 2.3		CA_n28-n40 3rd order harmonic mixing MSD analysis

To assess the potential MSD improvement for CA_n28-n40 due to n28 Rx 3rd order harmonic mixing, we have performed the link analysis based on the reference architecture as shown in Figure 2.3-1 and a set of front-end component isolation parameters as summarized in Table 2.3-1.

Table 2.3-2 presents the link analysis to derive the equivalent interference power levels at n28 main and diversity Rx inputs. The MSD for n28 5MHz DL channel BW after MRC assuming uncorrelated noise is summarized in Table 2.3-3.

Figure 2.3-1 Reference architecture for CA_n28-n40 n28 Rx 3rd order harmonic mixing MSD analysis

| Isolation | dB |
| --- | --- |
| Diplexer | 20 |
| n28 duplexer to n40 | 40 |
| n28 diversity Rx filter to n40 | 33 |
| PCB | 65 |
| Antenna | 10 |
| Rx 3rd harmonic rejection ratio | 25 |

Table 2.3-1 Front-end component isolation parameters for MSD calculation

| Insertion loss (dB) | 3 |  |  |
| --- | --- | --- | --- |
| PA output power (dBm) | 26 |  |  |
| Main Path | dBm | Diversity Path | dBm |
| n40 UL power at antenna port | 23 | n40 UL power at antenna port | 13 |
| n40 UL power after diplexer | 3 | n40 UL power after diplexer | -7 |
| n40 UL power at n28 LNA input (filter path) | -40 | n40 UL power at n28 LNA input (filter path) | -40 |
| n40 UL power at n28 LNA input (PCB) | -39 | n40 UL power at n28 LNA input (PCB) | -39 |
| Total n40 UL power at n28 LNA input | -36.5 | Total n40 UL power at n28 LNA input | -36.5 |
| Total equivalent interference power after harmonic rejection | -58.5 | LNA H2 referred to antenna | -58.5 |

Table 2.3-2 Link analysis to derive the interference power levels at n28 main and diversity Rx inputs

| BW |  | Floor | H2 | Total |
| --- | --- | --- | --- | --- |
| 5 MHz | Main Path (dBm) | -95.5 | -58.5 | -58.5 |
|  | Diversity Path (dBm) | -95.5 | -58.5 | -58.5 |
|  | After MRC (dBm) | -98.5 |  | -61.5 |
|  | MSD (dB) |  |  | 37.0 |

Table 2.3-3 MSD for 5MHz DL channel BW after MRC

Though there are many factors affecting the MSD value, from the link analysis we have observed that the most effective way to reduce the MSD due to UL 2nd harmonic interference is to improve PCB isolation and antenna isolation simultaneously. Thereby without taking into account the practical implementation feasibility, we have evaluated the MSD improvement over the improved PCB isolation up to 100 dB and antenna isolation up to 20 dB. The result for 5MHz DL channel BW is shown in Figure 2.3-2.

Based on the above analysis, it can be seen that even with 20dB antenna isolation and 100 PCB isolation, the improved MSD is still above 25 dB. As a result, it would be rather challenging to improve MSD caused by 3rd order harmonic mixing to below 30 dB based on practical UE implementation.

Observation 11: It would be rather challenging to improve MSD caused by 3rd order harmonic mixing to below 30 dB based on practical UE implementation.

Figure 2.3-2 3rd order harmonic mixing MSD improvement over improved PCB and antenna isolations

Though the RF parameter assumptions for MSD analyses may vary among companies, when considering the potential improvement, company shall present their assumptions for link analysis or measurement results instead of only showing the MSD numbers, otherwise, it would not be conceivable for other companies to envision how the MSD had been derived.

Proposal: When considering the potential MSD improvement, company shall present their assumptions for link analysis or measurement results instead of only showing the MSD numbers.

# 3	Conclusion

In this contribution, in addition to continuing our analyses from last RAN4 meeting [3] on MSD improvement for CA_n3-n78 which is a fallback combination of the example band combination CA_n1-n3-n78, we also share our analysis on MSD improvement for CA_n28-n40 where the n28 REFSENS degradation is caused by 3rd order Rx harmonic mixing.

Observation 1: When harmonic filter rejection is above 20 dB, the MSD would be dominated by other impairments.

Observation 2: When the combined diplexer+n78 filter rejection ratio to n3 Tx is above 60 dB, the MSD would be dominated by other impairments.

Observation 3: When the PA 2nd harmonic rejection ratio is above 56 dB, the MSD would be dominated by other impairments.

Observation 4: MSD though improved with increasing LNA IP2, the improvement is quite limited even with LNA IP2 up to 30 dBm.

Observation 5: Without PCB and antenna isolations improvement, the UL 2nd harmonic MSD from other improved RF parameters alone cannot be reduced to below 18 dB.

Observation 6: When the duplexer rejection ratio for IMD2 at n3 DL is above 70 dB, the MSD would be dominated by other impairments.

Observation 7: When the PA forward mixing IP2 is above 45 dB, the MSD would be dominated by other impairments.

Observation 8: MSD though improved with increasing LNA IP2, the improvement is quite limited even with LNA IP2 up to 20 dBm.

Observation 9: when the n3 diversity Rx filter rejection ratio to n3 and n78 UL is above 46 dB, the MSD would be dominated by other impairments.

Observation 10: Without PCB and antenna isolations improvement, the 2UL IMD2 MSD from other improved RF parameters alone cannot be reduced to below 18 dB.

Observation 11: It would be rather challenging to improve MSD caused by 3rd order harmonic mixing to below 30 dB based on practical UE implementation.

Proposal: When considering the potential MSD improvement, company shall present their assumptions for link analysis or measurement results instead of only showing the MSD numbers.

# 4	References

RP-221496 “Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)”, Huawei, HiSilicon, 3GPP TSG-RAN Meeting #96, Budapest, Hungary, June 6th – 9th, 2022

R4-2214452 “WF on study for lower MSD”, Huawei, 3GPP TSG-RAN WG4 Meeting #104-e, Online, August 16th – 27th, 2022

R4-2212382 “Analyses and views on MSD improvement for inter-band CA/DC”, Apple, 3GPP TSG-RAN WG4 Meeting #104-e, Online, August 16th – 27th, 2022