3GPP TSG-RAN WG4 Meeting #104bis-e 	R4-2215674

Electronic meeting, 10th – 19th October, 2022

Source:	Ericsson

Title:	Discussion on general issues for SAN and UE demodulation requirement

Agenda item:	4.2.7.1

Document for:	Discussion

# Introduction

In the last RAN4#104-e meeting, companies have discussions on the non-terrestrial network demodulation requirements. Following WF [1] was agreed on the general issue for SAN demodulation requirement issues.

### Sub-topic 1-1

Issue 1-1: Channel model for NTN-TDLA (NLOS)

- Proposals: to confirm the following tentative agreements:

  - Option 1: DS =100ns, Doppler = 200Hz (residual Doppler after UE compensation)

  - Option 2: specify other option if any.

- Agreement:

  - DS =100ns, Doppler = 200Hz

### Sub-topic 1-2

Issue 1-2: Channel model for NTN-TDLC (LOS)

- Proposals: continue to discuss DS and K-factor for NTN-TDLC (LOS)

  - Option 1: DS = 10ns, Doppler =200Hz, K-factor=3.59dB

  - Option 2: DS=12ns, Doppler=200Hz, K-factor=3dB

  - Option 3: DS=100ns, Doppler=200Hz, K-factor=21.6dB

  - Option 4: DS=3.5ns, Doppler=200Hz, K-factor=8.05dB

- Agreement:

  - Assuming elevation angle: 30 degree to decide K_offset and K-factor

    - DS = [3.5ns],   Doppler =200Hz, K_factor= 8.05 dB , K_offset = [8 slots]

### Sub-topic 1-3

Issue 1-3: Channel model parameter combination

- Proposals: to confirm the following tentative agreements:

  - Option 1: Use NTN-TDLX <DS>-<Fd> for channel parameter combination for NTN demodulation

  - Option 2: specify other option if any.

- Agreement:

  - Option 1 agreed

### Sub-topic 1-4

Issue 1-4: Doppler shift due to satellite motion for DL in service link

- Proposals: To confirm the following tentative agreements

  - Option 1: Confirm the agreement that do not verify the UE compensation prior to the baseband processing.

  - Option 2: specify other option if any.

- Agreement:

  - Option 1 agreed

### Sub-topic 1-5

Issue 1-5: Timing drift and sampling frequency offset

- Proposals: to confirm the following tentative agreements:

  - Option 1: Confirm the agreement that do not define sampling offset model.

  - Option 2: specify other option if any.

- Agreement:

  - Option 1 agreed

In this contribution, how to choose NTN channel model is further analyzed.

# Discussion

For agreed LOS channel model, DS = [3.5ns],   Doppler =200Hz, K_factor= 8.05 dB , K_offset = [8 slots]. It would be better to round DS to an integral value, i.e., 5ns, which would be easier for channel model expression. The corresponding impact on performance could be ignored.

Furthermore, the K_offset value could be captured in the parameter configuration table.

Proposal 1: Taking NTN-TDLD5-200 as LOS channel model. Capture K_offset in the parameter configuration table.

Proposal 2: Accept following delay profile for NTN demodulation requirements.

Table 1 NTN-TDLA100 delay profile (resolution 5ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 110 | -4.675 | Rayleigh |
| 3 | 285 | -6.482 | Rayleigh |

Table 2 NTN-TDLC5 delay profile (resolution 5ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.394 | LOS path |
|  | 0 | -8.668 | Rayleigh |
| 2 | 60 | -21.423 | Rayleigh |
| NOTE: The first tap follows a Rician distribution with a K-factor of K1 = 8.05 dB and a mean power of 0dB. |  |  |  |

# Conclusions

Proposal 1: Taking NTN-TDLD5-200 as LOS channel model. Capture K_offset in the parameter configuration table.

Proposal 2: Accept following delay profile for NTN demodulation requirements.

Table 1 NTN-TDLA100 delay profile (resolution 5ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 110 | -4.675 | Rayleigh |
| 3 | 285 | -6.482 | Rayleigh |

Table 2 NTN-TDLC5 delay profile (resolution 5ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.394 | LOS path |
|  | 0 | -8.668 | Rayleigh |
| 2 | 60 | -21.423 | Rayleigh |
| NOTE: The first tap follows a Rician distribution with a K-factor of K1 = 8.05 dB and a mean power of 0dB. |  |  |  |

# References

[1]	R4-2214386, WF for NTN demodulation requirements - general and PDSCH, Qualcomm