3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215737

# Electronic Meeting, 10th-19th, Oct, 2022

Source: 	Samsung

Title: 	Views on FR1 inter-band UL CA co-existence requirements

Agenda Item:	6.3.3

Document for:	Discussion

- Introduction

In last RAN4 meeting, a WF [1] was generated to follow up with the proposal and discussion about the potential approach to simplify the FR1 2UL inter-band CA UE co-existence requirements without an explicit coexistence table, for the purpose of specification simplification and efforts saving on manually update and checking. The WF records:

- Companies are encouraged to investigate whether the 2UL inter-band CA UE co-existence requirements can be specified based on the intersection set of the protected bands and frequency ranges from each constituent band without an explicit coexistence table.

- Investigate on how to handle the intersection band/ranges with different requirements from each constituent band.

- Identify if there are exceptions not applicable to the principle of the intersection set and how to handle them in either a generic or a combination specific approach.

In this paper, we present our views on this issue.

- 2. Discussion

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. Clause 6.5.3.2 of 38.101-1 specifies the requirements for single NR bands for coexistence with protected bands. Clause 6.5A.3.2.3 of 38.101-1 specifies the requirements for inter-band UL CA combinations for coexistence with protected bands.

Observation 1: The following rules are carried out when we are in preparation of defining the SE requirements for protected bands including the explicit band numbers and frequency ranges for a band combination:

- SE requirements of protected bands are derived based on the intersection set from each constituent band. The corresponding note should be applied as well. (i.e. The corresponding note should also be applied in case the protected band of one constituent band does not own this note while the other does);

- SE requirements of frequency ranges are derived based on the union set from each constituent band;

- Identify and specify the exceptional case, on top of 1), 2);

- Update is required when the SE requirement of the constituent band has been updated.

Careful consideration is required especially for Rule-1),-2) in Observartion1, if justified, replace the coexistence table with a normative text as illustrated in [2] is a potential way to go, for the purpose of specification simplification and efforts saving on manually update and checking. In addition, new SE requirement table for the exceptional cases is necessary in case the original one is replaced with the normative text if adopted.

Observation 2: SE requirement table for the exceptional cases is necessary in case the original one is replaced with the normative text if adopted.

Proposal 1: It is proposed to discuss whether the rules in Observation 1 are justified.

- 3. Conclusion

Observation 1: The following rules are carried out when we are in preparation of defining the SE requirements for protected bands including the explicit band numbers and frequency ranges for a band combination:

- SE requirements of protected bands are derived based on the intersection set from each constituent band. The corresponding note should be applied as well. (i.e. The corresponding note should also be applied in case the protected band of one constituent band does not own this note while the other does);

- SE requirements of frequency ranges are derived based on the union set from each constituent band;

- Identify and specify the exceptional case, on top of 1), 2);

- Update is required when the SE requirement of the constituent band has been updated.

Observation 2: SE requirement table for the exceptional cases is necessary in case the original one is replaced with the normative text if adopted.

Proposal 1: It is proposed to discuss whether the rules in Observation 1 are justified.

- 4. Reference

[1] R4-2214448, WF on FR1 2UL inter-band CA coexistence requirements, Apple, RAN4#104-e

[2] R4-2212357, On FR1 2UL inter-band CA coexistence requirements, Apple, RAN4#104-e