3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215743

Electronic Meeting, October 10 – October 19, 2022

Agenda item:			4.5.1.1

Source:			Samsung

Title:	Discussion on remaining issues of FeMIMO RRM core requirements for unified TCI state

Document for:		Discussion

- Introduction

In RAN4#104-e meeting, a WF on RRM core part for unified TCI state was approved in [1]. There are still some open issues for RRM core requirements maintenance on unified TCI state.

In this contribution, we provide our consideration of open issues and give our proposals.

- Discussion

As shown in [1], it is still open if UE needs to track UL time/frequency for UL TCI state activation when DL-RS is associated with serving cell or with cell with different PCI:

Issue1-1-1a Whether UE need to track UL time/frequency for UL TCI state activation when DL-RS is associated with serving cell - Proposal: - Option 1: - No, UL timing for cell with different PCI if derived from DL timing of serving cell - Option 2: - Depends on whether source RS in active UL TCI state is a subset of source RS in DL active TCI list

Issue1-1-1b Whether UE need to track UL time/frequency for UL TCI state activation when DL-RS is associated with cell with different PCI - Proposal: - Option 1: - No, UL timing for cell with different PCI if derived from DL timing of serving cell in Rel-17 - Option 2: - Depends on whether source RS in active UL TCI state is a subset of source RS in DL active TCI list

For serving cell, the uplink timing can be derived from the current serving cell DL timing. UE doesn't need to track time/frequency on DL-RS associated with active UL TCI. If the UL TCI state is for the cell with different PCI, it is needed for time/frequency tracking. But in Rel-17, the assumption is: serving cell and non-serving cell are timed within CP length. In addition, the same TAG is for serving cell and additional PCI. The only difference is spatial info but not timing. Under this assumption, we think UE doesn't need to track time/frequency in Rel-17.

Proposal 1: UE doesn’t need to track UL time/frequency for UL TCI state activation when DL-RS is associated with serving cell. UE doesn’t need to track UL time/frequency for UL TCI state activation when DL-RS is associated with non-serving cell in Rel-17.

In previous meetings, companies discussed about the scenario when SSB is indicated as PL-RS in UL TCI state for FR2 and no consensus are achieved.

Issue 1-2-2 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2 - Proposals - Option 1: - Longer delay is expected. - Option 2: - Reuse the existing delay requirement of MAC CE based UL TCI state switch.

In our understanding, UE needs to perform RX beam sweeping for SSB because there is no TCI info for SSB if SSB is indicated as PL-RS. But how long is the delay, it depends on UE implementation. One proposal is to specify the worst case to consider both 5 samples of PL-RS and 8 RX beams. Another approach is in RAN4 spec, it is just said longer delay is expected. We prefer option 1.

Proposal 2: For MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2, longer delay is expected.

Issue 1-4-1 Whether to consider unknown TCI state in the TCI state list - Proposals - If one or more TCI states in the active TCI state list is unknown, active DL TCI state list update delay is FFS. Issue 1-4-2 MAC CE based TCI state list update delay for unknown TCI state - Depends on issue 1-4-1.

In last meeting, for TCI state list update, unknown TCI state was discussed. And the agreement is achieved as:

Agreement:

- [Longer delay applies if any TCI state is unknown in TCI state list update]. Active TCI state list can contains known and unkown TCI states.

As discussed before, there is no requirement to consider unknown TCI state in R15/R16. For fast DCI based switch, unknown TCI state in not the common case. We agree that active TCI state list can contains known and unknown TCI states from the technical definition. But RAN4 won’t define the delay requirement for TCI state list update. We prefer to not define the delay requirement for unknown TCI state.

Proposal 3: For unknown TCI state in the TCI state list, follow the agreements in last meeting and no requirements for unknown TCI state.

- Conclusion

In this contribution, we provide our consideration of remaining issues of core requirements for unified TCI state and our proposals are:

Proposal 1: UE doesn’t need to track UL time/frequency for UL TCI state activation when DL-RS is associated with serving cell. UE doesn’t need to track UL time/frequency for UL TCI state activation when DL-RS is associated with non-serving cell in Rel-17.

Proposal 2: For MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2, longer delay is expected.

Proposal 3: For unknown TCI state in the TCI state list, follow the agreements in last meeting and no requirements for unknown TCI state.

- Reference

[1] R4-2214481, WF on FeMIMO RRM impact for unified TCI, Intel