3GPP TSG-RAN WG4 Meeting # 104-bis-e													R4-2215751

Electronic Meeting, 10– 19 Oct. 2022

Agenda item:			4.2.5.1

Source:	MediaTek Inc.

Title:	Discussion on measurement procedure requirements in NTN

Document for:	Discussion

# Introduction

In this contribution, our view on the measurement requirements for Fully Overlapping Concurrent MGs in NTN RRM is provided.

# Fully Overlapping Concurrent MGs

In the last meeting, the requirement on fully Overlapping Concurrent MGs was discussed as in WF [1].

### Issue 4. Fully Overlapping Concurrent MGs

Agreement:

- Option 1: Do not define requirements for fully overlapping concurrent MGs

- Option 2: For fully overlapped case, gap sharing rule is applied during the collided gap occasions, and the scaling factor is 2

  - Option 2A:

    - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms.

    - A MG with the lowest ID, i.e. 0, gets priority over the other, and the dropping rule starts from SFN=0, i.e. MG-ID#0 is selected and MG-ID#1 is dropped at the first collision instance after SFN=0, and it alternates afterwards.

    - [RAN4 introduce a new UE capability supporting “fully overlapping concurrent MGs” which is limited to NTN-only.]

  - Option 2B:

    - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms.

    - RAN4 introduce a new UE capability supporting “fully overlapping concurrent MGs” which is limited to NTN-only.

  - Option 2C:

    - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms.

Option 2 is preferred as the scaling factor is a common approach also used in other requirement such as inter-frequency measurement. In Option 2A, the specific measurement pattern is less preferable as it limits the UE implementation flexibility and may not be the best way for mobility performance.

Proposal 1: For fully overlapped case, gap sharing rule is applied during the collided gap occasions, and the scaling factor is 2 (Option 2).

# Reference

- R4-2214471, “WF on NR NTN RRM requirements”, Qualcomm, RAN4#104-e meeting, August 2022.