3GPP TSG-RAN WG4 Meeting #104bis-e 		 			     R4-2215786

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	L1 part enhancement for FR2 SCell activation

Agenda item:	6.9.2.2

Document for:	Discussion

Introduction

At RAN4#104e meeting, the WI Even Further RRM enhancement for NR and MR-DC was discussed for the first time. Companies brought contributions identifying the potential problems on FR2 SCell activation and the WF was agreed in [1] leaving all the proposals open for further discussion. In this paper, we are discussing the L1 part enhancement for FR2 SCell activation delay reduction.

Discussion

Beam sweeping factor reduction

Issue 2-3-1: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

- FFS on beam sweeping factor reduction for L1 measurement during unknown FR2 SCell activation.

  - Option 1a (vivo): Introduce the UE capability to support Rx beam sweeping factor can be less than 8 for FR2

  - Option 1b (Ericsson):

    - RAN4 to discuss the solutions or pre-conditions to reduce the scaling factor N1 in FR2 SCell activation.

    - RAN4 to define RX beam constant time. Where, RX beam constant time is a time duration or window within which the RX beams are assumed to be constant or non changing. Rx beam constant time to be agreed as [X=1280ms].

    - If UE performed a full RX beam sweeping for a procedure, next procedures or steps fall within RX beam constant time do not need to perform RX beam sweeping.

    - UE can speed up the remaining steps in SCell activation with a shorter beam scaling factor based on prior Rx beam information from the step before

  - Option 2 (LG, MTK, HW, ZTE): Consider reporting (rough) best Tx beam during cell detection to reduce L1-RSRP measurement delay

The first issue discussed on L1 part enhancement is also beam sweeping factor. Comparing to L3 part enhancements, the UE has completed cell detection before the start of L1-RSRP measurement. It is able to utilize the available measurement information to determine a suitable Rx beam setting thus the L1-RSRP measurement period can be reduced. In some cases e.g. if the reference signals used for L1-RSRP measurement has been measured during cell detection, the L1-RSRP measurement may be skipped.  The conditions for skipping the L1-RSRP measurement can be discussed.

Proposal 1: For FR2 unknown SCell activation, a smaller Rx beam sweeping factor is assumed for L1-RSRP measurement based on the measurement during cell detection.

Proposal 2: RAN4 to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell.

Sample number enhancement

Issue 2-3-3: Sample number enhancement in L1-RSRP measurement of FR2 unknown SCell activation

- FFS: the sample number of PHY filtering cannot be reduced since M=1 is used for L1-RSRP measurement in FR2 unknown SCell activation requirement.

In SCell activation delay requirement, the L1-RSRP measurement delay is defined already assuming M=1 as below, which means 1 sample is assumed for L1-RSRP measurement during SCell activation. Therefore, the sample number for L1-RSRP measurement cannot be further reduced.

TL1-RSRP, measure is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms or TL1-RSRP_Measurement_Period_CSI-RS based on applicability as defined in clause 9.5 assuming M=1.

Proposal 3: Do not consider the sample number reduction for L1-RSRP measurement as 1 sample has been assumed for SCell activation.

Prioritization of L1-RSRP measurement

Issue 2-3-4: Prioritization enhancement for L1-RSRP measurement of FR2 unknown SCell activation

FFS:

- Proposal 1 (Huawei, vivo, Apple): study whether L1-RSRP measurement during SCell activation shall have higher priority over L3 measurement and whether it should be performed in non-DRX mode even DRX is configured.

  - Proposal 1a (Huawei): agree that L1-RSRP measurement during SCell activation shall have higher priority over L3 measurement and it should be perform in non-DRX mode even DRX is configured.

Similar proposals were discussed in Rel15 but not agreed because of the negative impact on mobility performance. If L3 measurement on the target SCell is deprioritized over L1-RSRP measurement during SCell activation, the measurement reporting may not be timely transmitted hence delay the mobility decision. Even the SCell activation is completed, there is risk of connection failure due to mobility. It is uncertain if there is any benefit by prioritizing L1-RSRP measurement of FR2 unknown SCell activation. The prioritization of L1-RSRP measurement needs to be well justified taking into account the negative impact on mobility performance.

Proposal 4: The prioritization of L1-RSRP measurement needs to be well justified taking into account the negative impact on mobility performance.

Conclusion

This contribution discusses the L1 part enhancement for FR2 SCell activation delay reduction. The observations and proposals are summarized as below:

Proposal 1: For FR2 unknown SCell activation, a smaller Rx beam sweeping factor is assumed for L1-RSRP measurement based on the measurement during cell detection.

Proposal 2: RAN4 to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell.

Proposal 3: Do not consider the sample number reduction for L1-RSRP measurement as 1 sample has been assumed for SCell activation.

Proposal 4: The prioritization of L1-RSRP measurement needs to be well justified taking into account the negative impact on mobility performance.

References

- R4-2214345, WF on R18 eFeRRM, Apple