3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215790

# Electronic Meeting, 10th-19th, Oct, 2022

Agenda Item:	6.11.2

Title:	Discussion on the power imbalance requirement for Type-2 non-collocated intra-band NR-CA

Source:	KDDI

Document for:	Approval

1.  Introduction

In the RAN#95-e meeting, the new WI for intra-band non-collocated EN-DC/NR-CA deployment was approved [1]. In RAN4#104-e meeting, the discussion mainly focused on Type-2 non-collocated intra-band NR-CA and some remaining issue need further check [2]. In this contribution, we discuss the power imbalance requirements for intra-band non-contiguous NR-CA considering those for EN-DC already specified.

2.  Discussion

In this Clause, we present our views on below remaining issue of Type-2 non-collocated intra-and NR-CA in last RAN4 meeting [2].

2.3	 Sub-topic 2-3: RF requirements

< Issue 2-3-1: Power Imbalance and in-band blocking>

Agreement:

- 25dB power imbalance, 1dB REFSENS relaxation. RAN4 may revisit if there is technical concern.

In last RAN4 meeting, views on the power imbalance requirements was provided in [3]. In Rel-15 discussion, RAN4 have already concluded the possible UE architecture for DC_42_n77/78 would be that for intra-band non-contiguous CA with common LNA and PA due to the frequency range of n77/78 contains that of B42 [4]. And the shared filtering for both B42 and n77/78, such as intra-band non-contiguous CA, was assumed when the power imbalance requirements for DC_42_n77/78 was discussed [5].

Observation 1: The possible UE architecture for DC_42_n77/78 would be that for n77/78 intra-band non-contiguous NR-CA.

Considering the above, the power imbalance requirements for n77/78 intra-band non-contiguous NR-CA should be the same of that for DC_42_n77/78. One thing is that the possible UE architecture for those band combinations might be updated from Rel-15 discussion. However, updated UE architectures were not shown in last RAN4 meeting clearly. The 2 Rx per band based on 2 Rx chain per CC is currently a baseline on UE architecture for Type-2 UE of n77/78 intra-band non-contiguous NR-CA same as R16/R17 DC_42_n77/78.

Observation 2: The updated UE architectures were not shown in last RAN4 meeting

Observation 3: The 2 Rx per band based on 2 Rx chain per CC is a baseline on UE architecture for Type-2 UE of n77/78 intra-band non-contiguous NR-CA

If there are no technical issues on the power imbalance 25dB with REFSENS 1dB relaxation, RAN4 can conclude those requirements for Type-2 NR-CA in this meeting and focus on topics about new Type UE supporting 4-layer MIMO per CC.

In case of defining several power imbalance requirements for Type-2 or reducing 25dB power imbalance, it will cause fragmentation of UE Type. The fragmentation doesn’t have any benefits from aspects of network operation, or rather it increases complexity of network configuration and operation eventually.

Observation 4: Fragmentation of UE types doesn’t have any benefits from network configuration and operation perspective.

We think that RAN4 don’t need to revisit 25dB power imbalance with 1dB REFSENS relaxation for Type-2 Intra-band non-collocated NR-CA anymore. Our proposal is as follows.

Proposal: RAN4 conclude 25dB power imbalance with 1dB REFSENS relaxation for Type-2 Intra-band non-collocated NR-CA in this meeting.

3.  Conclusion

Observation 1: The possible UE architecture for DC_42_n77/78 would be that for n77/78 intra-band non-contiguous NR-CA.

Observation 2: The updated UE architectures were not shown in last RAN4 meeting

Observation 3: The 2 Rx per band based on 2 Rx chain per CC is a baseline on UE architecture for Type-2 UE of n77/78 intra-band non-contiguous NR-CA

Observation 4: Fragmentation of UE types doesn’t have any benefits from network configuration and operation perspective.

Proposal: RAN4 conclude 25dB power imbalance with 1dB REFSENS relaxation for Type-2 Intra-band non-collocated NR-CA in this meeting.

4.  Reference

RP-221012		New WI: Support of intra-band non-collocated EN-DC/NR-CA deployment, KDDI

R4-2214458		WF on WF on NonCol_intraB_ENDC_NR_CA, KDDI

- [3]	R4-2211752		Discussion on the power imbalance requirement for intra-band non-contiguous CA with non-collocated scenario, SoftBank Corp.

- [4]	R4-1709390		Feasibility of DC_42A-n77A and 42A-n78A, NTT DOCOMO, INC.

- [5]	R4-2111461		Imbalance Requirement for Type 2 UE RX, Qualcomm Incorporated