3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215799

# Electronic Meeting, October 10 – 19, 2022

Source:	LG Electronics

Title:	Discussion on TCI assumption for RSSI measurement for FR2-2

Agenda item:	4.3.5.1

Document for:	Discussion

Introduction

In this contribution, we provide our views on the QCL-ed assumption for inter-frequency RSSI measurement for FR2-2 based on the WF [1].

Discussion

In the last RAN4 meeting, RAN4 agreed on the QCL-ed assumption when TCI state information is configured, but no TCI state information is provided, there were no agreements.

Issue 1-2-2: Inter-frequency QCL-ed assumption Agreement in GTW: - The QCL reference serving cell is determined by serving cell ID indicated in TCI-StateInfo in RMTC-Config, when the TCI state is provided in the RMTC configuration. - For performing inter-frequency RSSI measurement in FR2-2, - if the TCI state is provided in RMTC configuration of FR2-2, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. - FFS on the following bullet - if no TCI state is provided in the RMTC configuration [or if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier] UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of [a or the] carrier in FR2-2. on which the RMTC configuration is provided - RAN4 to clarify the case with RAN1 when there is no serving cell in FR2-2. - If a UE has no serving cell in FR2-2, it is not clear if that explicit TCI state should be configured to the UE for FR2-2 RSSI measurement. If explicit TCI state should be configured, how does the UE use such explicit TCI?

Two parts are still open in the above agreements; yellow and cyan highlighted parts.

[if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier]

For the condition “if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier”, even if TCI state information can be provided by the FR1 serving cell, it is also the situation of no serving cell in an FR2-2 carrier. So, RAN4 can further discuss after receiving RAN1 reply LS, and the QCL-ed assumption in case of no serving cell in FR2-2 carrier could be separately captured from the condition in case no TCI state information is provided if needed.

- Proposal 1: Wait for the RAN1 reply LS in case there are no serving cells in FR2-2, and remove the “if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier” in the description of the case no TCI information.

[a or the] carrier in FR2-2

When no TCI state information is provided, RAN4 agreed that UE shall assume the QCL-ed TypeD one of the latest received PDSCH and the latest monitored CORESET in the active BWP of FR2-2 carrier. The problem is which serving cell on the FR2-2 carrier should be applied. If there are multiple serving cells on the FR2-2 band and RSSI measurements resources are configured FR2-2 non-serving cell carrier, it is not clear whether UE should assume QCL-ed TypeD as the latest received PDSCH and the latest monitored CORESET for which serving cell carrier, and it should be clarified in RAN4. One of the possible approaches is to refer to the lowest or highest cell index among the serving cells for which the corresponding TCI state ID is provided in another RMTC configuration.

- Proposal 2: Clarify which serving cell carrier’s the latest received PDSCH and the latest monitored CORESET in the active BWP should be referred to when there are multiple serving cells on the FR2-2 band if no TCI state is provided in the RMTC configuration.

Based on the above proposals, we provide the text proposal for inter-frequency RSSI measurement in case no TCI state is provided in the RMTC configuration.

- Proposal 3: Text proposal for inter-frequency RSSI measurement for an open issue is

    - If no TCI state is provided in the RMTC configuration UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier in FR2-2 having the [lowest or highest] cell index among the serving cells for which corresponding TCI state ID is provided in another RMTC configuration.

Conclusion

In this contribution, we provide our views on the QCL-ed assumption for inter-frequency RSSI measurement for FR2-2, and we propose

- Proposal 1: Wait for the RAN1 reply LS in case there are no serving cells in FR2-2, and remove the “if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier” in the description of the case no TCI information.

- Proposal 2: Clarify which serving cell carrier’s the latest received PDSCH and the latest monitored CORESET in the active BWP should be referred to when there are multiple serving cells on the FR2-2 band if no TCI state is provided in the RMTC configuration.

- Proposal 3: Text proposal for inter-frequency RSSI measurement for an open issue is

    - If no TCI state is provided in the RMTC configuration UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier in FR2-2 having the [lowest or highest] cell index among the serving cells for which corresponding TCI state ID is provided in another RMTC configuration.

Reference

[1] R4-2215159, “WF on NR extension to 71GHz RRM requirements (Part 1),” Huawei, HiSilicon.