3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215806

# Electronic Meeting, October 10 – 19, 2022

Source:	LG Electronics

Title:	Discussion on TCI state switching for simultaneous DL reception from different directions

Agenda item:	6.8.3.4

Document for:	Discussion

Introduction

In this contribution, we provide our views on TCI state switching for simultaneous DL reception from different directions based on the agreed WF [1] in the last meeting.

Discussion

Dual TCIs would be configured for each antenna panel, and TCI switching can be separately configured. So, legacy TCI switching delay requirements can be reused for independent TCI switching for dual TCI in the multi-Rx chain. For legacy TCI switching delay requirements, RAN4 discussed which legacy requirements would be the baseline for the multi-Rx chain; Rel-15/Rel-16 TCI switching delay or Rel-17 unified TCI switching delay. Rel-17 unified TCI is only applicable for a single TRP case, and RAN1 is discussing unified TCI for multi-TRP in the Rel-18. So, Rel-17 unified TCI switching delay requirements for multi-Rx chain is not feasible.

- Proposal 1: Independent TCI state switching delay requirements for multi-Rx chain could be based on Rel-15/Rel-16 TCI state switching delay requirements.

If one of the TCI states is unknown for dual TCI state switching, an L1-RSRP measurement should be performed. So, when L1-RSRP measurement for a TRP is performed, UE cannot receive the signal from other TRP, especially for mDCI case. RAN4 needs to study UE behavior in case one of the TCI states is unknown for dual TCI state switching.

- Proposal 2: Further study UE behavior in case one of the TCI states is unknown for dual TCI state switching.

Conclusion

In this contribution, we provide our views on TCI state switching for simultaneous DL reception from different directions, and we propose

- Proposal 1: Independent TCI state switching delay requirements for multi-Rx chain could be based on Rel-15/Rel-16 TCI state switching delay requirements.

- Proposal 2: Further study UE behavior in case one of the TCI states is unknown for dual TCI state switching.

Reference

[1] R4-2214344, “WF on FR2 multi-Rx RRM,” vivo.