3GPP TSG-RAN WG4 Meeting # 104bis-e						  R4-2215817

Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	6.20.3

Source: 	OPPO

Title: 	Discussion on L1/L2 based inter-cell mobility

Document for:	Approval

1	Introduction

A WF on Rel-18 Mobility enhancement was approved in RAN4#104 meeting [1]. In this contribution, we discuss the issues of L1/L2 based inter-cell mobility[2].

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized - To specify RRM core requirements for the following, as necessary [RAN4]: - L1/L2-based inter-cell mobility - Enhanced CHO configurations addressed by this WI

2	Discussion

RAN4 agreed to specify L1/L2 inter-cell mobility delay and analyses each component of L1/L2 inter-cell mobility delay. The target performance enhancements due to L1/L2 inter-cell mobility may include:

- Handover delay and interruption requirements

- L1 measurement and reporting requirements

- Timing management requirements

Since the requirements rely on the procedure defined by RAN1/RAN2, we can discuss the details of HO delay requirements after RAN2 has specified concrete procedures. As discussed in RAN2, HO interruption time for L1/L2-based inter-cell mobility is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. RAN2 is considering to reduce HO interruption time, and investigate solutions to reduce the time for UE reconfiguration, downlink and uplink synchronization after handover decision (other parts of dynamic switch are not precluded, e.g., preparation of target cell configurations capable of dynamic switching without need for full configuration.). So RAN4 can further study any enhancement of Tinterrupt in mobility delay requirements as well.

Proposal 1: RAN4 to discuss the details of HO delay requirements based on the conclusion of RAN1/2 on L1/L2 based procedures.

Proposal 2: RAN4 can further study any enhancement of Tinterrupt in mobility delay requirements.

Rel-18 L1/L2 mobility includes both non-CA (PCell only) and CA scenarios (PCell and SCell), at least including the case that the target PCell/target SCell(s) is not a current serving cell. From this point of view, target cell could be intra-frequency or inter-frequency cell. But how to define L1/L2 intra-frequency/inter-frequency or whether to consider inter-frequency, needs to be clarified in RAN4.

In our view, for the definition of intra-frequency for L1-RSRP measurement, we think it can be defined that the measured SSB or CSI-RS for L1-RSRP measurement is within active BWP(s) of the UE, and SCS of SSB or CSI-RS is the same as serving cell SSB or CSI-RS.

For other measurements, they are inter-frequency. In this case, inter-frequency L1-RSRP measurement needs measurement gaps. Since the intention to use L1 measurement for L1/L2 mobility is to get the channel state change timely to switch to a better beam, the benefit will be reduced due to gaps for inter-frequency measurement. Thus, we prefer not to define any requirements for inter-frequency case at this stage.

Proposal 3: Intra-frequency for L1-RSRP measurement can be defined as: the measured SSB or CSI-RS for L1-RSRP measurement is within active BWP(s) of the UE, and SCS of SSB or CSI-RS is the same as serving cell SSB or CSI-RS. Otherwise, it is inter-frequency L1-RSRP measurement.

Proposal 4: Not to define requirement s for inter-frequency L1-RSRP measurement.

In last meeting, RAN4 also discussed whether to support L1 measurement on multiple cells with PCI different from serving cell.

- the number of cells to be monitored on a FR2 band

- the total number of cells to be monitored on all bands

- the total number of cells to be monitored per frequency layer

- In last meeting, RAN2 confirmed to support L1/L2-based inter-cell mobility for inter-DU scenario (as well as intra-DU scenarios). Inter-cell beam management is one scenario considered for L1/L2 mobility, but is not the only one, and is not a prerequisite for using L1/L2 mobility. Since supporting large number of neighbor cells with L1 measurement would result in longer delay, we are ok to define the maximum number as UE capability. As we prefer to consider intra-frequency L1-RSRP measurement in R18, the total number of cells to be monitored per frequency layer could be good. If more than 1 non-serving cell are supported to perform L1/L2 mobility, the R17 scaling factor between serving cell L1 measurement and non-serving cells, i.e., Psc and PCDP needs update correspondingly.

Proposal 5: Define UE capability of supporting the total number of cells to be monitored per frequency layer, and update related RRM requirements，e.g., Psc and PCDP.

3	Conclusion

Proposal 1: RAN4 to discuss the details of HO delay requirements based on the conclusion of RAN1/2 on L1/L2 based procedures.

Proposal 2: RAN4 can further study any enhancement of Tinterrupt in mobility delay requirements.

Proposal 3: Intra-frequency for L1-RSRP measurement can be defined as: the measured SSB or CSI-RS for L1-RSRP measurement is within active BWP(s) of the UE, and SCS of SSB or CSI-RS is the same as serving cell SSB or CSI-RS. Otherwise, it is inter-frequency L1-RSRP measurement.

Proposal 4: Not to define requirement s for inter-frequency L1-RSRP measurement.

Proposal 5: Define UE capability of supporting the total number of cells to be monitored per frequency layer, and update related RRM requirements，e.g., Psc and PCDP.

4	Reference

[1] R4-2214348, WF on Further NR Mobility Enhancements, MediaTek Inc.

[2] RP-221799, “Revised WID on Further NR Mobility Enhancements”, MediaTek Inc., RANP#96