# 3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215824

# Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	6.12.5

Source: 	OPPO

Title: 	Discussion on RRM requirements for FR2 HST

Document for:	Approval

1	Introduction

The basic requirements for FR2 HST scenarios were introduced in Rel-17, mainly focus on open space deployment, single carrier and single active panel case. A revised WID [1] on enhanced NR support for high speed train scenario in frequency range 2 was approved in the last RAN-P meeting. The objectives are shown below and this contribution will give our initial consideration on this topic.

- Only train roof-mounted high power devices with target applicable carrier frequency up to 30GHz and up to 350km/h velocity are considered in this WI - Specify the RF requirements for intra-band carrier aggregation (CA) scenario, and investigate and specify the RRM requirements for intra-band carrier aggregation (CA) scenario [RAN4] - Specify the requirement for simultaneous multi-panel operation for train roof-mounted FR2 high power devices [RAN4]: - Maximum 2 active panels supporting the multi-panel simultaneous reception. - NOTE: Focus on FR2 HST specific requirements, and avoid the overlap with the scope of FR2 multi-Rx DL reception - Study on reference tunnel deployment scenario for FR2 HST and specify the channel model and corresponding core requirements if any [RAN4] - Specify UL timing adjustment solution, including explicit NW signalling assistance, for FR2 HST scenario with large UL/DL propagation delay difference from different RRHs/TRPs to UE [RAN4, RAN2]. - Note: RAN1/RAN2 work can be triggered by RAN4 LS.

2	Discussion

For intra-band carrier aggregation scenario, we think the enhanced RRM requirements for FR1 HST should also be discussed for FR2. Obviously, the SCell related requirements should be introduced, including time period for PSS/SSS detection, measurement period for intra-frequency measurements, and SCell activation/deactivation delay. Inter-frequency measurement is precluded in Rel-17 FR2 HST since only single carrier is considered. If multiple carriers are supported in Rel-18, we think inter-frequency measurement should also be discussed.

Proposal-1: Discuss the following RRM requirements for FR2 HST CA scenario:

- Time period for PSS/SSS detection and measurement period for activated/deactivated SCell

- SCell activation/deactivation delay

- Inter-frequency measurements

In Rel-17, the basic assumption for UE is equipped with 2 antenna panels pointing at forward or backward directions respectively, but only 1 panel could be active. In Rel-18, maximum 2 active panels are introduced for simultaneous reception in bi-directional RRH deployment. From the RRM measurement requirements, we think this is similar as the NR FR2 multiRx DL WI. Noticing that the requirements for L3/L1 measurements and TCI state switching are discussed in multiRx DL WI, it is better to follow the conclusions as much as possible and avoid repetitive discussion. We are also open to revise the conclusions if any critical issue is identified in FR2 HST.

Proposal-2: In case of 2 active panels, consider whether to reuse conclusions from NR FR2 multiRx DL WI.

Due to the large propagation delay difference between RRH (2.5us in uni-directional RRH deployment), a new UL timing mechanism is expected. However, this issue was raised in the very late stage in Rel-17, and a UE based solution, i.e. one-shot large timing adjustment mechanism is finally adopted. Several NW-involved solutions are also proposed but not pursed. NW assistance could helpful to identify large timing jump in case of wrong estimated DL timing by UE itself. Basically, RRC based pre-configured information and MAC-CE based information along with TCI switch command are proposed in Rel-17. We prefer RRC based assistance information, and the details could be further discussed after the related procedure is agreed.

Proposal-3: For UL timing adjustment, support RRC based assistance information.

3	Conclusion

This contribution gave our general views on RRM requirements for FR2 HST and the following proposals:

Proposal-1: Discuss the following RRM requirements for FR2 HST CA scenario:

- Time period for PSS/SSS detection and measurement period for activated/deactivated SCell

- SCell activation/deactivation delay

- Inter-frequency measurements

Proposal-2: In case of 2 active panels, consider whether to reuse conclusions from NR FR2 multiRx DL WI.

Proposal-3: For UL timing adjustment, support RRC based assistance information.

4	Reference

- RP-222272, Revised WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2), Samsung