3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215842

Electronic Meeting, 10-19 October 2022

Source:	Ericsson, Telstra

Title:	TP for TR 38.718-11-21: DC_1-n26-n78

Agenda item:

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.718-11-21 to include DC_1-n26-n78 configuration as defined in WID [1].

# ---Start of changes---

## 6.X		DC_1-n26-n78

### 6.X.1	 Operating bands for DC

Table 6.X.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-n26-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.X.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-n26A-n78A | DC_1A_n26A DC_1A_n78A |

### 6.X.2 	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-n26A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.X.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_1_n26 are shown below.

Table 6.X.3-1: The harmonic and IMD products caused by UL DC_1_n26

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1628 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 5760 | 5940 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3256 | 3396 | 7680 | 7920 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4070 | 4245 | 9600 | 9900 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1166 | 1071 | 2734 | 2829 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 352 | 222 | 2991 | 3146 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3548 | 3678 | 4654 | 4809 |
| Two-tone 4th order IMD products | ¦2*fx_low –2*y_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2332 | 2142 | 5468 | 5658 |
| Two-tone 4th order IMD products | ¦3*fx_low –1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – *fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 462 | 627 | 4911 | 5126 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4527 | 6574 | 6789 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7106 | 6831 | 1476 | 1276 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + *fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8494 | 8769 | 5176 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – *fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – *fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4312 | 4062 | 1293 | 1518 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7388 | 7638 | 6282 | 6507 |

The harmonic and IMD products caused by UL DC_1_n78 are shown below.

Table 6.X.3-2: The harmonic and IMD products caused by UL DC_1_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 3840 | 3960 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 9900 | 11400 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 13200 | 15200 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1880 | 1320 | 5220 | 5780 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 | 660 | 4620 | 5680 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 | 7760 | 8520 | 9580 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3760 | 2640 | 10440 | 11560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 | 2640 | 7920 | 9480 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 | 9740 | 11820 | 13380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 | 11220 | 4620 | 3880 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 | 17180 | 10980 | 11720 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7560 | 5940 | 660 | 1840 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 | 15360 | 12360 | 13540 |

The IMD3 interference of UL DC_1_n26 may have an impact on the Rx of band n78.

The IMD5 interference of UL DC_1_n78 may have an impact on the Rx of band n26.

### 6.X.4	 ∆TIB and ∆RIB values

Referring to DC_1-5_n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.X.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n26-n78 | 1 | 0.3 |
|  | n26 | 0.6 |
|  | n78 | 0.8 |

Table 6.X.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n26-n78 | 1 | 0.2 |
|  | n26 | 0.2 |
|  | n78 | 0.5 |

### 6.X.4	 MSD requirements

Referring to CA_n1-n5-n78, the MSD requirements can be specified as below:

Table 6.X.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n26A-n78A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n78 | 3610 | 10 | 50 | 3610 | 15.7 | IMD3 |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n26 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |

---End of changes---

# Reference

[1] 		RP-221944, Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL), LG Electronics Inc.