3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215849

Electronic Meeting, 10-19 October 2022

Source:	Ericsson, Telstra

Title:	TP for TR 38.718-21-11: DC_1-3-n26

Agenda item:			5.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.718-21-11 to include DC_1A-3A_n26A and DC_1A-3C_n26A configurations  as defined in WID [1].

# ---Start of changes---

## 5.X	DC_1-3_n26

### 5.X.1	Operating bands for EN-DC

Table 5.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n26 | CA_1-3 | n26 | No |

### 5.X.2	Configuration for DC

Table 5.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n26A | DC_1A_n26A DC_3A_n26A | CA_1A-3A | n26A |
| DC_1A-3C_n26A | DC_1A_n26A DC_3A_n26A DC_3C_n26A | CA_1A-3A CA_1A-3C | n26A |

### 5.X.3		∆TIB and ∆RIB values

For DC_1-3_n26, the TIB,c and RIB,c values are reused from DC_1-3_n5 and are given in the tables below.

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n26 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n26 | 0.3 |

Table 5.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n26 | 1 | 0 |
|  | 7 | 0 |
|  | n26 | 0 |

### 5.X.4	REFSENS requirements

Reusing the coexistence study results from DC_1-3_n5, there is no need to define MSD values for DC_1-3_n26.

---End of changes---

# Reference

[1] 		RP-222505, Rel-18 Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei