3GPP TSG-RAN4 WG4 Meeting #104-bis-e	R4-2215866

# Electronic meeting, October 10 – October 19, 2022

Agenda item:			6.9.2.2

Source:	vivo

Title:	Discussion on L1 part enhancement for FR2 SCell activation

Document for:	Discussion

- Introduction

In the RAN4#104-e meeting, the issues related to FR2 SCell activation delay reduction requirements were discussed. And the related WF are captured in [1]. In this contribution, we further discuss the L1 part enhancement for FR2 SCell activation.

- Discussion

Firstly, the Rx beam sweeping factor enhancement in L1-RSRP measurement was discussed in the last meeting:

Issue 2-3-1: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception) - FFS on beam sweeping factor reduction for L1 measurement during unknown FR2 SCell activation. - Option 1a (vivo): Introduce the UE capability to support Rx beam sweeping factor can be less than 8 for FR2 - Option 1b (Ericsson): - RAN4 to discuss the solutions or pre-conditions to reduce the scaling factor N1 in FR2 SCell activation. - RAN4 to define RX beam constant time. Where, RX beam constant time is a time duration or window within which the RX beams are assumed to be constant or non changing. Rx beam constant time to be agreed as [X=1280ms]. - If UE performed a full RX beam sweeping for a procedure, next procedures or steps fall within RX beam constant time do not need to perform RX beam sweeping. - UE can speed up the remaining steps in SCell activation with a shorter beam scaling factor based on prior Rx beam information from the step before - Option 2 (LG, MTK, HW, ZTE): Consider reporting (rough) best Tx beam during cell detection to reduce L1-RSRP measurement delay

For option 1a, similar to L3 part, we suggest to introduce the UE capability to support Rx beam sweeping factor can be less than 8 (i.e., 1, 2, 4, 6) for L1-RSRP measurement during unknown FR2 SCell activation. In addition, we understand that when the L3 part including cell search and AGC settling has been completed, UE may have some knowledges about Rx beams of activated SCell (e.g., which Rx beam or Rx beams can receive the SCell successfully). In that case, when the SSB is used to execute L1-RSRP measurement for UE, only these valid beams other than all beams need measurement. In this sense, the Rx beam sweeping factor of L1-RSRP measurement may be reduced. And in our view, it is also aligned with the fourth bullet of Option 1b.

For Option 2, we are not sure whether the information of Tx beam can be obtained during cell detection. UE may only need to perform PSS/SSS detection to obtain the cell ID and coarse timing without SSB index detection for cell search during unknown SCell activation. So the feasibility of Option 1b need further to be discussed.

Observation 1: During the cell detection, the UE only needs to perform PSS/SSS detection to obtain the cell ID and coarse timing without SSB index detection. There is no way to report the information of Tx beam for UE to reduce the L1-RSRP measurement delay.

Proposal 1: Introduce the UE capability to support Rx beam sweeping factor can be less than 8 (i.e., 1, 2, 4, 6) for L1-RSRP measurement during unknown FR2 SCell activation.

Proposal 2: When the L3 part including cell search and AGC settling has been completed, if the SSB is used to execute L1-RSRP measurement, the Rx beam sweeping factor of L1-RSRP measurement can be less than 8.

Secondly, the issue related to prioritization enhancement for L1-RSRP measurement of FR2 unknown SCell activation was also discussed and the details were copied as below:

Issue 2-3-4: Prioritization enhancement for L1-RSRP measurement of FR2 unknown SCell activation FFS: - Proposal 1 (Huawei, vivo, Apple): study whether L1-RSRP measurement during SCell activation shall have higher priority over L3 measurement and whether it should be performed in non-DRX mode even DRX is configured. - Proposal 1a (Huawei): agree that L1-RSRP measurement during SCell activation shall have higher priority over L3 measurement and it should be perform in non-DRX mode even DRX is configured.

In order to reduce the L1-RSRP measurement delay, we support Proposal 1 and Proposal 1a. As commented from some companies in the last meeting, we also agree that this may have some impact on mobility. But we understand SCell activation is not a periodical behaviour, when the UE needs to perform SCell activation, if L1-RSRP measurement priority can be raised temporarily, the SCell activation can be completed quickly. In addition, we notice that in the existing spec, when the UE perform cell detection and AGC settling during SCell activation, there is no consideration about the collision between SMTC occasions which is used to perform cell detection/AGC settling and L3 measurement. And the DRX configuration has been not taken into account. For L1-RSRP measurement for SCell activation, we can follow the same rule to reduce the SCell activation delay.

Proposal 3: L1-RSRP measurement shall have higher priority over L3 measurement and 	it shall be performed in non-DRX mode even DRX is configured during the SCell activation.

Another issue is related to TCI activation enhancement during FR2 SCell activation. The details were discussed as follows:

Issue 2-3-7: TCI activation enhancement during FR2 unknown SCell activation FFS: - Proposal 1 (Intel, Apple, Xiaomi,): further discuss the UE beam assumption after L1-RSRP report to reduce MAC CE based TCI activation delay. - Proposal 1a(Huawei, Apple, Intel, Xiaomi, OPPO, MTK, Ericsson, NTT DCM): the uncertainty of TCI configuration/activation can be saved when TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report. - Proposal 2 (ZTE): to consider whether the TCI state indication can be acquired by UE before SCell activation or not, which would impact the total latency. - Proposal 3 (ZTE): to consider whether and how to identify TCI state without L1-RSRP measurement and report, which is the core issue for latency reduction for unknown case.

For Proposal 1a, if the TCI configuration/activation is skipped, If the assumption is unknown for network, the network has no idea to schedule the data based on the correct beam. Due to the TCI state configuration of PDCCH/PDSCH or CSI-RS are both based on the result of L1-RSRP report, so it is very likely that PDCCH/PDSCH and CSI-RS follow the same TCI state information.

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is: - 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time. If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is: - max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

In the legacy spec, based on the highlighted part, we notice the SCell activation delay depends on the maximum of CSI-RS activation time and PDCCH/PDSCH TCI state activation time. Therefore, if only the TCI from CSI-RS is configured, it is feasible that PDCCH/PDSCH can follow the same TCI state information as CSI-RS. And in the clause 5.1.5 in the spec 38.214, we also understand this UE behaviour is supported in the RAN1 spec. So we suggest that only the TCI from CSI-RS needs to be configured. In this way, the PDCCH/PDSCH TCI configuration can be saved and the SCell activation delay can also be reduced accordingly.

For the DM-RS of PDCCH, the UE shall expect that a TCI-State or DLorJointTCIState except an indicated DLorJointTCIState indicates one of the following quasi co-location type(s): - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, 'typeD' with the same CSI-RS resource.

Observation 2: Due to the TCI state configuration of PDCCH/PDSCH or CSI-RS are both based on the result of L1-RSRP report, so it is very likely that PDCCH/PDSCH and CSI-RS follow the same TCI state information.

Proposal 4: During the SCell activation, only the TCI from CSI-RS used for CQI needs to be configured. The PDCCH/PDSCH can follow the same TCI state information as CSI-RS. In this way, the PDCCH/PDSCH TCI configuration can be saved and the SCell activation delay can be reduced accordingly.

- Summary

Observation 1: During the cell detection, the UE only needs to perform PSS/SSS detection to obtain the cell ID and coarse timing without SSB index detection. There is no way to report the information of Tx beam for UE to reduce the L1-RSRP measurement delay.

Proposal 1: Introduce the UE capability to support Rx beam sweeping factor can be less than 8 (i.e., 1, 2, 4, 6) for L1-RSRP measurement during unknown FR2 SCell activation.

Proposal 2: When the L3 part including cell search and AGC settling has been completed, if the SSB is used to execute L1-RSRP measurement, the Rx beam sweeping factor of L1-RSRP measurement can be less than 8.

Observation 2: Due to the TCI state configuration of PDCCH/PDSCH or CSI-RS are both based on the result of L1-RSRP report, so it is very likely that PDCCH/PDSCH and CSI-RS follow the same TCI state information.

Proposal 3: During the SCell activation, only the TCI from CSI-RS used for CQI needs to be configured. The PDCCH/PDSCH can follow the same TCI state information as CSI-RS. In this way, the PDCCH/PDSCH TCI configuration can be saved and the SCell activation delay can be reduced accordingly.

- References

- R4-2214345 WF on R18 eFeRRM

- R1-2112767 LS on lower Rx beam sweeping factor for latency improvement