# 3GPP TSG-RAN WG4 Meeting # 104bis-e                            R4-2215891

# Electronic Meeting, Oct. 10-Oct. 19, 2022

# Title: 	Discussion on power domain enhancements to reduce MPR/PAR

# Source: 	ZTE Corporation

# Agenda item:	6.23.3

# Document for:	Approval

Document for:	ApprovalIntroduction

In the RAN#95-e meeting, the WI on further NR coverage enhancements was approved [1] and was updated in RAN#96 meeting as [2]. The detailed objective about power domain enhancements is showed as below:

- Study and if necessary specify following power domain enhancements - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1) - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

In this contribution, the enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation, are analyzed and simulation results are provided. Moreover, similar contribution [5] is also submitted in this meeting, considering both RAN4 and RAN1 were the involved groups, and RAN1 may clarify and determine the work split between RAN1 and RAN4. Meanwhile, some parallel discussions on enhancement to reduce MPR/PAR may take place in RAN1 in Oct. Meeting.

MPR/PAR reduction schemes

## Analysis on candidate schemes

Based on the WID, there are three candidate schemes to reduce MPR/PAR, i.e., FDSS with and without spectrum extension and tone reservation. We provide our views on these schemes as follows.

FDSS without spectrum extension

In Rel-15, FDSS without spectrum extension for pi/2 BPSK is supported and captured in Clause 6.4.2.4.1 in TS 38.101-1 as shown below.

For Pi/2 BPSK modulation the UE shall be allowed to employ spectral shaping and the shaping filter shall be restricted so that the impulse response of the shaping filter itself shall meet │ãt(t,0)│ ≥ │ãt(t, τ)│ ∀τ ≠ 0 20log10│ãt(t,τ)│< -15 dB 1< τ < M - 1, where│ãt(t, τ)│=IDFT{│ãt(t,f)│ejφ (t,f)}, f is the frequency of the M allocated subcarriers, ã(t,f) and φ(t,f) are the amplitude and phase response. 0 dB reference is defined as 20log10│ãt(t,0)│.

As can be seen, FDSS processing can be viewed as adding a window function in the frequency domain for shaping filter. For FDSS without spectrum extension, the window length should be equal to the number of REs allocated for PUSCH transmission.

Observation 1: For FDSS without spectrum extension, the window length of the shaping filter in the frequency domain is equal to the number of REs allocated for PUSCH transmission.

In Rel-18 coverage enhancement WI, one potential enhancement is to extend FDSS without spectrum extension for higher modulation orders, e.g., QPSK. In terms of the studies on Pi/2 BPSK in Rel-15 and Rel-16, the common understanding was that FDSS filter can help to reduce the PAR/MPR. So similar with Pi/2 BPSK, it would be foreseen that the FDSS filter can also help to reduce the PAR/MPR for QPSK. Some evaluation are given below.

Therefore, there are some RAN4 specification impacts to extent FDSS to QPSK, where some RF requirements like MPR needs to be re-visited.

Observation 2: Some RAN4 specification impacts areexpected for QPSK supporting of FDSS.

FDSS with spectrum extension

FDSS with spectrum extension was proposed to reduce MPR/PAR back to RAN1 #42 [3]. The proposed method for spectrum extension is shown in Figure 1, where the extension PRBs are filled with the copy of data from the two edges of the original allocated PRBs. In such case, the window length of the shaping filter in the frequency domain is equal to (1+α) times of the number of REs allocated for original PUSCH transmission.

Figure 1. FDSS with spectrum extension

Observation 3: For FDSS with spectrum extension, the window length of the shaping filter in the frequency domain is equal to (1+α) times of the number of REs allocated for original PUSCH transmission, where α is ratio of the extended REs.

Tone reservation

In Rel-17 CE SI, tone reservation was proposed to reduce PAPR by allocating some reserved REs for better frequency domain shaping. An example is shown in Figure 2. Fundamentally, tone reservation is one kind of FDSS scheme, where the window length of the shaping filter in the frequency domain is equal to (1+β) times of the number of REs allocated for original PUSCH transmission, where β is ratio of the reserved REs.

Figure 2. Tone reservation

Observation 4: For tone reservation, the window length of the shaping filter in the frequency domain is equal to (1+) times of the number of REs allocated for original PUSCH transmission, where β is ratio of the reserved REs.

## Simulation results

To study whether the three schemes are necessary to be specified, the performance of CM/PAPR reduction as well as LLS performance should be evaluated. The simulation assumptions for evaluations are listed in Table-1. Several reservation or extension ratios are simulated in this section.

Table 1 Simulation assumptions

| Parameters | Value |
| --- | --- |
| Fc | 4 GHz |
| BW | 100 MHz |
| FFTNum | 4096 |
| Rx Antenna | 2 |
| FDSS | [0.28 1 0.28] |
| FFT number | 4096 |
| Symbol Number | 14 |
| RB Number (baseline) | 16 PRBs |
| MCS | 0 |
| Extension/reservation ratio | 12.5%, 25%, 50% In case of 16 PRBs, the number of extended/reserved PRBs at each edge of the allocated PRBs is 1, 2, 4 PRBs. |
| Waveform | Pi/2-BPSK, QPSK |

The CDF of CM and CCDF for PAPR curves are illustrated in Figure 3.

| Pi/2-BPSK | QPSK |
| --- | --- |

Figure 3 CM and PAPR distributions for pi/2 BPSK and QPSK based on different schemes

Based on Figure 3, we have the following observations.

- For pi/2-BPSK：

- FDSS without spectrum extension can reduce the PAPR significantly, more than 3dB PAPR reduction is observed at CCDF@10^-4 compared to pi/2-BPSK without FDSS. There is also 1dB gain in terms of mean CM.

- For tone reservation, neither PAPR nor CM reduction gain is observed for all the simulated reservation ratios compared to FDSS without spectrum extension. And as the reservation ratio increases, the PAPR/CM performance degrades.

- For FDSS with spectrum extension, similar PAPR performances are observed for different extension ratios, and all are similar to FDSS without extension. The performance gap would be within 0.2dB at CCDF@10^-4. And as the extension ratio increases, the CM performance even degrades at CCDF below 10^-4.

- For QPSK：

- FDSS without spectrum extension can reduce the PAPR about 2.3dB at CCDF@10^-4 compared to QPSK without FDSS. But marginal CM reduction is observed.

- For tone reservation, no PAPR reduction is observed for all the simulated reservation factors compared to FDSS without extension. And as the reservation ratio increases, the PAPR performance degrades. But the observations for CM are on the contrary. As the reservation ratio increases, the mean CM would reduce slightly compared to FDSS without extension.

- For FDSS with spectrum extension, both PAPR and CM will reduce as the extension ratio increases. About 1.6 dB PAPR reduction at CCDF@10^-4 is observed when the extension ratio is 50% compared to the case of FDSS without extension. For extension ratio of 25%, there is still more than 0.9dB PAPR reduction and 0.7dB CM reduction respectively.

In summary, we conclude the following observations based on above results.

Observation 5: For both pi/2-BPSK and QPSK, tone reservation cannot provide clear PAPR/CM reduction gain compared to FDSS with or without spectrum extension.

Observation 6: For pi/2-BPSK, FDSS without spectrum extension can achieve 3dB PAPR gain or 1dB CM gain, and on top of this, FDSS with spectrum extension provides no or minor additional PAPR/CM reduction gain.

Observation 7: For QPSK, FDSS without spectrum extension can achieve 2.3dB PAPR gain while marginal CM gain, and on top of this, FDSS with spectrum extension can provide additional PAPR/CM reduction gain about 0.51 dB, 0.9 dB and 1.63 dB PAPR gain or 0.27 dB, 0.71 dB and 1.17dB CM gain for extension ratio of 12.5%, 25% and 50% respectively.

In addition, we also evaluate the LLS BLER performance for the different schemes. Figure 4 shows the LLS results for the above mentioned schemes. Different reservation ratios or extension ratios are included.

| Pi/2-BPSK | QPSK |
| --- | --- |

Figure 4 LLS results for pi/2 BPSK and QPSK based on different schemes

Based on Figure 4, we have the following observations for both pi/2 BPSK and QPSK.

- For pi/2 BPSK, Compared to legacy case without FDSS, about 0.56dB and 0.79dB LLS performance loss is observed when FDSS without extension is applied at BLER@0.1 and 0.01 respectively.

- For QPSK, compared to legacy case without FDSS, about 0.56dB and 0.78dB LLS performance loss is observed when FDSS without extension is applied at BLER@0.1 and 0.01 respectively.

- The performance of tone reservation / spectrum extension would improve as the reservation / extension ratio increases. When the reservation / extension ratio increases to 25%, almost the same performance can be reached as legacy case without FDSS. Additional performance gain can be obtained if the reservation / extension ratio increases to 50%.

- The performances of tone reservation and FDSS with spectrum extension are almost same when the reservation and extension ratio are the same.

- Similar performances are observed for pi/2-BPSK and QPSK for the same MCS.

Observation 8: For pi/2-BPSK, FDSS without spectrum extension would cause about 0.56~0.79 dB link-level performance loss. For QPSK, FDSS without spectrum extension would cause about 0.56~0.78 dB link-level performance loss.

- To have a comprehensive comparison among different schemes, the performance of PAPR/CM and LLS are summarized in Table 2 and Table 3 for pi/2-BPSK and QPSK respectively.

- Table 2. Comparison summary of performance comparison for Pi/2-BPSK

| pi/2-BPSK | PAPR |  | CM |  | required SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CCDF@1e-4 | Max | Mean | Max | BLER@10% | BLER@1% |  |
| No FDSS | 5.23 | 6.6 | 0.2427 | 0.3824 | -10.24 | -7.53 |  |
| FDSS | 2.05 | 2.7 | -0.7944 | -0.7643 | -9.68 | -6.74 |  |
| 12.5% TR | 2.15 | 2.9 | -0.7937 | -0.7615 | -10.04 | -7.08 |  |
| 25% TR | 2.66 | 3.6 | -0.7120 | -0.6749 | -10.54 | -7.54 |  |
| 50% TR | 3.4 | 4.4 | -0.5262 | -0.4748 | -11.06 | -8.23 |  |
| FDSS with 12.5% SE | 1.85 | 2.5 | -0.8346 | -0.8176 | -10.04 | -7.04 |  |
| FDSS with 25% SE | 1.95 | 2.4 | -0.7987 | -0.7827 | -10.54 | -7.56 |  |
| FDSS with 50% SE | 1.96 | 2.5 | -0.5168 | -0.4999 | -11.07 | -8.19 |  |
| pi/2-BPSK | PAPR gain |  | CM gain |  | required SNR gain |  |  |
|  | CCDF@1e-4 | Max | Mean | Max | BLER@10% |  | BLER@1% |
| No FDSS(baseline) | 0 | 0 | 0 | 0 | 0 |  | 0 |
| FDSS | 3.18 | 3.9 | 1.0371 | 1.1466 | -0.56 |  | -0.79 |
| 12.5% TR | 3.08 | 3.7 | 1.0364 | 1.1438 | -0.2 |  | -0.45 |
| 25% TR | 2.57 | 3 | 0.9547 | 1.0573 | 0.3 |  | 0.01 |
| 50% TR | 1.83 | 2.2 | 0.7689 | 0.8572 | 0.82 |  | 0.7 |
| FDSS with 12.5% SE | 3.38 | 4.1 | 1.0773 | 1.2000 | -0.2 |  | -0.49 |
| FDSS with 25% SE | 3.28 | 4.2 | 1.0414 | 1.1651 | 0.3 |  | 0.03 |
| FDSS with 50% SE | 3.27 | 4.1 | 0.7596 | 0.8823 | 0.83 |  | 0.66 |
| pi/2-BPSK | PAPR gain |  | CM gain |  | required SNR gain |  |  |
|  | CCDF@1e-4 | Max | Mean | Max | BLER@10% |  | BLER@1% |
| No FDSS | -3.18 | -3.9 | -1.0371 | -1.1466 | 0.56 |  | 0.79 |
| FDSS (baseline) | 0 | 0 | 0 | 0 | 0 |  | 0 |
| 12.5% TR | -0.1 | -0.2 | -0.0007 | -0.0028 | 0.36 |  | 0.34 |
| 25% TR | -0.61 | -0.9 | -0.0824 | -0.0894 | 0.86 |  | 0.8 |
| 50% TR | -1.35 | -1.7 | -0.2682 | -0.2895 | 1.38 |  | 1.49 |
| FDSS with 12.5% SE | 0.2 | 0.2 | 0.0402 | 0.0534 | 0.36 |  | 0.3 |
| FDSS with 25% SE | 0.1 | 0.3 | 0.0043 | 0.0185 | 0.86 |  | 0.82 |
| FDSS with 50% SE | 0.09 | 0.2 | -0.2776 | -0.2644 | 1.39 |  | 1.45 |

- Table 3. Comparison summary of performance comparison for QPSK

| QPSK | PAPR |  | CM |  | required SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CCDF@1e-4 | Max | Mean | Max | BLER@10% | BLER@1% |  |
| No FDSS | 6.68 | 8 | 1.2167 | 1.6256 | -10.24 | -7.52 |  |
| FDSS | 4.38 | 5.1 | 1.1849 | 1.6374 | -9.68 | -6.74 |  |
| 12.5% TR | 4.44 | 5.2 | 1.1100 | 1.5627 | -10.04 | -7.08 |  |
| 25% TR | 4.78 | 5.6 | 1.0292 | 1.4734 | -10.53 | -7.54 |  |
| 50% TR | 5.35 | 6.4 | 0.9876 | 1.4262 | -11.05 | -8.23 |  |
| FDSS with 12.5% SE | 3.87 | 4.4 | 0.9139 | 1.3191 | -10.04 | -7.04 |  |
| FDSS with 25% SE | 3.48 | 4.1 | 0.4746 | 0.7349 | -10.54 | -7.56 |  |
| FDSS with 50% SE | 2.75 | 3.1 | 0.0127 | 0.1660 | -11.06 | -8.2 |  |
| QPSK | PAPR gain |  | CM gain |  | required SNR gain |  |  |
|  | CCDF@1e-4 | Max | Mean | Max | BLER@10% |  | BLER@1% |
| No FDSS(baseline) | 0 | 0 | 0.0000 | 0.0000 | 0 |  | 0 |
| FDSS | 2.3 | 2.9 | 0.0319 | -0.0118 | -0.56 |  | -0.78 |
| 12.5% TR | 2.24 | 2.8 | 0.1068 | 0.0630 | -0.2 |  | -0.44 |
| 25% TR | 1.9 | 2.4 | 0.1875 | 0.1523 | 0.29 |  | 0.02 |
| 50% TR | 1.33 | 1.6 | 0.2292 | 0.1994 | 0.81 |  | 0.71 |
| FDSS with 12.5% SE | 2.81 | 3.6 | 0.3028 | 0.3065 | -0.2 |  | -0.48 |
| FDSS with 25% SE | 3.2 | 3.9 | 0.7422 | 0.8907 | 0.3 |  | 0.04 |
| FDSS with 50% SE | 3.93 | 4.9 | 1.2040 | 1.4596 | 0.82 |  | 0.68 |
| QPSK | PAPR gain |  | CM gain |  | required SNR gain |  |  |
|  | CCDF@1e-4 | Max | Mean | Max | BLER@10% |  | BLER@1% |
| No FDSS | -2.3 | -2.9 | -0.0319 | 0.0118 | 0.56 |  | 0.78 |
| FDSS (baseline) | 0 | 0 | 0.0000 | 0.0000 | 0 |  | 0 |
| 12.5% TR | -0.06 | -0.1 | 0.0749 | 0.0748 | 0.36 |  | 0.34 |
| 25% TR | -0.4 | -0.5 | 0.1557 | 0.1641 | 0.85 |  | 0.8 |
| 50% TR | -0.97 | -1.3 | 0.1973 | 0.2112 | 1.37 |  | 1.49 |
| FDSS with 12.5% SE | 0.51 | 0.7 | 0.2710 | 0.3183 | 0.36 |  | 0.3 |
| FDSS with 25% SE | 0.9 | 1 | 0.7103 | 0.9025 | 0.86 |  | 0.82 |
| FDSS with 50% SE | 1.63 | 2 | 1.1722 | 1.4714 | 1.38 |  | 1.46 |

- With above summarized performance comparison, we have the following proposals.

- Proposal 1: For both pi/2-BPSK and QPSK, tone reservation is not supported in Rel-18 CE WI.

- Proposal 2: For pi/2-BPSK, FDSS with spectrum extension can be further studied in Rel-18 CE WI.

- Proposal 3: For QPSK, FDSS with or without spectrum extension can be further studied in Rel-18 CE WI.

Conclusion

Based on above analysis, we have the following observations and proposals.

Observation 1: For FDSS without spectrum extension, the window length of the shaping filter in the frequency domain is equal to the number of REs allocated for PUSCH transmission.

Observation 2: Some RAN4 specification impacts areexpected for QPSK supporting of FDSS.

Observation 3: For FDSS with spectrum extension, the window length of the shaping filter in the frequency domain is equal to (1+α) times of the number of REs allocated for original PUSCH transmission, where α is ratio of the extended REs.

Observation 4: For tone reservation, the window length of the shaping filter in the frequency domain is equal to (1+) times of the number of REs allocated for original PUSCH transmission, where β is ratio of the reserved REs.

Observation 5: For both pi/2-BPSK and QPSK, tone reservation cannot provide clear PAPR/CM reduction gain compared to FDSS with or without spectrum extension.

Observation 6: For pi/2-BPSK, FDSS without spectrum extension can achieve 3dB PAPR gain or 1dB CM gain, and on top of this, FDSS with spectrum extension provides no or minor additional PAPR/CM reduction gain.

Observation 7: For QPSK, FDSS without spectrum extension can achieve 2.3dB PAPR gain while marginal CM gain, and on top of this, FDSS with spectrum extension can provide additional PAPR/CM reduction gain about 0.51 dB, 0.9 dB and 1.63 dB PAPR gain or 0.27 dB, 0.71 dB and 1.17dB CM gain for extension ratio of 12.5%, 25% and 50% respectively.

Observation 8: For pi/2-BPSK, FDSS without spectrum extension would cause about 0.56~0.79 dB link-level performance loss. For QPSK, FDSS without spectrum extension would cause about 0.56~0.78 dB link-level

- Proposal 1: For both pi/2-BPSK and QPSK, tone reservation is not supported in Rel-18 CE WI.

- Proposal 2: For pi/2-BPSK, FDSS with spectrum extension can be further studied in Rel-18 CE WI.

- Proposal 3: For QPSK, FDSS with or without spectrum extension can be further studied in Rel-18 CE WI.

Reference

RP-220937 Revised WID on Further NR coverage enhancements, China Telecom

RP-221858 Revised WID on Further NR coverage enhancements, China Telecom

R1-050702, RAN1#42, DFT-Spread OFDM with Pulse Shaping Filter in Frequency Domain in Evolved EUTRA uplink, NTT Docomo, NEC, Sharp.

RP-212622, New WID: Increasing UE power high limit for CA and DC, RAN#93-e, China Telecom, Qualcomm.

R1-2208489, Discussion on power domain enhancements, ZTE