# 3GPP TSG-RAN WG4 Meeting # 104bis-e                            R4-2215896

# Electronic Meeting, Oct. 10-Oct. 19, 2022

# Title: 	Feasible on CA band combination of n5-n8

# Source: 	ZTE Corporation

# Agenda item:	6.2.2.1

# Document for:	Approval

# 1 Introduction

The CA band combination of the 700MHz, 800MHz and 900MHz bands (i.e. LB bands) were initial discussed in last meeting. For n5-n8, due to the n5 DL frequency range is partial overlapped with n8 UL frequency range, so the frequency restriction to avoid the overlapping are studied and be approved based on the operator’s input in last meeting.

For the UE RF architecture, it was agreed in the WF[1], where:

2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

In this contributions, we continue to discuss the feasible on n5-n8 for smartphone to support the NR CA configurations of LB bands.

# 2	Discussion

Although it was already approved that UE should support the full range of spectrum for single band operation on both n5 and n8, it is unclear that it is for pure single band operation(i.e. no CA in both UL and DL) or for the 1UL/2DL CA where UL CA configuration is single band operation (i.e. ‘-’ for UL CA configuration.).

It is no doubt that there are no frequency restriction applied to pure single band operation. However, like other normal CA band combination, RF architecture is shared by 1UL single band operation and 2UL CA to support the xUL/2DL CA.

Observation 1. RF architecture is shared by 1UL single band operation and 2UL CA to support the xUL/2DL CA.

For the RF implementation, there are two options for the RF filter, one is traditional full band filter and the other one is new n-plexer (i.e.dedicated filter). For the antenna implementation, either 2 antenna or 3 antenna is approved in the WF, where the antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

In our understanding, the relationship between the RF filter implementation with antenna number is:

1. 2 antenna (i.e. one is for common n5&n8 main UL/DL, the other one is for common n5&n8 diversity DL): new n-plexer (i.e.dedicated filter) should be used.

2. 3 antenna (i.e. one is for n5 main UL/DL and one is for n8 main UL/DL, and the left one is for common n5&n8 diversity DL): traditional full band filter is foreseen.

For the first one, simultaneous Tx/Rx can possibly be enabled due to the frequency range restriction, but the performance for the dedicated filter should be studied. There may less room to have good performance considering the gap is small which would cause difficult for the trade off between IL and rejection out of the band.

For the second one, simultaneous Tx/Rx could not be enabled when n8 UL is working if there are no frequency restrictions. However, if using the frequency restrictions, simultaneous Tx/Rx can still possibly be enabled. But in this case, the interference from n8 Tx to n5 DL should be studied. In addition, it seems the single band OOB blocking requirements for band n5 cannot be applied directly.

To avoid the the above issue, and considering the n5 UL is far away from the n8 DL, and the traditional n5 duplexer attenuation in the band n8 DL can up to 40dB, so we propose to discuss whether if it is feasible to restrict the UL configuration is only in n5 for 1UL/2DL NR CA.

Proposal: To discuss whether if it is feasible to restrict the UL configuration is only in n5 for 1UL/2DL NR CA_n5-n8.

# 3 Conclusion

In this contribution, we give some furture discussion on the feasibility for smartphone to support 2DL/2UL  CA_n5-n8.

The conclusions are summarized below:

Observation 1. RF architecture is shared by 1UL single band operation and 2UL CA to support the xUL/2DL CA.

Proposal: To discuss whether if it is feasible to restrict the UL configuration is only in n5 for 1UL/2DL NR CA_n5-n8.

# 4	Reference

[1]  RP-221464, Enhancement for 700/800/900MHz band combinations, CATT

[2]  R4-2107804, WF on architecture and device type for DC_8A-20A_n28A, Huawei, Qualcomm

[3]  R4-2114582, DC_8-20_n28 and other LB-LB-LB RF-FE challenges, Skyworks Solutions, Inc.

[4]  R4-2115012, Email discussion summary for [100-e][112] NR_Baskets_Part_1, Moderator(Skyworks)

[5]  TR 38.717-02-01

[1] R4-2214445, WF on study on FS_NR_700800900, CATT