3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215915

Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	4.3.7.2.3

Source:	Ericsson

Title:	Simulation results for SDR requirements in 52.6 – 71 GHz band

Document for:	Information

# 1	Introduction

In this paper, we present the simulation results to support our view on the SDR requirements in the 52.6 – 71 GHz band. It is worth mentioning that SDR is kept FFS in [1], since we must check whether the Max DL testable SNRs could be satisfied or not when using high MCS.

# 2	Simulation results and discussions

In this section, we present the simulation results following the SDR test steps as per TS 38.101-4 recommendations, where the purpose is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities.

Furthermore, RAN4 was not able to agree on a single representative phase noise (PN) model during the study item. Therefore, and for the sake of fairness, we performed our simulations considering both PN models in TR 38.808 [2], referring to as Set 1 and Set 2, where Set 1 has been proposed in [3] and Set 2 also known as TR 38.803 Example 2.

We will use as general parameters.

- Numerologies: 120 KHz/100 MHz and 480 KHz/400 MHz.

- Two PN models in TR 83.808 [2]:

    - Set 1 in TR 38.808: PN model in [3], referred to as R4_2010176

The design margin (DM) should be set to 0 dB for BS while to be set to 5 dB for UE

    - Set 2 in TR 38.808: PN model TR38.803 Ex2, Set 2 in TR 38.808

- Carrier frequency: 70 GHz

- Antenna configuration: 2x2 ULA Low

- Rank 1

- Phase noise at the UE receiver only with CPE compensation only

- Benchmark: No PN and No compensation

- Pre-loaded Tx EVM: 6%

- Additive Rx EVM: 0%

## 2.1	Simulation setup

According to TS38.101-4, the existing MCS index table has already covered the configuration applicable for UEs such as rank 1/2 and QPSK/16QAM/64QAM. RAN4 only need to evaluate the applicability rule for FR2-2 where rank 1 is supported as shown in Table 1 (based on MCS index Table in the Appendix, in clause 5.1.3.1 of TS 38.214).

| Received antenna | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 2Rx UE | 1 | 6 | 1 | 27 |
|  |  |  | 0.8 | 23 |
|  |  |  | 0.75 | 22 |
|  |  |  | 0.4 | 14 |
|  |  | 4 | 1 | 16 |
|  |  |  | 0.8 | 16 |
|  |  |  | 0.75 | 16 |
|  |  |  | 0.4 | 10 |
|  |  | 2 | 1 | 9 |
|  |  |  | 0.8 | 9 |
|  |  |  | 0.75 | 9 |
|  |  |  | 0.4 | 4 |

Summary of Test Cases

The TB success rate shall be higher than 85% when PDSCH is scheduled with MCS defined for the selected SCS/CBW. To this end, we used the following simulation setup table.

| Received/ antenna | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | PN compensation |
| --- | --- | --- | --- | --- | --- |
| 2Rx UE | 1 | 6 | 1 | 27 | CPE only |
|  |  |  | 0.8 | 23 | CPE only |
|  |  |  | 0.75 | 22 | CPE only |
|  |  |  | 0.4 | 14 | CPE only |
|  |  | 4 | 1 | 16 | CPE only |
|  |  |  | 0.4 | 10 | CPE only |
|  |  | 2 | 1 | 9 | CPE only |
|  |  |  | 0.4 | 4 | CPE only |

## 2.2	Simulation results

We present the SDR performance in terms of normalized throughput as function of SNR (dB), where the reference SNRs correspond to 85% of the normalized throughput. SNR0 refers to the case when no PN applies, while SNR1 and SNR2 refer to the cases when PN Set 1 and Set 2 have been used, respectively.

## 2.2.1 SCS/CBW = 120 KHz/100 MHz results

(a) QPSK (MCS 4 and 9), CPE                   (b) 16QAM (MCS 10 and 16), CPE

(c) 64QAM (MCS 14), CPE                    (d) 64QAM (MCS 22), CPE

Fig. 1 PDSCH simulation results depicting the normalized throughput as function of SNR to determine the practical MCS for SDR requirements when using 120 KHz/100 MHz (66 PRBs)

Fig. 1 depicts the PDSCH normalized throughput as function of the SNR according to parameters in Table 1 and using the PN compensation as per Table 2. The reference SNRs values, required to achieve 85% of PDSCH peak throughput under AWGN conditions, are summarized in the following table.

| Received antenna | Max number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | SNR0 (dB) @ 85% max Thpt | SNR1 (dB) @ 85% max Thpt | SNR2 (dB) @ 85% max Thpt |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 1 | 6 | 1 | 27 | - | - | - |
|  |  |  | 0.8 | 23 | - | - | - |
|  |  |  | 0.75 | 22 | 8.0 | 9.6 | 11.8 |
|  |  |  | 0.4 | 14 | 1.25 | 1.8 | 1.6 |
|  |  | 4 | 1 | 16 | 2.9 | 3.4 | 3.7 |
|  |  |  | 0.4 | 10 | -2.1 | -1.8 | -1.7 |
|  |  | 2 | 1 | 9 | -2.7 | -2.5 | -2.3 |
|  |  |  | 0.4 | 4 | -7.5 | -7.25 | -7.23 |

It is worth reminding that for 120 KHz/100 MHz, the maximum DL testable SNR is at 7.7 dB for PDSCH. Based on the results in the table above, only MCS16 can be tested which made the definition of SDR requirements questionable.

Observation 1: When using 120 KHz/100 MHz, only MCS 16 can be supported when using CPE compensation only. Therefore, higher MCS cannot be tested for SDR.

## 2.2.2 SCS/CBW = 480 KHz/400 MHz results

The simulation results are depicted as follow.

(a) QPSK (MCS 4 and 9), CPE                     (b) 16QAM (MCS 10 and 16), CPE

(c) 64QAM (MCS 14), CPE

Fig. 2 PDSCH simulation results depicting the normalized throughput as function of SNR to determine the practical MCS for SDR requirements when using 480 KHz/400 MHz (66 PRBs)

| Received antenna | Max number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | SNR0 (dB) @ 85% max Thpt | SNR1 (dB) @ 85% max Thpt | SNR2 (dB) @ 85% max Thpt |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 1 | 6 | 1 | 27 | - | - | - |
|  |  |  | 0.8 | 23 | - | - | - |
|  |  |  | 0.75 | 22 | - | - | - |
|  |  |  | 0.4 | 14 | 1.25 | 1.5 | 1.6 |
|  |  | 4 | 1 | 16 | 2.9 | 3.4 | 3.5 |
|  |  |  | 0.4 | 10 | -2.1 | -1.8 | -1.7 |
|  |  | 2 | 1 | 9 | -2.7 | -2.5 | -2.4 |
|  |  |  | 0.4 | 4 | -7.5 | -7.15 | -7.15 |

It is worth reminding that for 480 KHz/400 MHz, the maximum DL testable SNR is at [-0.6] dB for PDSCH. Based on the results in the table above, only MCS10 can be tested which made the definition of SDR requirements questionable.

Observation 2: When using 480 KHz/400 MHz, only MCS 160can be supported when using CPE compensation only. Therefore, higher MCS cannot be tested for SDR.

# 6	Summary

In this paper, we provide some initial simulation results to assess the SDR performance.

Observation 1: When using 120 KHz/100 MHz, only MCS 16 can be supported when using CPE compensation only. Therefore, higher MCS cannot be tested for SDR.

Observation 2: When using 480 KHz/400 MHz, only MCS 160can be supported when using CPE compensation only. Therefore, higher MCS cannot be tested for SDR.

# References

- R4-2214390, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

- 3GPP TR 38.808 v17.0.0, “Study on supporting NR from 52.6 GHz to 71 GHz”, (Release 17).

- R4-2010176, “On phase noise for 52.6-71 GHz”, Ericsson.

# Appendix

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |