3GPP TSG-RAN WG4 Meeting #104-bis	R4- 2215960

# Electronic Meeting, October 10 – 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.114 | CR | draftCR | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.114 Clause 4.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_repeaters-Core/Perf |  |  |  |  |  | Date: |  |  | 2022-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test configurations for NR repeater is not defined in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The missing content of clause 4.5 is introduced, based on the agreement in R4-2210506. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------- start of changes----------

## 4.5	NR repeaters test configurations

The present clause defines the NR repeaters test configurations that shall be used for demonstrating conformance. A single NR repeater carrier shall be used for testing of single-carrier capable NR repeaters.

Single carrier configuration (SC) tests shall be performed using signal with narrowest supported NR repeater channel bandwidth with the smallest supported subcarrier spacing declared per operating band in TS 38.115-1 [3] clause 4.6, and TS 38.115-2 [4] clause 4.6.

For other NR repeaters, the test configurations in table 4.5-1 and table 4.5-2 shall be used. The NR repeaters test configurations (RTCx) are defined in TS 38.115-1 [3], clause 4.7 for NR repeaters type 1-C and in TS 38.115-2 [4], clause 4.7 for NR repeaters type 2-O.

Table 4.5-1: Test configurations for NR repeaters type 1-C

| Repeater test case | Repeater capable of single or multiple passbands in a single band |  |  | Repeater capable of multi-band operation |  |
| --- | --- | --- | --- | --- | --- |
|  | Single passband repeater | Multiple passband capable repeater with identical parameters per passband | Multiple passband capable repeater with different parameters per passband | Common connector | Separate connectors |
| Emission tests | RTC1 | RTC1, RTC2 | RTC1, RTC2 | RTC1/2 (Note 1), RTC4 | RTC1/2 (Note 1, 2), RTC4 (Note 2) |
| Immunity tests | RTC1 | RTC1, RTC2 | RTC1, RTC2 | RTC1/2 (Note 1), RTC4 | RTC1/2 (Note 1, 2), RTC4 (Note 2) |
| NOTE 1: RTC1 and/or RTC2 shall be applied in each supported operating band. NOTE 2: For single-band operation test, other antenna connector(s) is (are) terminated. |  |  |  |  |  |

Table 4.5-2: Test configurations for NR repeaters type 2-O

| Repeater test case | Repeater capable of single or multiple passbands in a single band |  |  |
| --- | --- | --- | --- |
|  | Single passband repeater | Multiple passband capable repeater with identical parameters per passband | Multiple passband capable repeater with different parameters per passband |
| Emission tests | RTC1 | RTC1, RTC2 | RTC1, RTC2 |
| Immunity tests | RTC1 | RTC1, RTC2 | RTC1, RTC2 |

# 5	Performance assessment

---------- end of changes----------

Note: The content in clause 4.7 in TS 38.115-2 is not updated yet, please refer to R4-2214786 “TP to TS 38.115-2: Test Configurations and Requirement applicability”, which was endorsed during RAN4#104.