3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216008

# Electronic Meeting, 10th – 19th Oct., 2022

Title: 	Simulation results on UE FeMIMO CSI mTRP

Source: 	Huawei, HiSilicon

Agenda item:	4.5.3.2

Document for:	Information

Background

During RAN4#104-e meeting, WF [1] on UE demodulation and CSI requirements for FeMIMO was approved. In this contribution, we provide our updated simulation results about the UE FeMIMO mTRP CSI requirements.

Simulation

Here we provide the updated simulation results for the FeMIMO mTRP CSI requirements for alignment, as shown in Table 2-2 and Figure 2-1, based on simulation assumption as shown in Table 2-1.

Table 2-1 Simulation assumption for FeMIMO mTRP CSI reporting enhancement

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| CBW | 10 MHz | 40 MHz |
| Scheme | Single-DCI based SDM |  |
| Antenna configuration | 2x2 XPL High, 2x4 XPL High |  |
| Propagation condition | TDLA30-10 |  |
| Number of ports per TRP | 8 |  |
| DMRS type | Type 1 |  |
| Number of DMRS symbols | 1+1 |  |
| Number of layers | 2 (1MIMO layer per TRP) |  |
| Maximum number of HARQ transmission | 4 |  |
| TDD pattern | N/A | 7D1S2U, S: 6D 4G 4U |
| NZP CSI-RS resource | Row 8 |  |
| ZP CSI-RS resource | Row 5 |  |
| (N1, N2, O1, O2) | (4, 1, 4, 1) |  |
| CSI configuration | 5slot periodicity, 1 slot offset for all CSI resources | 10slot periodicity, 1 slot offset for all CSI resources |
| CSI delay | 8ms | 6.5ms |
| TRS configuration | 20ms periodicity, 10ms offset, 2 slot patterns, symbol 4 and 8 |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| CSI reporting | Wideband, Mode1, X=0 |  |
| Time/frequency offset between two TRxP | N/A |  |
| Codebook type | typeI-SinglePanel |  |
| Test metric | TP ratio between following PMI and Random PMI |  |
| Test point | 90% |  |

Table 2-2 Simulation results for FeMIMO mTRP CSI reporting enhancement

| Case Number | Duplex | Antenna configuration | γ |
| --- | --- | --- | --- |
| 1 | FDD | 2x2 | 2.06 |
| 2 | FDD | 2x4 | 2.39 |
| 3 | TDD | 2x2 | 2.06 |
| 4 | TDD | 2x4 | 2.38 |

Figure 2-1 Simulation results for FeMIMO mTRP CSI reporting enhancement

Conclusion

In this contribution, we provide our updated simulation results on UE CSI requirements for FeMIMO mTRP.

Reference

R4-2214397, WF on UE demodulation and CSI requirements for FeMIMO, RAN4#104-e, Samsung