3GPP TSG-RAN WG4 Meeting # 104bis-e									  R4-2216042

Electronic Meeting, October 10 – October 19, 2022

Title: 	Discussion on RF requirements for multi-carrier enhancement

Source: 	Xiaomi

Agenda item:	6.19.2

Document for:	Approval

- Introduction

In RAN4#104-e meeting, some agreements are achieved on switching period and a reply LS [1] was sent to RAN1. However, some open issues, e.g. switching period, UE complexity, still need further discussion. This contribution will provide our view on the remaining issues.

- Discussion

The remaining issues of UL Tx switching for single TAG and multiple TAG are captured in the WF [1] and [2] as below:

Single TAG: #### Issue 2-1-3: Exact value of Tx switching period - Further discuss the two options in the next meeting: - Option 1: Reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18 Tx switching. - Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 Tx switching with 3/4 bands configured. #### Issue 2-2-2: Impact on the band with the number of Tx chain unchanged due to switching - As baseline UE assumption, neither of Tx chains is expected to be used for transmission on band C during the switching period. - RAN4 will further discuss optional advanced features to allow the other Tx chain can be expected to be used for transmission on band C during the switching period as advanced/optional UE assumption. #### Issue 2-5-5: UE complexity aspect - The complexity related aspects would be discussed in RAN4 in future meetings. Multiple TAG: - Issue 3-1-1 UL switching time Agreement: - RAN4 further discuss if the UL switching time is the same for single TAG and 2 TAGs - UL switching time should not include timing difference up to MTTD between two TAGs. FFS: - UE may omit the uplink transmissions corresponding to any TAG during the UE switching time.

In the last meeting, RAN4 has agreed to reuse the same set of values as in Rel-16/17 for UL CA and SUL. Also for each band pair, the switching period can be the same or different for 1Tx-2Tx switching and 2Tx-2Tx switching based on UE reporting. There is no difference compared to Rel-16/17. Actually the switching case in Rel-18 is either 1Tx-2Tx or 2Tx-2Tx because the maximum number of uplink transmission is the same as Rel-16/17, i.e., 2. Compared to Rel-16/17, the switching case in Rel-18 does not change.

Taking 3 band case as an example, the potential switching cases for option 1 (SwitchedUL) and option 2 (DualUL) are given in table 1 and table 2. Even though up to three bands are involved, the switching case still remains as 1Tx-2Tx or 2Tx-2Tx. Based on the latest agreement in RAN1, Alt 1 is considered as baseline for Rel-18 UL Tx switching mechanism. RAN1 will also consider complexity reduction for switching cases in terms of at least additional preparation time, additional interruption time, and RF complexity for certain switching cases/patterns. Addition preparation time can be discussed in RAN1 while interruption time can be discussed in RRM session. From the RF perspective, the switching period is not impacted by increasing 2 band to 3 or 4 band switching. So it is proposed that the switching period specified for each band pair specified in Rel-16/17 can be reused for Rel-18 single TAG case.

Table 1: UL Tx switching for Option 1 (SwitchedUL) (3 band)

|  | Number of Tx Carrier 1+Carrier 2+Carrier 3 |
| --- | --- |
| Case 1 | 2T+0T+0T |
| Case 2 | 0T+2T+0T |
| Case 3 | 0T+0T+2T |

Table 2: UL Tx switching for Option 2 (DualUL) (3 band)

|  | Number of Tx Carrier 1+Carrier 2+Carrier 3 |
| --- | --- |
| Case 1 | 2T+0T+0T |
| Case 2 | 0T+2T+0T |
| Case 3 | 0T+0T+2T |
| Case 4 | 0T+1T+1T |
| Case 5 | 1T+0T+1T |
| Case 6 | 1T+1T+0T |

Proposal 1: The switching period specified for each band pair specified in Rel-16/17 can be reused for Rel-18 single TAG case.

RAN4 has discussed the UE assumption/behavior considering two cases: - Case 1: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on either band A or band B. For Case 1, RAN4 agreed that neither of Tx chains is expected to be used for transmission during the switching period. - Case 2: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on a different band (named “band C”). For Case 2, RAN4 agreed that, as baseline UE assumption, neither of Tx chains is expected to be used for transmission on band C during the switching period.

Based on the reply LS, it is observed that the following two cases for 3 band and 4 band is not considered but still within the scope.

- Case 3: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is triggered to switch from one band (named “band A”) to another band (name “band C”)

- Case 4: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is triggered to switch from one band (named “band C”) to another band (name “band D”)

Table 3: Case 3

| Band | A | B | C |
| --- | --- | --- | --- |
| Switch from | 2T | 0T | 0T |
| Switch to | 0T | 1T | 1T |

Table 4: Case 4

| Band | A | B | C | D |
| --- | --- | --- | --- | --- |
| Switch from | 1T | 1T | 0T | 0T |
| Switch to | 0T | 0T | 1T | 1T |

For Case 3 and Case 4, both Tx chains need to perform switching so there are two switching periods for each band pair on each Tx chain. These switching cases are not captured in the reply LS [3] to RAN1. RAN4 is expected to study the Tx chain impact for Case 3 and Case 4 and to inform RAN1 if any consensus reached.

The issue is UE behavior during the switching process. It would be feasible two different band pairs is switched on both two Tx chains at the same time based on UE RF architecture. In case that two Tx chains have the same switching period, UL transmission will be allowed on each Tx chain after the same switching completion time. But in case that two Tx chains have different switching periods, UE behavior should be discussed in terms of whether the Tx chain with smaller switching period can allow UL transmission once the switching is completed. For example, one of the two Tx chains is to switch from band A to band B with 35us switching period, and the other Tx chain is to switch from band C to band D with 140us switching period. Whether band B on the Tx chain with smaller switching period can perform UL transmission after the switching completion during the followed 105us during which other Tx chain is in process of switching.

Proposal 1: RAN4 to study the Tx chain impact for Case 3 and Case 4 and to inform RAN1 if any consensus reached.

Proposal 2: In case that two Tx chains have different switching periods, UE behavior should be discussed in terms of whether the Tx chain with smaller switching period can allow UL transmission once the switching is completed.

For UE complexity, it is expected UE complexity is not increased compared to Rel-16/17 because up to 2 Tx simultaneous transmissions are still restricted for 2 RF chain architecture.

For multiple TAG case, RAN4 agreed the target scenario in Rel-18 is dual TAG. Whether the switching time is the same for single TAG and 2 TAG is FFS. However, RAN4 reached consensus that UL switching time should not include timing difference. That is to say, the number of TAG has no impact on the switching time and the switching time should be the same for single TAG and 2 TAG. The switching time mask should not capture the timing difference cause by two TAGs. The similar issue was also discussed in Rel-17 NR SL. The result is that timing difference is not captured in the switching time mask. The same principle can be followed for UL Tx switching.

Proposal 3: The switching time for UL Tx switching should be the same for single TAG and 2 TAG.

- Conclusion

This contribution gives our views on UL Tx switching across 3 or 4 bands. The following proposal are derived:

Proposal 1: RAN4 to study the Tx chain impact for Case 3 and Case 4 and to inform RAN1 if any consensus reached.

Proposal 2: In case that two Tx chains have different switching periods, UE behavior should be discussed in terms of whether the Tx chain with smaller switching period can allow UL transmission once the switching is completed.

Proposal 3: The switching time for UL Tx switching should be the same for single TAG and 2 TAG.

- Reference

- R4-2214463, WF on UL Tx switching with single TAG, China Telecom, RAN4#104-e

- R4-2215163, WF on UL Tx switching with multiple TAGs, Ericsson, RAN4#104-e

- R4-2214464, Reply LS on UL Tx switching across 3 or 4 bands, China Telecom, RAN4#104-e