3GPP TSG-RAN WG4 Meeting #104-bis-e	 R4-2216053

Electronic Meeting, 10 - 19 October, 2022

Source: 	Huawei, HiSilicon

Title:  	TP for TR 38.876 ATG

Agenda Item:	6.13.4

Document for:	Approval

- Introduction

RAN4 has approved TR 38.876 ATG v0.0.1 skeleton [1]. This paper is a text proposal to capture the agreements for NR ATG in RAN4#104-bis meeting [2].

- References

[1] R4-2214912, “TR 38.876 ATG v0.0.1 skeleton”, CMCC

[2]R4-2214247, “Email discussion summary for [104-e][136] NR_ATG”, CMCC

[3] R4-2214461 WF on ATG BS RF requirements_GTW_v5.docx

# Text proposal for TR 38.876

----- Start of TP -----

<Unchanged sections omitted>

7.2 ATG BS specific

7.2.1  ATG BS class and BS type

ATG Base Stations are characterized by requirements derived from ATG scenarios with a ground BS to air UE with typical vertical altitude range [TBD km].Further discuss the typical vertical altitude range.

The BS type of ATG BS is 1-C, 1-H and 1-O.

7.2.2 Tx requirements

| Tx requirements | Proposal |
| --- | --- |
| gNB output power | Define a new BS-class which adopts the properties from the Wide area BS, similar to what have been done for HAPS and its maximum output power should be left up to the declaration. |
| Output power dynamics |  |
| RE power control dynamic range | To reuse the existing requirement defined in TS 38.104 |
| Total power dynamic range | To reuse the total power dynamic range of TS 38.104 |
| Transmit ON/OFF power | To reuse the existing requirement defined in TS 38.104 |
| Transmitted signal quality |  |
| Frequency error | To reuse the same requirement defined in TS 38.104 |
| Modulation quality | To reuse the same requirement defined in TS 38.104 FFS:256QAM |
| Time alignment error | This is not applicable for ATG BS since MIMO or CA is not applicable for ATG BS. |
| Unwanted emissions |  |
| Occupied bandwidth | To reuse the same requirement defined in TS38.104 which is following ITU-R Recommendation SM.328 |
| Adjacent Channel Leakage Power Ratio | Depend on the coexistence study outcome. |
| Operating band unwanted emissions | Depend on the coexistence study outcome. |
| Transmitter spurious emissions | To reuse the same spurious emission requirement defined in TS 38.104 |
| Transmitter intermodulation | Not applicable since it’s supposed to have no surrounding interfering gNB next to gNB |

7.2.3 Rx requirements

| Rx requirement |  |
| --- | --- |
| Reference sensitivity level | To reuse the same reference requirements for Wide Area BS defined in TS 38.104 |
| Dynamic range | Since IoT of ATG BS is supposed to quite low compared with IoT level of the legacy TN BS, it’s sufficient to reuse the existing requirement for Wide Area BS defined in TS 38.104. |
| ACS | Depend on the coexistence study outcome . |
| In-band blocking | This depends on the outcome of coexistence study. |
| Out of band blocking | To reuse OOBB power level as -15dBm for ATG BS which should be sufficient enough. |
| Receiver spurious emission | To reuse the requirements defined in TS 38.104 |
| Receiver intermodulation | Similar as transmitter intermodulation requirements, not applicable since it’s supposed to have no surrounding interfering gNB next to gNB |
| In-channel selectivity | Similar as dynamic range requirements, the IoT of ATG BS is supposed to quite low compared with IoT level of the legacy TN BS, it’s sufficient to reuse the existing requirement for Wide Area BS defined in TS 38.104. |

<Unchanged sections omitted>

----- End of TP -----