3GPP TSG-RAN WG4 Meeting #104-bis-e                           	R4-2216068

Electronic Meeting, 10 October– 19 October, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on UE requirements for ATG scenario

Agenda Item:	6.13.3

Document for:	Approval

Introduction

In RAN4#104 meeting, a way forward [1] was approved with some options for each issue to further discuss. In this paper, we’d like to provide our views on CPE mounted in the aircraft, especially for power class / UE type considerations.

General discussion

Referring to the latest work item [2], the main objectives for CPE mounted in the aircraft is listed as below.

    - Identify key characteristics where it is necessary to differentiate ATG ground-based BS and UEs from conventional ground based BS and UEs

      - Aim to reuse existing requirements for BS and UE where possible, e.g.,

        - Reuse TN BS requirements for ATG BS

    - Specify RF requirements for ATG UE/BS

      - Considering the results of co-existence simulations in terms of impact on emissions and RX requirements, cell sizes and link budgets, technology capabilities, likely BS and UE architectures and other relevant aspects.

      - Taking into account identified differences between ATG and fully ground based systems

      - Consider BS type 1-C/1-H/1-O and specify the requirements

      - Consider conductive requirements for UE

Based on these objectives, the scope for this WI is very clear. Firstly, working group should aim to reuse existing requirements for UE where possible. Secondly, to consider conductive requirements for UE. Based on these two objectives, that means OTA requirements for UE are excluded and the existing requirements and frameworks for UE should be considered as first priority.

Proposal 1: Since only conductive requirements for UE are considered in this work item, the OTA requirements for UE and considerations related to OTA can be excluded in this release.

Proposal 2: Since this work item aims to reuse the existing requirements for UE where possible, the existing requirements and frameworks for UE should be considered as first priority.

Based on the way forward [3], working group has agreed that only aircraft is considered in this WI. ATG UE seems more general and may cause some ambiguity about the UE type. It’s better to standardize a general term for it.

Proposal 3: To use the term “CMA: CPE mounted in the aircraft” for the target UE type for this R18 air-to-ground work item.

Maximum output power for CPE mounted in the aircraft

When we derive the UE maximum output power requirements, we should consider the following factors, e.g. deployment scenario, frequency band and coverage.

Bands n1, n78 and n79 are considered as exemplary bands, but the UE maximum output power requirements for FDD band 1 are different from TDD bands n78 and n79. It’s proposed to assume the UE maximum output power requirements for FDD band n1 and TDD bands n78 and n79 separately.

In addition, the UL coverage should be evaluated based on the specific deployment scenario. It’s very important to define the network layout for ATG scenario firstly before working group have a good way forward on how to specify UE maximum output power. For example, the typical cell range should be assumed firstly.

Observation 1: It’s very important to define the network layout for ATG scenario firstly before working group have a good way forward on how to specify UE maximum output power.

Since the aircraft isn’t fixed in the network and it isn’t like a network node, not sure whether the UE maximum output power can be declared by UE vendors independently. From network performance perspective, the minimum requirement for UE MOP should be clearly specified instead of depending on the declaration from UE vendors.

Proposal 4: From network performance perspective, the minimum requirement for UE MOP should be clearly specified instead of depending on the declaration from UE vendors.

Proposal 5: In order to reuse the current industry eco-system, 23dBm per element for FDD band n1 and 23dBm (26dBm) per element for TDD band n78, n79 can be assumed. The UE maximum output power can be further discussed after antenna array configurations are concluded.

Antenna model for CPE mounted in the aircraft

Since UE orientation aims to the ground, omni antenna assumption for CPE mounted in the aircraft can be excluded. It’s unnecessary to consider the direction which is opposite to the ground.

Proposal 6: Omni antenna assumption for CPE mounted in the aircraft can be excluded.

It’s proposed to adopt the following array antenna model in Table 1. It’s highlighted that the antenna array configuration can be further discussed based on the expected maximum output power and antenna gain of CPE mounted in the aircraft. Based on the discussion in last meeting, there are no conclusion on whether CPE mounted in the aircraft has beam steering capability. RAN4 never assume FR1 UE has the capability of beam steering. Thus, if we assume that FR1 UE has the capability of beam steering, it’s very challenging for RAN4 to specify the UE requirements based on beam steering assumption. Given the objective of this WI is to reuse the existing requirements as soon as possible, the TU budget is limited. Assuming beam steering capability for CPE mounted in the aircraft will enlarge the scope of ATG WI and may cause some risks of progress.

Proposal 7: Assuming beam steering capability for CPE mounted in the aircraft will enlarge the scope of ATG WI and may cause some risks of progress as all RF requirements related to the beamforming should be developed under the limited TU. Based on the concerns above, it’s proposed to focus on the CPE mounted in the aircraft without beam steering capability in Rel-18 ATG WI.

Table 1 Array antenna pattern and parameters for CPE mounted in the aircraft

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ,φ=-min12θ-90θ3dB2,SLAv, θ3dB=90°, SLAv=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hθ,φ=-min12φφ3dB2,Am, φ3dB=90°, Am=25dB |
| Combining method for 3D antenna element pattern (dB) | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Maximum directional gain of an antenna element GE,max | 5 dBi GE,max=DE,max-LE |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| (Mg, Ng, M, N, P) | FFS |
| (dv, dh) | (0.5λ, 0.5λ) |
| LE (dB) | 2.0 dB |
| UE orientation | UE orientation is perpendicular to the ground θetilt=90° and φescan=0° |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max. |  |

Proposal 8: It’s recommended to adopt Table 1 UE array antenna pattern and parameters for ATG scenario.

- 3 Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: Since only conductive requirements for UE are considered in this work item, the OTA requirements for UE and considerations related to OTA can be excluded in this release.

Proposal 2: Since this work item aims to reuse the existing requirements for UE where possible, the existing requirements and frameworks for UE should be considered as first priority.

Proposal 3: To use the term “CMA: CPE mounted in the aircraft” for the target UE type for this R18 air-to-ground work item.

Observation 1: It’s very important to define the network layout for ATG scenario firstly before working group have a good way forward on how to specify UE maximum output power.

Proposal 4: From network performance perspective, the minimum requirement for UE MOP should be clearly specified instead of depending on the declaration from UE vendors.

Proposal 5: In order to reuse the current industry eco-system, 23dBm per element for FDD band n1 and 23dBm (26dBm) per element for TDD band n78, n79 can be assumed. The UE maximum output power can be further discussed after antenna array configurations are concluded.

Proposal 6: Omni antenna assumption for CPE mounted in the aircraft can be excluded.

Proposal 7: Assuming beam steering capability for CPE mounted in the aircraft will enlarge the scope of ATG WI and may cause some risks of progress as all RF requirements related to the beamforming should be developed under the limited TU. Based on the concerns above, it’s proposed to focus on the CPE mounted in the aircraft without beam steering capability in Rel-18 ATG WI.

Table 1 Array antenna pattern and parameters for CPE mounted in the aircraft

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ,φ=-min12θ-90θ3dB2,SLAv, θ3dB=90°, SLAv=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hθ,φ=-min12φφ3dB2,Am, φ3dB=90°, Am=25dB |
| Combining method for 3D antenna element pattern (dB) | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Maximum directional gain of an antenna element GE,max | 5 dBi GE,max=DE,max-LE |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| (Mg, Ng, M, N, P) | FFS |
| (dv, dh) | (0.5λ, 0.5λ) |
| LE (dB) | 2.0 dB |
| UE orientation | UE orientation is perpendicular to the ground θetilt=90° and φescan=0° |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max. |  |

Proposal 8: It’s recommended to adopt Table 1 UE array antenna pattern and parameters for ATG scenario.

- References

[1] R4-2215147, WF on UE requirements for ATG, Apple

[2] RP-220962, New WID: Air-to-ground network for NR, CMCC

[3] R4-2214459, WF on co-existence evaluation for ATG, Huawei, HiSilicon